3GPP TSG RAN Meeting #109								     RP-252653

Beijing, China, September 15-18, 2025

Agenda Item:	9.2.3

Source:	Xiaomi

Title:	Discussion on Rel-20 Ambient IoT SI scope

Document for:	Discussion and Decision

- Introduction

In the SID [1] of Rel-20 Ambient IoT, the objective 2 mentioned that RAN#109 to decide whether to include an objective on positioning / proximity determination in the scope for Rel-20 study item.

[…] 2.RAN#109 to decide whether to include an objective on positioning / proximity determination in the scope for Rel-20 study item […]

This contribution discusses about the views on whether to include objective on positioning / proximity determination in the scope for Rel-20 study item.

- Discussion

Background

Service requirements in SA1

In SA1’s TS 22.369 [2], the service requirements for ambient IoT are provided. Ambient IoT use cases can be characterized in four different categories in which Asset tracking is to determine the location of goods. Positioning, as one of the functional services, is also discussed that the 5G system shall support location services for Ambient IoT devices (e.g., to locate Ambient IoT devices using absolute or relative positioning methods). The corresponding performance services requirements can be referred in following table:

Table 6.4-1 KPIs for tracking [2]

| Deployment | Scenarios | Max. allowed end-to-end latency | Communication service availability | Reliability | User-experienced data rate | Message size | Device density | Communication range (Note 1) | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | Indoor tracking | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | 25 devices /100 m² - 250 devices /100 m² | 10 m | 200 m² | up to 3km/h | 60 mins | 1 s | 90% | 1 m - 3 m, 90% availability |
| Outdoor | Outdoor tracking | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | ≤10 devices/ 100 m² | 500 m | Up to the whole PLMN | up to 10 km/h | 60 mins | 1 s | 95% | several 10 m |
| NOTE: The communication range is the communication distance between the ambient IoT device and the 5G network or between the ambient IoT device and an ambient IoT capable UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Study output by RAN-P in Rel-18

In TR 38.848 [3], Ambient IoT was studied by RAN Plenary level about use cases, design targets and preliminary feasibility assessment. The 8 representative use cases (rUC) include rUC3 Indoor positioning and rUC7 outdoor positioning. According to the mapping between RAN representative use cases and SA1 use cases, positioning has some applicable traffic scenarios, such as finding remote lost item, location service, personal belongings finding, positioning in shopping center, etc. According to the design targets, the absolute positioning accuracy when performed by the cellular network (including assisting nodes when present) is 1~3 meters @ 90% indoor location, several tens of meters @ 90% outdoor location.

Study output by RAN WGs in Rel-19

In Rel-19 Ambient IoT, proximity determination was studied and concluded, which is summarized as to solutions in TR 38.769 [4]. Two solutions were studied and concluded as feasible. The difference between the two solutions is that whether the determination is based on measurement or not at the reader side.

### 6.9.4 Proximity determination Proximity determination is feasible with either of the two following solutions. Potential specification impact or not will not be determined in this study item. Solution 1: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader. Solution 2: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side. Proximity determination based on device-side measurements is not considered.

Positioning / proximity determination in Rel-20 A-IoT SI

Positioning or Proximity determination

Proximity determination is to determine whether the device is near to the reader. The detection result is near or not to the reader, but how to quantitively define the “near” is not clear. According to the study on coverage, the maximum distance target between reader and device in Rel-19 is 10m~50m, while the maximum distance between reader and device is 50m~500m for Rel-20. With such a large maximum distance between reader and device, the terminology “near” is ambiguous.

In NR, positioning is to determine location by using measurement/calculation based on reference signal(s). Considering about the ultra-low complexity of device, even for active device C, the feasibility should be firstly studied. To achieve accurate location information, the relative aspects should also be studied and designed for adaptation in Ambient IoT system, such as reference signal, measurement/calculation procedures. It may require dedicated design and enhancement that has higher complexity compared with Proximity, but it is more accurate.

Based on the analysis above, Proximity is simpler but with quite low accuracy to locate device, while positioning requires extra design with higher complexity which can achieve more accurate location of device.

Observation 1: The positioning and proximity determination have following characteristics:

- Proximity determination is simpler, and the accuracy is quite low.

- Positioning requires higher complexity, and the accuracy is higher.

Measurement and reference signal

Based on the conclusion in Rel-19 [4], Proximity Solution 2 is defined as that the device is determined as near to the reader based on measurement at the reader side. Even it is based on measurement, the result is still determined as “near”. The difference between Solution 1 and Solution 2 is that Solution 1 is only based on D2R reception and decoding by reader, while Solution 2 additionally requires reader to perform measurement (e.g., D2R signal strength) and then to determine the device is near or not. In Rel-20 Ambient IoT, active device is supported with much higher capability and transmission power. If proximity determination is considered, feasibility and necessary enhancement should be studied in order to achieve reasonable accuracy.

Positioning is to determine the relative or absolute location of the device by measurement based on reference signal(s). If positioning is considered in Rel-20 Ambient IoT, the following aspects should be studied:

- Reference signal(s): In Rel-19 Ambient IoT, X-amble signal(s) (i.e., preamble/midamble/postamble) is supported to assist R2D/D2R detection and reception. In Rel-20 Ambient IoT, the X-amble signal(s) can be used to do positioning measurement, whether the existing signal(s) is feasible / need enhancement can be studied. To obtain accurate location of device, new reference signal can be considered in Ambient IoT system, such as DMRS, PTRS, SRS. To support positioning based on new reference signal(s), many aspects should be studied, such as signal design, R2D/D2R pattern, reference signal resource allocation, etc.

- Positioning method(s): In NR, it supports several positioning methods, such as RTT, AoA, TDOA, AoD, which can be studied about the feasibility in Rel-20 Ambient IoT. These existing methods requires large frequency resources to acquire accurate positioning result. However, Ambient IoT signal only consumes very limited frequency resource, e.g., R2D with 180 kHz Tx bandwidth and D2R with 15 kHz Tx bandwidth. Such narrow bandwidth cannot provide acceptable positioning result, and further enhancement or new positioning method can be studied.

- Measurement/calculation perform side: In Rel-19 Ambient IoT, passive device is not capable to perform measurement due to backscattering transmission and ultra-low complexity and power consumption. It is also not possible to transmit positioning reference signal by using backscattering. Therefore, the measurement can be done only by reader side. In Rel-20, active device is supported with higher complexity, especially device C with much higher power consumption. Reader can perform the measurement based on D2R reference signal, final location can be calculated and determined by reader or LMF. Besides, whether active device (e.g., device C) is capable to do the measurement and determination needs to be studied.

Observation 2: In Rel-20 Ambient IoT, if proximity determination is considered, feasibility and necessary enhancement should be studied in order to achieve reasonable accuracy.

Observation 3: In Rel-20 Ambient IoT, if positioning is considered, the relative technical aspects should be studied, e.g., reference signal(s), positioning method, measurement/calculation perform by reader or device.

TU consideration

Rel-20 Ambient IoT is planned to have 18 months to complete SI and WI. The WG discussion time allocation in RAN1 is 3.5 TU. Based on the SID [1] and WID [5], the scope is already large that can fulfill current 3.5 TU. Based on the discussion in RAN1#122 meeting, many detailed technical aspects are agreed to be studied for Rel-20 Ambient IoT, such as new signal(s) for R2D and D2R for active device, DO-A traffic procedures. These potential works are expectable to fully occupy the allocated TU. If positioning/proximity determination is additionally considered in Rel-20 Ambient IoT, the current 3.5 TU is not enough. Extra required TU depends on whether positioning or proximity determination method is included. Additionally, if positioning is considered to be studied, the exact method(s) also have impact on the TU requirement.

Observation 4: The current 3.5 TU cannot accommodate additional work on positioning / proximity determination.

Whether/how to include it or not

According to the above analysis, positioning/proximity determination is an independent mechanism from the existing scope of Rel-20 Ambient IoT SI and WI. If it is not considered, there is no impact on the normative work of Rel-20 Ambient IoT. If it is considered in Rel-20, additional TU is required to accommodate the additional work.

In case additional TU can be allocated to Rel-20 Ambient IoT for positioning/proximity determination, it is still important to have a clear and limited scope to fit in the TU, including e.g., whether proximity determination or positioning is included etc.

A simple method with limited solutions for positioning/proximity determination is slightly preferred in Rel-20 Ambient IoT.

Proposal: If additional TU is available for Rel-20 Ambient IoT positioning/proximity determination, the scope should be clear and limited to fit the additional TU.

- Conclusion

This contribution discusses about the views on whether to include objective on positioning / proximity determination in the scope for Rel-20 study item. Based on the discussion and analysis, the following observations and proposal are concluded:

Observation

Observation 1: The positioning and proximity determination have following characteristics:

- Proximity determination is simpler, and the accuracy is quite low.

- Positioning requires higher complexity, and the accuracy is higher.

Observation 2: In Rel-20 Ambient IoT, if proximity determination is considered, feasibility and necessary enhancement should be studied in order to achieve reasonable accuracy.

Observation 3: In Rel-20 Ambient IoT, if positioning is considered, the relative technical aspects should be studied, e.g., reference signal(s), positioning method, measurement/calculation perform by reader or device.

Observation 4: The current 3.5 TU cannot accommodate additional work on positioning / proximity determination.

Proposal

Proposal: If additional TU is available for Rel-20 Ambient IoT positioning/proximity determination, the scope should be clear and limited to fit the additional TU.

- References

- RP-251884, “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, Moderator (RAN1 Vice-Chair), Prague, Czech Republic, June 9-13, 2025.

- 3GPP TS 22.369, v19.3.0, Service requirements for ambient power-enabled IoT, Stage 1, Release 19, September, 2024.

- 3GPP TR 38.848 v18.0.0, Study on Ambient IoT (Internet of Things) in RAN, Release 18, September, 2023.

- 3GPP TR 38.769, v19.0.0, Study on solutions for Ambient IoT (Internet of Things) in NR, Release 19, December, 2024.

- RP-251885, “New WID on Solutions for solutions for Ambient IoT (Internet of Things) in NR Phase 2”, Moderator (RAN1 Vice-Chair), Prague, Czech Republic, June 9-13, 2025.