3GPP TSG RAN Meeting #109	         RP-252697

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.2

Source:	Lenovo

Title:	Discussion on 6GR Numerology

Document for:	Discussion

# 1	Introduction

In RAN#108, a new SI “Study on 6G Radio” was approved [1]. The study aims to develop a new 6G Radio (6GR) access technology to meet a broad range of use cases, requirements and deployment scenarios as defined in [2]. An overview of the new 6GR interface is provided in [3]. The contribution discusses the recent RAN1 discussion on numerology for 6GR interface from 6G-IoT perspective.

# 2	Discussion

RAN#108, a new SI study on 6G Radio mentions to avoid multiple numerologies for the same band/sub-range as shown below. Furthermore, RAN1#122 discussed and agreed on numerology for various frequency ranges as follows:

d) Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

One of the controversial points is the proposal identifying multiple numerologies especially in the sub 6GHz frequency range, such as 15kHz, 30kHz to support FDD and TDD respectively while additional numerology such as 30kHz to be supported in and around 1-2.5GHz and 7.5kHz for sub1GHz FDD.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

## 6G Numerology

Numerology is a fundamental to the design of 6GR Radio, and are key to meet the many requirements and scenarios, including

- Support different services including MBB, Immersive communication and IoT in the same network.

- Support carrier frequencies from below 1GHz to 52.6GHz (FR2-1).

- Support carrier bandwidths from 3[or 5] MHz for IoT @FR1 to 200MHz @FR3 and 400MHz @ FR2-.

- Support different duplex modes including FDD, TDD and SBFD.

- Support coexisting with and migration from 5G and possibly 4G (e.g., IoT).

Since the RAN1 study item for 6G has only recently started and many of the RAN requirements are still under discussion, the evaluation of numerologies for different frequency ranges, device types, and TN/NTN deployments should remain open to multiple options.

- Target scalable and forward compatible design for diverse device types.

Prematurely converging on a single numerology—without thorough evaluation against diverse requirements—risks suboptimal performance for certain use cases. While some companies may prefer to limit the scope early to accelerate decision-making, such an approach could overlook critical requirements. Therefore, the final selection of numerology should only be made after comprehensive evaluation has been completed.

Observation 1: 6G study item objective agreed to support scalable design to include diverse types e.g., low-tier IoT / LPWA.

Observation 2: Prematurely converging on a single SCS at the early phase of 6G study without proper evaluation against diverse requirements risks suboptimal performance for certain use cases.

Proposal 1: 6G study needs proper evaluation of one or more SCS considering use cases and requirements and the down-selection of numerology should be done after finishing the evaluation work.

The numerology requirements is analyzed in general for IoT requiring extended coverage. IoT requires higher MCL coverage requirement compared to MBB. The coverage requirement for IoT can be +10-15dB more than the MBB coverage as shown in Figure 1. In the basic coverage, both MBB and IoT device type can be supported with the same numerology to offer improved network energy efficiency while at the extended coverage, IoT device type needs more repetitions to meet the extended UL coverage.

Figure 1: Illustration of Coverage for 6G MBB and LPWA

For extended IoT uplink coverage, especially with narrowband allocations, a smaller SCS (e.g., 7.5 kHz) can be beneficial as shown in the Figure 2. It can be observed that at 10% BLER, ~3 dB and 1.8 dB gain compared to 15 kHz SCS can be achieved for 1 RB and 6 RB allocations, respectively. Note in LTE, 3.75 kHz SCS was specified for NB-IoT UL. The corresponding simulation parameters are provided in Table 1 of the Appendix.

Figure 2: Comparison of 15kHz and 7.5kHz numerology for extended UL coverage

However, 15kHz SCS is sufficient to be supported in the downlink direction in sub-1 GHz enabling MRSS support for all DL channels.

Observation 3: A Smaller SCS of 7.5kHz in sub-1 GHz UL provides ~3 dB and ~1.8 dB gain compared to 15 kHz SCS at 10% BLER for 1 RB and 6 RB allocations, respectively.

Proposal 2: A smaller SCS e.g., 7.5kHz is not needed to be supported in DL.

Proposal 3: Continue to assess the need for a smaller SCS e.g., 7.5kHz in sub-1 GHz UL ONLY for UL coverage enhancement i.e., PUSCH, PUCCH supporting IoT/LPWA devices.

# 3	Conclusion

This contribution discusses 6G scope for Release 20

Observation 1: 6G study item objective agreed to support scalable design to include diverse device types e.g., low-tier IoT / LPWA.

Observation 2: Prematurely converging on a single SCS at the early phase of 6G study without proper evaluation against diverse requirements risks suboptimal performance for certain use cases.

Proposal 1: 6G study needs proper evaluation of one or more SCS considering use cases and requirements and the down-selection of numerology should be done after finishing the evaluation work.

Observation 3: A Smaller SCS of 7.5kHz in sub-1 GHz UL provides ~3 dB and ~1.8 dB gain compared to 15 kHz SCS at 10% BLER for 1 RB and 6 RB allocations, respectively.

Proposal 2: A smaller SCS e.g., 7.5kHz is not needed to be supported in DL.

Proposal 3: Continue to assess the need for a smaller SCS e.g., 7.5kHz in sub-1 GHz UL ONLY for UL coverage enhancement i.e., PUSCH, PUCCH supporting IoT/LPWA devices.

# References

[1] RP-251881, “New SID: Study on 6G”, NTT DOCOMO.

[2] 3GPP TR 38.914, “Study on 6G Scenarios and Requirements”.

[3] R1-2505790, “Overview of 6GR radio interface”, Lenovo.

[4] 3GPP TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks”.

# Appendix

The following table provides the simulation parameters used to evaluate the UL coverage with different numerologies in Figure 2.

Table 1: Simulation parameters corresponding to Figure 2

| Parameter | Value |
| --- | --- |
| Carrier frequency | 900 MHz |
| System BW | 5 MHz |
| Channel Model | TDL-C |
| Delay Spread | 1000 ns |
| UE speed | 0 |
| Channel Estimation | Ideal |
| MCS | Fixed to index 3 |
| Modulation | QPSK |
| Channel Coding Scheme | 5G LDPC with 24-bit CRC |
| Repetition | 20, and 80 for 1 RB, and 6RB allocations, respectively |
| Payload Size | 32 bits for 1 RB allocation, 192 bits for 6 RB allocation |