3GPP TSG RAN Meeting #109	RP-252739

Beijing, China, September 15-18, 2025

Title: 	Proposals for RAN4-led Rel-20 BS RF topics

Agenda Item:	9.1.4.2

Source: 	Huawei, HiSilicon

Document for:	Approval

1	Introduction

During RAN#108 (June 2025) meeting, a WF for RAN4 led Rel-20 topics was Endorsed in [1]. BS RF aspects were extracted below for reference.

In this contribution we provide our views of further handling of Rel-20 BS RF topics in RAN4, on top of agreements in the Endorsed WF for RAN4 led Rel-20.

BS RF centric evolution Agreements: The high-level scope for WI on BS RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - Enhancement of co-location requirements - It is FFS on the applicable frequency ranges, e.g., >3GHz requirements, for OTA - R19 outcome should be considered - Coexistence requirement enhancement can be further discussed in RAN#109 - Down select from the following four options in RAN#109 - Additional Hybrid requirement sets for BS type 1-H/large antenna array - SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios - It is limited to RAN4 only solution - Neither - Both - No RAN1/2 impact is expected - Target 0.5RF TU in Q4’25 and 1 TU allocation from Q1’26.

2	Discussion

2.1	SBFD BS to SBFD BS adjacent channel coexistence

At RAN4 meeting RAN4#115, the following way forward is approved on the deployment scenarios for FR1 SBFD BS. Although RAN4 identified the importance of the deployment scenarios on SBFD-SBFD co-ex, due to the work load and limited time in Rel-19, RAN4 agreed not to introduce the specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19, and SBFD-SBFD co-ex case can be considered for Rel-20 [4].

Way forward on Whether SBFD-SBFD co-ex case should be in Rel-19 scope

- RAN4 do not introduce the specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19. SBFD-SBFD co-ex case can be considered for Rel-20.

At last RAN4 meeting #116, the work of core part for Rel-19 is completed [5] where SBFD-SBFD co-ex case is not included. We think SBFD-SBFD co-ex case and corresponding RF requirements should be included in the Rel-20 5G-Advanced RAN4 Package. The justifications are discussed as below.

Firstly, for TDD-SBFD co-ex, ACIR enhancement is limited by ACLR of TDD BS. Limited ACIR enhancement cannot help much for better co-existence performance. Co-existence is only feasible for 100% grid-shift, which is a quite limited deployment scenario.

Secondly, for lower grid shift, e.g. 10%/20%/50%, for co-existence the required ACIR enhancement is far exceed the legacy BS ACIR, whose enhancement cannot exceed 1.2 dB in FR1 based on the limitation of the legacy BS ACLR simulation assumption.

However, for SBFD-SBFD co-ex, ACIR can be improved thanks to the nature of SBFD design (e.g. uplink subband some distance away from inter-operator band), so better co-existence performance can be achieved.

For a new wide TDD band without existing deployment of legacy TDD base stations, it would be more beneficial to deploy SBFD BS to increase UL coverage and reduce latency to realize the full potential of SBFD technology. In this case improved adjacent channel co-existence between an SBFD BS with another SBFD BS would be feasible by enhancing the ACIR, i.e. defining additional requirements both transmitter side and receiver side. As shown in below figure, thanks to the nature of SBFD design, for protection of SBFD UL of operator B, the SBFD DL of operator B can be adopted as guard band for filter implementation of adjacent operator A. It would not impose additional implementation difficulty for taking the advantage of narrow band filter.

Considering Rel-19 SBFD WI progress, and considering support from multiple operators, completion of related SBFD BS to SBFD BS adjacent channel coexistence work within Rel-20 is the top priority for BS RF package, to secure efficient feature implementation possibilities in future.

Proposal 1: It is proposed to define RF requirements for SBFD BS to SBFD BS adjacent channel coexistence in the Rel-20 5G-Advanced WI on BS RF centric evolution.

Proposal 2: the following WI objective is proposed for SBFD BS to SBFD BS adjacent channel coexistence in the Rel-20 5G-Advanced WI on BS RF centric evolution:

4 Objective 4.1 Objective of SI or Core part WI or Testing part WI SBFD BS to SBFD BS adjacent channel coexistence - Define FR1 Macro/Micro RF requirements for SBFD-SBFD adjacent operator scenario, considering the expected spectrum allocation and sub-band sizes, and relevant co-existence and co-location deployment scenarios. Compliance with these requirements may be declared by manufacturer for SBFD capable BS. - TX unwanted emission - RX in-band blocking 4.2 Objective of Performance part WI SBFD BS to SBFD BS adjacent channel coexistence: - Specify the conformance testing of the introduced core requirements for SBFD-SBFD adjacent channel coexistence

2.2	Enhancements of co-location requirements

The first of the identified Rel-20 BS RF item was enhancements of co-location requirements, as extracted below:

- Enhancement of co-location requirements - It is FFS on the applicable frequency ranges, e.g., >3GHz requirements, for OTA - R19 outcome should be considered - Coexistence requirement enhancement can be further discussed in RAN#109

In general, co-location requirements enhancements were seen as important for Rel-20 work on BS RF. Still, the WF [1] requires further work to add more details and clarifications.

The first of the above sub-bullets treats about the applicable frequency range. Referring to the ongoing Rel-19 work on OTA co-location test aspects, the WF in [2] from RAN4#115 (Malta) meeting has captured agreement on the applicable frequency range. Based on the BS-to-BS isolation data collected in Rel-19 WI, no RF (including core and conformance test) co-location requirements impact below 4.9 GHz was agreed.

Sub-topic 1-2: Impacted RF requirements Way forward: - Based on the BS-to-BS isolation data, no RF (including core and conformance test) co-location requirements impact below 4.9 GHz was agreed. - OTA TX IMD: - Impact on the OTA TX IMD requirement is FFS, considering the argument that this is a regulatory requirement. - Provide further inputs to address the testability issue. - Further investigate impact of higher than 43dBm BS output power.

Based on the above, it is clear that any future study or work item for the OTA co-location requirements shall be limited to frequencies above 4.9 GHz. To address some of the previous comments received, the above derived frequency boundary could be relaxed to also consider other mid-band operating bands, i.e. in this case to also account for 3.5 GHz operating bands.

Proposal 3: For Rel-20 BS RF scope on enhancements of co-location requirements, clarify that the applicable frequency range for OTA co-location requirements enhancements is above 4.9 GHz, and could be extended down to above 3.5 GHz, if necessary.

Secondly, the WF in [1] stated to consider Rel-19 BS RF evolution (NR_BS_RF_req_evo) WI outcomes. BS-BS isolation topic was well elaborated with simulation and measurement results collected from multiple companies. This topic was concluded within Rel-19 Core deadline. No need to further discuss BS-BS isolation aspects in Rel-20. Discussion on impacted co-location requirements was subject to the conclusions on the BS-BS isolation result conclusions.

During RAN4#116, a WF in [6] was approved, capturing the following on impacted BS RF OTA co-location requirements:

OTA co-location requirements are not modified in Rel-19 based on the discussion on BS-BS isolation data analysis.

RAN4 aims to re-evaluate OTA co-location requirements in Rel-20, with Rel-19 BS-BS isolation data as baseline.

Proposal 4: For Rel-20 5G-Advanced WI on BS RF centric evolution, agree on objective to re-evaluate OTA co-location requirements in Rel-20, with Rel-19 BS-BS isolation data as baseline.

Proposal 5: Consider the following OTA co-location objectives for the Rel-20 BS RF WID:

4 Objective 4.1 Objective of SI or Core part WI or Testing part WI Enhancements for OTA co-location requirements: - Evaluate impact of Rel-19 BS-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Investigate necessity of OTA co-location requirements above 4.9 GHz. - Consider impact on the following OTA co-location requirements: - OTA transmitter OFF power requirement, - OTA co-location spurious emission requirement, - OTA transmitter intermodulation requirement, - OTA co-located OOBB requirement. 4.2 Objective of Performance part WI Enhancements for OTA co-location requirements: - Reflect modifications introduced for OTA co-location Core requirements, in related OTA co-location test requirements.

On the coexistence requirements enhancements, the WF in [1] stated that those can be further discussed in RAN#109. As co-existence requirements were discussed in RAN4 only from the RAN4#115 (Malta) meeting, its progress and completion of related objectives within Rel-19 requires more RAN4 meetings, due to system level simulation campaign still ongoing. Based on offline coordination among interested companies, it is suggested to extend related Core objective of Rel-19 WI on NR_BS_RF_req_evo by one quarter. Decision on potential need for Rel-20 follow-up of the ongoing Rel-19 work on co-existence requirements could be postponed till Rel-20 RAN4 check-point in June 2026.

Proposal 6: Co-existence requirements objective shall be separated from the co-location requirements, as those two (i.e., co-location, and co-existence requirements) are separate and unrelated topics.

Proposal 7: Decision on potential need for Rel-20 follow-up of the ongoing Rel-19 work on co-existence requirements to be postponed till Rel-20 RAN4 check-point in June 2026.

2.3	Additional Hybrid requirement sets

Due to limited TU assigned and multiple items pre-selected, there may be need for further down-scoping. Considering priority of the items, introduction of additional Hybrid requirement sets can be postponed to a later release, once other high importance items are advanced or completed.

Proposal 8: De-prioritize Rel-20 objective on additional Hybrid requirement sets for BS type 1-H/large antenna array.

3	Summary

Based on the above discussion, the following proposals were formulated:

Proposal 1: [BS RF: SBFD] It is proposed to define RF requirements for SBFD BS to SBFD BS adjacent channel coexistence in the Rel-20 5G-Advanced WI on BS RF centric evolution.

Proposal 2: [BS RF: SBFD] The following WI objective is proposed for SBFD BS to SBFD BS adjacent channel coexistence in the Rel-20 5G-Advanced WI on BS RF centric evolution:

4 Objective 4.1 Objective of SI or Core part WI or Testing part WI SBFD BS to SBFD BS adjacent channel coexistence - Define FR1 Macro/Micro RF requirements for SBFD-SBFD adjacent operator scenario, considering the expected spectrum allocation and sub-band sizes, and relevant co-existence and co-location deployment scenarios. Compliance with these requirements may be declared by manufacturer for SBFD capable BS. - TX unwanted emission - RX in-band blocking 4.2 Objective of Performance part WI SBFD BS to SBFD BS adjacent channel coexistence: - Specify the conformance testing of the introduced core requirements for SBFD-SBFD adjacent channel coexistence

Proposal 3: [BS RF: co-location] For Rel-20 BS RF scope on enhancements of co-location requirements, clarify that the applicable frequency range for OTA co-location requirements enhancements is above 4.9 GHz, and could be extended down to above 3.5 GHz, if necessary.

Proposal 4: [BS RF: co-location] For Rel-20 5G-Advanced WI on BS RF centric evolution, agree on objective to re-evaluate OTA co-location requirements in Rel-20, with Rel-19 BS-BS isolation data as baseline.

Proposal 5: [BS RF: co-location] Consider the following OTA co-location objectives for the Rel-20 BS RF WID:

4 Objective 4.1 Objective of SI or Core part WI or Testing part WI Enhancements for OTA co-location requirements: - Evaluate impact of Rel-19 BS-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Investigate necessity of OTA co-location requirements above 4.9 GHz. - Consider impact on the following OTA co-location requirements: - OTA transmitter OFF power requirement, - OTA co-location spurious emission requirement, - OTA transmitter intermodulation requirement, - OTA co-located OOBB requirement. 4.2 Objective of Performance part WI Enhancements for OTA co-location requirements: - Reflect modifications introduced for OTA co-location Core requirements, in related OTA co-location test requirements.

Proposal 6: [BS RF: co-existence] Co-existence requirements objective shall be separated from the co-location requirements, as those two (i.e., co-location, and co-existence requirements) are separate and unrelated topics.

Proposal 7: [BS RF: co-existence] Decision on potential need for Rel-20 follow-up of the ongoing Rel-19 work on co-existence requirements to be postponed till Rel-20 RAN4 check-point in June 2026.

Proposal 8: [BS RF: hybrid] De-prioritize Rel-20 objective on additional Hybrid requirement sets for BS type 1-H/large antenna array.

References

[1]	RP-251879, WF for RAN4 led REL-20 topics, RAN#108

[2]	R4-2508657, Way Forward for [115][303] NR_BS_RF_Part2_CLTA_SE: CLTA, RAN4#115

[3]	RP-251881, Study on 6G Radio, RAN#108

[4]	R4-2508713, Way Forward for [115][307] NR_duplex_evo_BSRF

[5]	R4-2511255, CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS

[6]	R4-2512577, Way Forward on CLTA, RAN4#116