3GPP TSG RAN Meeting #110					RP-253038

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.0

Source:	Ericsson

Title:	On deployment scenarios

Document for:	Approval

# Introduction

In this paper we propose further details on scenarios for evaluation of the 6G Radio (6GR), based on the moderator summary RP-252915 after RAN#109.

As the 6GR is intended to fulfill the IMT-2030 requirements, the IMT-2030 test environments need to be a subset of the evaluation scenarios used in 3GPP. Beyond these, 3GPP should study additional scenarios of interest. These 3GPP related scenarios are the focus of this paper.

A note on terminology:

- In ITU, a test environment is a combination of a service type, a geographical environment with associated propagation models, and system configurations.

- In 3GPP, a deployment scenario is a combination of a geographical environment with associated propagation models, and system configurations.

- Note: Although some deployment scenarios are defined with a service in mind, the selection of which deployment scenarios a service is evaluated in is typically done outside the definition of the deployment scenario.

# 2	Discussion

The proposals for 6G deployment scenarios are based on multiple inputs; those described for 3GPP NR in TR 38.913, what is discussed for IMT-2030 in ITU, and additional aspects that Ericsson have identified as relevant for 6G.

In short, the main additions since NR and IMT-2020 are:

- Addition of a new services: Fixed Wireless Access (FWA) and Sensing

- Addition of a suburban macro scenario and an indoor factory scenario

- Addition of a new frequency bands around 7GHz and 15 GHz

- Modification of parameters for the 30GHz band

- Simultaneous use of multiple frequencies/carriers

Note that we distinguish between deployment scenarios and service types. Table 1 gives an overview of our proposals, where the combinations of deployment scenarios and service types to be evaluated is indicated. The yellow highlighted ones are supported by IMT-2030 while the rest are additional combinations for 3GPP evaluations to be added in TR 38.914.

## 2.1 Scenarios and services

As discussed in the introduction we have scenarios and services/use cases. In 6GR, we will have many more deployment scenarios compared to earlier. The ITU has already expanded the number of scenarios, increasing from five in ITU-2020 to ten in ITU-2030 . 3GPP shall include the ITU scenarios, but has in addition defined new scenarios, such as Suburban macro, and intends to study additional such as NTN, Air to Ground, high-speed trains and highway. Therefore, that absence of an “X” in Table 1 doesn’t imply that the service is not supported in that deployment in 6GR. The sparse distribution of “X” reduces the need for evaluations of similar deployments where the developed 6GR features to support that service, likely will not differ much.

Table 1 Combinations of deployment scenarios and service types to be captured in TR 38.914

| Deployment scenarios |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 4.1 Indoor hotspot | 4.X Indoor factory | 4.2 Dense urban | 4.4 Urban | 4.5 Suburban | 4.3 Rural | 4.10 NTN | 4.9 Air to Ground | 4.6 High Speed train | 4.7 Extreme coverage | 4.8 Urban for MTC |
| Service types | eMBB | X |  | X | X | X | X | X |  |  | X |  |
|  | FWA |  |  |  | X | X | X |  |  |  |  |  |
|  | HRLLC |  | X |  | X |  |  |  |  | X |  |  |
|  | ISAC |  | X |  | X |  | X |  |  |  |  |  |
|  | Positioning |  | X |  | X |  |  |  |  |  |  |  |
|  | MTC |  |  |  | X |  |  | X |  |  |  | X |
|  | Commercial Air |  |  |  |  |  |  |  | X |  |  |  |

If we had followed the monolithic approach, where each scenario corresponds to one service, the result would be several pages of scenario descriptions with a high degree of redundancy, as the same deployment assumptions would need to be repeated in multiple scenarios.

Hence, we propose to define multiple use cases/services that share the same deployment scenario assumptions within a single scenario, we can achieve a compact and efficient way of capturing the expanding need for new cases while minimizing redundancy in number of deployment scenarios.

Define multiple use cases/services that share the same deployment scenario assumptions within a single scenario

A deployment scenario is basically defined by:

- whether the BS is indoor/outdoor/sky

- the ISD of the grid (when applicable) and

- the propagation model (such as InH, InF, UMi, UMa, RMa, +their derivatives for UAV and SMa -no UAV for SMa).

The approach in the proposal above and illustrated in Table 1  is not entirely new. For example, the UMa scenario in NR includes two services—eMBB and URLLC as defined in TR 38.802. So overall, we have the magnificent-6 scenarios (InH, InF, Dense Urban, Urban, Suburban, Rural), along with some special structure scenarios (NTN, High-speed train, Air-to-Ground, Urban grid, Highway) and for each of these deployment scenarios a number of services or use cases can be defined.

With the modest approach of mapping services to deployment scenarios that is captured in the table above, the number of scenarios can increase from twelve for 5G to twenty for 6G.

For example, in the Urban Macro table in the next section, we can have eMBB, HRLLC, Positioning, MTC and Sensing of UAV types of services.

This approach also has the advantage that we can simulate each service independently or a combination of two or more services in a certain deployment scenario (for example eMBB and sensing services in the Urban deployment scenario or eMBB and FWA in Rural scenario). Example of the latter is described in Table A.2.4-1 of TR 38.802, where eMBB is multiplexed with URLLC traffic.

The approach where each deployment scenario can have multiple service types defined, allows for simulation of combination of service types, e.g. eMBB and FWA together in Rural.

Note, however, combining services in a single simulation (simulating different slices) is advanced and can be considered in the working groups at a later stage of the 6GR study.

## 2.2 New scenarios

We have one newly added scenario:

Indoor Factory (InF)

- For HRLLC and positioning, propose an additional UE speed of 0 km/h to represent non-moving machines (e.g., production belts).

## 2.3 Sensing related issues

For sensing, we propose a minimum set of scenarios in the discussion below, while we are open that the study item adds additional scenarios for this service, if identified to be necessary (e.g., sensing of humans in Rural). In addition, for UEs performing sensing, we assume this is not a handheld device, but a sensing capable UE device that is more conformant with the CPE form factor and assumptions on antennas etc. This has also been reflected below.

We don’t see the bracketed urban grid necessary for 3GPP work and suggest not to include it.

The urban grid scenario corresponds to a deterministic building grid combined with a fabricated site layout that fits the grid geometry. Although, the main argument for using such a scenario is being more realistic, we believe that it primarily adds complexity to the simulations setup without bringing any benefits in terms of realism.

From a communications study perspective, we foresee several shortcomings of urban grid:

- The site deployment in a deterministic building grid, requires extensive coverage analysis where site locations are picked carefully in order to ensure a contiguous service coverage. Such an analysis is beyond the interest of 3GPP and the scope of evaluations. Deploying sites based solely on the building grid is not realistic and can actually lead to a suboptimal network performance operating point. For example, the site position relative to the building, if deployed in the middle of the roof or at the roof edges, as well as the sectors orientation, play a very important role in network performance. If such parameters are not designed carefully, it can lead to biased network performance and flawed feature development.

- The urban grid model, despite having deterministic components, such as buildings, roads, etc., it still relies on the stochastic 3GPP UMa propagation model. As a result, it does not provide any additional benefit in terms of realism, since the underlying channel model is identical.

- Finally, the urban grid model focuses on outdoor vehicles and pedestrians, neglecting indoor traffic which comprises most of the traffic in urban areas. Therefore, we believe that urban grid is not suitable to study communication services.

Additionally, there are a few strong weaknesses in this scenario from ISAC perspectives which makes this scenario unrealistic for simulating sensing of vehicles.

- A scenario specifically proposed for sensing does not go well along with the concept of ISAC. To make sensing integrated with communication, sensing should be evaluated in communication scenarios. Since this scenario is not considered for evaluating communication for the aforementioned reasons, this scenario should be excluded for ISAC too.

- The dimensions of buildings and roads proposed in this scenario are not proportional in the picture depicting this scenario.  The roads are much narrower compared to the sizes of the buildings. Sensing of vehicles and pedestrians alongside on such narrow streets separated by opaque (in radio sense) wide and big buildings presents an extremely difficult sensing scenario which is unrealistic and should be avoided.

- The deployment of base stations (Urban Macro) in this scenario does not provide good line of sight probabilities with vehicle on roads. This makes sensing of vehicles more difficult and less efficient. Reasonable performance of sensing cannot be expected in such a lack of line of sight environments.

## 2.4 Other modifications to TR 28.914 V0.2.0

In addition, for coverage limited scenarios, solutions related to multi-band operation (e.g. uplink in a lower band) are important for 3GPP to evaluate and to study to understand the possible needed standard support. Hence, we are supportive of e.g., 4GHz + 7GHz combinations in Urban scenario for example, as will be seen in the tables below.

For mTRP, we suggest not capturing this in TR 38.914, it is up to WGs to define assumptions for mTRP evaluations.

The  Heterogenous Networks (Macro+Microlayers) are not necessary to capture in TR 38.914 deployment scenario tables, these can be defined in the WGs, discuss further at RAN#110

Traffic models can be defined in the respective WGs and need not be defined for the scenarios in TR 38.914

In addition, the highway scenario has been integrated with the rural and extreme long distance coverage scenarios. Both of these scenarios include users in cars driving with high speeds, which allow for studies targeting high performance with high reliability and frequent handovers. Furthermore, automotive sensing is possible to be evaluated under the RMa scenario. Therefore, we propose to not include the highway scenario as an effort also to minimize the number of special scenarios.

# 3 Device antenna elements per service

The discussion on device (UE) antenna elements can be separated from the deployment scenarios and is copied into the different deployment scenarios below according to Table 1. We see different number of antenna elements for devices that are handheld vs. non-handheld, where non-handheld devices are larger and can have more antennas and possibly also more TXRUs.

## 3.1 eMBB/Positioning service

“handheld” UE

- Around 700 MHz: 1 Tx and 2 Rx antenna elements (ITU up to 4Tx/Rx)

- Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements

- Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements

- Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements

- Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements

Optional “non-handheld” UE (computer):

- Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements

- Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements

- Around 30 GHz: Up to 32 Tx and Rx antenna elements

## 3.2 HRLLC service

“handheld” UE

- Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements

- Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements

Optional “non-handheld” UE

- Around 4 GHz: Up to 4 Tx and 8 Rx antenna elements

- Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements

## 3.3 FWA service

CPE

- Around 700 MHz: Up to 2 Tx and 4 Rx antenna elements

- Around 2 GHz: Up to 4 Tx and 8 Rx antenna elements

- Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements

- Around 7 GHz: Up to 8 Tx and 8 Rx antenna element

- Note that for scenarios that have indoor deployment of CPEs, procedures for pre-condition mechanisms to determine whether CPE is indoor or outdoor should be considered

## 3.4 MTC service

- Around 700 MHz: 1 Tx/Rx antenna elements

- Around 2 GHz: 1 Tx/Rx antenna elements

- Around 4 GHz: 1 Tx and up to 2 Rx antenna elements

## 3.5 Sensing capable UE for ISAC service

- Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements

- Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements

- Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements

# 4 Motivation for updates to the deployment scenarios

Here we give some motivation for the changes we have made in the text proposal to TR 38.914 that is found in the next section of this document.

## 4.1 Indoor hotspot

The attributes for Indoor hotspot from the endorsed TR 38.914 V0.2.0 is shown in Table 4.1 in Section 5. In Indoor hotspot we propose to evaluate the eMBB service with both smartphones and laptops/desktops. On our view, the use of 6G in laptops working at desks and in meeting room in offices is an important service in the indoor hotspot deployment and such users are essentially stationary. Therefore, we choose 0 km/h for these stationary eMBB devices. Note that simulators may need to set a small nonzero UE speed in practice, due to simulator implementations, but this is up to each company to address, as it is a simulator dependent practicality.

- Adding a second Profile “2” for User distribution and 0 km/h UE speed  for non-held devices to capture stationary seated desktop/laptop stations

- These non-held devices have a larger upper bound of the number of TX and RX antennas compared to handheld devices

## 4.2 Dense Urban

The attributes for Dense Urban  from the endorsed TR 38.914 V0.2.0 is shown in Table 4.2 in Section 5. In Dense Urban we propose to evaluate the eMBB service with smartphones only (handheld). Here we also add large city square service deployment to accommodate high load outdoor user scenarios in busy squares, touristic areas such as Time Square in New York, Shibuya crossing in Tokyo or The Bund in Shanghai.

We thus propose the following changed/additions:

- Propose an outdoor traffic split of 80% Outdoor / 20% Indoor as to differentiate from the UMa split; otherwise, the only distinction between DU and UMa is ISD

- In such dense deployments, the Macro layer should include around 30 GHz option. Dense urban is a high capacity demand scenario, therefore the high bandwidth along with the short ISD can be benefit from the high available bandwidth offered by the around 30 GHz band option.

- Remove the Macro+Micro option, instead based on the study objectives in the working groups, each proponent can select the combination of Macro and Micro frequencies from the provided list of frequencies supported by each layer

- We propose to add a “Times Square” service deployment profile of UEs, characterized by a large portion (60%) of outdoor users at pedestrian speed. 20% users in slowly moving cars and 20% indoor users.

- FWA service is added in this scenario

## 4.3 Rural

The attributes for Rural  from the endorsed TR 38.914 V0.2.0 is shown in Table 4.3 in Section 5.In Rural we propose to evaluate the eMBB service with smartphones (handheld only), FWA service and Sensing service.

We thus propose the following changed/additions:

- For sensing we propose to focus on UAV and automotive scenarios. Sensing can be performed either by the Macro BS or by specially deployed ground sensing units (e.g. VRU, or RSU-type UEs). Antenna characteristics if the latter one are provided in section 3.5 and in table 4.3.

- On user distribution for eMBB services we propose an alternative to the classic 50/50 indoor/outdoor-in-car split that captures both outdoor pedestrian and in car users. The new profile 2 assumes a higher indoor traffic of 70% capturing households in rural communities and following a similar principle on indoor assumption as in other eMBB scenarios in urban and suburban areas.

- For FWA service in rural, we assume rooftop mounted antennas, i.e. 100% outdoor and 0 km/h. The CPE height can be further decided in the working group.

## 4.4 Urban Macro

The attributes for Urban Macro from the endorsed TR 38.914 V0.2.0 is shown in Table 4.4 in Section 5.

In Urban Macro we propose to evaluate the eMBB service with smartphones (handheld only), HRLLC. Sensing, Positioning and MTC

We thus propose the following changed/additions:

- We propose to remove the explicit bands combinations of “Macro + Micro”. Instead, we support the approach of defining a list of bands per layer. Depending on the objectives of the future studies, the proponents can select the set of bands to be assigned in each layer. Also, we propose to remove the quad-band option, since the triplet of 2+4+7 will capture most of the potential phenomena future studies would like to investigate.

- For the case of the two-layer scenario, we propose small cells to be randomly dropped within each Macro cell following the same methodology as in the dense urban scenario. This also reflects real deployments, where small cells are deployed on grids with higher ISDs.

  - Since, the cell range of the urban cell is twice as large of the cell range of the dense urban cell, we propose 6 Micro TRxPs per Macro TRxP. Fixed or deterministic Micro positions are not encouraged since they might impact collection of performance statistics.

- For eMBB and HRLLC services, we propose a similar indoor-to-outdoor split of 80-to-20, reflecting that most of the traffic is generated indoors. Same traffic split is proposed for MTC type of services with an optional proposal of 0km/h speed for indoor traffic to capture static devices such as utility meters and in-building sensors.

- For positioning services, we propose a 50-to-50 indoor-to-outdoor split, targeting more outdoor location services.For sensing we propose to focus on UAV, human and automotive scenarios. Sensing can be performed either by the MacroBSs or by specially deployed ground sensing capable UEs  together with BSsAntenna characteristics if the latter one are provided in section 3.5 and in table 4.3.

- Propose splitting eMBB outdoor traffic between pedestrian and in-car users

## 4.5 Sub-Urban Macro

The attributes for Suburban Macro from the endorsed TR 38.914 V0.2.0 is shown in Table 4.5 in Section 5.

In Suburban Macro we propose to evaluate the eMBB service with smartphones (handheld only) and FWA service

We thus propose the following changed/additions:

- A major service that has gained a lot of traction in sub-urban areas is FWA. The high bandwidth of around 30 GHz band, offers a good opportunity to support such high-capacity demand services as well as offloading bands with less available capacity. Therefore, we propose to add around 30 GHz band in the Macro layer, targeting primarily serving FWA users.

- For FWA service, we propose two user deployment profiles, either 100% indoor or 100% outdoor. Indoor CPEs could be deployed behind windows or walls, to ensure good service coverage, capturing experience from real networks. Similarly, outdoor CPEs could be mounted on exterior side of building walls or pole mounted above rooftop. Nevertheless, a mix of indoor and outdoor, along with prequalification criteria on indoor versus outdoor CPE deployment, is not precluded in the WG evaluations.

## 4.6 High speed

The attributes for High speed from the endorsed TR 38.914 V0.2.0 is shown in Table 4.6 in Section 5.

In High speed we propose to evaluate the eMBB service with smartphones (handheld only) with HRLLC service

We thus propose the following changed/additions:

- Add around 15 GHz as an option to be evaluated.

- Support a linear Macro deployment as depicted in the figure 6.1.5-1 found in TR 38.913

- Service will be provided either via direct connection of the UE to the BS or via a data aggregator deployed on the roof of the train. We propose to eliminate the option of relay.

## 4.7 Extreme long distance

The attributes for Extreme long distance from the endorsed TR 38.914 V0.2.0 is shown in Table 4.7 in Section 5.

In Extreme long distance scenario, we propose to evaluate the eMBB service with smartphones (handheld only) for 700 MHz carrier frequency.

## 4.8 Urban coverage for massive connection

The attributes for Urban coverage for massive connection from the endorsed TR 38.914 V0.2.0 is shown in Table 4.8 in Section 5.

In Urban coverage for massive connection, we propose to evaluate the MTC service.

The assumptions are copied from the corresponding clause in TR 38.913. If the deployment scenario needs to be used for evaluation of other aspects than uplink connection density, it may be necessary to extend the assumptions with downlink-related parameters.

## 4.9 Air-to-ground scenario

The attributes for Air-to-ground scenario from the endorsed TR 38.914 V0.2.0 is shown in Table 4.9 in Section 5.

In Air-to-ground scenario we propose to evaluate the commercial air service. Around 7 GHz band will play an important role in 6G systems, therefore we propose to include this band in the list of evaluated bands.

## 4.10 NTN

The attributes for NTN from the endorsed TR 38.914 V0.2.0 is shown in Table 4.5 in Section 5.

In NTN we propose to evaluate the eMBB service with smartphones (handheld only), FWA and MTC.

## 4.X Indoor Factory

The attributes for Indoor Factory from the endorsed TR 38.914 V0.2.0 are shown in Table 4.X in Section 5.

In Indoor Factory we propose to evaluate the positioning service, sensing and HRLLC. For HRLLC service, we propose two user speed profiles, targeting both mobile (such as devices carried by human or AGVs) and fixed factory components (such as stationary machines, production belts, etc.). For sensing we propose to focus on human and AGV scenarios. Sensing can be performed either by the Macro BS or by specially deployed ground sensing units. Antenna characteristics if the latter one are provided in section 3.5 and in table 4.3.

# 5 Text proposals

Based on the discussion above we propose the following text proposal for TR 38.914.

# 4.	Deployment scenarios

[…]

## 4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

The evaluation of indoor hotspot deployment scenario supports the eMBB service with both handheld and non-handheld (e.g. laptop) devices.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 2 GHz: Up to 200 MHz (DL+UL) NOTE3. Around 4 GHz: Up to 300 MHz (DL+UL) NOTE3 Around 7 GHz: Up to 400 MHz (DL+UL) NOTE3 Around 15 GHz Up to 400 MHz (DL+UL) NOTE3 Around 30 GHz: Up to 1GHz (DL+UL) NOTE3 |
| Layout | - Indoor floor (Open office) , 120m x 50m |
| ISD | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) Other candidate site numbers: 3, 6 |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | TBD Handheld for eMBB service - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements Optional non-handheld (computer) for eMBB service - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| Service deployment and speed options | 100% Indoor, 3km/h, eMBB services - Profile 1 100% Indoor: 3km/h - Profile 2 (Optional, for seated people, desktop stations, using UE antenna elements of the optional device) 100% Indoor: 0 km/h [ |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| NOTE 1: TBD NOTE 2: TBD NOTE 3: TBD NOTE 4: TBD |  |

## 4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor (for example large city squares) and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

The evaluation of Dense urban deployment scenario supports the eMBB service with handheld devices,

Some of its attributes are listed in Table 4.2.

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700MHz Around 2GHz Around 4 GHz Around 7 GHz Around 15GHz Around 30 GHz Around 2 GHz + Around 4 GHz Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700 MHz |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 200MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | 200m |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | TBD Handheld for eMBB service - Around 700 MHz: 1 Tx and 2 Rx antenna elements - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements |
| Service deployment and speed options | Uniform/Macro TRxP, - eMBB Service Profile 1 (Majority indoor) 20% Outdoor in cars: 30 km/h 80% Indoor: 3 km/h - eMBB Service Profile 2 (Majority outdoor, “Time Square” scenario) 60% Outdoor: 3 km/h, 20% Outdoor in cars: 30 km/h 20% Indoor: 3 km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| NOTE 1: TBD NOTE 2: TBD |  |

## 4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

The evaluation of Rural deployment scenario supports the eMBB service with handheld devices, FWA service and sensing service.

Some of its attributes are listed in Table 4.3

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7 GHz (ISD 1) Around 700 MHz + Around 7 GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements | Handheld for eMBB service - Around 700 MHz: 1 Tx and 2 Rx antenna elements - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements FWA service (CPE) or Sensing capable UE: - Around 700 MHz: Up to 2 Tx and 4 Rx antenna elements - Around 2 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements |
| Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded) | eMBB Service Profile 1 - 15% Outdoor: 3 km/h, 15% Outdoor in cars: 120 km/h 70% Indoor: 3 km/h eMBB Service Profile 2 - 50% Outdoor in cars: 120 km/h, 50% Indoor: 3 km/h FWA service Profile: - 80% Indoor: 3 km/h - 20% Outdoor rooftop mounted CPE: 0 km/h - Procedures for pre-condition mechanisms to determine whether CPE is indoor or outdoor should be considered |
| Sensing target distribution and Sensing target speed | UAV sensing: - 100% Outdoor - Sensing target assumptions as in Table 7.9.1-1 (TR. 38.901) Automotive sensing: - Speed: 30 km/h , 120km/h - Sensing target assumptions as in Table 7.9.1-2 (TR. 38.901) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

The evaluation of Urban Macro deployment scenario supports the eMBB service with handheld devices, HRLLC  service, positioning service, MTC service, FWA service and sensing service.

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 2 GHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz |  |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |  |
| Layout | Single layer: - Hex. Grid |  |
| ISD | Macro layer: 500m |  |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |  |
| UE antenna elements |  | Handheld (used in eMBB, HRLLC, Positioning services): - Around 700 MHz: 1 Tx and 2 Rx antenna elements (ITU up to 4Tx/Rx) - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements Sensing capable UE: - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements MTC device: - Around 700 MHz: 1 Tx/Rx antenna element - Around 2 GHz: 1 Tx/Rx antenna element - Around 4 GHz: 1 Tx and up to 2 Rx antenna elements FWA service CPE : - Around 700 MHz: Up to 2 Tx and 4 Rx antenna elements - Around 2 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements |  |
| Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded) | Uniform/Macro TRxP eMBB Service Profile: - 10% Outdoor: 3 km/h, - 10% Outdoor in cars: 30 km/h - 80% Indoor: 3 km/h HRLLC Service Profile: - 20% Outdoor: 3 km/h, 80% Indoor: 3 km/h Positioning Service Profile: - 25% [10%] Outdoor: 3 km/h - 25% [10%] Outdoor in cars: 30 km/h - 50% [80%] Indoor: 3 km/h MTC Service Profile: - 20% Outdoor in cars: 30 km/h, or Outdoor: 3 km/h (optional 0 km/h) - 80% Indoor: 3 km/h, (optional 0 km/h for utility meters, sensors etc) FWA Service Profile (Indoor CPE) Placed behind wall or window 100% Indoor: 0 km/h |  |
| Sensing target distribution and Sensing target speed | UAV sensing targets: - 100% Outdoor - Sensing target assumptions as in Table 7.9.1-1 (TR. 38.901) Automotive sensing targets: - Speed: 30 km/h , 120km/h - Sensing target assumptions as in Table 7.9.1-2 (TR. 38.901) Human sensing targets: - Sensing target assumptions as in Table 7.9.1-3 (TR. 38.901) with target speed according to Option 2 and Option 3 |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |

## 4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving,] This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

The evaluation of Sub-Urban Macro deployment scenario supports the eMBB service with handheld devices and FWA service with both indoor and external mounted CPE.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz Around 700 MHz + Around 7 GHz Around 4 GHz +Around 7 GHz Around 2 GHz + Around 7 GHz + Around 30 GHz Around 2 GHz+ Around 700 MHz |
| Aggregated bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer Hex. Grid |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | Handheld for eMBB service - Around 700 MHz: 1 Tx and 2 Rx antenna elements (ITU up to 4Tx/Rx) - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements FWA service CPE device : - Around 700 MHz: Up to 2 Tx and 4 Rx antenna elements - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements |
| Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded) | eMBB Service Profile: - 10% Outdoor: 3 km/h, - 10% Outdoor in cars: 40 km/h - 80% Indoor: 3 km/h FWA Service Profile 1 (Indoor CPE) - Behind wall or window - 100% Indoor: 0 km/h, FWA Service Profile 2 (Outdoor mounted CPE) - Exterior wall mounted or rooftop - 100% Outdoor: 0 km/h, |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.6	High speed

Editor note: Highway scenario will be considered and whether highway scenario is merged to section 4.6 can be further discussed

The high speed deployment scenario focuses on train  scenario will be noise-limited and/or interference-limited, using macro TRxPs.

The evaluation of high speed deployment scenario supports the eMBB service with HRLLC service.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.6.

Editor note: airborne will not be captured in section 4.6.

Table 4.6: Attributes for high speed scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz Around 7GHz Around 15GHz |
| Aggregated system bandwidth | Around 4 GHz: Up to 300 MHz (DL+UL) Around 7GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Linear Grid along railway line |
| ISD | 1732m |
| BS antenna elements | Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements | Handheld for eMBB service (eMBB and HRLLC) - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements Optional Non- Handheld for eMBB and HRLLC service - Around 4 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements |
| Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded) | eMBB Service Profile with high reliability (HRLLC): - 100% Outdoor in train: Up to 500 km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.7	Extreme long distance coverage in low density area

The extreme Long Range deployment scenario is defined to allow for the provision of services for very large areas with low density of users whether they are humans and machines (e.g. wilderness, river or water, remote village or island areas, etc). The key characteristics of this scenario are Macro cells with very large area coverage supporting [at least] basic data speeds and voice services, with low to moderate user throughput and low user density.

Table 4.7: Attributes for extreme long distance

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz |
| Aggregated System Bandwidth | 40 MHz (DL+UL) |
| Layout | Single layer: Isolated Macro sites |
| Cell range | 50 km |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements |
| UE antenna elements | eMBB UE “handheld” device - Around 700 MHz: 1 Tx and 2 Rx antenna elements (ITU up to 4Tx/Rx) |
| User density and UE speed | eMBB Service Profile 1 - 100% outdoor, 3 km/h eMBB Service Profile 2: - 20% indoor (3km/h) - 40% outdoor in car (60km/h) - 40% outdoor in car (120km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.8	Urban coverage for massive connection

Editor note: focusing on MTC

The urban coverage for massive connection scenario focuses on large cells and continuous coverage to provide mMTC. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas, with very high connection density of mMTC devices. This deployment scenario is for the evaluation of the KPI of connection density.

Some of its attributes are listed in Table 4.8.

Table 4.8: Attributes of urban coverage for massive connection

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 700MHz, 2100 MHz as an option |
| Network deployment including ISD | Macro only, ISD = 1732m, 500m |
| Device deployment | Indoor, and outdoor in-car devices |
| Maximum mobility speed | 20% of users are outdoor in cars (100km/h) or 20% of users are outdoors (3km/h) 80% of users are indoor (3km/h) Users dropped uniformly in entire cell |
| Service profile | Non-full buffer with small packets |
| BS antenna elements | 2 and 4 Rx ports (8 Rx ports as optional) |
| UE antenna elements | 1Tx |

## 4.9	Air-to-Ground Scenario

The commercial Air to Ground deployment scenario is defined to allow for the provision of services for commercial aircraft to enable both humans and machines aboard the aircraft to initiate and receive mobile services. It is not for the establishment of airborne based base stations.

The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput that are optimized for high altitude users that are travelling at very high speeds. The commercial airlines aircrafts are likely equipped with an aggregation point (e.g. Relay).

Some of the characteristics of this deployment scenario are listed in Table 4.9.

Table 4.9: Attributes for commercial Air to Ground Scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz |
| Aggregated System Bandwidth | 100 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| Cell range | 300 km |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements | Belly mounted antenna |
| Service deployment and speed options | eMBB service Profile with high reliability (HRLLC): - 100% Outdoor in aircraft: Up to 1200 km/h - Altitude up to 12 km |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.10	Non-Terrestrial Network

Editor Table 4.10: Attributes for Non-Terrestrial scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2 GHz Around 15 GHz Around 30 GHz (UL) and 20 GHz (DL) |
| Aggregated system bandwidth | Around 2 GHz: Up to 30 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 500 MHz (DL+UL) |
| Layout | Single layer: - Hex. Pattern (at least 19 spot beams) |
| Satellite Orbit | LEO 600 km (optional 1200 km) altitude GEO 35786 km altitude |
| BS antenna pattern | Bessel function as in Section 6.4.1 in TR 38.811 |
| UE antenna elements | Handheld for eMBB service - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 15 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements MTC device - Around 2 GHz: 1 Tx/Rx antenna element |
| Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded) | eMBB Service Profile 1 - 100% Outdoor: 0 km/h eMBB Service Profile 2 - 100% Outdoor in car: 120 km/h MTC Service Profile: - 100% Outdoor: 0 km/h, |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

note: orbit type and payload type will be captured in section 4.10

## 4.X	Indoor factory

The indoor factory deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency NOTE1 | Around 4 GHz Around 7 GHz Around 4GHz+Around 7GHz Around 30 GHz |  |
| Aggregated system bandwidth NOTE2 | Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |  |
| Layout | Single layer: - Indoor floor (Factory), 300m x 150m |  |
| ISD | 50m (Equivalent to 18 sites per 300m x 150m) |  |
| BS antenna elements | Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |  |
| UE antenna elements | “handheld” eMBB service device (used in HRLLC, Positioning): - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements Optional eMBB service “non-handheld” device (used in HRLLC, Sensing ): - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements |  |
| Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded) | eMBB Service Profile 1 with High reliability (HRLLC) 100% Indoor: 3km/h eMBB Profile 2 with High reliability (HRLLC) (Optional, stationary machine) 100% Indoor: 0 km/h Positioning Service: 100% Indoor: 3km/h |  |
| Sensing target distribution and Sensing target speed | Human sensing targets - Sensing target assumptions as in Table 7.9.1-3 (TR. 38.901) - Target speed according to Option 2 and 3 AGV sensing targets: - Sensing target assumptions as in Table 7.9.1-4 (TR. 38.901) |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |

Some of its attributes are listed in Table 4.1X.

Table 4.1X: Attributes for indoor factory

# 6	Conclusion

In this contribution we propose adding the text proposal in section 5 to Section 4 of TR 38.914.