3GPP TSG RAN Meeting #110						RP-253061

Baltimore, USA, December 8-11, 2025

Agenda Item:		8.2.1.2

Source:		AT&T

Title:				Architecture Requirements for Data Collection

Document for:		Discussion/Decision

# 1	Introduction

In this document, we provide AT&T’s views on 6G architecture and migration to be captured in 38.914, following the agreements from RAN#109, as well as the discussions in the RAN2 #131bis, RAN2#133 and RAN3#129bis and RAN3#130.

# 2	Discussion

The following is captured in draft 38.914 on 6G architecture requirements [1].

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

Further to these agreements, RAN2 and RAN3 working groups had corresponding discussions related to 6G RAN architecture and the data collection framework considerations on the architecture. The following discussions and agreements in RAN2 and RAN3 are of particular relevance:

RAN2 discussed the requirements on standardized data collection for different use cases, which includes, AI/ML, sensing, QoE, etc., during RAN2#131bis and RAN2#132. Following agreements are made during RAN2#132 [2]:

Agreements Study the follow aspects as a starting point: Data types (non-real time) - AI/ML (data collection for offline training) - SON/MDT, QoE - (details on what is included in 6G depends on RANP) - Sensing (the detailed characteristics (including what cases are real time or not) will be discussed in later phase, after some RAN1 and RAN2 discussions in the sensing session) QoS (e.g. data volume and latency) - Define some categories we want to base our discussion on. Details of QoS for some cases may depend on other WGs. Termination points (impacted group) - UE to RAN - UE to CN - UE to OAM - UE to UE server Understand the consumer (may be different than the termination) of the data and whether data has to be decodable by RAN.

Similarly, RAN3 made progress on the data collection framework during RAN3#130. Following agreements have been made 3G

The 6G RAN architecture supports data collection according to the following principles: - Reusability of collected data is supported. - Data collected or generated by the 6G RAN can be used by the RAN and it can be made available to other entities, if needed. FFS which entities. - The 6G RAN can request data, and use the data collected from other entities. FFS which entities. NOTE 1: The collection, storage and usage of data follows the security principles which will be defined by relevant WGs.

## In addition, chair also noted the following statement:

## Triggering of data collection needs further discussion

## Agreements in RAN2 and RAN3 indicate that a 6G architecture must support multiple termination points for data collection. The RAN may trigger data collection from the UE, or it may consume data collected by the UE. The collected data may terminate at the RAN, at OAM, or in the CN. There is also a clear understanding that the termination point of the data can be different from the consumer of the data.

## As noted in the RAN2#123 agreement, multiple data types are considered for data collection. Some are non–real-time, such as AI/ML data collected for training, and certain sensing use cases in which measurements are collected in RAN while the sensing result resides in the OAM or the CN. There are also more real-time use cases, including AI/ML data collection for monitoring and sensing applications with tighter latency requirements.

## AI/ML use cases in cellular networks are heterogeneous in their objectives, latency budgets, data volumes, and operational constraints. Consequently, it is essential for the system to support multiple data collection and processing termination points, so that each use case can be served at an appropriate location in the architecture. For use cases that require fast responses with tight control-loop time scales, it is advantageous to collect and, where applicable, pre-process data within the Radio Access Network (RAN). Locating data collection closer to the point of actuation reduces round-trip latency, minimizes transport overhead on the fronthaul/backhaul, and enables timely inference and closed-loop control.

## Conversely, for use cases where immediate responses are not required (i.e., relaxed control-loop time scales), data can be collected at Operations, Administration, and Maintenance (OAM) functions. OAM nodes can offer higher compute and storage capacity and are better suited to handling network management, analytics, and automation activities. As a result, AI/ML use cases that focus on network automation and management, and that have less stringent response-time requirements, can leverage OAM-based data collection and processing to benefit from these richer resources without adversely impacting latency-sensitive paths.

## In addition, certain use cases may require the Core Network (CN) to have visibility into data originating from the RAN and/or from the User Equipment (UE). In such scenarios, the relevant data can be collected directly at CN functions, using standardized interfaces and procedures to obtain RAN- and UE-derived measurements, events, and telemetry. This approach allows the CN to perform analytics, coordinate ML pipelines, or support service-level optimizations while remaining consistent with applicable security, privacy, and policy constraints.

## Overall, the selection of the termination point for AI/ML data collection is guided by the use case’s latency and responsiveness requirements, available compute and storage resources, transport efficiency considerations, and operational manageability. For all these termination points considerations, the MNO should have visibility and control over the data collected.

## The 6G architecture should thus support data collection for different types of data, and different types of use cases corresponding to multiple services, such as AI/ML and sensing. The 6G architecture should be flexible to accommodate data collection for various services in a uniform and efficient way.

## Proposal 1: The 6G RAN architecture shall support an integrated data collection framework with MNO visibility and control over data collection

## Proposal 2: The 6G RAN architecture shall support an integrated data collection framework for diverse data types which allows data collection terminated in RAN, OAM, and CN

# 4 Conclusion

In this contribution, we discussed the architecture requirements for data collection for 6G. The following proposals were made to be captured in Section 5.2 of 38.914.

## Proposal 1: The 6G RAN architecture shall support an integrated data collection framework with MNO visibility and control over data collection

## Proposal 2: The 6G RAN architecture shall support an integrated data collection framework for diverse data types which allows data collection terminated in RAN, OAM, and CN

# 5	References

- RP-252870 pCR for 38.914 on Requirements for architecture and migration for 6G

- 3GPP RAN2#132 Chair Notes, Dallas, USA

- 3GPP RAN3#130 Chair Notes, Dallas, USA