3GPP TSG RAN Meeting #110					RP-253069

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.0

Source:	T-Mobile USA, Deutsche Telekom, Ericsson, Nokia, Jio Platforms, Verizon

Title:	Requirements for support of FWA service in 6G deployment scenarios

Document for:	Discussion and Decision

# 1	Introduction

Fixed Wireless Access (FWA) service has experienced significant growth in recent years, driven by technological advancements. Wireless operators, particularly those in North America and India, have recognized the increased business potential for in-home high-speed internet in areas with limited access to traditional broadband infrastructures such as cable or fiber. The global deployment of 5G has provided unique opportunities for operators to open new revenue streams by offering in-home broadband services to their customers. The rising demand in both residential and commercial sectors for faster, reliable, and high-speed connectivity is propelling the 5G fixed wireless access market.

The U.S. 5G fixed wireless access service market size was estimated at USD 8.35 billion in 2023 and is projected to be worth around USD 343.18 billion by 2034, growing at a compound annual growth rate (CAGR) of 40.18% from 2024 to 2034, according to Precedence Research. With FWA becoming the primary traffic in operators’ networks, it is critical that on Day One of 6G, FWA as a primary service offered by operators can meet customer demand with a sustainable and profitable business model.

# 2	Discussion

The existing network, designed primarily for enhanced Mobile Broadband (eMBB) services, may not be able to meet the demand of FWA services, which have different mobility profiles, devices and usage patterns. With the increased demand and projected growth of FWA in both enterprise and consumer areas, solutions that can further improve spectrum efficiency and economies of scale for FWA are essential to ensure the long-term success of the business model. Mobile operators seek to enhance FWA's functionality by optimizing systems, such as physical layer (PHY) optimizations, to address the unique needs of FWA service users. By revisiting key elements like mobility management, control channel overhead, and other PHY layer enhancements, network capacity could be significantly increased to meet the higher bandwidth demands of fixed users.

Observation

- Current 5G specification/protocol/architecture were designed for general eMBB service, without special consideration of FWA service

- One-pipe-fits-all type approach may not work well for eMBB and FWA

- While eMBB remains an important service provided by operators, FWA service, with its specific characteristics and significantly higher traffic volume, needs to be treated as a primary service when the specification is developed in 3GPP

- At the same time it is important to acknowledge that part of the success of FWA is due to the fact that it can technically leverage eMBB design. On high-level a similar approach should be taken for 6G. That said FWA is such a successful service, it would warrant targeted enhancements.

In order to support FWA service in a sufficient manner in 6G it is important that FWA specific enhancements are studied within the working groups. Aspects to study are air interface enhancements, protocol enhancements and mobility specific enhancements. Further it is clear that FWA devices can support specific HP UE classes

In more detai,it is proposed to study potential enhancements in the following areas

- The potential air interface modifications that provide better spectrum efficiency for FWA

- RAN signalling and protocols that are specifically designed to support FWA deployment

- Support for link adaptation, channel estimation, and RRM schemes that leverage the unique characteristics of FWA service user profiles (mobility, usage pattern etc)

- Device capabilities that are unique for FWA service, including exploring the regulatory requirements.

- FWA device enhancement (BF, IC, HPUE)

- Use of unlicensed spectrum to support FWA services for capacity and coverage improvements

- Support for interference management/coordination schemes that are uniquely beneficial for FWA deployment.

- RAN architecture options, e.g. modular NAS/AS for FWA.

It is important that candidate enhancements are studied and evaluated in relevant scenarios.

In addition to studied multiple different technical areas within the working groups it is important to put focus on FWA service by having a suitable evaluation scenario. This could e.g. be based on the urban, suburban, and rural scenarios used for eMBB, with modifications of e.g. UE positions, mobility, and antenna types to reflect FWA.

It is essential that when deployment scenarios and performance evaluation of new Radio Interface Technology (RIT) are being defined, FWA devices and their parameters should be included in the TR 38.914.

When FWA devices with outdoor, roof-top deployments, directional antennas and with capabilities of beam forming are added into the simulation studies of 6G RIT, then the outcomes can be significantly different compared to the case where traditional eMBB type of UEs are used.

From a telecom operator’s perspective, It is important that the studies on various deployment scenarios should mimic as much as possible the practical deployment scenario.

Thus, 6G deployment scenarios that do not include FWA devices with higher order MIMO capabilities into the mix of devices used for study do not give a clear indication to a telecom operator about the potential benefits of 6G and will be only academic in nature.

The FWA deployment scenario has the following specific attributes that need to be catered for when evaluating such:

- CPE/FWA device capabilities, i.e. antennas and Tx power may be typically higher than for other types of device.

- CPE/FWA may have certain characteristics more typical to such a device (i.e. stationary, typical antenna height may be higher in Urban deployments especially).

We assume attributes such as Tx power, associated power control modelling and any differences in antenna heights will be further defined by the WGs. Therefore, the attributes proposed in this document reflect those normally captured at RAN plenary level.

Since FWA is a new service which generally is offered on existing eMBB sites, FWA and enhanced Mobile Broadband (eMBB) may share spectrum in some deployments, while others may use dedicated carriers [R1-2509079]. To have flexibility to evaluate each service independently or a combination of two or more services simultaneously in a certain deployment scenario (for example eMBB and FWA services in the suburban deployment scenario), we add the FWA service to the evaluation assumptions of three key deoplyment scenarios in 38.914.

Here, the Dense Urban, Urban Macro, Sub-urban and Rural scenarios needs to add asusmptions related to FWA service, and the Text Proposal for 38.914 is given in Section 3 below.

Note that although other deployment scenarios such as Deep Rural (known as Extreme long distance coverage in low density area in TR 38.914) and Non Terrestrial Network (NTN) are also attractive scenarios for FWA service [R1-2509079], adding them to 38.914 would lead to excessive evaluations with likely no impact on the defined 6GR features. Therefor we select Dense Urban, Urban Macro, Sub-urban and Rural,  as four  scenarios for FWA service from TR 38.914 perspective as they span the space of possible solutions well and likely use of both indoor and externally mounted CPEs. If necessary, evaluation assumptions for addition scenarios, could be added to the evaluation assumptions defined by the WGs.

# 3 Text proposal based on TR 38.914 V0.2.0

[…]

## 4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

The evaluation of Rural deployment scenario supports the eMBB service with handheld devices and FWA service

Some of its attributes are listed in Table 4.3

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7 GHz (ISD 1) Around 700 MHz + Around 7 GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements | eMBB UE “handheld” device (used in eMBB) - Around 700 MHz: 1 Tx and 2 Rx antenna elements - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements FWA CPE (used in FWA): - Around 700 MHz: Up to 2 Tx and 4 Rx antenna elements - Around 2 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements |
| Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded) | eMBB service: Profile 1 - 15% Outdoor: 3 km/h, 15% Outdoor in cars: 120 km/h 70% Indoor: 3 km/h Profile 2 - 50% Outdoor in cars: 120 km/h, 50% Indoor: 3 km/h FWA service: - 80% Indoor: 3 km/h - 20% Outdoor rooftop mounted CPE: 0 km/h - Procedures for pre-condition mechanisms to determine whether CPE is indoor our outdoor should be considered |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

The evaluation of Urban deployment scenario supports the eMBB service with handheld devices and FWA service.

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 2 GHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 2 GHz + Around 30GHz Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | eMBB UE “handheld” device (used in eMBB) - Around 700 MHz: 1 Tx and 2 Rx antenna elements - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements FWA CPE (used in FWA): - Around 700 MHz: Up to 2 Tx and 4 Rx antenna elements - Around 2 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements |
| User distribution and UE speed | TBD [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.] This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

The evaluation of Sub-Urban Macro deployment scenario supports the eMBB service with handheld devices and FWA service with both indoor and external mounted CPE.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 700 MHz + Around 7 GHz Around 4 GHz +Around 7 GHz Around 2 GHz + Around 7 GHz + Around 30 GHz Around 2 GHz+ Around 700 MHz Around 7 GHz +Around 4 GHz + Around 2 GHz+Around 700 MHz |
| Aggregated bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer Hex. Grid [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | eMBB UE “handheld” device - Around 700 MHz: 1 Tx and 2 Rx antenna elements - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements FWA CPE device : - Around 700 MHz: Up to 2 Tx and 4 Rx antenna elements - Around 2 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements |
| Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded) | eMBB: - 10% Outdoor: 3 km/h, 10% Outdoor in cars: 40 km/h 80% Indoor: 3 km/h FWA: Profile 1 (Indoor CPE) - 100% Indoor: 0 km/h, Profile 2 (Outdoor mounted CPE) - Exterior wall mounted or rooftop - 100% Outdoor: 0 km/h, |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

The evaluation of Dense Urban deployment scenario supports the eMBB service with handheld devices and FWA service.

Some of its attributes are listed in Table 4.2

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro layer: Around 700MHz Around 2GHz Around 4 GHz Around 7 GHz Around 15GHz Around 2 GHz + Around 4 GHz Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700 MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 200MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100]m All micro TRxPs are all outdoor |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | eMBB UE “handheld” device - Around 700 MHz: 1 Tx and 2 Rx antenna elements - Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements - Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements - Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements FWA CPE device : - Around 700 MHz: Up to 2 Tx and 4 Rx antenna elements - Around 2 GHz: Up to 4 Tx and 8 Rx antenna elements - Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements - Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements |
| User distribution and UE speed | eMBB: - 10% Outdoor: 3 km/h, 10% Outdoor in cars: 40 km/h 80% Indoor: 3 km/h FWA: Profile 1 (Indoor CPE) - 100% Indoor: 0 km/h, Profile 2 (Outdoor mounted CPE) - Exterior wall mounted or rooftop - 100% Outdoor: 0 km/h, |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| NOTE 1: TBD NOTE 2: TBD |  |

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914V0.2.0.

We further propose that RAN shall task the RAN working groups to study FWA specific enhancements specifically within the areas of air interface enh, protocol enh, mobility specific enhancements and device enhancements (e.g. HPUE).

Proposal 1: RAN shall task the RAN working groups to study FWA specific enhancements related to, e.g., air interface, protocol, mobility, and device enhancements (e.g., HPUE). Accordingly, update the deployment scenarios in TR 38.914 V02.0 to include the FWA relevant aspects per the TP in section 3 of this document.