3GPP TSG RAN Meeting #110	RP-253073

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.3.2

Source:	Futurewei

Title:	Diverse device types for 6GR

Document for:	Discussion and Decision

# Introduction

The study of diverse device types is included in the SID, and has been discussed in RAN1#122, 122bis, and 123, as well as in RAN#109. So far there have been agreements on a low-tier device type for (at least) massive communication, which can have 1TX/RX (from RAN#109) and a reduced bandwidth (3, 5, 10, 20MHz TBD) (from RAN1#122bis). Logically, there is also a non-lowest-tier device type that (so far) covers everything else. In RAN#110 discussion is expected to continue from the two RAN#109 device type agreements [1].

Proposal 3: To investigate further:

  - Motivations/justifications behind the proposed diverse device types, which should be a limited set

  - Whether/how to have one or more device types for eMBB or 6G IoT

  - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

  - Whether/how to explicitly standardize device types

  - Ensuring forward compatibility

  - Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

Proposal 4: In terms of diverse device types, study further:

- Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

Note: some of the above parameters/factors may be related with form factor

Note: aim to have a focused/limited set of parameters/factors for a device type

- The value(s) for the identified parameters for a device type

In this paper we present our views on the next steps of the device type study.

# Discussion

Terminology

Per the SID we will study diverse device types, as we have done at RAN1#122, 122bis, and 123, as well as RAN#109. In RAN#109 we noted we would investigate further whether/how to explicitly standardize device types, and that the terminology of “device type” is open to further refinement. In our view it is not so urgent to change the term “device type”, or to discuss how or whether it will show up in the normative specifications. Even if we list the different types in a table for convenience in the study, such a table may not show up in the specifications. As an example, “normal/smartphone” and “FWA” device types could be handled via the usual capability signaling, low-end IoT could be handled like NR RedCap (i.e., a paragraph and some capability differences listed in 38.306), and smart glasses exceptions could be handled in the same manner as developed in Rel-19. It is also important (and aligned with the RAN#109 agreements) to investigate having a limited set of device types which are characterized by a focused/limited set of parameters. Such an investigation does not make much sense if we replace “device types” with “devices”.

Observation 1: It is not urgent to change the study terminology of “device type” or to discuss whether/how device types will show up in the specifications.

Wearables

As mentioned in the introduction, RAN1 and RAN agreements define a low-tier device type, and effectively everything else is then the non-low tier device type until we define more device types.

Observation 2: Currently there are only two 6GR device types, a low-tier device type and a non-low tier device type. The low-tier device type has a maximum bandwidth of either 3, 5, 10, or 20MHz (TBD) and can have 1 TX/RX chains.

The question is then whether wearables should be introduced as a third device type? We see three options for handling wearables in 6GR.

- Option 1: Select 20MHz as the lowest tier IoT bandwidth

  - Similar data rates as Rel-17/18 NR RedCap wearables

  - Greatest economy of scale for the lowest tier

- Option 2: Introduce a wearables (or non-smartphone) device type with 40-50MHz BW and at most 2RX

  - Higher data rates than NR RedCap wearables

- Option 3: Apply the smart glasses exception for XR agreed in Rel-19

  - Highest data rates for wearables

Options 1 and 3 introduce no new device type (thereby fulfilling the goal of a limited set of device types).

The bandwidth gap of Option 2 compared to lowest-tier IoT may reduce market fragmentation.

Proposal 1: Consider handling wearables in 6GR with one or more of the following options:

- Option 1: Select 20MHz as the lowest tier IoT bandwidth

- Option 2: Introduce a wearables (or non-smartphone) device type with 40-50MHz BW and at most 2RX

- Option 3: Apply the smart glasses exception for XR agreed in Rel-19

As a side note, in NR RedCap we considered 1RX and 2RX devices to be part of the same device type. For 6GR we could consider that multiple numbers of RX chains could also be allowed in the same device type.

Bandwidth for lowest-tier device type

In RAN1#122 we agreed:

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

In RAN1#123 we agreed:

Agreement

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - Overall device complexity

  - Overall system performance impact

  - Energy efficiency for both BS and UE

  - Aim at a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority

Given the RAN direction to have a limited set of device types, the focus should be on the main selection of Opt 1, 2, 3, or 4, not on the (RAN1#122) FFS or optimizing for the FFS. There are three points that we would like to make. First, the 6GR lower-tier device type is going to achieve cost-reduction from economies of scale, not from a custom (non-common/non-scalable) design. Second, a larger maximum bandwidth will be able to meet the data rate requirements of more services/applications, without the need for introducing more device types. This is aligned with our paper [2], where we propose that the data rate for massive communication needs to be at least 10Mbps, and possibly even higher. Third, at RAN#109 a RAN4 WI was approved for NR RedCap UEs that can work in a 3MHz bandwidth. The WID discussion included possible complexity reductions from allowing the device to only support 3MHz; however it was argued that this was a new NR RedCap device that had little complexity benefit and a greater negative system impact than existing NR RedCap, and consequently the complexity reduction was not agreed. Rel-18 RedCap also did not introduce 5MHz RF BW, in part because of concerns on limited complexity benefit. The same arguments also apply to 6GR, and as such suggest adopting a larger BW device with a larger peak data rate.

Observation 3: The 6GR lower-tier device type is going to achieve cost-reduction from economies of scale, not from a custom (non-common/non-scalable) design.

Observation 4: A larger maximum bandwidth will be able to meet the data rate requirements of more services/applications, without the need for introducing more device types.

Proposal 2: To limit the number of device types, minimize market fragmentation, and maximize economies of scale, a single maximum bandwidth option is selected.

Proposal 3: Opt 1: 3MHz should not be considered.

# Conclusion

The following observations and proposals were discussed in this contribution.

Observation 1: It is not urgent to change the study terminology of “device type” or to discuss whether/how device types will show up in the specifications.

Observation 2: Currently there are only two 6GR device types, a low-tier device type and a non-low tier device type. The low-tier device type has a maximum bandwidth of either 3, 5, 10, or 20MHz and can have 1 TX/RX chains.

Proposal 1: Consider handling wearables in 6GR with one or more of the following options:

- Option 1: Select 20MHz as the lowest tier IoT bandwidth

- Option 2: Introduce a wearables (or non-smartphone) device type with 40-50MHz BW and at most 2RX

- Option 3: Apply the smart glasses exception for XR agreed in Rel-19

Observation 3: The 6GR lower-tier device type is going to achieve cost-reduction from economies of scale, not from a custom (non-common/non-scalable) design.

Observation 4: A larger maximum bandwidth will be able to meet the data rate requirements of more services/applications, without the need for introducing more device types.

Proposal 2: To limit the number of device types, minimize market fragmentation, and maximize economies of scale, a single maximum bandwidth option is selected.

Proposal 3: Opt 1: 3MHz should not be considered.

# References

RP-252873, “Summary of 6G Diverse Device Types (including Minimum UE Bandwidth).”

RP-253074, “Massive communication needs a minimum peak data rate of at least 10Mbps,” FUTUREWEI.