3GPP TSG RAN Meeting #110					RP-253088

Baltimore, USA, Dec 8-11, 2025

Agenda Item:	8.2.1.3.3

Source:	Ericsson, Airbus, BDBOS, Deutsche Telekom, DISA, Erillisverkot, FirstNet, LGUplus, Motorola Solutions, Nkom, Nokia, Softil, SyncTechno Inc, Telstra, Thales, The Netherlands Police, T-Mobile USA, Verizon

Title:	Resiliency requirements in 6G

Document for:	Approval

# 1	Introduction

The 6G system must provide robust and resilient communication. The goal is to maximize service availability and reliability by e.g., minimizing outages of the connectivity, reducing their number as well as the duration of each of them (see e.g., 3GPP TS 22.261 for different related definitions such as reliability or communication service availability). This paper discusses corresponding requirements for 6G in the scope of RAN.

# 2	Discussion

Resilience has always been a consideration across all generations of mobile networks, from early mobility needs to evolving use cases and requirements. However, as networks become more complex and critical to daily life and society critical operations, resilience can no longer be treated as an afterthought. It must become a fundamental design principle in the architecture of future networks. Resilience is a characteristic that is different from and not fully addressed by URLLC features or the QoS framework in 5G. Robust and resilient communication are important for enabling critical communications (e.g., business critical, mission critical, society critical, etc.) but also for supporting high-value use cases which depend on communication services with restrictions on the allowable service interruption time.

In the past, resiliency has been considered at node level (e.g., for 5G NR, TR 38.913 defines ‘RAN availability’ as a measure of the availability of a base station for communication). In our view, it is important to consider the availability of connectivity services, beyond availability of the base station.

Like other requirements (e.g., capacity, throughput, etc.), the level of resiliency that can be offered depends on multiple considerations such as deployment, available features, etc. 6G should enable resilience in communications for all networks and deployments, but the tools available (e.g., availability of one/multiple frequencies, backhaul redundancy, etc.) in each case will vary and so will the level of resilience provided. In general, the most suitable set of solutions for providing resiliency may vary depending on the deployment, etc. For example, in an urban deployment the availability of multiple sites and frequencies may provide a good foundation for delivering resiliency. In contrast, in a rural deployment it may be necessary to consider e.g., other terrestrial or non-terrestrial networks to provide resiliency to failures.

Robustness within a single carrier is important for achieving high levels of reliability in typical operation conditions. However, such robustness is typically delivered by single elements in the network infrastructure (e.g., a single radio or baseband serving a cell), which become single points of failure. In many cases, it is possible to provide resilience by leveraging redundancy in the network (e.g., multiple radios covering a certain area; multiple software instances; redundant compute; etc.). Although we do not think that 3GPP solutions should assume specific types of redundancy, it is important that the features and the architecture allow for leveraging them whenever available. For such cases, the design of 6G and the signaling and procedures should allow to quickly recover from failures.

It is important to note that different users and verticals may have different requirements regarding resilience and, similarly, some services may have stricter requirements regarding service availability than others. Since providing resiliency will come at some cost, it is necessary that 6G allows for identifying which users and services need differential treatment. Besides, delivering resilience is sometimes in conflict with other goals (e.g., energy saving, etc.) so being able to identify users and services subject to resiliency requirements will allow for fulfilling the different targets to the largest extent. Providing the possibility of differential treatment for different services creates new business opportunities for MNOs.

We believe that resilience should be a central consideration in the design of 6G and not a collection of additional features. In our view, protocols and signaling should be robust by design and capable of providing solutions when issues arise. Robustness and resilience should target typical operational conditions (e.g., radio link issues) as well as unforeseen issues (e.g., power outages; node failures; interruptions of front-haul or back-haul; etc.). Whenever possible, general-purpose features should be designed to provide resilience (e.g., mobility procedures; reconnection procedures; etc.). For example, in case of node failures (e.g., due to a power outage, due to a hardware failure, etc.) mobility features may be used to ensure continuity of the connectivity services for the users leveraging overlapping coverage from multiple cells. However, it is important that the mobility procedures are robust, allowing the UE to maintain a connection even if the serving node fails. 3GPP added support for NTN in 5G (and also for IoT NTN using IoT-based solutions, e.g., NB-IoT). Including satellite connectivity allows for providing coverage in unserved and underserved areas but also broadens the set of tools that can be used for resilience. For example, NTN connectivity may be used as a fallback for scenarios where having overlapping TN coverage from multiple cells may not be realistic.

In earlier generations, 3GPP introduced radio features enabling different types of diversity (e.g., spectrum aggregation; transmission from different TX/RX points, etc.). These features are instrumental in providing robustness. However, the design of some of these features included single points of failure that made them vulnerable to certain types of issues (e.g., PCell dependency in CA, etc.). These problems should be avoided in 6G.

As stated above, the goal of resilience is not only to reduce communication outage duration, but to avoid outages altogether. In some cases, issues impacting the connectivity service (user plane) cannot be foreseen and 6G should provide tools to address this situation. However, in many cases it is possible to anticipate the problems (e.g., interruptions in the control plane typically lead to interruptions in the user plane, etc.). We believe that 6G should support both approaches by having:

- Means to detect issues before (e.g., predictions) or when affecting the user plane and the control plane, both at the network and the UE.

- Control plane tools (e.g. mobility, reconfiguration) to address detected issues.

Configuring and managing a network to provide resilience is a complex task. To facilitate it, 6G should include support for collecting the necessary UE measurements (e.g., related to coverage, failures, etc.).

To meet some of the requirements described here, involvement of the 6G Radio and CN is necessary. Thus, coordination and cooperation between RAN WGs and SA/CT WGs is needed.

# 3 	Text proposal

Based on the discussion above we propose the following text proposal.

### 5.3.5 Network Resilience The 6G Radio and 6G RAN architecture shall be designed to facilitate implementations minimizing occurrence and duration of interruption of the user plane and control plane caused by environmental factors (e.g., radio link issues, electromagnetic interference, etc.) as well as issues affecting the UE and the network infrastructure (e.g., power outages, failures in hardware or software, etc.). The 6G Radio and the 6G RAN architecture shall facilitate implementations suitable for varying levels of redundancy in the network deployment, avoiding in general specific assumptions on availability of redundancy options (e.g., redundant serving cells, etc.). The 6G Radio procedures and the 6G RAN architecture (including TN and NTN deployments) shall be robust (e.g., to avoid failures) and shall allow for utilizing redundancy (e.g., overlapping radio coverage, duplicated hardware or software, etc.), whenever available, to recover from failures (e.g., link interruptions, radio unit failures, baseband failures, hardware/software failures, etc.) with a predefined behaviour (e.g., based on configuration) and a configurable recovery timeline (e.g., limit the service interruption time by handing over users to well-functioning cells, by re-establishing the connection, etc.) The 6G Radio procedures and the RAN architecture shall be designed to facilitate RAN maintenance with minimal service interruption times, including tools for recovering the connectivity. The 6G Radio shall support anticipation and early detection of issues affecting user plane and control plane. The 6G Radio and the 6G RAN shall know which users and services (e.g., QoS flows) have requirements regarding resilience. The 6G Radio and 6G RAN architecture shall be designed to ensure that features relevant for resilience can be configured and used for specific users and services (e.g., specific QoS flows) based on resiliency requirements. The 6G Radio shall support collection of information, including UE reports, that allow for managing the network to address issues and provide the desired level of resiliency (e.g., information about coverage, failures, etc.), including automation.

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914.