3GPP TSG RAN Meeting #110 								                  RP-253114

Baltimore, US, Dec. 8-11, 2025

Agenda Item:	10.3.1

Source:	OPPO

Title:	Consideration on supporting IMS voice via NTN GEO access (GEO voice)

Document for:	Discussion

Introduction

For IMS voice via NTN GEO, both RAN2 and SA1/SA2 discussed in past few meetings. Some progress has been made especially on the choice between UP and CP solution, as well as IP and non-IP direction. In this paper progress made in both SA and RAN side is put on the table and propose how to proceed with this item in coming meetings. The TP for revised WID can be found in Annex.

Discussion

UP vs CP based solution

For UP solution, packets for IMS voice and IMS signaling are carried on DRB over radio after establishing a single PDN connection for EPS bearer for voice call.

For CP solution, packets for IMS voice and IMS signaling are carried on SRB over radio. A PDN connection is established in advance for EPS bearer and relevant context is stored by UE and core network before establishing voice call.

In SA2 the proposal for KI#1 in [1] is approved. Here is the approved text:

In addition, about PDN connection, another TP in [2] is approved. And here is the approved text:

Observation 1: From SA2 perspective, it is concluded that UP solution shall be specified.

RAN2 concluded that UP solution is recommended for this item in [3].

Observation 2: From RAN2 perspective, it is recommended that UP solution is used for this item.

Based on both observation 1 and observation 2, it is clear that UP solution should be specified to support IMS voice service over NTN IoT (GSO).

Proposal 1: Only UP solution is specified to support IMS voice service over NTN IoT (GSO)

RAN2 also concluded that different RLC modes will be used for IMS signaling packet and IMS voice packet. Here is the conclusion:

	In UP solution, RAN2 understands that voice data packets will be transmitted via UM DRB while the voice signalling packets will be transmitted via AM DRB (no intention to mandate this in the specification)

Table 2-1

So, two DRBs are needed to support IMS voice service, which is already supported in current spec of NB IoT. The question is whether another DRB is needed to accommodate other data service e.g. internet. If parallel data service is allowed, then it seems that another DRB is needed since the QoS of data service and voice service should be different. Our preference is that parallel voice service and data services are not allowed considering NTN IoT system is a narrow band system while performance of voice service should be guaranteed with high priority. It means that a voice service and data services can run in TDM manner. Another issue is whether data service should be preempted by voice service if a voice service needs to be established? We think this issue should be addressed by SA2.

Proposal 2: Parallel voice service and data service are not allowed, and voice service is of higher priority.

Proposal 3: If P2 is approved, SA2 needs to discuss whether any solution is needed to preempt data service when voice service is to be established.

Proposal 3a: If P2 is not approved, RAN2 needs to discuss how to add at least one more DRB and SA2 needs to discuss whether and how parallel data service can be supported.

IP vs non-IP solution

For both IP and non-IP solutions, a PDN connection will be established to allocate IP address for UE and establish ESP bearer to carrier IP packets. The difference between IP and non-IP based solutions is that for IP based solutions, ROHC algorithms will be adopted in PDCP layer to reduce signaling overhead over Uu interface. While for non-IP based solution, UDP/IP header can be deleted/compressed between the PGW and UE and restored at the other end. In addition, AGW is used to remove/compress RTP header in IMS layer [4].

We understand that both IP and non-IP based solutions will be specified based on approved text proposal in [5], while IP based solution is mandatory and non-IP based solution is optional solution.

It means non-IP based solution could be negotiated when both UE and network support it.

Observation 3: IP based solution is mandatory solution to be specified while non-IP based solution is optional solution.

For IP based solution, voice packet size may change upon transition of ROHC state. If such issue is resolved by SPS (Semi-Persistent Scheduling), it means SPS needs to be always configured based on the worst case. While it is a rare case based on RAN1’s evaluation [6]. So, we think dynamic schedule schemes should be studied and specified apart from schemes based on SPS. For both IP and non-IP based solution, packet size will change between voice burst and silence period. However, such change can be addressed by switching between different SPS resources.

Proposal 4: Dynamic schedule should be studied and specified in RAN2 to address variable packet size due to transition of ROHC state.

Conclusion

Here are observations and

Observation 1: From SA2 perspective, it is concluded that UP solution shall be specified.

Observation 2: From RAN2 perspective, it is recommended that UP solution is used for this item.

Proposal 1: Only UP solution is specified to support IMS voice service over NTN IoT (GSO)

Proposal 2: Parallel voice service and data service are not allowed, and voice service is of higher priority.

Proposal 3: If P2 is approved, SA2 needs to discuss whether any solution is needed to preempt data service when voice service is to be established.

Proposal 3a: If P2 is not approved, RAN2 needs to discuss how to add at least one more DRB and SA2 needs to discuss whether and how parallel data service can be supported.

Observation 3: IP based solution is mandatory solution to be specified while non-IP based solution is optional solution.

Proposal 4: Dynamic schedule should be studied and specified in RAN2 to address variable packet size due to transition of ROHC state.

References

[1] S2-2511304 Conclusion for KI#1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC, vivo, China Mobile, MediaTek Inc., Qualcomm Incorporated, ZTE, China Unicom, CATT, Ericsson, Nokia, Samsung, Spreadtrum, Xiaomi, OPPO, SHARP, Thales, EchoStar, CSCN, Skylo, NTT DOCOMO, NEC

[2] S2-2511305 Interim conclusion for KI#1 of FS_5GSAT_Ph4_ARC, ZTE, CMCC, vivo, China Telecom, MediaTek Inc, Sanechips, LG Electronics, NTT DOCOMO, CALTTA

[3] R2-2509213	LS on Voice over NB-IoT NTN	vivo

[4] S2-2510961 Compromise proposal for KI#1 and KI#2, Qualcomm Incorporated, Deutsche Telekom, vivo, MediaTek Inc., Samsung, Skylo

[5]S2-2510962 Conclusion for KI#1 Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC, vivo, Qualcomm Incorporated, China Mobile, MediaTek Inc., CATT, China Unicom, Spreadtrum, NTT DOCOMO, Xiaomi, OPPO, SHARP, Thales, EchoStar, CSCN, TNO, Deutsche Telekom, NEC, China Telecom, LG Electronics, Skylo, Samsung

[6] R1-2508096 Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC, RAN1

Annex : Text proposal to revise RAN WID RP-252473:

- 4.1	Objective of SI or Core part WI or Testing part WI

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives:

- Only UP solutions for voice support over NB-IoT-NTN is specified for both IMS signalling and voice packet transmission

- Support of semi-persistent scheduling and dynamic scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1]

- Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2]

- Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2]

- Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4]

- Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements

- Note: Coordination with SA4 is expected