3GPP TSG RAN Meeting #110   											   RP-253117

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.1

Source:	CMCC

Title:	Discussion on 3GPP energy efficiency requirement

Document for:	Approval

Introduction

In RAN#107, the SID of Study on 6G Scenarios and requirements has been approved [1]. During the discussion in RAN#109, energy efficiency had been agreed as a quantitative KPI [2]:

Agreement Proposal 1: For 3GPP internal evaluation, define energy efficiency as a quantitative KPI in 3GPP for both network and device. Proposal 2: For 3GPP internal evaluation, at least empty load and partial load cases should be evaluated in 6GR study. - Evaluate via system level simulations (SLS) - Other evaluation methods are not precluded when relevant Proposal 3: For 3GPP internal evaluation, UE/network power model is to be discussed in RAN1.

Simultaneously, a current version of definition on energy efficiency for test performance requirement (TPR) in ITU-R WP5D is captured in TR 38.914 [3].

In this paper, we provide our views on 3GPP energy efficiency requirement, including the definition of energy efficiency in 3GPP and further consideration on energy efficiency design between RAN and RAN1.

Definition of energy efficiency in 3GPP

In this section, we discuss the definition of energy efficiency in 3GPP.

During the discussion on TPR for ITU-R WP5D, energy efficiency and is defined as the relative power saving ratio under different load cases. From our point of view, such methodology is a good starting point for 3GPP internal discussion and can be further considered from the following aspects:

- The first point is how to calculate the power consumption for both NW and UE. Since RAN1 is currently discussing and formulating the new BS/UE power model for 6GR [4], such model should be considered as the baseline.

- The second point is how to ensure the experience while enhancing the energy efficiency. Regarding such issue, we prefer to introduce the constraint on certain metric while evaluate the network EE or Device EE. To be specific:

  - The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant like certain percent (e.g. 90%) of the UE fulfils the target should be achieved while evaluating the relative power saving for each load case.

  - In addition, since different service may sensitive to different type of metric, it is recommended that the certain metric can be selected according to the service type, for instance:

    - For Vo6G (i.e. VoNR-like) service, the certain metric is latency

    - For burst-buffer service, the certain metric is QoS or UPT

    - For XR service, the certain metric is latency

- The third point is the definition for reference case of relative power saving under UE EE. From our understanding, when NW is fully loaded, a certain UE may not be fully loaded due to there is more than one UE under each BS. Moreover, the load of UE under NW fully loaded case is strongly relevant to the number of UEs under a certain BS, and simply reuse the definition of reference cases under NW EE will cause ambiguity. Therefore, it is better to clarify that the reference case of UE EE is when UE is fully-loaded/has full-buffer service, and it is also necessary to report the number of UE per BS and the traffic model.

Therefore, we propose the following definition for energy efficiency in TR 38.914:

Proposal 1: For energy efficiency, support the following definition to be captured in TR 38.914:

5.1.15 Energy efficiency Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of 6GR to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of 6GR to support device modem power saving in relation to the traffic characteristics. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. The requirement is the relative energy savings between the selected load case(s) and a reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. The power model defined for 6GR BS and 6GR UE in RAN1 should be used as baseline. Otherwise, proponents should report the power model used for evaluation. Proponents should also report the number of UE under each BS and the traffic model. The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant that certain percent (e.g. 90%) of UE fulfils the target should be achieved while evaluating the relative power saving for each load case. The certain metric can be selected according to the service type: - For Vo6G (i.e. VoNR-like) service, the certain metric is latency - For burst-buffer service, the certain metric is QoS or UPT - For XR service, the certain metric is latency The minimum requirements for energy efficiency are summarized in Tables below. Table 5.1.15-1: The minimum requirements for Network Energy efficiency Test environment(s) **; Selected load case(s); The reference case; Relative power saving (%). N. A; empty load; Fully loaded case *; [X0%]. Indoor Hotspot-IC; empty load, low load, light load and/or medium load *; [X1%, X2%, ...]. Dense Urban-IC; empty load, low load, light load and/or medium load *; [Y1%, Y2%, ...]. Rural-IC; empty load, low load, light load and/or medium load *; [Z1%, Z2%, ...]. *empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation Table 5.1.15-2: The minimum requirements for Device Energy efficiency Test environment(s) **; Selected load case(s); The reference case; Relative power saving (%). N. A; empty load; Device is fully-loaded/has full-buffer service; [A0%]. Indoor Hotspot-IC; empty load, low load, light load and/or medium load *; [A1%, A2%, …]. Dense Urban-IC; empty load, low load, light load and/or medium load *; [B1%, B2%, …]. Rural-IC; empty load, low load, light load and/or medium load *; [C1%, C2%, …]. *empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50 ** needed only in case of simulation

Further consideration on energy efficiency design between RAN and RAN1

In this section, we discuss further consideration on energy efficiency design between RAN and RAN1.

During the discussion in RAN1#122, the following proposal was made for EE metric in 6G BS and 6G UE [5]:

Agreement - Study metric(s) for UE energy efficiency. - Study metric(s) for BS energy efficiency.

Given consideration on the current progress in RAN1 6GR EE and RAN KPI agendas, we suggest RAN plenary to further focus on the following aspects and give some guidance to RAN1:

- How to handle the parallel discussion on EE metric between RAN and RAN1. From our understanding, to avoid the duplicate work between RAN and RAN1, it is recommended that discussion on EE metric is limited to RAN 6G SI, and RAN can inform RAN1 the information once there is some progress.

- Moreover, since RAN1 6G SI is discussing the design of 6G BS and 6G UE power model, RAN plenary can wait for RAN1 progress before further discuss the target value on NW EE and UE EE if needed.

Proposal 2: To avoid duplicate work between RAN and RAN1, support the followings:

- The discussion on EE metric for both NW and UE is limited to RAN 6G SI discussion, and RAN can inform RAN1 the information once there is some progress.

- RAN plenary wait for RAN1 progress on the design of 6G BS and 6G UE power model before further discuss the target value on NW EE and UE EE if needed.

Proposal

In this paper, we provide our views on 3GPP energy efficiency requirement, and the following proposals are made:

Proposal 1: For energy efficiency, support the following definition to be captured in TR 38.914:

5.1.15 Energy efficiency Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of 6GR to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of 6GR to support device modem power saving in relation to the traffic characteristics. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. The requirement is the relative energy savings between the selected load case(s) and a reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. The power model defined for 6GR BS and 6GR UE in RAN1 should be used as baseline. Otherwise, proponents should report the power model used for evaluation. Proponents should also report the number of UE under each BS and the traffic model. The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant that certain percent (e.g. 90%) of UE fulfils the target should be achieved while evaluating the relative power saving for each load case. The certain metric can be selected according to the service type: - For Vo6G (i.e. VoNR-like) service, the certain metric is latency - For burst-buffer service, the certain metric is QoS or UPT - For XR service, the certain metric is latency The minimum requirements for energy efficiency are summarized in Tables below. Table 5.1.15-1: The minimum requirements for Network Energy efficiency Test environment(s) **; Selected load case(s); The reference case; Relative power saving (%). N. A; empty load; Fully loaded case *; [X0%]. Indoor Hotspot-IC; empty load, low load, light load and/or medium load *; [X1%, X2%, ...]. Dense Urban-IC; empty load, low load, light load and/or medium load *; [Y1%, Y2%, ...]. Rural-IC; empty load, low load, light load and/or medium load *; [Z1%, Z2%, ...]. *empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation Table 5.1.15-2: The minimum requirements for Device Energy efficiency Test environment(s) **; Selected load case(s); The reference case; Relative power saving (%). N. A; empty load; Device is fully-loaded/has full-buffer service; [A0%]. Indoor Hotspot-IC; empty load, low load, light load and/or medium load *; [A1%, A2%, …]. Dense Urban-IC; empty load, low load, light load and/or medium load *; [B1%, B2%, …]. Rural-IC; empty load, low load, light load and/or medium load *; [C1%, C2%, …]. *empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50 ** needed only in case of simulation

Proposal 2: To avoid duplicate work between RAN and RAN1, support the followings:

- The discussion on EE metric for both NW and UE is limited to RAN 6G SI discussion, and RAN can inform RAN1 the information once there is some progress.

- RAN plenary wait for RAN1 progress on the design of 6G BS and 6G UE power model before further discuss the target value on NW EE and UE EE if needed.

References

- RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom.

- RP-252883, Moderator's summary for RAN led 6G SI: Key performance indicators

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements.

- R1-2509592, Summary #8 of 6GR Energy Efficiency Study

- R1-2506602, Final Summary of 6GR Energy Efficiency Study