3GPP TSG RAN Meeting #110                                                   RP-253124

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.3.1

Source:	CMCC

Title:	Discussion on 6GR spectrum

Document for:		Discussion

Introduction

Spectrum is a scarce non-renewable resource that is the basis of a mobile communication network. How to fully efficiently utilize the spectrum has always been critical and captured the focus of key industry stakeholders.

In last meeting it is approved that 1) 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR, 2) 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

This contribution focus to discusses other spectrum requirements from following aspects, channel bandwidth, DL and UL resource flexible and spectrum range.

Spectrum

6GR network aims to support highly diverse bandwidth allocations considering more and more fragmented and irregular spectrum may be utilized or re-farmed for 6GR. Furthermore, 6GR network aims to possess the flexibility to accommodate extreme variance in demand simultaneously: it needs to efficiently support massive deployment of IoT devices requiring only narrow spectrum, while concurrently delivering the dozens of gigahertz needed for bandwidth-hungry applications. To manage this complexity efficiently, the network's spectrum resource allocation must be inherently agile, capable of adaptation to changing traffic patterns and different operators’ spectrum allocation.

Proposal 1: 6GR aims to support inherent agility in channel bandwidth, allowing systems to adapt to changing traffic patterns across diverse applications. Furthermore, this adaptive capability is crucial for navigating the fragmented, irregular and diverse spectrum allocations held by different operators globally.

Last meeting agreements mainly focus on supporting flexible utilization of spectrum resources for DL and UL over different carriers/bands. Furthermore, within any single operationing band, the capability for flexible and asymmetric utilization of spectrum resources between DL and UL remains significant for future 6GR network. The ongoing evolution toward 5G-A is already demonstrating the importance of enhanced uplink capacity to support growing industrial applications and robust data transmission from user devices. 6G aims to build upon this trajectory to manage an even wider and more flexible traffic patterns. Besides, massive and instantaneous demands for uplink capacity from sources like IoT devices can shift back to heavy downlink requirements. Consequently, flexible DL and UL resource allocation is significant for single carriers/bands. Therefore, 6G aims to support the agility to rebalance DL and UL allocations and ensure optimal spectrum efficiency.

Proposal 2: 6GR aims to support flexible spectrum resources for DL and UL for carriers/bands to ensure optimal spectrum efficiency.

Following lists the current frequency ranges defined for NR. Although FR2-2 has been defined, real-world deployment remains scarce.

WRC-27 will consider the identification of frequency band(s) for the terrestrial component of IMT which are less than 52.6GHz:

– 4 400-4 800 MHz, or parts thereof, in Region 1 and Region 3;

– 7 125-8 400 MHz, or parts thereof, in Region 2 and Region 3;

– 7 125-7 250 MHz and 7 750-8 400 MHz, or parts thereof, in Region 1;

– 14.8-15.35 GHz,

Furthermore, from a technology perspective, the 6G study item only address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). t’s noted frequency ranges beyond 52.6 GHz are not in scope of the work. For 6GR, it’s suggested to focus on the frequency range less than 52.6GHz.

Proposal 3: 6GR aims to support frequency range less than 52.6GHz.

Conclusion

The RAN plenary is suggested to consider following spectrum related requirements for 6GR system, and the following texts in annex are proposed to add in the TR.

Proposal 1: 6GR aims to support inherent agility in channel bandwidth, allowing systems to adapt to changing traffic patterns across diverse applications. Furthermore, this adaptive capability is crucial for navigating the fragmented, irregular and diverse spectrum allocations held by different operators globally.

Proposal 2: 6GR aims to support flexible spectrum resources for DL and UL for carriers/bands to ensure optimal spectrum efficiency.

Proposal 3: 6GR aims to support frequency range less than 52.6GHz.

- References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

Annex

Text proposal to TR

-------------Start of text proposal-----------------

## 5.3	Operational requirements

Editor note: Spectrum, co-existence, topology and other operational requirements

### 5.3.1	Spectrum

Editor note: e.g. spectrum and co-existence related

6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR.

6GR aims to support flexible utilization of spectrum resources for DL and UL over single and multiple different carriers/bands.

6GR aims to support diverse channel bandwidth allocation.

6GR aims to support frequency range less than 52.6GHz.

-------------End of text proposal-----------------