3GPP TSG-RAN WG Meeting #110       	                      RP-253132

Baltimore, USA, December 8-11 2025

Agenda item:	8.2.1.4

Source:	CMCC

Title:	Requirements of AI for 6G RAN

WID/SID:	FS_6G_RAN_Scen_Req

Document for:	Discussion

- Introduction

It can be observed that AI/ML is a crucial component of 6G based on 6G SID [1].

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] (6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

This contribution will focus on the requirements of AI/ML for 6G RAN.

- Discussion

During the study on 5G NR and AI integration, RAN1/RAN2/RAN3 have studied or standardized multiple AI/ML for RAN use cases, such as AI/ML based beam management, CSI prediction, CSI compression, AI/ML for Mobility, AI/ML enabled network energy saving, load balancing, slicing, etc..

However, the existing 5G-A network architecture remains unchanged to support the enhancements on AI/ML, which has limitations on AI data collection and model transfer/delivery.

- Data Collection

In 5G-A, the data collection procedure is triggered by OAM, and cannot be initiated by gNB autonomously which is not flexible especially for the RAN-centric use cases.

For the transmission of collected data, there are some restrictions using the legacy CP/UP solution as follows:

- The existing SRB cannot transmit the large volumes of data for model training (the maximum size of UL/DL RRC signaling is 144kbytes/45kbytes).

- The existing DRB cannot be setup between gNB and UE directly, which is always associated with PDU session terminated in UPF. Therefore, the large volumes of data collected by the UE cannot be transmitted to the gNB directly via DRB.

On the other hand, the data for model training is collected per use case for now, which may result in duplicated data collection and reporting, especially for basic measurement results. For example, L1 beam measurement results can be used for both AI/ML based beam management and AI/ML for mobility use cases.

- Model transfer/delivery

For model transfer/delivery from gNB to UE, the similar challenges to data transmission needs to be resolved:

- The existing SRB cannot transmit the large model (the maximum size of UL/DL RRC signaling is 144kbytes/45kbytes).

- The existing DRB cannot be setup between gNB and UE directly, which is always associated with PDU session terminated in UPF. Therefore, the model trained at gNB cannot be transferred to the UE directly via DRB, especially for use cases with two-sided model.

Observation 1: There are some challenges for AI data collection and AI/ML model transfer/delivery, including the transmission of large volume of data and AI/ML model, as well as duplicated data collection and reporting.

For 6G, the issues on AI data collection and model transfer/delivery mentioned above should be resolved in Day 1. There may be several potential solutions to support the transmission of large volume of data and AI/ML model, such as enhanced CP solution, enhanced UP solution, or introduce a new data plane. From the perspective of flexibility and scalability, we prefer to introduce a RAN data plane to support the large volumes of AI data collection and transmission, especially for model training, and AI/ML model transfer/delivery.

Proposal 1: It is proposed to introduce a RAN data plane to support large volumes of AI data collection/transmission and AI/ML model transfer/delivery.

In 5G-A, a unified Life Cycle Management (LCM) for AI/ML for NR air interface illustrated in the following Figure [2] is defined.

Figure 1: Functional framework for AI/ML for NR Air Interface

For 6GR AI/ML LCM framework, RAN2#131bis meeting [3] agreed to study at least the following functions/procedures which is similar to 5G-A AI/ML LCM:

- UE AI/ML Capabilities Exchange

- Applicable Functionality Reporting

- Inference Configuration and Reporting

- Performance Monitoring Configuration and Reporting

We think 5G-A AI/ML LCM can be taken as starting point for 6GR.

Proposal 2: It is proposed that 6GR AI/ML LCM takes 5G-A LCM as starting point.

For requirements on AI/ML data collection, 5G-A agreed that MNO has full visibility and full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose. For 6GR, the same requirements should be followed for AI/ML data collection.

Proposal 3: For 6GR AI/ML data collection, MNO has full visibility and full control of the standardized data collection transfer process without the need of Service Level Agreement (SLA).

# 3	Conclusion

Here are the proposals for requirements of AI for 6G RAN.

Observation 1: There are some challenges for AI data collection and AI/ML model transfer/delivery, including the transmission of large volume of data and AI/ML model, as well as duplicated data collection and reporting.

Proposal 1: It is proposed to introduce a RAN data plane to support large volumes of AI data collection/transmission and AI/ML model transfer/delivery.

Proposal 2: It is proposed that 6GR AI/ML LCM takes 5G-A LCM as starting point.

Proposal 3: For 6GR AI/ML data collection, MNO has full visibility and full control of the standardized data collection transfer process without the need of Service Level Agreement (SLA).

# 4	Text Proposal to TR 38.914

## 5.4	Requirements of New and Existing Services

#### 5.4.X	AI/ML for RAN

6G AI/ML LCM takes 5G-A LCM as illustrated in the following figure as starting point.

Figure X: Functional framework for AI/ML for NR Air Interface

For 6GR AI/ML data collection, MNO has full visibility and full control of the standardized data collection transfer process without the need of Service Level Agreement (SLA).

# 5	Reference

- RP-251881 New SID: Study on 6G Radio

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R2-2508002 RAN2#131bis Meeting Report