3GPP TSG RAN Meeting #110	RP-253163

# Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.3.3

Source:	Samsung

Title:	Discussion on 6G coverage target

Document for:	Discussion and Decision

1	Introduction

One objective for 6GR design is to allow the reuse of the existing TDD mid-band site grid for 6GR deployments in the 7 GHz band. Another design goal for 6GR is to offer comparable or better coverage when operating in the legacy FR1 NR bands, i.e., below 4 GHz.

In RAN#109, it was agreed that for 3GPP internal evaluation, link budget is used as the evaluation methodology for coverage when applicable. The target for coverage is to be determined by RAN. The exact coverage target value(s) or any additional details were left FFS.

For reuse of the existing TDD mid-band site grid for 6GR deployments in the 7 GHz band, it can be expected that MIMO operation using more TRX and a larger number of AEs with typical/expected MIMO panels for 6GR in the 7 GHz band can mostly compensate for both higher pathloss and penetration losses incurred due to the use of the higher frequency band when compared to today’s NR deployments in the 3.5-3.8 GHz bands when considering the DL. During the 6GR deployment lifecycle, it can later be expected that 6GR 7 GHz MIMO capabilities will increase, i.e., from 128 TRX to more TRX and supporting even higher Tx EIRPs. Therefore, achieving a balanced and matching coverage for NR in the TDD mid-band and 6GR in 7 GHz reusing the same site grid is most critical during initial deployment stages.

The achievable DL/UL cell edge and service data rates in a band are also highly dependent on other factors such as the supported UE power class in the band or the number of DL or UL antenna paths for the UE form factor. Different 6GR UE device classes, e.g., handheld/smartphone, FWA/CPE, or Wearable offer different DL and UL performance in the same network deployment.

Matching or exceeding the UL coverage of NR operating in the 3.5-3.9 GHz bands with 6GR operating in the 7 GHz band from the same site can therefore be seen as much more challenging than DL coverage. This is due to the device limitations, especially in the power limited urban macro deployment case. For example, UL coverage/performance for common channels/signals during initial access can become a critical design aspect for 6GR in the case that 6GR in the 7 GHz band is deployed with standalone initial access.

In this contribution, we provide our views on the initial coverage targets for 6GR.

2	Discussion

Real-life performance of NR in commercial network deployments has been extensively benchmarked. NR coverage has been further evaluated and improved by 3GPP and a number of later release coverage enhancements features were introduced.

For the 6GR coverage targets, the existing NR evaluation methodology/metrics defined by the NR coverage enhancements can be reused as a natural starting point. There are three primary metrics for consideration: MCL/MaxCL, MIL and MPL, including some possible modification of these.

It was already agreed in RAN1#122bis to consider two candidates for the link budget templates.

Agreement (RAN1#122bis) For link budget template, consider the following candidates: - Candidate 1: Reusing the link budget template from TR38.830, - The values of the parameters are TBD. - MCL in row (22bis) is TBD. - FFS: whether/how/why to update. - (…) - Candidate 2: Template as Table 7.10.1-1 from TR38.913. - FFS: whether/how/why to update. - (…)

Candidate 2 is based on MCL methodology and candidate 1 is based on either the MIL or the MPL methodology. It is well understood that each of these methodologies, or MCL, MIL, or MPL metrics has its own pros and cons for purpose of meaningful 6GR coverage evaluation and the comparison with NR.

MCL is a simple and straightforward metric excluding complex factors like hardware impairments, allowing a quick analysis without the need for alignment across vendors’ implementation and almost independent of deployment assumptions. It is already clear that the 6GR will need to provide a baseline MCL ~ 144 dB target comparable to the MCL for 5G NR. Unfortunately, due to the independence of MCL methodology with respect to almost all deployment and band specific assumptions, these MCL numbers do not allow to meaningfully compare the achievable coverage for 6GR in the 7 GHz band and NR in the 3.4-3.9 GHz bands. The MCL based methodology can also be insufficient for other reasons. For example, MCL as defined in Rel-16 only contains the transmit power, receiver sensitivity and BS antenna gains. Only a part of the BS antenna gains is accounted for. Ideally, the BS antenna gains should account for all the antenna elements and all the per element antenna gains. It also needs to be considered that for later ITU evaluations of the 6G RAT, coverage analysis based on the MCL methodology (Candidate 2) is insufficient.

MIL has been defined based on the MCL with some additional considerations given to implementation impairments such as the Tx loss and Rx loss. Components of BS antenna gain and UE antenna gains are also considered in the MIL. The difference between MCL and MIL is, however, not often meaningful in practice. The assumed Tx losses or Rx losses can much depend on implementation, similar to many vendor-specific antenna assumptions, and have resulted in wide variability across companies results, which has made it difficult to agree on observations. We note however, that a MIL based methodology is still meaningful under specific assumptions such as when the coverage analysis is done to identify coverage gaps for different channels/signals in the same band.

MPL is defined based on the MIL with additional considerations given to budget for shadowing, BS selection/macro-diversity gain, penetration margin, etc. For example, shadowing and the penetration losses can vary according to different deployment scenarios. MPL contains deployment- or band-specific assumptions. MPL methodology allows to assume the same site grid for NR TDD 3.4-3.9 GHz bands and 6GR in the 7 GHz band. MPL is therefore the only meaningful metric which can provide an actual link budget comparison for coverage comparison across bands, i.e., with respect to coverage analysis for 6GR in the 7 GHz and NR in the 3.4-3.9 GHz bands reusing the same site grid.

However, the drawback with MPL based methodology, or the MPL metric, based on Candidate 1 is that alignment across companies on several key parameters including the shadow fading margin, antenna gains, penetration losses and other implementation losses are prerequisite. Furthermore, link/system level simulation-based metrics are required as input which first assumes an alignment between companies on basic 6GR operational scenarios such as 6GR channel/signal formats, scheduling or HARQ principles (where different from NR).

It is therefore unavoidable to set the initial coverage targets for 6GR using MCL while later/specific MPL-based coverage targets can be set following more progress on 6GR channel/signal design.

Note that in RAN1#123 it was agreed that link budget candidates 1 and 2 are used as starting point to calculate the metrics for Urban Macro and Suburban Macro (both for O2I and Outdoor). Email discussion is planned prior to RAN1#124 for collecting company inputs on the values for link budget template.

Proposal #1:

- MCL is used to set the initial coverage target requirements for 6GR.

- MPL is used to set later/specific coverage target requirements across bands for 6GR in 7 GHz and NR 3.4-3.9 GHz in a same/shared site deployment.

For a basic MBB service characterized by a DL data rate of 1 Mbps and an UL data rate of 30 kbps for stationary users, 38.913 sets the MCL =143dB as coverage target for 5G NR. The examples of Table 1 (DL) and Table 2 (UL) show the link budget tables for 20 MHz channel BW, SCS=15kHz and 1 PRB PDSCH or PUSCH. It is assumed that 2-Rx DL is supported by the UE.

Table 1: DL link budget (using Table 7.10.1-1 from TR38.913)

| Item | Value |
| --- | --- |
| Transmitter |  |
| (1) Tx power (dBm) | 25.8 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 9 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 180000 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log (5) (dBm) | -112.4 |
| (7) Required SINR (dB) | -4.8 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -117.2 |
| (9) MaxCL = (1) - (8) (dB) | 143 |

Table 2: UL link budget (using Table 7.10.1-1 from TR38.913)

| Item | Value |
| --- | --- |
| Transmitter |  |
| (1) Tx power (dBm) | 23 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 5 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 180000 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log (5) (dBm) | -116.4 |
| (7) Required SINR (dB) | -3.6 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -120.0 |
| (9) MaxCL = (1) - (8) (dB) | 143 |

While basic MBB data rates are not particularly ambitious for setting the initial 6GR coverage targets, it is clear that more meaningful target values relative to 5G mid-band coverage can only be set later using MPL, and after first agreeing on the assumed 6GR target eMBB cell edge and service data rates.

Proposal 2: 6GR supports a baseline MCL=143 dB coverage target for FR1 urban and suburban macro deployments with basic eMBB data rates.

Existing NR DL demodulation requirements have been set assuming 4-Rx DL in a number of NR FR1 bands and assuming 2-Rx DL in some other FR1 bands. A better DL demodulation performance may be achieved by the FWA/CPE device class. Existing NR REDCAP devices can support 1-Rx DL in 2-Rx DL bands. Expected wearable devices or 6GR LPWA/IoT devices may not be able to support 2-Rx or 4-Rx DL in a number of bands due to their reduced form factors. Other considerations include more limited heat dissipation/thermal budget management and higher RF interference across the Tx or Rx paths with reduced device form factors. In the UL, some of these device classes may not support the default PC3 (23 dBm) in the UL.

Therefore, it is important for 6GR to support a modest number of repetitions from the initial Rel-21 to overcome inherent DL or UL link penalties anticipated for such devices. It can be expected that an MCL compensation up to around 10 dB may be needed and to guarantee sufficient margin even in some more challenging deployments. This is comparable to LTE CE Mode A supporting a moderate number of repetitions. A margin for more robust DL/UL link performance would also be required for NTN. Using 1-Rx DL instead of 2-Rx DL may in a number of cases result in 3-5 dB degradation. For link mismatches between the NR TDD mid-band and 6GR in 7 GHz using the same/shared site grid, for example, 2-4 repetitions would then be necessary to achieve the same link budget for the data rate.

Proposal 3: 6GR supports an extended MCL coverage target of 10 dB better (or MCL=153 dB)

3	Conclusions and Proposals

In this contribution, we made the following proposals:

Proposal #1:

- MCL is used to set the initial coverage target requirements for 6GR.

- MPL is used to set later/specific coverage target requirements across bands for 6GR in 7 GHz and NR 3.4-3.9 GHz in a same/shared site deployment.

Proposal 2: 6GR supports a baseline MCL=143 dB coverage target for FR1 urban and suburban macro deployments with basic eMBB data rates.

Proposal 3: 6GR supports an extended MCL coverage target of 10 dB better (or MCL=153 dB).