3GPP TSG RAN Meeting #110					RP-253174

Baltimore, MD, December 8 – 11, 2025

Agenda Item:	8.2.1.1

Source:	Ericsson, T-Mobile USA, Nokia, AT&T, MediaTek

Title:	Internal 3GPP requirements on coverage

Document for:	Approval

# 1	Introduction

Proposal 4: For 3GPP internal study, link budget is used as the evaluation methodology for coverage when applicable

Proposal 5: For 3GPP internal study, the target for coverage is to be determined by RAN.

-	FFS: Exact coverage target value(s).

-	FFS: Additional details considering control/data channel

In this contribution, we propose an internal 3GPP quantitative requirement on overall coverage for 6GR using a similar methodology as was used for NR in TR 38.913 [1]. This builds on the following agreement from RAN#109 [2]:

In addition, we propose a qualitative requirement corresponding to the following objective in the SID [3]:

  - Re-use of existing 5G mid-band (~3.5 GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

# 2	Discussion

During RAN#107 the 6G RAN requirement SID was updated in [3]. Regarding coverage, the following related statements were captured:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage

- Re-use of existing 5G mid-band (~3.5 GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

Coverage is essential and it is important that 6GR can be introduced and perform well using existing site grids, without the need for densification. This objective can be divided into two parts:

- When 6GR is introduced in existing bands, using existing sites and antennas, the coverage of 6GR should be at least as good as that of NR.

- When 6GR is introduced on new frequency bands, higher than those used for NR on the site grid in question, larger  antennas (i.e., more antenna elements) can be used to compensate for the increased path loss. In addition other beneficial approaches exist such as deployment of lower band carriers enabling load balancing, UL/DL decoupling, etc., as well as longer duration transmissions, e.g., repetition over multiple slots, HARQ retransmissions, TBoMs, etc. The choice of which technique or combination of techniques to use is a deployment decision. It is clear that the specification should support a wide variety of tools to meet operators’ needs in different use cases and scenarios.

## 2.1 	Quantitative requirement for overall coverage

Regarding the first part of the coverage objective (6GR in existing bands), a similar objective existed when NR was introduced and compared to LTE. The resulting coverage requirements for NR are captured in TR 38.913 Clause 7.10 [1] and are based on maximum coupling loss (MaxCL). MaxCL is the limit value of the coupling loss at which a service can be delivered and therefore defines the coverage of the service. MaxCL is independent of both the carrier frequency the BS/UE antenna configurations, and is defined in the UL and DL as:

- UL MaxCL = UL Max Tx power - eNB Sensitivity

- DL MaxCL = DL Max Tx power - UE Sensitivity

MaxCL is evaluated via link budget analysis supported by link level simulations. Table 7.10.1-1 in [1] provides a template for calculation of MaxCL.

MaxCL is a suitable and simpler metric to use compared to other alternatives such as maximum isotropic loss (MIL) and maximum path loss (MPL). The latter two metrics depend on numerous additional parameters (e.g. antenna configuration, antenna gain correction factors, etc.). An overall coverage requirement should not depend on a specific antenna configuration, since the standard will support many different configurations and the choice depends on the deployment scenario.

The text proposal in Section 3 provides a quantitative requirement for overall coverage based on MaxCL similar to [1].

## 2.2 	Qualitative requirement for deployment of 6GR around 7 GHz on same site grid as NR midband (~3.5 GHz)

For the second part of the above coverage objective (6GR in new, higher frequency bands), as we stated above, achieving comparable coverage to NR mid band (~3.5 GHz) on the same site grid can be achieved with various techniques to compensate for higher path loss around 7 GHz such as larger antennas, multi-carrier deployment, UL/DL decoupling, multi-slot transmission, etc. It is clear that the 6GR specification should support all of these tools; however, the choice of which tool or combination of tools to use is a deployment decision. For this reason, it is not appropriate to introduce a quantitative RAN-level requirement on 6GR deployment around 7 GHz compared to NR mid-band deployment. To address the SID objective we instead propose a qualitative requirement that does not restrict deployment freedom on which tools are used to achieve comparable coverage. How such a requirement is met can be evaluated quantitatively in RAN1 with link and system level studies according to the scenarios, services, and air interface features under study.

The text proposal in Section 3 provides a qualitative RAN-level requirement on coverage for deployment of 6GR around 7 GHz on the same site grid as NR midband (~3.5 GHz).

# 3	Text proposal for 38.914

Based on the discussion above, we propose the following text proposal for TR 38.914 [4]. The TP introduces a new section 5.1.20 entitled “Coverage” under Section 5.1 entitled “Key performance indicators.” This is similar to the structure of TR 38.913. For the quantitative requirement on overall coverage, we propose to reuse the link budget table and calculation assumptions from TR 38.913 for NR. Those assumptions were applicable to 2-Rx antennas. Here we extend the requirement to also include devices with 1-Rx antenna since it was agreed in RAN#109 that 6G shall support massive IoT services for the case of a single receive antenna. We also include an extended coverage requirement of +10 dB.

*** Start Text Proposal for TR 38.914 ***

Note to editor: This is a new section under 5.1 “Key Performance Indicators”

## 5.1.20	Coverage

### 5.1.20.1	Overall coverage – Maximum Coupling Loss (MaxCL)

The coupling loss is defined as the total long-term channel loss over the link between the BS and UE. The maximum coupling loss (MaxCL) is the limit value of the coupling loss at which the service can be delivered and therefore defines the coverage of the service. MaxCL is independent of the carrier frequency. It is defined in the UL and DL as:

- UL MaxCL = UL Max Tx power - BS Sensitivity

- DL MaxCL = DL Max Tx power - UE Sensitivity

MaxCL is evaluated via link budget analysis, supported by link level simulations. The  MaxCL calculation template is given in following table:

Table X: MaxCL calculation template

| Item | Value |
| --- | --- |
| Transmitter |  |
| (1) Tx power (dBm) |  |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) |  |
| (3) Receiver noise figure (dB) |  |
| (4) Interference margin (dB) |  |
| (5) Occupied channel bandwidth (Hz) |  |
| (6) Effective noise power = (2) + (3) + (4) + 10 log(5) (dBm) |  |
| (7) Required SINR (dB) |  |
| (8) Receiver sensitivity = (6) + (7) (dBm) |  |
| (9) MaxCL = (1) - (8) (dB) |  |

The following assumptions are used.

Table X: MaxCL calculation assumptions

| UE Tx power | 23dBm |
| --- | --- |
| DL Tx power | 46dBm |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| Interference margin | 0dB |

For a basic MBB service characterized by a downlink data rate of 1Mbps and an uplink data rate of 30kbps for stationary users, the target on maximum coupling loss is MaxCL = 143dB. At this maximum coupling loss, relevant downlink and uplink control channels shall perform adequately. These data rates correspond to devices with 2-Rx antennas. For devices with 1-Rx antenna, the target DL data rate shall be lower due to the extended duration transmissions  (e.g., retransmissions/repetitions/more symbols) required to compensate for the single Rx antenna.

6GR shall also support extended coverage which can be achieved by extending the duration of the transmission.  For extended coverage, the target on maximum coupling loss is MaxCL = 153 dB. This requirement applies under the assumption of both 1-Rx and 2-Rx antennas. The following table lists the data rate targets for both normal and extended coverage. These are instantaneous (per slot) data rates, not accounting for HD-FDD or TDD operation.

Table X: Target data rates for normal and extended coverage under the assumption of different number of UE Rx antennas

|  | Normal coverage (MaxCL = 143 dB) |  | Extended coverage (MaxCL = 153 dB) |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| 2 UE Rx antennas | 1 Mbps | 30 kbps | 100 kbps* | 3 kbps* |
| 1 UE Rx antenna | 250 kbps** |  | 25 kbps*** |  |

* Assuming 10 dB for extended coverage (e.g. 10 retransmissions/repetitions)

** Assuming 3~6 dB to compensate for the single UE Rx antenna (e.g. up to 4 retransmissions/repetitions)

*** Assuming 13~16 dB to handle single antenna and extend coverage (e.g. up to 40 retransmissions/repetitions)

### 5.1.20.2	Coverage for deployment of 6GR around 7 GHz on same site grid as NR midband (~3.5 GHz)

For 6G deployment at least around 7 GHz reusing the same site grid as NR midband (~3.5 GHz), the 6GR design targets comparable coverage as NR midband, e.g., through one or more of the following: increased number of antenna elements, multi-carrier deployment of 6GR including lower frequency bands, longer duration transmissions etc.

*** End Text Proposal ***

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914 [4].

# References

3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies,” v.14.3.0, June 2017.

RP-252883, “Moderator's summary for RAN led 6G SI: Key performance indicators,” Moderator (CMCC), RAN#109, September 2025.

RP-250810, “Revised SID: Study on 6G Scenarios and requirements,” RAN#107, March 2025.

3GPP 38.914, “Study on 6G Scenarios and requirements”, v0.2.0, October 2025.