3GPP TSG-RAN Meeting #110	RP-253206

# Baltimore, USA, 8th Dec 2025 – 11th Dec 2025

Agenda item:	8.2.1.3.3

Source: 	Viasat, Novamint, Airbus, Sateliot, Gatehouse Satcom, ESA, EchoStar, Thales, Iridium, Skylo, Terrestar

Title: 	NTN specific proposals related to co-existence between NB-IoT NTN and 6GR NTN

Document Type:	Discussion

Document for:	Approval

## 1.	Introduction

This paper provides our views on the co-existence requirements related to NTN as part of 6G Radio.

## 2.	Discussion

NTN support is going to be an integral and native component of the first release of 6G specification.

NTN has been specified in 3GPP since Release 17. The following access technologies are supported within 3GPP:

- NR NTN for FR1 and FR2 NTN specific bands

- LTE for eMTC based IoT NTN for sub-3 GHz FR1 NTN bands

- NB-IoT NTN for sub-3 GHz FR1 NTN bands

With respect to NB-IoT NTN, both standalone operation as well as in-band operation in NR NTN via the same satellite access node (SAN) have been specified. Guard band operation is left to implementation based on channel size and availability of adequate guard band.

Given the existing assets of the NTN operators, it is quite likely that IoT NTN and 6GR NTN may operate via satellites in different orbits.  6GR NTN must facilitate in-band and guard band operation of NB-IoT NTN via a satellite that is in a different orbit than the 6GR NTN (e.g., NB-IoT NTN deployed via GSO, while 6GR NTN deployed via NGSO).

Finally, for IoT NTN, single channel raster of only 100 kHz is supported. This results in very few PRB locations at which an NB-IoT channel can operate in-band within an NR NTN channel such that it does not overlap with the SSB. For example, NB-IoT channel can be deployed in-band at a single location within a 5 MHz channel that does not overlap with SSB transmission.

## 3. Proposals

Proposal 1: The 6G RAN shall support configuration and co-existence of in-band and guard band NB-IoT NTN channel(s) in a 6GR NTN channel.

Proposal 2: The 6G RAN shall support co-existences of in-band NB-IoT NTN in one orbit (e.g., GSO) within a 6GR NTN channel deployed via another orbit (e.g., NGSO).

Proposal 3: The 6G RAN shall support frequency shift and associated capability signalling to support co-existence between NB-IoT and 6GR.

Proposal 4: To facilitate co-existence, NB-IoT channel raster values shall be aligned with 6GR channel raster to allow flexibility in placement of an NB-IoT channel within a 6GR channel while still maintaining orthogonality across the sub-carriers of NB-IoT and 6GR channels.

A potential incorporation of above proposals in Clause 5.3 is depicted in the Annex of this document.

## 4.		References

[1]	TR 38.914, v0.2.0, Study on 6G Scenarios and Requirements

======================

## Annex	Potential Updates to TR 38.914 [1]

Based on the above discussion we propose the following text proposal to clause 5.3.

====================Start of the changes==================

## 5.3	Operational requirements

Editor note: Spectrum, co-existence, topology and other operational requirements

### 5.3.1	Spectrum

Editor note: e.g. spectrum and co-existence related

6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR.

6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

### 5.3.2	Diverse device types

### 5.3.3	RAN sharing

### 5.3.4	Ease of operation and Self Organization

### 5.3.5	Network Resilience

### 5.3.6	[Service awareness in RAN]

### 5.3.7 Multi-RAT co-existence

The 6G RAN shall support configuration and co-existence of in-band and guard band NB-IoT NTN channel(s) in a 6GR NTN channel.

The 6G RAN shall support co-existences of in-band NB-IoT NTN in one orbit (e.g., GSO) within a 6GR NTN channel deployed via another orbit (e.g., NGSO).

The 6G RAN shall support frequency shift and associated capability signalling to support co-existence between NB-IoT and 6GR.

### To facilitate co-existence, NB-IoT channel raster values shall be aligned with 6GR channel raster to allow flexibility in placement of an NB-IoT channel within a 6GR channel while still maintaining orthogonality across the sub-carriers of NB-IoT and 6GR channels.

==================End of the changes========================