3GPP TSG RAN Meeting #110						RP-253223

Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.4

Source: 	Qualcomm Incorporated

Title: 	Requirements for NTN

Document for:	Discussion

# Introduction

In this contribution we present our views on the support of NTN under 6GR.

# General principles

NR NTN was first specified in Rel-17 in 3GPP, supporting FR1 frequencies. Several features have been added in Rel-18+, including the support of higher frequency bands with VSAT.

At the time of writing this contribution there is no commercial deployment of NR NTN, although several constellations have been announced and trials / demonstrations have been conducted. Given the deployment timeline of NR NTN, it is quite unlikely that operators will deploy 6GR NTN based on Rel-21.

On the other hand, there seems to be consensus in the industry that 6GR NTN should be supported, providing an update cycle to satellite systems once NR NTN is mature. In order to ensure that the 6GR system properly supports NTN, it would be beneficial to introduce the basic functionality for NTN in day-1 even though some aspects such as band definitions may be postponed to later releases. This would ensure that 6GR NTN would work without the need of resorting to “patches” in later releases while giving NR NTN the time to achieve commercial success.

Proposal 1: 6GR to introduce basic functionality to support NTN in day-1 (“NTN-ready”)

- Some aspects such as band definitions can be postponed to later releases.

One of the key (if not the main) advantages of developing specifications for satellite communications in 3GPP is the economies of scale: the additional functionality needed to be implemented on top of the terrestrial cellular features is minimal (with respect to implementing a separate SatCom standard), which reduces the bar to commercialize this feature, especially for the case of handsets. While we acknowledge that specific optimizations for NTN may be beneficial, RAN should assess the cost-benefit tradeoff of these optimizations and, in general, maximize commonality between 6GR TN and 6GR NTN. Similarly, we do not think RAN should specify a migration path for NB-IoT, specifically regarding support for 200kHz channel bandwidth.

Proposal 2: Try to maximize commonality between 6GR TN and 6GR NTN

- 6G NTN should provide an evolution / migration path from NR NTN.

- Do not specify a radio evolution path for NB-IoT NTN in day 1

It is also expected that the first wave of devices that would support 6GR would be handsets, and these may be the device types that may first benefit from the introduction of the NTN features under 6GR. Although we acknowledge the interest in NTN support in higher bands, we propose prioritizing support of handsets in Day 1.

Proposal 3:  Prioritize handsets (not VSAT) in the first release

# Essential features

As part of the migration strategy from NR to 6GR NTN, one would expect some degree of “soft migration” similar to the one envisioned for terrestrial networks. Therefore, it would be essential to ensure that multi-RAT spectrum sharing (MRSS) between 6GR NTN and NR NTN is possible. This would imply that basic building blocks of NR NTN (e.g. assumptions on time-frequency synchronization) would need to be replicated in 6GR.

Proposal 4: 6GR NTN should support MRSS with NR NTN.

Since Rel-17 to Rel-19, NR NTN operation requires precompensation based on UE location. This assumption is being relaxed in Rel-20 in the RAN1-led study on GNSS resiliency. In our view (and potentially reusing the conclusions from the Rel-20 5G-A study), 6GR should consider GNSS-less/resilient operation from day 1.

Proposal 5: 6GR NTN should support GNSS-less/resilient operation:

- UEs with GNSS location information and without GNSS location information can operate in an NTN cell.

- Further study whether the lack of GNSS location information can be due to absence of GNSS support in the device, temporary GNSS unavailability, or both.

One of the features introduced late in the 5G-A lifecycle is the so-called “beam hopping”, where the SSB periodicity was adapted to cope with the on-board limitation on the number of RF chains that can be simultaneously active. While we acknowledge that this limitation will likely remain in the 6G era, we think RAN should carefully reflect on the acceptable parameters for the number of RF chains. For instance, it would seem extreme to study scenarios with a ~1% ratio of RF chains to beam footprints when considering potential deployments after 2030.

Proposal 6: 6GR NTN should support “beam hopping” with more capable satellites than those considered in Rel-19.

# Potential enhancements (not necessarily day-1)

In this section, we list two of the potential enhancements for 6GR that have been discussed in previous RAN TSG and WG meetings and that may result in new topics for further study to be considered in the RAN WGs.

LEO PNT / positioning:

In the last few years there has been increased worldwide interest in enabling additional sources of location / timing in case GNSS is not available (due to e.g. spoofing or jamming) or to complement GNSS to achieve better accuracy. In our opinion, and also assuming 3GPP-based LEO constellations will be widespread soon, RAN should study how to make use of this new satellite infrastructure to provide positioning services (and potentially timing services).

Proposal 7: Study positioning / PNT based on LEO constellations in 6GR

Support of TDD:

In Rel-19, 3GPP has specified B249 in IoT NTN. This MSS band has TDD duplexing, and it was specified with very strict constraints (90ms frame structure, with 8ms DL and 8ms UL duty cycles) due to compatibility with the legacy system deployed in this band. For 6GR, RAN should consider how to efficiently operate an NTN TDD system without these constraints. There are several issues caused by the large distances involved between the UE and the SAN, as well as cross-link interference (CLI), that the RAN WGs may need to consider when conducting this study. UE to UE CLI may also happen in some corner cases (e.g. UEs served by different satellites with large pre-compensation difference), but in our view the system should work without explicitly solving this issue.

Proposal 8: Study support of TDD in 6GR NTN for LEO / vLEO, taking into account the following aspects:

- Target spectrum for NTN TDD

- Aspects related to TDD frame structure / guard interval, including the following:

  - Half duplex constraints at both the UE and satellite.

  - Varying propagation delay across the satellite footprint.

  - Satellite-to-satellite CLI.

# Summary

In this contribution we proposed our views on NTN support in 6GR. We made the following proposals:

Proposal 1: 6GR to introduce basic functionality to support NTN in day-1 (“NTN-ready”)

- Some aspects such as band definitions can be postponed to later releases.

Proposal 2: Try to maximize commonality between 6GR TN and 6GR NTN

- 6G NTN should provide an evolution / migration path from NR NTN.

- Do not specify a radio evolution path for NB-IoT NTN in day 1

Proposal 3:  Prioritize handsets (not VSAT) in the first release

Proposal 4: 6GR NTN should support MRSS with NR NTN.

Proposal 5: 6GR NTN should support GNSS-less/resilient operation:

- UEs with GNSS location information and without GNSS location information can operate in an NTN cell.

- Further study whether the lack of GNSS location information can be due to absence of GNSS support in the device, temporary GNSS unavailability, or both.

Proposal 6: 6GR NTN should support “beam hopping” with more capable satellites than those considered in Rel-19.

Proposal 7: Study positioning / PNT based on LEO constellations in 6GR

Proposal 8: Study support of TDD in 6GR NTN, taking into account the following aspects:

- Target spectrum for NTN TDD

- Aspects related to TDD frame structure / guard interval, including the following:

  - Half duplex constraints at both the UE and satellite.

  - Varying propagation delay across the satellite footprint.

  - Satellite-to-satellite CLI.