3GPP TSG RAN Meeting #110						RP-253223

Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.4

Source: 	Qualcomm Incorporated

Title: 	Requirements of new and existing services

Document for:	Discussion

# Introduction

In this contribution we present our views on the requirements of several services.

# Massive communication (IoT)

RAN#109 has agreed the following TP for TR38.914:

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

In addition, RAN#109 had the following agreement for lowest-tier device:

For 6G Massive Communication (IoT), the minimum number of UE receive/transmit antenna is 1 for lowest-tier device - Reflect this agreement in the device type section of the TR

In this paper, we discuss the smallest maximum bandwidth, half-duplex, and coverage along with minimum data rate for the lowest-tier device type and propose a TP on massive communication (IoT) for TR38.914.

## Smallest maximum bandwidth

Agreement (RAN1#122bis)

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

FFS: whether RF and BB UE BW are same or different

Agreement (RAN1#123)

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - Overall device complexity

  - Overall system performance impact

  - Energy efficiency for both BS and UE

  - Aim at a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority

In 6G SI overview discussion, RAN1 has the following agreements on smallest maximum bandwidth for low-tier device types:

We compare different options in terms of the considerations based on RAN1 agreement. Note that it is assumed for FR1 FDD with SCS of 15kHz.

|  | Device complexity (e.g., RF/BB complexity in terms of filters/ADCs/FFT sizes/memory) | System performance impact (e.g., coverage/initial access reliability, access latency, throughput) | Energy efficiency (e.g., both UE and BS across idle/low/medium loads) | Common signals/channels design (e.g., unified/scalable design for idle mode and initial access and maintain flexibility for diverse device types) |
| --- | --- | --- | --- | --- |
| Opt1 3MHz | Lowest cost/power (small RF passband with simpler UL chain, small FFT/BB and silicon area) | Poor fragmentation if initial access optimized for 3 MHz only; 3MHz-SSB/SIB1 penalize higher-tier devices with worse coverage and longer access delay; Constrained throughput for higher-tier devices | UE: lower sampling and narrower filtering reduce dynamic power; BS: need to use longer SI and DTX/DRX periodicity for power saving | Not scalable with 5/10/20MHz if design is optimized for 3 MHz Sub-optimal design for higher-tier devices |
| Opt2 5MHz | Low cost/power (meaningful RF/BB simplifications) | Better fragmentation for initial access with larger CBW than 3MHz while keeping MBB requirements high priority Good coverage without excessive time-domain consumption | UE: low sampling and narrow filtering reduce dynamic power BS: efficient DTX/DRX power saving while enough bandwidth to avoid prolonged initial access latency | Best candidate for common signals/channel >=5MHz; minimum scalability intact and good compatibility with NR baseline for common channels and evaluations |
| Opt3 10MHz | Higher cost/power than 5MHz However, less suitable for the lowest tier | Better coverage and decoding margins and smaller access latency than 3/5 MHz Better MBB throughput but not necessary for low-tier devices | UE: higher power than 5MHz; BS: slightly better energy/bit but idle/low-load overhead still requires DTX/DRX | Viable common design but no meaningful benefit for IoT use cases |
| Opt4 20MHz | Higher cost/power than 5MHz and 10MHz However, not compatible with the goal of IoT cost/power reduction | Better coverage and decoding margins and smaller access latency than 10 MHz Better MBB throughput However, mismatched to low-tier devices and likely to fragment the market if mandated | UE: higher power than 10MHz; BS: requires optimization for DTX/DRX and SI periodicity, only suitable for traffic profiles consistently benefit from higher BW (not for massive IoT) | Easy common design with MBB but over-provisioned for low-tier devices |

Based on the analysis above, we propose to support a smallest maximum supported bandwidth (RF and BB) of 5MHz.

Proposal 2.1: The smallest maximum supported RF and BB UE BW at least one low-tier device type is 5 MHz for FR1 FDD with 15 kHz SCS.

## Half duplex

For LTE MTC/NB-IoT, HD-FDD Type B is supported with slow TX/RX switching (e.g., 1ms retuning gap in FDD), which is implementable with a single LO. The scheduling gaps and HARQ timing are adapted for super low complexity.

For NR (e)RedCap, HD-FDD Type A is supported as an option alongside FD-FDD and TDD with smaller TX/RX switching time (e.g., 13 µs retuning gap in FDD/TDD). The (e)RedCap has 1T2R or 1T1R configurations and higher complexity than MTC but fewer antennas and relaxed RRM than eMBB. The scheduling gap and HARQ timing are designed for higher throughput than MTC while reducing complexity versus full-duplex transmission/reception.

For 6G massive IoT, HD-FDD is required because it eliminates the need for a duplexer. This reduction in RF front-end and baseband requirements benefits low-tier devices by reducing cost and improving energy efficiency through decreased instantaneous power consumption.

Proposal 2.2: Support half-duplex FDD operation for 6G low-tier device type.

- Further study the half-duplex type and required retuning gap/scheduling gap.

## Coverage and minimum uplink data rate

One of the key selling points of eMTC / NB-IoT in Rel-13 was the adoption of large coverage enhancements with respect to baseline LTE. NB-IoT was designed to operate with an MCL of 164dB (~20dB higher than LTE). This extreme coverage enhancement of eMTC / NB-IoT, however, results in extreme reduction in data rates to the point of not being able to run any useful service over them. For instance, the minimum data rate allowed by NB-IoT specifications is around 10bps. The RAN4 reference channel for SNR = -9.8dB results in 144bps. In our view, it is not necessary to achieve these levels of coverage for 6GR.

The 6G target of coverage enhancement should be based on the minimum UL data rate over which we can run a service. We consider the target of 1kbps to offer a reasonable level of coverage enhancement while maintaining enough throughput to run a service over it. For this minimum data rate, based on simulations for 1Rx / 1Tx devices, the MCL is ~153dB. In the figure below, we observe that some physical channels in NR fall short of this requirement (PBCH / PDCCH / PRACH fall short of this coverage level)

Fig. 1 DL/UL performance of 5MHz (e)RedCap UEs

In our view, the coverage on UL 1kbps at 153 dB MCL ensures robust connectivity for critical uplink signaling and IoT-type traffic in deep coverage condition.

Proposal 2.3: Consider a target coverage corresponding to a minimum UL data rate of ~1kbps (target MCL~153dB) for 6G low-tier device type.

## Text proposal for massive communication (TR 38.914)

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. - The minimum number of UE receive/transmit antenna is 1 for lowest-tier device. - The coverage extension is ~[153]dB MCL with a minimum uplink data rate of [1]kbps. - Half-duplex FDD operation is supported for lowest-tier device. - The smallest maximum channel bandwidth is 5MHz (RF and baseband, for 15kHz SCS) in both uplink and downlink for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

# UAV

During 4G and 5G specifications development, 3GPP has defined several features for supporting UAV connectivity. Several of these techniques are essential for operating UAV (e.g. identification, authorization and compliance with regulatory requirements), while others have been proven to facilitate network operation in the presence of UAVs (flight path information, altitude reporting, mobility enhancements). We propose to port these

Proposal 3.1: 6GR to support the following basic functionality for support of UAV (port functionality from NR):

- Identification / authorization of aerial UA.

- Support of minimum necessary features to comply with regulatory requirements (NTZ, emissions)

- Signaling of flight path information and altitude reporting.

- Altitude dependent configurations / mobility procedures.

## Text proposal for UAV (TR 38.914)

### 5.4.x Uncrewed Aerial Vehicles (UAV) ### The 6GR and 6G RAN architecture shall support the following minimum requirements for UAV: - Support of identification and authorization of Aerial UE. - Support of signalling of flight path information. - Support of altitude reporting. - Support of altitude-dependent configurations and mobility procedures. - Support of minimum necessary features to comply with regulatory requirements (e.g. no-transmit zone, emission requirements). ### NOTE: Transmission of BRID and DAA messages can be enabled through LTE and NR PC5 interface.

# Sidelink

During the several releases of 5G, sidelink has been developed with different functionality (V2X, Public Safety, A2X, unlicensed, etc.). At the time of writing this contribution, the sidelink ecosystem is still developing, and we do not see the need to have a refresh of this technology by Rel-21 timeline.

Proposal 4.1: Do not consider sidelink in Day-1 6GR.

- If necessary, consider mechanisms to use NR PC5 when under coverage of 6GR Uu.

# Other services

The following table summarizes our view on several other services:

Proposal 5.1: On other services in 6GR:

| Service | View |
| --- | --- |
| Fixed wireless access | To be considered in 6G study |
| Dedicated broadcast | No need to consider in the 6G study |
| Broadcast / multicast | No need to consider in the 6G study |
| TSN | Support in day 1 |
| HST | Support in day 1 |
| Unlicensed | No support of LBT |
| HRLLC | Supported in day 1, avoid optimizations. |

# Summary

In this contribution we presented our views on requirements for several services. We made the following proposals:

On massive communications:

Proposal 2.1: The smallest maximum supported RF and BB UE BW at least one low-tier device type is 5 MHz for FR1 FDD with 15 kHz SCS.

Proposal 2.2: Support half-duplex FDD operation for 6G low-tier device type.

- Further study the half-duplex type and required retuning gap/scheduling gap.

Proposal 2.3: Consider a target coverage corresponding to a minimum UL data rate of ~1kbps (target MCL~153dB) for 6G low-tier device type.

On UAV

Proposal 3.1: 6GR to support the following basic functionality for support of UAV (port functionality from NR):

- Identification / authorization of aerial UA.

- Support of minimum necessary features to comply with regulatory requirements (NTZ, emissions)

- Signaling of flight path information and altitude reporting.

- Altitude dependent configurations / mobility procedures.

On Sidelink

Proposal 4.1: Do not consider sidelink in Day-1 6GR.

- If necessary, consider mechanisms to use NR PC5 when under coverage of 6GR Uu.

Proposal 5.1: On other services in 6GR:

| Service | View |
| --- | --- |
| Fixed wireless access | To be considered in 6G study |
| Dedicated broadcast | No need to consider in the 6G study |
| Broadcast / multicast | No need to consider in the 6G study |
| TSN | Support in day 1 |
| HST | Support in day 1 |
| Unlicensed | No support of LBT |
| HRLLC | Supported in day 1, avoid optimizations. |