3GPP TSG-RAN Meeting #110	RP-253233

Baltimore, US, December 08-11, 2025

Agenda item	: 8.2.1.0

Source	: Jio Platforms Limited

Title	: 6G Intra-frequency Het-Net deployment scenarios

Document for	: Discussion

# Introduction

6G deployment scenarios were discussed in RAN#108 and RAN#109, while agreements were captured in TR 38.914 [1]. To our opinion there are missing key aspects of 6G deployment scenarios in the TR. This contribution provides Jio Platforms view on 6G intra-frequency Het-Net deployment scenarios.

# Discussion

- Most operator networks nowadays are densely deployed with scarce spectrum availability.

- The propagation as well as penetration losses in spectrum bands like 2300 MHz (n40) and 3500 MHz (n78) are significantly higher than voice centric bands like 900 MHz or 850 MHz. As a result, Macro cell deployment alone is not sufficient to ensure ubiquitous coverage in the desired geographical area.

- For example, Jio’s radio network consists of close to 300 K Macro eNodeB/gNodeB on Towers of height 25 meters and yet it cannot meet the coverage and capacity demands of its consumers.

- Hence Jio has also deployed , more than 150 K small cells sites on street light poles / roof tops / other street furniture and around 150 K Indoor Small Cells .

There are many locations (i.e. such as train stations, marketplaces, etc) where huge crowds congregate daily. Macro cells are not sufficient to meet the capacity needs for such locations.

Observation 1: Current radio coverage assessments show that the macro layer alone cannot ensure consistent nationwide service.

Moreover, the Macro cell coverage is not sufficient for serving the capacity at “Hot Spot Locations”. Beyond a certain inter-site distance, densification of Macro gNodeB sites creates interference and causes per site capacity reduction as well as customer experience issues.

Observation 2: Locations with daily high footfall regularly exceed the capacity that macro cells can provide.

Observation 3: Macro densification beyond certain inter-site distances becomes counter-productive, as it increases interference and degrades user experience.

Thus, for a country like India with huge land mass, highest population in world and cities with very high population densities, Macro cell layer alone is not sufficient to provide true 4G/5G mobile broadband experience.

## Current Deployment Scenario

The amount of spectrum available in many countries is scarce and it is also very costly. So, splitting the spectrum in any band and dedicating a part of spectrum for small cells will be wasteful.

For example, imagine a small cell location which receives Macro cell signal of -105 dBm or less from 4 surrounding cells. Such scenario is very commonplace in Jio network. If 100 MHz of spectrum available in 3500 MHz band is split into two parts – 50 MHz for Macro cells and 50 MHz for Small Cells, there will be a permanent loss of 4 x 50 MHz = 200 MHz capacity on four Macro cells surrounding the small cell and additionally a loss of 50 MHz capacity on Small Cell. This kind of permanent loss of capacity is not acceptable.

Furthermore this kind of specialized configurations around small cell sites will complicate the mobility configurations and neighbor relationship management also.

Instead of splitting the spectrum into two parts, in many operator networks like Jio deployment, the HetNet interference is mitigated to some extent by deploying small cells at locations where Macro cell coverage is poor e.g. -105 dBm or less.

Observation 4: It is both commercially and practically unviable to split any spectrum and allocate dedicate spectrum for small cell deployment. Hence, Macro cells and Small Cells have to deployed in same frequency band and the radio network of this type is called Intra-frequency Heterogenous Network (HetNet)

Observation 5: In-spite of the best practices of spatial isolation used in small cell deployments, the small cell performance is not optimal, and it suffers from several issues.

## Interference issues in Intra-frequency HetNet

Downlink Interference Issue

For downlink configurations, the 5G Macro cells are typically deployed with mMIMO configurations with max capacity as 32-port CSI-RS which spans across multiple OFDM symbols. On the other hand, 5G small cells use only 4-port CSI-RS which can be transmitted in single OFDM symbol.

As a result of this mis-alignment, the UE in coverage of a Small Cell faces “Reference Signal Interference” from the always-on Reference Signal of Macro gNodeB. This leads to CRC failures and high BLER in specific slots where Macro reference signals are transmitted. High BLER in one slot tends to bring down the MCS allocation by scheduler and overall loss of spectral efficiency of small cells. This is depicted in figure 1 below.

Figure 1. Reference Signal Interference from Macro to Small Cell in case of Intra-frequency HetNet

Uplink Interference Issue

For uplink, the uplink interference issue is the classical HetNet issue that was acknowledged even in 4G days. The key point is that the UEs in vicinity of small cell but connected to Macro cell (due to better downlink) have high path loss with Macro. Owing to higher path loss with Macro, such UEs transmit at higher power. However, as they are closer to the small cell, their path loss with small cell is smaller and hence their transmission raises the noise floor of the small cells.

In worst case, the amount of interference will be same as the Transmit Power difference between the Macro cell and the Small cell, as explained in figure 2.

Figure 1. Het-Net scenario for interference mitigation and coverage enhancement

Observation 6: Persistent DL RS reference-signal collisions together with UL power-asymmetric interference from macro-served UEs near small cells, continue to create significant HetNet performance degradation.

# Proposed deployment Scenario

To better characterize and optimize performance in such scenarios, it is proposed to study the possible issues that come with Intra-frequency HetNet.

Proposal 1: RAN Plenary to consider Intra-frequency HetNet of Macro cells and Small Cells deployment scenario.

# Conclusion

In this contribution, the following observations and proposals are provided:

Observation 1: Current radio coverage assessments show that the macro layer alone cannot ensure consistent nationwide service.

Observation 2: Locations with daily high footfall regularly exceed the capacity that macro cells can provide.

Observation 3: Macro densification beyond certain inter-site distances becomes counter-productive, as it increases interference and degrades user experience.

Observation 4: It is both commercially and practically unviable to split any spectrum and allocate dedicate spectrum for small cell deployment. Hence, Macro cells and Small Cells have to deployed in same frequency band and the radio network of this type is called Intra-frequency Heterogenous Network (HetNet)

Observation 5: In-spite of the best practices of spatial isolation used in small cell deployments, the small cell performance is not optimal, and it suffers from several issues.

Observation 6: Persistent DL RS reference-signal collisions together with UL power-asymmetric interference from macro-served UEs near small cells, continue to create significant HetNet performance degradation.

Proposal 1: RAN Plenary to consider Intra-frequency HetNet of Macro cells and Small Cells deployment scenario.

# References

3GPP TR 38.914 V0.2.0: Study on 6G Scenarios and Requirements

# Annex

3GPP TR 38.914

>>>>>>>>>>>>>>>>>>>>>>>>>>>>> skip unchanged text <<<<<<<<<<<<<<<<<<<<<<<<

4. Deployment scenarios

Editor note: Sensing related aspects will be included in section 4

Editor note: More deployment scenarios can be added

High-level descriptions on deployment scenarios including carrier frequency, aggregated system bandwidth, network layout / ISD/ Orbit, BS / UE antenna elements, UE distribution / speed and service profile are proposed in this TR. It is assumed that more detailed attributes and simulation parameters, for example, the channel model, BS / UE Tx power, number of antenna ports, etc. should be defined in the new RAT study item.

…..

>>>>>>>>>>>>>>>>>>>>>>>>>>>>> skip unrelated text <<<<<<<<<<<<<<<<<<<<<<<<

…..

4.14 Intra-Frequency Het-Net Deployment scenario

Intra-frequency HetNet are inevitable in higher frequency bands like 2300 MHz and 3500 MHz band. When Intra-frequency HetNets are deployed, the issues described in 2.2 corresponding to Downlink and Uplink interference are also unavoidable.

It is proposed for 3GPP to consider Intra-frequency HetNet of Macro cells and Small Cells as one important deployment scenarios.

Some of its attributes are listed in Table 4.14.1.

Table 4.14.1: Attributes for High Rise Building Coverage scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 850 MHz 900 MHz 2300 MHz Around 3500 MHz Around 7 GHz |
| Aggregated system bandwidth | Around 800-900 MHz: Up to [20] MHz (DL+UL) |
|  | Around 2GHz: Up to [40] MHz (DL+UL) Around 3.5GHz: Up to [100] MHz (DL+UL) |
|  | Around 7 GHz: Up to [400] MHz (DL+UL) |
| Layout | Single layer: |
|  | - Indoor floor (Open office) - outdoor dense urban/urban - outdoor suburban - outdoor rural |
|  | Note: Co-site & single-layer deployment for Around 4GHz+Around 7GHz is considered. |
| ISD | 1 Km urban and 4 Km rural, for below 1 GHz 0.5 Km urban and 3 Km rural, for around 1.8 -2.3 Ghz 0.3 Km urban and 2 Km Rural, for around 3.5 GHz 200m urban to 2 Km rural, for around 7 GHz |
|  | TBD on other carrier frequencies |
|  | (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements | Around 1 GHz: Up to 32 Tx and Rx antenna elements Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7GHz: Up to 1024 Tx and Rx antenna elements |
|  | TBD on other carrier frequencies |
| UE antenna elements | Around 1GHz: Up to 2 Tx and Rx antenna elements Around 4 GHz: Up to 4 Tx and Rx antenna elements |
|  | Around 7GHz: Up to 8 Tx and Rx antenna elements |
|  | TBD on other carrier frequencies |
| Multi-TRP operation | Below 1 GHz: single TRP Around 2 GHz: single TRP, or coherent joint transmission multi-TRP Around 2 GHz to 3.5 GHz: single TRP, or coherent joint transmission multi-TRP Around 7 GHz: single TRP, or coherent joint transmission multi-TRP |
|  | TBD on other carrier frequencies |
|  | Note: TRP(s) can be all DL+UL TRP, or randomly selected from {DL+UL TRP, UL-only TRP}. |
| User distribution and UE speed | 75% indoor 10% mobile 3km/h up to 60km/h 15% outdoor pedestrian, up to 3km/h |
|  | [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>> end of proposed changes <<<<<<<<<<<<<<<<<<<<<<<<