3GPP TSG-RAN Meeting #110			RP-253234

Baltimore, US, December 08-11, 2025

Source:	Sony

Title:	6G diverse device types

Agenda Item:	8.2.1.3.2

Document for:	Discussion and decision

# 1	Background

In RANP#109, the following proposals in [1] were endorsed:

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, at least form factor limitations is considered. Sstudy further: - The parameters related to form factor limitations, e.g., the number of Tx antennas/chains, Rx antennas/chains, power classes, etc. - Other Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

This document considers the motivation for and definition of device types in 6GR. Our view is that a device type represents a set of common assumptions about UE hardware capabilities and can be specified as a set of UE parameters / capabilities / features with different values. Form factor limitations will also play a role in differentiating between device types. We conclude that devices that share similar hardware capabilities or performance requirements should be categorized into the same device type.

# 2	Device Types

2.1 Classification of device types

The following proposal was endorsed in RANP#109:

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement.

The 6GR design targets a scalable and forward-compatible design for diverse device types. Each device type may represent a set of common assumptions about UE hardware implementation and can contain a set of UE parameters/capabilities/features with different values, e.g., UE CBW, number of antennas, TX power, etc.

Observation 1: Each device type may represent a set of common assumptions about UE hardware implementation and can be specified as a set of UE parameters/capabilities/features e.g., UE CBW, number of antennas, TX power, etc., with different values

The basic methodology for device type definition can be considered as below:

- Common mandatory baseline parameters/capabilities/features set for all 6G devices to ensure basic interoperability

- Device-type-specific parameters/capabilities/features to cater for the diverse performance, cost, and power consumption targets

- Optional parameters/capabilities/features can be supported for each device type, e.g., AI, sensing, NTN etc.

With the above methodology, introducing UE device types in 6GR can help to streamline the UE capability framework and avoid excessive UE capability reporting. Additionally, with a clear set of features to be defined for each device type, it can also help to reduce the effort on Interoperability and Development Testing (IoDT) among UE vendors, network vendors and operators.

Observation 2: Introducing device type in 6GR can help to avoid excessive UE capability reporting and the effort on Interoperability and Development Testing (IoDT).

On how to determine the device types, we think the device usage scenario and the form factors are the two key factors to be considered.

- The intended usage scenario or use case of a device is inevitably one of the key factors to be considered first when it comes to determining the device type in 6GR in our view. The main differentiator in terms of the usage scenario when it comes to the device type discussion is whether the device is for massive IoT or mobile broadband.  An IoT device would have a lower device complexity and cost structure than mobile broad band, which leads to different assumptions for UE hardware capabilities regardless of form factor of the device.

Observation 3: The targeted usage scenario, e.g., if it is massive IoT or if it is a mobile broadband device, is a key factor to be considered when determining the hardware assumption for device type, due to the different cost and complexity considerations.

- On top of the usage scenario, form factor is a critical aspect to be considered as well when determining the device type as it plays a major role in constraining the device implementation. For example, the assumption on the number of antennas can be different between a handheld device, e.g., smartphone, and a non-handheld device, e.g., FWA/CPE devices, although the cost/complexity assumption could be similar.

Observation 4: For a given usage scenario, e.g., mobile broadband, the form factor limitation plays a key role to differentiate device types.

With the consideration above, it is proposed to consider the following device types as a starting point and further discuss whether more device types would be needed.

- Massive IoT device (e.g., tracker, wearable devices, meters)

  - Aim for device capability above 4G IoT and below 5G NR (e)Redcap

  - with reduced device complexity than legacy devices (e.g., 5G (e)Redcap)

- Mobile broadband device:

  - Broadband device with limited form factor (e.g., smartphone, glass type XR)

    - Improved data rate but limited additional complexity to legacy devices (e.g., 5G eMMB)

    - Form factor limitation considered when determining the device parameters/capabilities

  - Ultra Broadband devices (e.g., FWA/CPE, HMD XR)

    - focus on fulfilling the extraordinary performance bounds

    - no major constraints due to device form factor but with realistic implementation assumption

Proposal 1: Considering 1) Massive IoT device, 2) Broadband device with limited form factor e.g., smartphone, 3) Ultra Broadband devices (e.g., FWA/CPE, XR) as a starting point for definition, further discuss if more device types need to be defined in 6GR.

6GR supports a diverse range of service classes, from immersive multimedia and massive IoT to integrated communications and sensing. To avoid over-fragmentation, the device type definitions in 6GR should consider grouping devices that share similar hardware capabilities or performance requirements. For example, both FWA devices and HMD XR devices may need to offer superior throughput to the end user while facing fewer constraints in terms of the device's form factor, which can be categorized into the same device type.

Proposal 2: Devices that share similar hardware capabilities or performance requirements shall be categorized into the same device types.

The following proposal was endorsed in RANP#109:

Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

From the table above, the parameters that we think are critical to characterize a 6G device type are:

- Number of antennas/chains (Tx/Rx)

- Default power classes

- Maximum UE bandwidth (DL/UL)

- Peak data rate (DL/UL)

- Maximum MIMO layers (DL/UL)

- Duplex mode

- Max modulation order (DL/UL)

- UE processing capabilities

- Energy efficiency

Proposal 3: A 6G device type can be characterized by the following parameters:

- Number of antennas/chains (Tx/Rx)

- Default power classes

- Maximum UE bandwidth (DL/UL)

- Peak data rate (DL/UL)

- Maximum MIMO layers (DL/UL)

- Duplex mode

- Max modulation order (DL/UL)

- UE processing capabilities

- Energy efficiency

The detailed parameters/capabilities of device types that are proposed above are listed in Table I below. We further elaborate the motivation behind our proposed values particularly for massive IoT devices in the later section of the paper.

Table I. 6GR device type and associated device capability

|  | Massive IoT device, e.g., tracker, wearable devices, meters | Broadband device with formfactor limitations, e.g., smartphone, glass type XR devices | Ultra broadband device e.g., FWA/CPE, Vehicle, HMD type XR devices. |
| --- | --- | --- | --- |
| Design considerations | - Aim for device capability above 4G IoT and below 5G NR, - with reduced device complexity than legacy devices (e.g., 5G (e)Redcap) | - Improved data rate but limited complexity additional to legacy devices (e.g., 5G eMMB) - consider the formfactor limitation when determining the device parameters/capabilities | - focus on fulfilling the extraordinary performance bounds - no major constraints due to device formfactor but with realistic implementation assumption |
| Number of antennas (Tx/Rx) | 1T/1R | FR1&FR2-1 antenna assumption to be the same as in 5G NR, 2T/4R in FR3. | 4T/8R in FR1 and FR3 FR2-1 antenna assumption to be the same as in 5G NR. |
| Default power class | 23 dBm | 23 dBm in FR1 26 dBm in FR3 Considering enhancement on PC3 in FR2-1 | 26 dBm in FR1 29 dBm in FR3 PC4 in FR2-1 |
| Maximum UE CBW (single CC) | UL: 3MHz/ DL: [5-20] MHz | - 100 MHz in FR1 (same as 5G NR) - 200 MHz in FR3 - 400 MHz in FR2-1(same as 5G NR) | - 200 MHz in FR1 - 400 MHz in FR3 - 400 MHz in FR2-1 |
| Peak data rate | 10Mbps/10Mbps | TBD | TBD |
| Maximum MIMO layer | Equal to the number of antennas | Equal to the number of antennas | Equal to the number of antennas |
| Duplex mode | Focus on HD-FDD, FD-FDD/TDD can be supported | FDD/TDD | FDD/TDD |
| Maximum Modulation order | 64QAM UL/64QAM DL | 256 QAM UL/1024 QAM DL in all frequency ranges | 1024 QAM UL/1024 QAM DL in all frequency ranges |
| Processing capability | Relaxed | Standard | Supports low latency |
| Energy efficiency | high | Standard | Standard |
| Mobility | Idle mode mobility as baseline | Connected mode mobility | Connected mode mobility |

2.2 Consideration on parameters / factors for IoT device types

As 6G-IoT is likely to be the lowest tier of device in 6G, which will determine the minimal set of features and minimal capability that 6G is required to operate with, therefore, it is particularly critical to consider and determine the 6G-IoT parameters. Therefore, we particularly discuss the motivation behind our proposed parameters for 6G-IoT in this section.

Observation 5: the device capability/parameters of 6G-IoT will determine the minimal set of features and minimum capabilities that are required to operate in 6GR.

In the table below, we share our consideration for parameters on 6G-IoT

Table II. consideration on 6G-IoT

| Parameter | Value | 6G-IoT consideration |
| --- | --- | --- |
| Number of Tx antennas/chains | 1 |  |
| Number of Rx antennas/chains | 1 |  |
| Power classes | 23dBm |  |
| Maximum UE bandwidth (DL/UL) | UL: 3MHz DL: [5-20MHz] | For a SAWless global 1SKU design, a 3MHz UL bandwidth is beneficial. Some detailed discussion can be found in our accompanied paper [2] The DL bandwidth is not impacted by SAWless considerations. A proper DL bandwidth can be supported to minimize the impact to other device types. |
| Peak data rate (DL/UL) | UL: 10Mbps DL: 10Mbps | UL data rate constrained by UL bandwidth. DL data rate is limited to 10Mbps to minimize device complexity especially if a relatively large DL bandwidth needs to be supported, e.g., 10 or 20 MHz |
| Maximum MIMO layers (DL/UL) | 1 |  |
| Duplex mode | HD-FDD (priority) TDD/FDD can be supported | HD-FDD is an enabler for a 1 SKU design and significantly reduces device complexity and formfactor |
| Max modulation order (DL/UL) | 64QAM |  |
| UE processing capabilities | relaxed | A relaxed processing timeline significantly reduces device complexity |
| Energy efficiency | High, with potential IoT specific power saving features | Battery life is a key consideration for IoT devices and it would be beneficial for a high energy efficiency operation. It can be expected that a 6G-IoT device can support UE power saving and NES features, while IoT specific power saving features, e.g., eDRX should also be enabled. In addition, from hardware implementation aspect, UE power is saved with a SAWless design since the insertion loss for a SAWless design is lower when there are no band specific filters or switches. |
| Mobility/speed | Idle mode | Idle mode mobility supported as a baseline to reduce the required processing capability, power consumption and memory footprint |

Some of the parameters above may be more intuitive and can be naturally considered, for example number of antennas and MIMO layer. Meanwhile there are some parameters among the list above that may trigger more considerations, including the IoT device complexity and the potential impact to the mobile broadband and these need to be very carefully considered in our view, which includes the maximum UL/DL bandwidth, the peak data rate and etc. Based on the discussion above, we make the following proposal to highlight  parameters whose determination is particularly critical for a 6G-IoT device:

Proposal 4: Critical parameter values for a 6G-IoT device type are:

- Maximum UL bandwidth: 3MHz

- Maximum DL bandwidth: 10MHz

- Peak UL and DL data rates: UL/DL = 10Mbps

- HD-FDD support

- Relaxed processing capabilities / timelines: at least 4ms between UL and DL

- Improved energy efficiency

# 3	Conclusion

In this contribution we made the following observations and proposals:

Observation 1: Each device type may represent a set of common assumptions about UE hardware implementation and can be specified as a set of UE parameters/capabilities/features e.g., UE CBW, number of antennas, TX power, etc., with different values

Observation 2: Introducing device type in 6GR can help to avoid excessive UE capability reporting and the effort on Interoperability and Development Testing (IoDT).

Observation 3: The targeted usage scenario, e.g., if it is massive IoT or if it is a mobile broadband device, is a key factor to be considered when determining the hardware assumption for device type, due to the different cost and complexity considerations.

Observation 4: For a given usage scenario, e.g., mobile broadband, the form factor limitation plays a key role to differentiate device types.

Observation 5: the device capability/parameters of 6G-IoT will determine the minimal set of features and minimum capabilities that are required to operate in 6GR.

Proposal 1: Considering 1) Massive IoT device, 2) Broadband device with limited form factor e.g., smartphone, 3) Ultra Broadband devices (e.g., FWA/CPE, XR) as a starting point for definition, further discuss if more device types need to be defined in 6GR.

Proposal 2: Devices that share similar hardware capabilities or performance requirements shall be categorized into the same device types.

Proposal 3: A 6G device type can be characterized by the following parameters:

- Number of antennas/chains (Tx/Rx)

- Default power classes

- Maximum UE bandwidth (DL/UL)

- Peak data rate (DL/UL)

- Maximum MIMO layers (DL/UL)

- Duplex mode

- Max modulation order (DL/UL)

- UE processing capabilities

- Energy efficiency

Proposal 4: Critical parameter values for a 6G-IoT device type are:

- Maximum UL bandwidth: 3MHz

- Maximum DL bandwidth: 10MHz

- Peak UL and DL data rates: UL/DL = 10Mbps

- HD-FDD support

- Relaxed processing capabilities / timelines: at least 4ms between UL and DL

- Improved energy efficiency

# References

RP-252873. “Moderator’s summary on 6G diverse device types (incl. minimum UE bandwidth). Apple (moderator). RANP#109. Beijing. September 2025.

RP-253235. “6G massive IoT requirements”. Sony, Nordic Semiconductor, AT&T, Semtech. RANP#110, Baltimore. December 2025.