3GPP TSG-RAN Meeting #110			RP-253235

Baltimore, US, December 08-11, 2025

Source:	Sony, Nordic Semiconductor ASA, AT&T, Semtech

Title:	6G massive IoT requirements

Agenda Item:	8.2.1.4

Document for:	Discussion and decision

# 1	Background

Massive communication is among the six usage scenarios which are targeted by IMT-2030 [1] for 6G communications. Meanwhile, from the 3GPP perspective, it has been agreed to study 6GR so that it can support diverse device types, including massive IoT devices [2].

Cellular IoT devices (cat-M1/NB-IoT, (e)Redcap) have been successfully deployed and have supported various use cases over the years. Those devices are expected to last for decades and continuously provide service even in the 6G era. Meanwhile, new 6G massive IoT devices are expected to be designed and introduced in 6G-day1, which can support existing use cases with a smooth migration path to 6G in the future and can also support new use cases that bring additional revenue streams.

Based on the reflections from legacy IoT services and demand from potential new use cases, the following aspects shall be considered when designing the 6G massive IoT:

- Adding new features in later stages poses challenges (technical and market). Therefore, target a 6GR that supports from day-1 diverse device types scaling up from the lowest capability device (e.g., 6G massive IoT) to higher capability devices.

- The multiple 3GPP cellular IoT solutions create some market confusion and fragmentation and have increased the implementation effort and complexity, and thus a single massive IoT solution as part of 6GR would be preferred. However, two IoT device types may be considered if sufficient differentiation between them is identified.

- Considering the longevity of IoT services in the field, it is important to “plan ahead” and make critical decisions in the initial design (e.g. minimum bandwidth), to ensure the future proofness of the 6G massive IoT design.

- Based on experience from legacy IoT, there are areas where complexity reduction can be improved.  for example, overhead reduction for small payload communication, SAW-less frontend implementation, mobility relaxation, etc. The complexity reduction on IoT device implementation can benefit the growth of the cellular IoT ecosystem and unlock new markets.

In this contribution, we particularly focus on the last point, where we share our understanding on the importance of SAW-less frontend implementation and the potential solution that can be studied in 3GPP.

# 2	Design Consideration of 6G massive IoT

To facilitate the economy of scale, while offering a highly efficient, compact device design with low complexity, HD-FDD is the most important duplex mode for 6G massive IoT devices, which offers improved power efficiency, reduced device complexity, and enhanced global scalability by eliminating band- or regional-specific SAW/BAW duplexers/filters from the RF front end of the devices.

However, based on the lessons learned from 5G NR eRedcap implementation, the full advantages of HD-FDD are not realized since band-specific SAW filters would still be required to fulfil the band-to-band protections (-50 dBm), especially for selected bands in sub-1GHz where the offset between the aggressor band and victim band are small (below 12 MHz-ish), where the high order intermodulation distortion (IMD) will fall into the protected band, which is illustrated in Fig. 1.

Fig. 1. Illustration of a issue of enable a full SAW-less implementation.

The difference between an HD-FDD design with a few SAW filters vs. a fully SAW-less HD-FDD design is illustrated in Fig. 2. It can be observed that adding a SAW filter does not only require the filter itself, but also leads to the need for additional switches and higher output power PA and more complicated RFIC design. Consequently, the device complexity and module size will be increased with a reduced power efficiency. While multiple regional stock-keeping units (SKUs) can partially address the module size and complexity, these = would jeopardize the economies of scale of the device. Finally, adding new bands in supported frequency range can be done without hardware changes.  A detailed comparison is given in appendix A.

Fig. 2. Comparison of massive IoT device with SAW filter/duplexer design vs a fully SAW-less design.

Therefore, achieving a true SAW-less HD-FDD design should be a key consideration for 6G massive IoT, which can offer:

- A true single SKU design for global operations allowing for true economies of scale.

- A smaller IoT module which may unlock new IoT use-cases.

- The number of components is reduced compared to FD-FDD/HD-FDD with SAW filters for selected bands which reduces the module bill of material (BOM) as well as reduces the maintenance effort of the module.

- Additional supported bands can be introduced without a need to re-design hardware.

- Last, but not least, the insertion loss on Tx and Rx chains can be reduced, and the power efficiency of the module can be enhanced along with the module’s longevity.

Observation 1: a true SAW-less HD-FDD designed massive IoT device can enable a true single stock-keeping unit (SKU) design for global operations, along with

- improved power efficiency

- significantly reduced module size.

- with lower overall BoM associated with the module.

- with greater longevity of the module.

Proposal-1: 6GR shall support low-tier IoT module implementation without band-specific analogue filters (e.g. without SAW filters)

# 3	Enable Full SAW-less HD-FDD design

As illustrated in Fig. 1, the main technical challenge to enable a full SAW-less HD-FDD design is to avoid the high order IMD components, especially IMD 7, falling into the protected bands and exceeding the emission limit. Therefore, to resolve the issue, one or combination of multiple solutions can be adopted in the 6GR design to enable a full SAW-less design for 6G massive IoT.

Solution 1: Since the issue is the most severe when the device is configured on the edge of the transmission bandwidth that is near the protected band, the most straightforward solution would be to avoid configuring the UE in such an allocation as illustrated in Fig. 3. Based on our initial assessment assuming the 5G NR waveform, introducing an additional  2-3 MHz guardband from the lower edge of the band could be sufficient.

Fig. 3 Allocation restriction based approach to enable a full SAW-less implementation

Solution 2: A less restrictive approach would be limiting the UE transmission bandwidth to a relatively smaller number, e.g., 3MHz, when the UE is configured less than X MHz from theprotected band. In such a configuration, the corresponding IMD region will also be narrower which can effectively avoid the IMD7 component falling into the protected band. Theoretically, such a reduced UE transmission bandwidth can be either configured as RF bandwidth or baseband bandwidth, where the corresponding solutions are shown in Fig. 4 (a) and (b), respectively.

(a)

(b)

Fig. 4 (a) RF bandwidth limitation and (b) BB bandwidth limitation-based approach to enable a full SAW-less implementation

Solution 3: As a final approach, reducing the output power when the UE is configured on the edge that is near to the protected band as shown in Fig. 5. Based on the assessment of a few commercial PAs in the market, about 2-3 dB power backoff may be needed to enable full SAW-less implementation.

Fig. 5 Power backoff based approach to enable a full SAW-less implementation

The impact on the standard to achieve a true SAW-less implementation is still pending on the basic system parameters required for 6G massive IoT devices, especially for the uplink bandwidth. For example, if the device only needs to support a relatively small bandwidth, e.g., 3MHz, it can be designed naturally as a true SAW-less design without any further impact as explained in Solution 2. If a larger bandwidth needs to be supported, especially in the uplink direction, other approaches, e.g., limited number of allocated RBs, allowed power backoff for certain bands and/or certain RB allocations or scheduling restrictions on where a SAW-less device can be allocated when transmitting a large number of RBs may need to be considered instead. Overall, we see that the impact is very limited and would not jeopardize the overall 6GR design, and especially would not negatively impact 6GR broadband services.

Observation 2: if the device only needs to support a relatively small bandwidth in the uplink, e.g., 3MHz, it can be designed naturally as a true SAW-less design without any further specification impact.

Observation 3: If a large bandwidth needs to be supported for certain frequency locations in certain bands and regions, other approaches, e.g., limit the operated UL bandwidth to a smaller bandwidth, limited number of allocated UL RBs, allowed power backoff for certain bands and/or certain UL RB allocations restrictions, may need to be considered instead.

It is proposed to capture a requirement in TR38.914 that 6GR supports a lowest tier device type without band-specific analogue filters to support global operation in multiple bands. During the RAN WG study, RAN4 and RAN1 can determine a preferred approach for meeting this requirement. Some potential approaches are discussed above.

Proposal 2: Include the following objective in the “Study on 6G Radio” study item:

- Study the scheduling restrictions, power reductions and/or UL bandwidth reductions necessary to enable a lowest tier device type without band-specific analogue filters to support global operation in multiple bands with minimum incremental device complexity.  [RAN4, RAN1]

Proposal 3: Adopt the following text proposal for 38.914 section 5.4.3.

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. - The 6GR design shall allow/enable implementation of a lowest tier device type without band-specific analogue filters to support global operation in multiple bands. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

# 4	Conclusion

In this contribution we made the following observations and proposals and also provide a text proposal for TR38.914:

Observation 1: a true SAW-less HD-FDD designed massive IoT device can enable a true single stock-keeping unit (SKU) design for global operations, along with

- improved power efficiency

- significantly reduced module size.

- with lower overall BoM associated with the module.

- with longer longevity of the module

Observation 2: if the device only needs to support a relatively small bandwidth in the uplink, e.g., 3MHz, it can be designed naturally as a true SAW-less design without any further spec impact.

Observation 3: If a large bandwidth needs to be supported for certain frequency locations in certain bands and regions, other approaches, e.g., limit the operated UL bandwidth to a smaller bandwidth, limited number of allocated UL RBs, allowed power backoff for certain bands and/or certain UL RB allocations restrictions, may need to be considered instead.

Proposal-1: 6GR shall support low-tier IoT module implementation without band-specific analogue filters (e.g. SAW filters)

Proposal 2: Include the following objective in the “Study on 6G Radio” study item:

- Study the scheduling restrictions, power reductions and/or UL bandwidth reductions necessary to enable a lowest tier device type without band-specific analogue filters to support global operation in multiple bands with minimum incremental device complexity.  [RAN4, RAN1]

Proposal 3: Adopt the following text proposal for 38.914 section 5.4.3.

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. - The 6GR design shall support a lowest tier device type without band-specific analogue filters to support global operation in multiple bands with minimum incremental device complexity. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

# References

Recommendation ITU-R M.2160-0: Framework and overall objectives of the future development of IMT for 2030 and beyond, ITU-R, November 2023.

RP-252912, “Study on 6G Radio,” 3GPP TSG RAN Meeting #109, Beijing, September 2025.

# Appendix A Comparison of FD-FDD, HD-FDD with SAW filters and full SAW-less HD-FDD

| Subject | Details | FD-FDD with Duplexers | HD-FDD with Tx SAWs in critical bands | SAW-less HD-FDD |
| --- | --- | --- | --- | --- |
| BOM | Complexity | High | Mid | Low |
|  | PA / FEM | Discrete PA with band switches (LB/MB) - 27dBm @ PA out | Discrete PA with band switch - 27dBm @ PA out | Optimized integrated FEM - 25dBm @ PA out |
|  | External band switch | + | + | - |
|  | External antenna switch - multiport | + | + | - (included in FEM) |
|  | External LPFs | - | + (Rx and Tx) | + (Rx only) |
|  | External Duplexer and filters | + (Duplexer all bands) | + (Tx SAW for crit. bands only) | + (n71 Rx) |
| Other gaps | PCB complexity | High | Mid | Low |
|  | Size | Large | Mid | Small |
|  | PA Power efficiency | Suboptimal - 27dBm @ PA out | Suboptimal - 27dBm @ PA out | Most optimal - 25dBm at PA out |
| RFIC impact |  | Multiple Rx ports Increased Tx power Stricter noise and linearity requirement. | Increased Tx power | Optimal design |
| Performance |  | Reduced Rx sensitivity Increase power consumption | Increase power consumption | Optimal performance |
| Maintenance | Engineering requirement | High Updated module design required if components change | Mid | Low Few components |
| Longevity |  | Medium | Medium | High Fewer components lead to less chance of single point of failure, as well as components are less stressed by heat caused by RF losses |