3GPP TSG RAN Meeting #110					RP-253237

Baltimore, USA, Dec 8-11, 2025

Agenda Item:	8.2.1.4

Source:	Jio Platforms

Title:	FWA in 6G

Document for:	Approval

# 1	Introduction

The growth of Fixed Broadband services in India has been very slow whereas post the launch of 5G services in n78 band, the Fixed Wireless Access based Home Broadband has picked up rapidly due to ease of deployment. The absolute subscriber count as well as the year on year (YoY) growth rate for both types of broadband services is as shown in table below (Data Reference: TRAI Telecom Subscription Data published every month)

Table 1: FWA Subscription ]Growth Rate in Indian Telecom Market

|  | Subscribers in Million |  | YoY % Growth |  |
| --- | --- | --- | --- | --- |
| Time | FWA | Fixed BB | FWA | Fixed BB |
| Dec, 2023 | 0.91 | 38.35 |  |  |
| Dec, 2024 | 5.21 | 41.19 | 472.5% | 7.4% |
| Sep, 2025 | 12.31 | 44.4 | 136.3% | 7.8% |

With current subscriber numbers, the market penetration for Fixed Broadband in India is only around 15-20%. The demand for fixed broadband in Indian homes is rising as subscribers shift from traditional linear TV to the on-demand OTT content available on a variety of streaming platforms.

On the other hand, the Mobile Broadband market with close to 1 Billion subscribers and an urban tele density of more than 125% is saturated.

Thus, one of the major use cases for 6G in the context of Indian market, especially in the upper mid-band spectrum, is affordable Fixed Wireless Access (FWA).

# 2	Discussion

The cellular services have evolved from 2G to 5G primarily to cater to a variety of service requirements for mobile devices.

With the specification of higher channel bandwidths in 5G NR and availability of larger chunks of spectrum in mid-band (3.5 GHz) and mmWave bands, it became possible for the operators to deliver Fixed Wireless Access (FWA) service using an air interface technology that was predominantly designed for enhanced Mobile Broadband (eMBB).

The advent of Network Slicing in 5G made it possible for the operators to reserve a part of the spectrum for FWA so that the Quality of Service for FWA subscribers can be guaranteed when there is growth of eMBB traffic from the mobile subscribers.

Now that 5G FWA is a successful service across multiple markets including India, US and others, it is important that in 6G, 3GPP should recognize FWA as a New and Important Service Type.

FWA has a distinct set of requirements and characteristics that are quite different from the eMBB service type. These are listed in table below:

Table 2: Differences between eMBB and FWA service requirements

| Sr. No. | eMBB | FWA |
| --- | --- | --- |
| 1 | Services are consumed on Mobile Phone and hence limited by the form factor of the device | Service are not limited by the form factor of the FWA device as consumption can happen by a variety of devices such as Smart TVs, Home Appliances and many more |
| 2 | Services are consumed by single user carrying the mobile phone | Services can be consumed by multiple homes that are connected through FWA device |
| 3 | Throughput requirement is limited by the capabilities of the mobile phone | Throughput requirement is much higher than eMBB service as multiple homes and multiple devices within a home consume services |
| 4 | Connection reliability requirements are baseline eMBB requirements | Connection reliability for FWA needs to be higher than eMBB as service disruption can impact more than one home and large number of consumption devices |
| 5 | Battery life is critical | Battery life is not a concern as devices are powered from a power source |
| 6 | Mobility at various speeds is essential | Mobility is not required. Mobility needs to be restricted to avoid frequent handovers due to multipath fading/shadowing effects |

Observation 1: Fixed Wireless Access (FWA) service requirements in terms of throughput, reliability, battery life and mobility are quite different compared to the eMBB service requirements.

If FWA is recognized as a separate service type compared to eMBB, the RAN WGs will be motivated to develop the new 6G Air Interface keeping in mind a distinct set of service requirements, distinct type of UEs and distinct type of UE environment.

Following are the key differences between eMBB and FWA in terms of UE type and UE’s environment which can open up possibilities for designing the new 6G radio interface that can enhance the FWA performance in 6G.

Table 3: Key differences between eMBB and FWA device type and possibility of building differentiation in 6G Air Interface for FWA

| Sr. No. | Key Differences between eMBB and FWA device type and environment | Possibility of differentiation in Air Interface for FWA |
| --- | --- | --- |
| 1 | eMBB: UEs are Mobile Phones which have screen size of the order of 6-7 inches. FWA: UEs can be roof-mount outdoor Customer Premise Equipment (CPE) or indoor, table-top CPE. In either case, the form factor is large than mobile phones. | As FWA UEs will have larger form factor, UE’s with higher number of single user Receiver MIMO chains and higher number of Transmitter MIMO chains can be standardized on day 1 of 6G. For example, Air Interface can be designed to support 8x8 SU-MIMO in DL and 4x4 SU-MIMO in UL |
| 2 | eMBB: UEs are typically indoor and hence face penetration loss FWA: Indoor Penetration Loss can be avoided by outdoor deployment | FWA UEs are likely to have better link margins compared to the eMBB Mobile Phones. Hence Air Interface can be designed to support higher order modulation schemes like 1024 QAM for both DL and UL |
| 3 | eMBB: UEs have omni directional antenna FWA: UEs can have directional antennas with beam forming capabilities | As FWA UEs can have beam forming capabilities, the 6G Air Interface can develop UL beamforming related solutions specifically for FWA devices. |
| 4 | eMBB: UEs are mobile and hence mobility is essential. FWA: UEs are fixed. Mobility is not required. | If FWA UEs can be identified separately compared to eMBB UEs, provisions for restricting mobility can be built at higher layers. |
| 5 | eMBB: Battery life is important FWA: Battery life is not a concern | FWA UEs can have higher power class e.g. 29 dBm as baseline. If FWA UEs can be identified separately compared to eMBB UEs, provisions for restricting CDRX and other battery saving measures for FWA can be built at higher layers |
| 6 | eMBB: Serves single user FWA: Multiple end-users can be served by one FWA device | Mechanism to identify end-users (homes) served by single FWA device can be studied so that Slicing/QoS can be applied at end-user level rather than the FWA device level |

Observation 2: FWA service uses different types of UEs compared to the eMBB. Also, the FWA UE environment is very different from eMBB UEs. Different UE type and UE environment open up several possibilities to design 6G air interface that can enhance FWA performance significantly and fulfil the distinct requirements of FWA service.

Proposal 1: Fixed Wireless Access (FWA) needs to be recognized as a separate service type in 6G, and a section for the same needs to be added in section 5.4 of draft TR 38.914.

# 3	Text Proposal

# Based on the discussion above we propose the following text proposal.

5.x	Fixed wireless access (FWA)

The 6GR and 6G RAN architecture shall support fixed wireless access (FWA).

# 4	Conclusion

In this contribution we make proposal to recognize FWA as a service type distinct from eMBB.

Observation 1: Fixed Wireless Access (FWA) service requirements in terms of throughput, reliability, battery life and mobility are quite different compared to the eMBB service requirements.

Observation 2: FWA service uses different types of UEs compared to the eMBB. Also, the FWA UE environment is very different from eMBB UEs. Different UE type and UE environment opens up several possibilities to design 6G air interface that can enhance FWA performance significantly and fulfil the distinct requirements of FWA service.

Proposal 1: Fixed Wireless Access (FWA) need to be recognized as a separate service type in 6G and a section for the same need to be added in section 5.4 of draft TR 38.914.