3GPP TSG RAN meeting #110										RP-253245

Baltimore, USA, December 8-11, 2025

Status Report to TSG

Agenda item:	9.2.1

| WI / SI Name | Study on Integrated Sensing And Communication (ISAC) for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_Sensing_NR_bis |  |  |  |  |
| Unique ID |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252819 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2026 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 58% | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Yingyang Li, Zheng Jiang |
|  | Company | Xiaomi, China Telecom |
|  | Email | liyingyang@xiaomi.com, jiangzheng@chinatelecom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1 #122bis, Prague, CZ, October 13-17, 2025

Agreement

On False alarm probability Type 2 (targets dropped in simulation area), the parameter K is the number of drops with at least one detected object.

Conclusion

Sensing resolution is not considered as a performance metric for evaluation of NR ISAC.

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |  |
| --- | --- | --- |
| BS antenna configuration | For 4GHz, 4.9GHz: - Option 1 as optional - Tx: (12,16, 2,1,1;2,16) - Rx: (12,16,2,1,1;2,16) - Option 2 - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) | For 6GHz: - Option 1 as optional - Tx: (16,16, 2,1,1;4,16) - Rx: (16,16,2,1,1;4,16) - Option 2: - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) |
|  | Mandatory: dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Optional: dH,dV = (0.5, 0.5)λ, +45°/-45° polarization |  |

- Note: Tx antenna elements and Rx antenna elements are operating simultaneously

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| BS antenna radiation pattern | Table 9 in Report ITU-R M.2412 |
| BS antenna mechanic tilt (downtilt angle without electrical tilt) | 90° in GCS (pointing to horizontal direction) |
| BS antenna electrical tilt (downtilt angle after mechanic and electrical tilt) | Option 1: no electrical tilt Option 2: 102° in GCS |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| gNB-target link | gNB-aerial UE of UMa/RMa parameters in 36.777 with Alternative 3 |
| Concatenation of TX-target and target-RX links | Up to company choice between two options for concatenation defined in Step 9 in section 7.9.4.1, TR 38.901 |
| The power threshold for path dropping after concatenation for target channel | -25dB and -40dB are respectively used for the two options for concatenation |

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| Co-site inter-sector interference | Not modelled |
| Adjacent channel interference | Not modelled |
| BS receiver noise figure | 5dB |

Agreement

When Tx/Rx operates simultaneously (baseline case), the following parameters are used for the evaluation on NR ISAC:

| Antenna isolation | - Option 1: 65dB - Option 2: 80dB |
| --- | --- |
| Maximum BS Tx power | - Option 1: 37dBm - Option 2: 52dBm The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively |

- Optionally: companies should report the maximum BS Tx power when it is assumed that Tx and Rx don’t operate simultaneously. Companies should report how the maximum BS Tx power is derived.

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| CPI (coherent processing interval) | Companies to report the CPI value assumed |
| Waveform | CP-OFDM as baseline, other waveform is up to companies report |

- Agreement

When Tx/Rx operates simultaneously, the following evaluation parameters are agreed for the evaluation on NR ISAC.

| Self-interference | The residual leakage interference/noise is modelled e.g. by additional additive white Gaussian noise, -94+X dBm in 100 MHz, X is up to company report. Companies to provide details on their modelling. Note: X = -Infinity corresponds to not modelling self-interference |
| --- | --- |

Agreement

For the evaluation on NR ISAC,

- 3 kinds of resources are defined

- Type 1: resources that are used for sensing signal transmission

- Type 2: part of Type 1 resources that are used for communication purpose

  - Note: it is possible the resource type 2 doesn’t exist

- Type 3: resources that are not used for sensing signal transmission, and cannot be used for communication purpose due to sensing operation

- Both options should be reported for sensing resource ratio

- Option 1: (Type_1 + Type_3) resources over all radio DL and UL resources

- Option 2: (Type_1 - Type_2 + Type_3) resources over all radio DL and UL resources

- Note: if Type 2 resource doesn’t exist, two options are the same

For the evaluation on NR ISAC,

- Company should report T/F RE mapping of sensing RS, assumed TDD UL/DL configuration if applicable.

- Company should report which sensing RS resources are considered as Type 2 resource and related reason.

Agreement

Companies are encouraged to report the high-level sensing signal/data processing method used in the evaluation, e.g., 2D FFT, MUSIC, and any other methods.

Agreement

Up to company to model low power cluster in the evaluation of NR ISAC

Agreement

Companies should report details of the Tx beam information (number of Tx beams, wide/narrow Tx beam) being used at TRP.

- The details of the Tx beam information (number of Tx beams, wide/narrow Tx beam) being used at TRP will be captured along with the corresponding evaluation results in the Rel-20 TR

Agreement

The following performance objectives are adopted for evaluation purpose of NR ISAC.

| Metric | Value |
| --- | --- |
| Missed detection Probability | [5]% |
| False Alarm Probability Type 1 | [5]% |
| False Alarm Probability Type 2 | [5]% |
| Horizontal Positioning Accuracy | [10] m with confidence level 90% |
| Vertical Positioning Accuracy | [10] m with confidence level 90% |
| Velocity Accuracy | [5] m/s with confidence level 90% |

Agreement

The TR skeleton in R1-2507422 is endorsed.

RAN1 #123, Dallas, US, November 17-21, 2025

Agreement:

- In section 6.2 Performance evaluation results in TR 38.765,

- Multiple configurations of the following parameters are defined to respectively analyze the evaluation results

  - At least Two baseline configurations 1 & 2 are defined for evaluation purpose

|  | Parameters | Configuration 1 Baseline | Configuration 2 Baseline |
| --- | --- | --- | --- |
| 1 | Scenario | (UMa-AV, 500m) | (UMa-AV, 500m) |
| 2 | Sensing Tx/Rx operating simultaneously or not | Sensing Tx/Rx operating simultaneously | Sensing Tx/Rx operating simultaneously |
| 3 | Carrier frequency | 4 or 4.9GHz | 4 or 4.9GHz |
| 4 | Max BS Tx power | 52 dBm | 37 dBm |
| 5 | BS antenna configuration | - (M, N, P, Mg, Ng, Mp, Np) - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization | - (M, N, P, Mg, Ng, Mp, Np) - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization |
| 6 | Number of targets per sector in center site | N=5 | N=5 |
| 7 | Target vertical distribution | 25-300m | 25-300m |

  - Additional configurations can be defined for other assumptions of the parameters, based on reported evaluation results in RAN1 #124.

Agreement:

- Companies should report whether a same target is modeled in the ISAC channel of single, multiple or all STXs/SRXs

  - Company should report how to determine the single or multiple STXs/SRXs for a target

  - The details of STX/SRX determination will be captured along with the corresponding evaluation results in the TR for NR ISAC

  - If the evaluation results are derived by measurement reports from multiple/all STXs/SRXs, companies should report how measurement reports from multiple/all STXs/SRXs are used

Agreement

From RAN1 perspective, the following measurements that may be reported from RAN are identified in the study of NR ISAC. Note: Down selection of Level(s)/Option(s) for recommendation can be discussed separately.

- Level A: Raw data per Tx antenna port per OFDM symbol per RX antenna port per TRP for a given time stamp

  - Option A1: Amplitude and phase samples in time/delay domain of the estimated channel, i.e., Amplitude and phase values of channel impulse response

  - Option A2: Amplitude and phase per subcarrier in frequency domain of the estimated channel

- Level B: Amplitude and phase profile of delay, and/or Doppler, and/or angle per TRP for a given time stamp by using window(s) of the [consecutive] samples in delay, Doppler and/or angle domain

  - Option B1: Delay-Doppler profile per Tx antenna port per Rx antenna port, which includes the amplitude and phase samples distributed across different delays and Doppler shifts.

  - Option B2: Delay-Angle profile per Tx antenna port per OFDM symbol, which includes the amplitude and phase samples distributed across different delays and spatial angles (e.g., Angle of Arrival).

  - Option B3: Delay-Doppler-Angle profile per Tx antenna port, which includes the amplitude and phase samples distributed across delay, Doppler, and angle domains.

  - Option B4: Delay profile per Tx antenna port per OFDM symbol per Rx antenna port, which includes the amplitude and phase samples distributed across different delays.

  - Note: Level B is applicable for either LCS or GCS

- Level C: per detected path/point measurements per Tx antenna port per TRP which may be reflected/scattered from scattering point(s).

  - Option C1: Delay/range, Doppler/velocity, one or multiple 3D angles, and [power/confidence metric] per detected path for a given time stamp.

    - A path is associated with one couple {delay/range, Doppler/velocity, [power/confidence metric]}, and one or multiple 3D angles.

  - Option C2: Doppler/velocity, position, and [power/confidence metric] per detected path for a given time stamp.

    - A path is associated with one doppler/velocity, [power/confidence metric], one or multiple positions

  - Option C3: Delay/range, Doppler/velocity, 3D angle, and [power/confidence metric] per detected path for a given time stamp.

    - A path is associated with one delay, and one or multiple triple {Doppler/velocity, 3D angle, [power/confidence metric]}

  - Option C4: Delay/range, Doppler/velocity, 3D angle, and [power/confidence metric] per detected point for a given time stamp

    - A point is associated with one range/delay, one Doppler/velocity, one 3D angle

  - Option C5: Position, velocity, and [power/confidence metric] per detected point for a given time stamp

    - A point is associated with one position, one velocity

  - FFS: Velocity can be

    - Opt 1: radial velocity

    - Opt 2: 3D velocity

  - Note:

    - Position can be defined in either LCS or GCS

    - Angle can be defined in either LCS or GCS

    - Ambiguity issue will be further discussed

    - 3D angle refers to a pair of horizontal and vertical angles

- Level D: Object/target level measurement [per TRP or per gNB]

  - One or more value pair(s) {position, velocity} in GCS for a given time stamp is reported for a detected object/target,

    - Option D1: Only one value pair {position, velocity} in GCS for a given time stamp is reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is not reported.

    - Option D2: Only one value pair {position, velocity} in GCS for a given time stamp is reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is reported.

    - Option D3: One or more value pairs {position, velocity} in GCS for a given time stamp are reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is reported.

    - Option D4: One or more value pairs {position, velocity} in GCS for a given time stamp are reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is not reported.

  - Note: Ambiguity issue can be further discussed if any

  - Note: velocity is 3D velocity for Option D2/D3

  - FFS: velocity can be radial velocity or 3D velocity for Option D1/D4

- Note: the possible information from RAN to SF for further processing can be further discussed for all levels

- Note: the possible information from SF to RAN can be further discussed for all levels

- Note: confidence metric can be further discussed for options of Level C/D

- Note: definition of time stamp for each of all options of all Levels can be further discussed

- For Level A/B/C, Tx antenna port means reference signal antenna port for sensing purpose

Agreement

RAN1 confirms the performance objectives in the early agreement from RAN1 #122bis by removing the brackets

Agreement The following performance objectives are adopted for evaluation purpose of NR ISAC. Metric; Value. Missed detection Probability; [5]%. False Alarm Probability Type 1; [5]%. False Alarm Probability Type 2; [5]%. Horizontal Positioning Accuracy; [10] m with confidence level 90%. Vertical Positioning Accuracy; [10] m with confidence level 90%. Velocity Accuracy; [5] m/s with confidence level 90%.

Agreement

Confidence level of the X% represents X percentile point of the cumulative distribution function (CDF) of the estimation errors

Conclusion

Sensing service latency and refreshing rate are not considered as performance metrics for evaluation of NR ISAC.

Agreement

- Up to company to report evaluation results for UAV tracking in NR ISAC study.

  - Proponent company should clarify the details on how target trajectory is modeled

Agreement

- The following general procedure for performance evaluation of NR ISAC is to be captured in TR 38.765.

- Simulation parameter configuration

- Sensing scenario generation, including the deployment of sensing Tx/Rx (STXs/SRXs)

- Dropping N target(s), where N is equal to 0 or larger than 0

- Channel generation and STX/SRX determination for the targets

- Sensing signal generation and passing the sensing signal to the generated channel

- Sensing signal processing at each SRX, optionally, sensing signal processing based on fusion from multiple STXs/SRXs

  - E.g., optionally, sensing signal processing based on tracking

- Sensing performance metric calculation.

Note: company should report the details on sensing signal processing and ISAC channel generation

Agreement

- The following evaluation assumptions for optional FR2-1 carrier frequency are considered for evaluation of NR ISAC

| Parameters | Values |
| --- | --- |
| Scenario | UMi-AV |
| Carrier Frequency | 30 GHz |
| System bandwidth | 400 MHz |
| SCS | 120 kHz |
| Inter-BS (2D) distance | 200 m |
| BS antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (16,16,2,2,1;1,1), (dH,dV) = (0.5, 0.5 or 0.8)λ, +45°/-45° polarization for whole panel; (16,16,2,1,1;1,1) for Tx and (16,16,2,1,1;1,1) for Rx |
| Beam set at TRxP | Up to company report |
| Antenna isolation | 80 ~ 100 dB |
| Max BS Tx power | 30 dBm |
| BS noise figure | 7 dB |

- Up to company to report evaluation results and additional configuration information for FR2-1.

Agreement

- The attached template in R1-2509242 (Proposal 5.3-1-rev2) is used as starting point for evaluation results collection

R1-2509522	Draft TR 38.765 v010: Study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

R1-2509522 is endorsed

2.1.2	Remaining Open issues

The following open issues need to be addressed:

- Study on performance evaluation of gNB-based mono-static sensing for UAV use case

  - Observations/conclusions on the reported evaluation results.

  - Measurement quantization.

  - Other measurement related issues, including fixing FFS in the measurement Level/Option definitions, payload size, possible assistance information, Level/Option down-selection.

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

RAN3 #129bis, Prague, CZ, October 13-17, 2025

- Network architecture

Capture logical architecture for ISAC

RAN3 focus on sensing protocol in coordination with SA2

Agreed TP for BL pCR:

- R3-257325, (TP to BL pCR to 38.765) Network Architecture, ZTE Corporation, Huawei, Xiaomi, Nokia, Nokia Shanghai Bell, NEC, Ericsson, China Telecom, Samsung, LG Electronics, CATT, InterDigital.

- RAN-CN procedures and signaling

Agreed TP for BL pCR:

- R3-257326, (TP to BL pCR to 38.765) for basic sensing call flow, Xiaomi, Huawei, Samsung, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, China Telecom, LG Electronics, NEC, OPPO, InterDigital, Telecom Italia, CATT, CMCC.

RAN3 #130, Dallas, US, November 17-21, 2025

- General

Endorsed baseline pCR:

- R3-258088, (pCR to TR 38.765) Study on Integrated Sensing and Communication (ISAC) for NR, China Telecom, Xiaomi.

- Network architecture

For Control Plane protocol stack, capture it is SCTP-based with NxAP (i.e. NGAP or new AP)

Agreed TP for BL pCR:

- R3-258820, (TP to pCR 38.765) ISAC general aspects and protocol stacks, Xiaomi, Huawei, CATT, LG Electronics, CMCC, Ericsson, NEC, ZTE, Samsung, China Telecom.

- RAN-CN procedures and signaling

WA: gNB decides TRP(s) for sensing.

Agreed TP for BL pCR:

- R3-258867, (TP for pCR) RAN-CN procedures and signaling for ISAC, Huawei, CATT, Xiaomi, Nokia, Nokia Shanghai Bell, LG Electronics, CMCC, Ericsson, ZTE, NEC, Samsung, China Telecom.

2.3.2	Remaining Open issues

The following open issues need to be addressed:

- Network architecture

Focus on the following candidate protocols for sensing data transmission, based on RAN1 and SA2 progress, considering the transmission requirements (e.g., data volumes, efficiency).:

- SCTP-based

- GTP-U-based

-Websocket-based (Streaming based data reporting for trace and MDT, refer to TS 28.532)

Coordination with SA2 is needed.

- RAN-CN procedures and signaling

Discuss interface connection setup if direct connectivity is agreed by SA2.

Discuss the need of sensing modify/update procedure

Discuss which procedure is used to transfer the gNB information (e.g., supported sensing area) from gNB to SF for gNB selection, if signalling approach is needed.

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RP-252819, “Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, China Telecom

TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz, V19.1.0 (2025-09)”

RAN1 #122bis

R1-2507421	Updated work plan for study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

R1-2507422	Updated TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR			Xiaomi, China Telecom

R1-2506811	Discussion on evaluation assumptions and metrics for ISAC	Spreadtrum, UNISOC

R1-2506895	Evaluation methodology and assumptions for 5G-A ISAC	vivo

R1-2506920	Evaluation assumptions and performance evaluation of ISAC for NR	InterDigital, Inc.

R1-2506946	Performance metric, methodologies, and initial evaluation results for ISAC	Huawei, HiSilicon

R1-2506986	Discussion on performance evaluation for ISAC	Xiaomi

R1-2507011	Discussion on ISAC evaluation methodology and assumptions	CMCC

R1-2507116	Discussion on evaluation assumptions and performance evaluation for R20 ISAC for NR	CATT, CICTCI

R1-2507173	Discussion of ISAC evaluation in 5GA	OPPO

R1-2507193	Discussion on 5G-A ISAC evaluation	ZTE Corporation, Sanechips

R1-2507204	Associating a detected object with its true counterpart in ISAC	Tejas Network Limited

R1-2507250	Discussion on ISAC evaluation assumptions and performance evaluation	Samsung

R1-2507337	Evaluation assumptions and performance evaluation for ISAC	EURECOM

R1-2507367	Discussion on ISAC Performance Evaluation for 5G-A   	NIST

R1-2507369	Views on evaluation assumptions and performance evaluation for ISAC	Sharp

R1-2507376	Discussion on evaluation assumptions and performance evaluation	Panasonic

R1-2507380	Discussion on evaluation assumptions and performance evaluation for NR ISAC	LG Electronics

R1-2507401	Views on R20 ISAC Study	SK Telecom

R1-2507428	Views on evaluation assumptions for NR ISAC	KPN N.V., TNO

R1-2507472	Evaluation assumptions and performance evaluations of ISAC for NR	Nokia, Nokia Shanghai Bell

R1-2507473	On ISAC performance evaluations and assumptions	Lenovo

R1-2507503	Discussion on evaluation assumptions and performance evaluation for NR ISAC	ETRI

R1-2507555	Joint views on ISAC measurement report	China Telecom, ZTE, CAICT, CATT, Pengcheng Laboratory

R1-2507573	Discussion on ISAC for NR	Ericsson

R1-2507593	Discussion on Evaluation of ISAC for NR	Sony

R1-2507627	Discussion on ISAC evaluation assumptions and performance evaluation	MediaTek Inc.

R1-2507634	Discussion on Performance Evaluations for Rel-20 ISAC	Google

R1-2507674	On Rel-20 Evaluation assumptions and performance evaluation for 5G-A ISAC	Apple

R1-2507718	Considerations on UAV gNB-monostatic sensing	Qualcomm Incorporated

R1-2507762	Views on Performance Metrics and Evaluation Methodology for ISAC	Tiami Networks

R1-2507810	Evaluation assumptions and performance evaluation for ISAC	NTT DOCOMO, INC.

R1-2507836	Evaluation assumptions and performance evaluation for NR ISAC	Hanbat National University

R1-2507423	Summary #1 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2507424	Summary #2 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2507425	Summary #3 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2507426	Summary #4 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2507427	Summary #5 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2507421	Updated work plan for study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

R1-2507422	Updated TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR			Xiaomi, China Telecom

RAN1 #123

R1-2509237	Draft TR 38.765 v010: Study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

R1-2508372	Views on evaluation assumptions and performance evaluation for ISAC	Sharp

R1-2508384	Discussion on evaluation assumption and performance evaluation for ISAC	Spreadtrum, UNISOC

R1-2508428	Evaluation methodology and performance evaluation  for 5G-A ISAC	vivo

R1-2508451	Discussion on evaluations and measurements for NR ISAC	CMCC

R1-2508464	Evaluation assumptions and performance evaluation of ISAC for NR	InterDigital, Inc.

R1-2508473	Discussion on Evaluation Assumptions and Performance Evaluation for ISAC	Tiami Networks

R1-2508485	Performance metric, methodologies, and initial evaluation results for ISAC	Huawei, HiSilicon

R1-2508593	Discussion on evaluation assumptions and performance evaluation for R20 ISAC for NR	CATT, CICTCI

R1-2508613	Discussion on ISAC evaluation assumptions and performance evaluation	China Telecom

R1-2508618	Evaluation assumptions and performance evaluation for ISAC	EURECOM

R1-2508634	On ISAC performance evaluations and assumptions	Lenovo

R1-2508680	Discussion on performance evaluation for ISAC	Xiaomi

R1-2508723	Discussion of ISAC evaluation in 5GA	OPPO

R1-2508798	Discussion on ISAC evaluation assumptions and performance evaluation	Samsung

R1-2508859	Evaluation assumptions and performance evaluations of ISAC for NR	Nokia, Nokia Shanghai Bell

R1-2508877	Considerations on Evaluations for NR ISAC	CAICT

R1-2508883	Discussion on ISAC Performance Evaluation for 5G-A   	NIST

R1-2508938	Discussion on evaluation assumptions and performance evaluation for NR ISAC	Panasonic

R1-2508969	Discussion on evaluation assumptions and performance evaluation for NR ISAC	ETRI

R1-2509002	Discussion on ISAC for NR	Ericsson

R1-2509040	Evaluation assumptions and performance evaluation for NR ISAC	Hanbat National University

R1-2509054	Discussion on Performance Evaluations for Rel-20 ISAC	Google

R1-2509070	Discussion on Evaluation Assumptions and Performance Evaluation of NR ISAC	Sony

R1-2509106	Rel-20 Evaluation assumptions and performance evaluation for 5G-A ISAC	Apple

R1-2509124	ISAC Performance metrics and overhead	T-Mobile USA

R1-2509157	Discussion on ISAC evaluation assumptions and performance evaluation	MediaTek Inc.

R1-2509227	Evaluation Assumptions and performance evaluation for UAV gNB-monostatic sensing	Qualcomm Incorporated

R1-2509246	Discussion on 5G-A ISAC evaluation	ZTE Corporation, Sanechips,VIAVI

R1-2509276	Evaluation assumptions and performance evaluation for ISAC	NTT DOCOMO, INC.

R1-2509293	Discussion on Evaluation assumptions and of ISAC for NR	SK Telecom

R1-2509305	Discussion on evaluation assumptions and performance evaluation for NR ISAC	LG Electronics

R1-2509326	Evaluation assumptions and Performance Evaluation for Self-Interference Modeling for ISAC	Tejas Network Limited

R1-2509238	Summary #1 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2509239	Summary #2 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2509240	Summary #3 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2509241	Summary #4 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2509242	Summary #5 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2509243	Summary #6 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2509237	Draft TR 38.765 v010: Study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

R1-2509522	Draft TR 38.765 v010: Study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

RAN3 #129b

R3-256589	TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR	China Telecom, Xiaomi	Zheng Jiang

R3-257332	TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR	China Telecom, Xiaomi	Zheng Jiang

R3-257325	(TP to BL pCR to 38.765) Network Architecture	ZTE Corporation, Huawei, Xiaomi, Nokia, Nokia Shanghai Bell, NEC, Ericsson, China Telecom, Samsung, LG Electronics, CATT, InterDigital	Jiren Han

R3-257326	(TP to BL pCR to 38.765) for basic sensing call flow	Xiaomi, Huawei, Samsung, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, China Telecom, LG Electronics, NEC, OPPO, InterDigital, Telecom Italia, CATT, CMCC	Lisi Li

R3-256528	Discussion on ISAC Architecture and General Aspects	Xiaomi	Lisi Li

R3-256529	Discussion on Sensing procedures and singalling	Xiaomi	Lisi Li

R3-256558	Discussion on Network Architecture for ISAC	ZTE Corporation	Jiren Han

R3-256559	Discussion on RAN-CN Procedures and Signalling for ISAC	ZTE Corporation	Jiren Han

R3-256586	(TP for 38.765) Network architecture solution for gNB-based mono-static sensing	China Telecom	Zheng Jiang

R3-256587	On RAN-CN procedures and signalling for supporting sensing	China Telecom, BUPT	Zheng Jiang

R3-256588	Work plan for study on Integrated Sensing And Communication (ISAC) for NR	China Telecom, Xiaomi	Zheng Jiang

R3-256676	(TP for ISAC TR) Network architecture for ISAC	Huawei	Feng Han

R3-256677	(TP for ISAC TR) RAN-CN procedures and signaling for ISAC	Huawei	Feng Han

R3-256685	Discussion on Network Architecture for NR ISAC	Tejas Network Limited	Sriganesh Rajendran

R3-256686	Discussion on RAN-CN procedures and signaling for ISAC	Tejas Network Limited	Sriganesh Rajendran

R3-256712	Discussion on RAN3 impact of gNB-based mono-static sensing	OPPO	Xinlei Yu

R3-256771	Discussion on network architecture for ISAC	CATT	Yang Lu

R3-256772	RAN-CN procedures and signaling on ISAC	CATT	Yang Lu

R3-256784	The discussion on ISAC network architecture	NEC	Chenghock Ng

R3-256785	The discussion on ISAC RAN-CN procedure and signaling	NEC	Chenghock Ng

R3-256826	Network Architecture and Protocol Aspects for NR Sensing Support	Qualcomm Incorporated	Sven Fischer

R3-256827	Signalling and Procedures for NR Sensing Support	Qualcomm Incorporated	Sven Fischer

R3-256854	Discussion on Network Architecture for Sensing	CMCC	Miaoqi Zhang

R3-256855	Discussion on RAN-CN Procedures Supporting ISAC	CMCC	Miaoqi Zhang

R3-256895	Discussion on architecture for ISAC	Nokia, Nokia Shanghai Bell	Steven XU

R3-256896	Discussion on RAN-CN procedures and signaling for ISAC	Nokia, Nokia Shanghai Bell	Steven XU

R3-256914	Discussion on ISAC network architecture	Lenovo	Mingzeng Dai

R3-256915	Discussion on general procedures for gNB-based sensing	Lenovo	Mingzeng Dai

R3-256922	Discussion on Network Architecture for ISAC	Ericsson, Jio Platforms	Gino Masini

R3-256941	Discussions on the framework of ISAC	China Unicom	Xufei Dong

R3-256942	RAN-CN procedures and signaling of ISAC	China Unicom	Xufei Dong

R3-256947	Network architecture enhancements for NR ISAC	Hanbat National University	Eunkyung Kim

R3-257025	Discussion on RAN-CN procedures and signaling to support ISAC	Ericsson, Jio Platforms	Yazid Lyazidi

R3-257111	Discussion paper on ISAC network architecuture	CEWiT	Pardhasarathy Jyothi

R3-257118	NR ISAC Network Architecture	InterDigital, Inc.	James Miller

R3-257119	NR ISAC RAN-CN Procedures and Signaling	InterDigital, Inc.	James Miller

R3-257123	Discussions for ISAC network architecture in RAN3	LG Electronics Inc.	Jaemin Han

R3-257124	Discussions on ISAC RAN-CN procedures and signalling	LG Electronics Inc.	Jaemin Han

R3-257144	Discussion on network architecture for ISAC	Samsung	fanhua kong

R3-257145	Discussion on RAN-CN procedures and signaling	Samsung	fanhua kong

R3-257294	TP to TR 38.765 for Network Architecture	ZTE Corporation	Jiren Han

R3-257297	TP to TR38.765 for basic sensing call flow	Xiaomi	Lisi Li

R3-257298	TP to TR38.765 for Terms, Abbreviations and References	Ericsson	Yazid Lyazidi

R3-257299	Summary of Offline Discussion for CB: # 21_ISAC	China Telecom	Zheng Jiang

RAN3 #130

R3-258088	(pCR to TR 38.765) Study on Integrated Sensing and Communication (ISAC) for NR	China Telecom, Xiaomi

R3-258114	(TP to pCR 38.765) ISAC general aspects and protocol stacks	Xiaomi

R3-258115	(TP to pCR 38.765) Sensing procedures and singalling	Xiaomi

R3-258116	(TP for pCR) Network architecture for ISAC	Huawei

R3-258117	(TP for pCR) RAN-CN procedures and signaling for ISAC	Huawei

R3-258130	Network Architecture and Protocol Aspects for NR Sensing Support	Qualcomm Incorporated

R3-258131	Signalling and Procedures for NR Sensing Support	Qualcomm Incorporated

R3-258137	NR ISAC Network Architecture	InterDigital Inc.

R3-258138	NR ISAC RAN-CN Procedures and Signaling	InterDigital Inc.

R3-258147	(TP to pCR 38.765) Further discussion on Network Architecture for ISAC	ZTE Corporation

R3-258148	(TP to pCR 38.765) Further discussion on RAN-CN Procedures and Signalling for ISAC	ZTE Corporation

R3-258184	Discussion on RAN-CN procedures and signaling for ISAC	Google

R3-258196	Network Architecture and Protocol stack for NR ISAC	Tejas Network Limited

R3-258197	RAN-CN procedures and signaling for NR ISAC	Tejas Network Limited

R3-258207	Discussion on architecture for ISAC	Nokia, Nokia Shanghai Bell

R3-258208	(TP to pCR) Discussion on RAN-CN procedures and signaling for ISAC	Nokia, Nokia Shanghai Bell

R3-258243	(TP to BL pCR of TR38.765) The discussion on ISAC network architecture	NEC

R3-258244	(TP to BL pCR of TR38.765)The discussion on ISAC RAN-CN procedure and signaling	NEC

R3-258257	Discussion on RAN-CN procedures and signaling for ISAC	KPN N.V.

R3-258289	Discussion on protocol stack for sensing reporting	OPPO

R3-258290	Discussion on RAN-CN procedures and signalling for sensing	OPPO

R3-258309	(TP to BL pCR for TR38.765) Discussion on network architecture for ISAC	CATT

R3-258310	(TP to BL pCR for TR38.765) RAN-CN procedures and signaling on ISAC	CATT

R3-258357	Work plan for study on Integrated Sensing And Communication (ISAC) for NR	China Telecom, Xiaomi

R3-258358	On protocol stack solution for ISAC	China Telecom

R3-258359	(TP to 38.473) On RAN-CN functions and procedures for supporting ISAC	China Telecom, BUPT

R3-258371	(TP to TR 38.765) Discussion on ISAC network architecture	Lenovo

R3-258372	(TP to TR 38.765) Discussion on general procedures for gNB-based sensing	Lenovo

R3-258376	Discussion on network architecture for ISAC	Samsung

R3-258377	Discussion on RAN-CN procedures and signaling	Samsung

R3-258454	ISAC Logical Architecture and Protocol Stack	Ericsson

R3-258501	Network architecture enhancements for NR ISAC	Hanbat National University

R3-258533	Discussion on RAN-CN procedures and signaling to support ISAC with TP to pCR	Ericsson

R3-258591	Discussions on the architecture design of ISAC	China Unicom

R3-258592	Standard impacts of RAN-CN procedures for ISAC	China Unicom

R3-258612	Discussion on Network Architecture for ISAC	CMCC

R3-258613	Discussion on RAN-CN Procedures Supporting ISAC	CMCC

R3-258630	(TP to BL pCR to TR 38.765) Discussions for ISAC network architecture	LG Electronics Inc.

R3-258631	(TP to BL pCR to TR 38.765) Discussions for ISAC RAN-CN aspects	LG Electronics Inc.

R3-258682	Discussion paper on ISAC network architecuture	CEWiT

R3-258683	Discussion paper on ISAC procedures and signaling	CEWiT

R3-258819	Summary of the offline discussion on CB#17 ISAC	Xiaomi

R3-258820	(TP to pCR 38.765) ISAC general aspects and protocol stacks	Xiaomi, Huawei, CATT, LG Electronics, CMCC, Ericsson, NEC, ZTE, Samsung, China Telecom

R3-258821	(TP for pCR) RAN-CN procedures and signaling for ISAC	Huawei

R3-258867	(TP for pCR) RAN-CN procedures and signaling for ISAC	Huawei, CATT, Xiaomi, Nokia, Nokia Shanghai Bell, LG Electronics, CMCC, Ericsson, ZTE, NEC, Samsung, China Telecom

R3-258879	(pCR to TR 38.765) Study on Integrated Sensing and Communication (ISAC) for NR	China Telecom, Xiaomi

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template