3GPP TSG RAN Plenary #110			RP-253250

Baltimore, USA, December 8-11, 2025

Agenda item:		8.2.1.4

Source:	China Telecom, Huawei, ZTE

Title:	Considerations on Immersive Communication scenarios

Document for:		Discussion

Introduction

In the last 3GPP TSG RAN meeting #109, the requirements of new and existing services have been discussed, while no consensus on Immersive Communication was agreed. The skeleton of TR 38.914 Study on 6G Scenarios and Requirements has been listed as follows [1].

- 5.4 Requirements of New and Existing Services ### 5.4.0 General This section describes the requirements of new and existing services. ### 5.4.1 Mobile Broadband ### 5.4.2 Immersive Communication ### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture) ### 5.4.4 Sensing Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs. ### 5.4.5 AI ### 5.4.6 Voice ### 5.4.7 Regulatory Services

Based on the above progress, we will provide considerations on Immersive Communication scenarios in this contribution.

Considerations on Immersive Communication

Immersive Communication is widely recognized as a critical scenario and a key driver in the evolution of wireless technology for 6G. It represents a fundamental shift from mere data transfer to the seamless integration of physical and digital realities, promising to bring unprecedented convenience and transform human life. We see potential use cases of Immersive Communication services including VR/XR, immersive gaming, real-time digital twins, Tele-education, Tactile Robotic Telesurgery and so on, which would demand ultra-low latency, extreme reliability, and immense data throughput to make interactions more intuitive and seamlessly integrated with reality.

Based on the description in TR 22.870 [2], it mentioned significantly different QoS requirements of latency, jitter, reliability and throughput for 6G services or applications for immersive communications. Additionally, for the use case on real time digital twins, some terminals have limited specific QoS requirements—such as strict guarantees on latency, jitter, and packet loss for critical service flows.

- 5.9.8 Enhanced Network Service Awareness 5.9.8.1 Description The 5G system provides capabilities (e.g. QoS, network slicing) that enable a network operator to offer differentiated services to their customers. These service awareness capabilities provide the network with the means to identify applications, services, or flows, service characteristics (e.g. latency sensitivity, bandwidth needs, security requirements), and the ability to dynamically adapt network settings to meet the needed/requested service requirements. This is a key capability to comply with SLAs. 6G services or applications (e.g. immersive communications or integrated sensing and communication services) may include multiple data streams (e.g. video, voice, text, sensor data, beyond communication services such as AI and sensing) that exhibit varied traffic patterns, and each component may have significantly different QoS requirements (e.g. latency, jitter, reliability, and throughput). The diversity and dynamism of such traffic patterns challenge traditional static or coarse-grained network management approaches. - 11.4 Use case on real time digital twins - 11.4.3 Service Flows Scenario 2: The second scenario indicates the real time digital twins with assistance from network, as shown in Figure 11.4.3-2. This scenario addresses cases where real time digital twin services are required, but some terminals in the scene have limited computing power or specific QoS requirements—such as strict guarantees on latency, jitter, and packet loss for critical service flows. In this case, in addition to Scenario 1, more 6G network capabilities are needed to compensate for terminal limitations and meet stringent QoS requirements, thereby ensuring the ultra-low latency, high reliability, and real time response required by users for digital twin applications. Figure 11.4.3-2: Real-time Digital Twins Example Scenario with Limited Computing Power and Specific QoS Requirements for Some Terminals.

To effectively enable immersive communication, it is imperative to meet appropriate jitter requirements and ensure deterministic latency. Based on the use case on 3D factory model based AR guided task in TR 22.870 [2], it mentioned the jitter is 10% of communication latency in general.

- 11.8 Use case on 3D factory model based AR guided task 11.8.1 Description Uplink user experienced data rate: The calculation of the uplink data rate needs to comprehensively consider the various data types generated by AR glasses or devices during real time interaction, including video streams, audio streams, sensor data (such as position, gestures, depth), environmental data, etc. Based on [24], for AR the typical steaming video and audio bitrates for real time 4K Ultra HD is from 5 Mbps to 25 Mbps with 4K resolution (3840 × 2160), audio (Stereo, 48 kHz, AAC encoding) needs 64 kpbs~128 kbps. The amount of sensors (including position, posture, gesture, High-resolution depth sensor, etc.) data depends on the data frequency and resolution which in general are 25 Mbps. In summary, the Uplink data rate is about 50 Mbps. If it is 8K or 12K, the data rate will be increased more. Regarding to the reliability and latency, TS 22.261 [14], it has specified 99.999% for industry control. So, for 6G, it is suggested to also consider 99.999% for similar scenarios. As to the jitter, according to the practice of typical smart factory, in general, the jitter is 10% of communication latency.

For various 6G services and applications in immersive communications, including haptic, visual and audio, different level requirements of composite performance of delay, jitter, data loss rate and data rate are mentioned [3]. It proposed jitter of 1-10ms for haptic, less than 30ms for visual and less than 30ms for audio. Different use cases may need different level of jitter requirements for immersive communication.

According to Recommendation ITU-T Y.3149 Quality of service assurance requirements for the tactile Internet [4], for haptic interpersonal communication (HIC), remote industry operation and tactile VR games, it needs less than 2ms to maintain extremely stringent stability and synchronization.

- Haptic interpersonal communication (HIC)

- Remote industry operation

- Tactile VR games

Hence, we have the following observations.

Observation: Different use cases may have different level of jitter, which are required to be addressed to ensure deterministic latency for immersive communication.

We see the performance indicators of latency, jitter, reliability, throughput or data rate, and packet loss are essential and crucial to Immersive Communication scenarios, based on SA study on 6G Use Cases and Service Requirements. However, the jitter has not yet been thoroughly discussed in 6G RAN study.

Jitter is a critical network performance metric defined as the variation in latency between successive data packets traversing a network, which has been formally known as Packet Delay Variation (PDV). It is quantitatively expressed as the statistical variance or standard deviation of the end-to-end delay. Additionally, jitter is not merely a secondary metric but a fundamental determinant of network utility for immersive communication applications. While latency defines the speed of communication, jitter defines its timely reliability and predictability. The presence of significant jitter necessitates larger de-jitter buffers at the receiver, which in turn increases overall latency, creating a design trade-off that is often unacceptable for ultra-responsive systems. Therefore, the controlling jitter is a prerequisite for achieving deterministic latency to obtain a guaranteed, bounded end-to-end delay for time-sensitive operations.

Hence, we propose to support jitter requirements for immersive communication.

Proposal 1: 6G RAN shall Support the requirement of jitter management to ensure user experience for immersive communication.

Text proposal for TR 38.914

We propose the following TP for Section 5.4 Requirements of New and Existing Services in TR 38.914.

### 5.4.2	Immersive Communication

6G RAN shall Support the requirement of jitter management to ensure deterministic latency for immersive communication.

Proposal 2: Adopt the following TP into TR 38.914.

### 5.4.2 Immersive Communication 6G RAN shall Support the requirement of jitter management to ensure deterministic latency for immersive communication.

Conclusions

In this contribution, we have the following observations and proposals:

Observation: Different use cases may have different level of jitter, which are required to be addressed to ensure deterministic latency for immersive communication.

Proposal 1: 6G RAN shall Support the requirement of jitter management to ensure user experience for immersive communication.

Proposal 2: Adopt the following TP into TR 38.914.

### 5.4.2 Immersive Communication 6G RAN shall Support the requirement of jitter management to ensure deterministic latency for immersive communication.

- References

- RP-252882, TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025.

- 3GPP TR 22.870, Study on 6G Use Cases and Service Requirements (Release 20), Oct. 2025.

- X. Wei, M. Zhang and L. Zhou, "Cross-Modal Transmission Strategy," in IEEE Transactions on Circuits and Systems for Video Technology, vol. 32, no. 6, pp. 3991-4003, June 2022, doi: 10.1109/TCSVT.2021.3105130.

- Recommendation ITU-T Y.3149, Quality of service assurance requirements for the tactile Internet, Aug. 2025.