3GPP TSG RAN Meeting #110                                                       RP-253251

Baltimore, USA, December 8-11, 2025

Source:	vivo

Title:	Discussion on 6G deployment scenarios

Agenda Item:	8.2.1.0

Document for:	Discussion and Decision

Introduction

In RAN#109, TR 38.914 V0.2.0 [1] was endorsed which captures the agreed scenarios and assumptions. However, some items for the agreed scenarios and some new deployment scenarios are still TBD. In this contribution, we provide our views on the UE antenna configurations, UE velocity etc. for the agreed scenarios. In addition, we provide our considerations on the Urban grid scenario, which can be used to evaluate the performance of sensing, mobility, AI related techniques etc.

Discussion

Urban grid scenario

Regarding the urban grid deployment scenario, we believe it is close to the real deployment and should be used to justify the 6G system performance. In TR 38.913 [2] and TR 38.775 [3], urban grid is defined for the V2X communication study. This scenario was also used in Rel-19 ISAC channel modelling with the additional 20m height assumption for the green blocks simulating the buildings as shown in the figures below, wherein there are 9 buildings (the green blocks in the figure) with four lanes and two sidewalks in between. TR 38.901 [4] already defines the urban grid for ISAC evaluation with 250m or 500m ISD. With the 3D buildings modeling in this scenario, BS can be located at the building sideways or at the corner of the intersection. Vehicular UEs are dropped on the roads according to spatial Poisson process and moving. In this setup, a vehicle changes its direction at the intersection as follows as described in [5]:

-	Go straight with probability 0.5

-	Turn left with probability 0.25

-	Turn right with probability 0.25

Figure 1, Urban grid with 500m ISD

Figure 2, Urban grid with 250m ISD

The urban grid deployment scenario where buildings and vehicles are placed in the urban area and vehicles are moving. It could cover a scenario where freeways cutting through an urban grid/area. The main KPI to evaluate under this scenario can be consistence UE experience, sensing-related performance and AI-related performance.

Therefore, this deployment scenario shall be considered for 6G and the assumptions are described in the following Table 1.

Table 1: Attributes for Urban grid

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz Around 7 GHz |
| Aggregated system bandwidth | Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 500m ISD 2: 250m |
| BS height | 25m |
| Building size | 413m x 230m x 20m |
| BS antenna elements | Around 4 GHz: Up to 576 for Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements | Up to 8 Tx and Rx antenna elements Note: Possible device types include handheld device, CPE, vehicle, sensing dedicated device etc. Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. |
| User distribution and UE speed | Urban grid model (car lanes and pedestrian/bicycle sidewalks are placed around a road block. 2 lanes (3.5m) in each direction, 4 lanes in total, 1 sidewalk (3m), one block size: 433m x 250m) 100% outdoor Vehicular UE in the lanes with up to 120km/h Pedestrian UE in the sidewalk with 3km/h UE changes its direction at the at the intersection as follows: - Go straight with probability 0.5 - Turn left with probability 0.25 - Turn right with probability 0.25 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

Support Urban gird as deployment scenario for 6G evaluation.

Sensing scenarios and parameters

In Rel-19 ISAC channel modelling study item, nearly all of the scenarios used for communication evaluation were considered for sensing where the BS and UE can be used as sensing nodes. The deployment scenarios in TR 38.914 and the key parameters as well as BS and UE antenna configurations are defined to model the typical deployment scenarios in real world. According to the use cases of communication and sensing, both communication and sensing can happen in any of these scenarios. Hence, the deployment scenarios in TR 38.914 and the corresponding parameters can be used to evaluate the performance of sensing. Accordingly, we have the following proposal:

All of the deployment scenarios in TR 38.914 can be used for evaluation of sensing performance.

The parameters of sensing targets, such as distribution and velocity, are highly related to the use cases to support sensing evaluation, these parameters can be discussed in RAN plenary or in working group.

The parameters assumption of sensing target can be discussed in RAN plenary or in working group.

UE antenna and velocity

Considering practical UE implementation and diverse device types to be supported in 6GR, we suggest to use same working assumption of maximum number of UE antenna elements as in TR 38.913 for around 4GHz and 30GHz. For around 7Ghz and 2GHz, we suggest the same working assumption with around 4GHz.

Considering the practical smart phone implementation, we consider 2Tx and 4Rx as baseline, propose a NOTE for further clarification.

The following number of UE antenna elements can be considered:

- Around 30 GHz: Up to 32 Tx and Rx antenna elements

- Around 2 GHz, around 4 GHz, around 7 GHz and around 15 GHz: Up to 8 Tx and Rx antenna elements Note

  - NOTE: Possible device types include handheld device, CPE, vehicle, sensing dedicated device, etc. Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone.

- Around 700 MHz: Up to 2 Tx and Rx antenna elements

On the supported UE velocity, considering ITU requirement of 500km/h UE speed and performance degradation at mid to high-speed scenarios in 5G deployment, we suggest to consider 3km/h, 30km/h, 60km/h, 120km/h, 500km/h for evaluation options. Considering the sensing scenarios and use cases, UE can be transmitter and receiver of sensing signal. For the use case of intruder detection, UE with sensing capability could be placed on some location to detect the intrusion. For the use case of health monitoring, especilly sleep monitoring, the UE with sensing capability could be placed around the human to monitor the sleep. Considering the IoT device, such as industrial sensors, which are widely used in the general environmental monitoring use cases such as monitoring of critical infrastructure (e.g., buildings, bridges, water dams, etc.), these UEs are also placed on some location. We sugget to add 0km/h as a UE velocity option.

The UE velocity of 0km/h, 3km/h, 30km/h, 60km/h, 120km/h, 500km/h can be considered for evaluation.

Conclusion

In this contribution, we provide our considerations on 6G deployment scenarios and the proposals are as following:

- Support Urban gird as deployment scenario for 6G evaluation.

- All of the deployment scenarios in TR 38.914 can be used for evaluation of sensing performance.

The parameters assumption of sensing target can be discussed in RAN plenary or in working group.

The following number of UE antenna elements can be considered:

- Around 30 GHz: Up to 32 Tx and Rx antenna elements

- Around 2 GHz, around 4 GHz, around 7 GHz and around 15 GHz: Up to 8 Tx and Rx antenna elements Note

  - NOTE: Possible device types include handheld device, CPE, vehicle, sensing dedicated device, etc. Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone.

- Around 700 MHz: Up to 2 Tx and Rx antenna elements

The UE velocity of 0km/h, 3km/h, 30km/h, 60km/h, 120km/h, 500km/h can be considered for evaluation.

Based on discussion in section 2, it is proposed to agree on the TPs below.

TPs for 38.914

=====================Start of the changes======================

4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 2 GHz: Up to 200 MHz (DL+UL) NOTE3. Around 4 GHz: Up to 300 MHz (DL+UL) NOTE3 Around 7 GHz: Up to 400 MHz (DL+UL) NOTE3 Around 15 GHz Up to 400 MHz (DL+UL) NOTE3 Around 30 GHz: Up to 1GHz (DL+UL) NOTE3 |
| Layout | Single layer: - Indoor floor (Open office) , 120m x 50m |
| ISD | 20m for around 30GHz 20m for around 4 GHz and 7 GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | TBD Around 700 MHz: Up to 2 Tx and Rx antenna elements Around 2 GHz: Up to 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 15 GHz: Up to 8 Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements Possible device types include handheld device, CPE, vehicle, sensing dedicated device etc. Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. |
| User distribution and UE speed | 100% Indoor, 3km/h, 0km/h [10] users per TRxP NOTE4 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| Sensing target | Option 1: TBD Option 2: To be discussed in working group. |
| NOTE 1: TBD NOTE 2: TBD NOTE 3: TBD NOTE 4: TBD |  |

4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro layer: Around 700MHz Around 2GHz Around 4 GHz Around 7 GHz Around 15GHz Around 2 GHz + Around 4 GHz Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700 MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 200MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100]m All micro TRxPs are all outdoor |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz: Up to 2 Tx and Rx antenna elements Around 2 GHz: Up to 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 15 GHz: Up to 8 Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements Possible device types include handheld device, CPE, vehicle, sensing dedicated device etc. Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP [10] users per TRxP with single-layer only 80% indoor (3km/h), 20% outdoor (30km/h, 60km/h, 0km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| Sensing target | Option 1: TBD Option 2: To be discussed in working group. |
| NOTE 1: TBD NOTE 2: TBD |  |

4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.3.

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7 GHz (ISD 1) Around 700 MHz + Around 7 GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz: Up to 2 Tx and Rx antenna elements Around 2 GHz: Up to 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Possible device types include handheld device, CPE, vehicle, sensing dedicated device etc. Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. |
| User distribution and UE speed | [15% outdoor vehicles (120km/h), 20% outdoor (3 km/h) and 70% indoor (3 km/h) 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| Sensing target | Option 1: TBD Option 2: To be discussed in working group. |

4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 2 GHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 2 GHz + Around 30GHz Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz: Up to 2 Tx and Rx antenna elements Around 2 GHz: Up to 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 15 GHz: Up to 8 Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements Possible device types include handheld device, CPE, vehicle, sensing dedicated device etc. Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. |
| User distribution and UE speed | 20% Outdoor: 30km/h, 60 km/h, 0km/h 80% Indoor: 3km/h [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| Sensing target | Option 1: TBD Option 2: To be discussed in working group. |

4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.] This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 700 MHz + Around 7 GHz Around 4 GHz +Around 7 GHz Around 2 GHz + Around 7 GHz + Around 30 GHz Around 2 GHz+ Around 700 MHz Around 7 GHz +Around 4 GHz + Around 2 GHz+Around 700 MHz |
| Aggregated bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer Hex. Grid [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz: Up to 2 Tx and Rx antenna elements Around 2 GHz: Up to 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 15 GHz: Up to 8 Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements Possible device types include handheld device, CPE, vehicle, sensing dedicated device etc. Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. |
| User distribution and UE speed | [10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 40km/h 80% Indoor in houses: 3km/h] [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| Sensing target | Option 1: TBD Option 2: To be discussed in working group. |

======================End of the changes======================

=====================Start of the changes======================

4.11	 [Urban grid]

The urban grid deployment scenario where buildings and vehicles are placed in the urban area and vehicles are moving. It could cover a scenario where freeways cutting through an urban grid/area. The main KPI to evaluate under this scenario can be consistence UE experience, sensing-related performance and AI-related performance.

Some of its attributes are listed in Table 4.11.

Table 11: Attributes for Urban grid

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz Around 7 GHz |
| Aggregated system bandwidth | Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 500m ISD 2: 250m |
| BS height | 25m |
| Building size | 413m x 230m x 20m |
| BS antenna elements | Around 4 GHz: Up to 576 for Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements | Up to 8 Tx and Rx antenna elements Possible device types include handheld device, CPE, vehicle, sensing dedicated device etc. Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. |
| User distribution and UE speed | Urban grid model (car lanes and pedestrian/bicycle sidewalks are placed around a road block. 2 lanes (3.5m) in each direction, 4 lanes in total, 1 sidewalk (3m), one block size: 433m x 250m) 100% outdoor Vehicular UE in the lanes with up to 120km/h Pedestrian UE in the sidewalk with 3km/h UE changes its direction at the at the intersection as follows: - Go straight with probability 0.5 - Turn left with probability 0.25 - Turn right with probability 0.25 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| Sensing target | Option 1: TBD Option 2: To be discussed in working group. |

======================End of the changes======================

Reference

[1]    RP-252882-3GPP TR 38.914 V0.2.0

[2]	3GPP TR 38.913: “Study on Scenarios and Requirements for Next Generation Access Technologies”.

[3]	3GPP TR 37.885: “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR”.

[4]	3GPP TR 38.901: “Study on channel model for frequencies from 0.5 to 100 GHz”.

[5]	3GPP TR 36.885: “Technical Specification Group Radio Access Network; Study on LTE-based V2X Services”.