3GPP TSG RAN Meeting #110					                     RP-253286

Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.4

Source:	Huawei, HiSilicon, China Telecom

Title:	Consideration on supporting immersive communication services for 6G

Document for:	Discussion

# Typical use cases

Immersive services are supposed to be the mainstream for future wireless communication networks. Recently, advanced achievements in AI have further broadened the scope of immersive applications, from 5G-era devices like XR to innovative mobile AI devices. These immersive experiences are expected to be seamlessly integrated into our daily life, from traveling and entertainment to working, education, and eldercare.

The typical use cases of immersive communication services may include:

- High resolution immersive gaming: Immersive gaming with 8K resolution display with 90fps has  been defined in Section 9.1 of TR 22.870 [1]. However, for XR display devices with angular resolution below around 60 PPD (Pixels Per Degree), a so-called “Screen Door Effect” is expected, whereas users can perceive individual display pixels (i.e. pixel boundaries), creating a mesh-like visual obstruction [2]. Therefore, higher solution display such as 12K/16K is preferred for immersive user experience.

- UL heavy video uploading: The real-time video upload are common for immersive services. Specifically, for use cases defined in section 6.28 of TR 22.870 [1], videos captured by high-resolution cameras of VR/AR glasses or embodied robots should be uploaded to cloud servers for AI inference. For use cases defined in section 9.3 & 9.4 of TR 22.870 [1], the videos captured by the cameras by VR/AR glasses or mobile phones need to be uploaded to the cloud in XR rendering task offloading operation. For use cases defined in section 9.8 & 9.10 of TR 22.870 [1], the videos captured by the cameras of XR devices should be uploaded and transmitted to remoted places for real-time healthcare. The real-time video upload to support these use cases are generally UL-heavy, and exhibits several key characteristics including: quasi-periodic data arrivals, large packet sizes, and low packet delay budget, just like XR traffic model defined in TR38.838 [3].

# Different level of composite requirement

The aforementioned typical use cases for immersive communication brings new requirements for future network. The service flow of these use cases is firstly discussed in the following Section 2.1 and then the composite requirement with different level of data rate and latency is discussed in Section 2.2.

2.1 Service flow

For the aforementioned UL heavy video uploading cases, the service workflow is illustrated in Figure 2.1.1, which consists of the following steps.

Figure 2.1.1 Service flow of mobile AI services

- Step 1: Device collects necessary data unit for AI inference.

- Step 2: Device uploads the data unit to cloud server through 6G network.

- Step 3: Cloud server performs AI inference/rendering based on received data unit.

- Step 4: Cloud server sends the results to the device through 6G network.

For the high resolution immersive gaming, the service flow can be referred to TR 38.838 [3]. This is similar to Figure 2.1.1, where the upload data includes pose/control information collected by XR devices, while the download data includes the high-resolution rendering video from the cloud server.

2.2 Composite Requirement for different use cases

2.2.1 DL/UL Service Data Rate Requirement

Based on the aforementioned service flow, it can be expected that the high resolution immersive gaming and video uploading are downlink-heavy and uplink heavy, respectively.

For high resolution immersive gaming, the downlink service data rate can be from 100Mbps to 500Mbps. Taking cloud gaming for example, the refresh rate should be 90 fps, or even 120 fps to provide smoother user experience as defined in TR 22.870 [1]. In the meanwhile, the resolution should be 8K or even 12K/16K to avoid the “Screen Door Effect”. The corresponding downlink service data rate can be computed as shown in Table 2.1.1.

For UL heavy video uploading, the uplink service data rates could scale from 20Mbps to 100Mbps, depending on different AI workloads. Taking the embodied AI robots as an example, there are several options for robots: 1) 20m gesture recognition, where 6*1080p cameras are equipped in the intelligent terminal; 2) 30m 180° micro expression capture capability, where the front view cameras (dual camera similar to “human eyes”) are replaced with 4K resolution and 30 fps refresh rate; 3) 30m 360° micro expression capture capability, where two additional cameras are upgraded to 4K.  The corresponding uplink service data rate can be computed as shown in Table 2.1.2.

2.2.2 DL/UL Latency Requirement

For high resolution immersive gaming, TR 38.838 has already preserved a low 50-70ms motion-to-render-to-photon (M2R2P) latency [3], which is often treated as the end-to-end (E2E) latency. Deducting the processing time at the XR devices, the round trip time at transport network, and the rendering time at cloud server, a minimum downlink RAN latency of 10ms is considered for DL cloud gaming frame with the packet success probability of 99%, as described in TR 38.838 [3].

For UL heavy video uploading, a typical E2E latency of 200ms for human involved services should be supported to maintain comfortable and smooth human interaction [4]. This value can be decomposed as follows: 1) 160ms cloud inference latency according to the latest Gemini 2.0 capability [5]; 2) 10ms UE processing latency; 3) 10ms transport network latency; 4) 20ms uplink and downlink RAN latency. Considering that the uplink traffic is heavier for mobile AI, a minimum RAN latency should be 15ms with the packet success probability of 99%.

2.2.3 Summary of different level of composite requirement

In conclusion, different levels of composite requirement are summarized in Table 2.1.1 and Table 2.1.2, following the latest working assumption of immersive traffic model in RAN1 #123 [6]. Note that the number of satisfied users supported for these services also matters, as the network needs to support multiple users at the same time to meet the composite requirements. The values are obtained based on 7GHz with 400 bandwidth and 256TRX MIMO, while further considering uplink coverage enhancement, multiple TRP coordination and interference management, and cross-layer PDB negotiation.

Table 2.1.1: Downlink composite requirements for immersive gaming

| Use case | Refresh rate | Service data rate[NOTE 1] | DL RAN latency | Packet success probability | Number of users ([/km2 or /TRxP]) |
| --- | --- | --- | --- | --- | --- |
| 8K immersive gaming | 90fps | 100Mbps | 10ms | 99% | ~30/TRxP （ISD 200m） |
| 12K immersive gaming | 90fps | 300Mbps | 10ms | 99% | ~6/TRxP （ISD 200m） |
| 16K immersive gaming | 120fps | 500Mbps | 10ms | 99% | ~2/TRxP （ISD 200m） |
| NOTE 1: Data Rate = Resolution × Bit Per Pixel × Refresh Rate / Compression Ratio, where compression ratio of 400 and 12 bits per pixel (YUV4:1:1 compression encoding) are considered [7]. |  |  |  |  |  |

Table 2.1.2: Uplink composite requirements for UL heavy video uploading

| Use case | Refresh rate | Service data rate[NOTE 1] | UL RAN latency | Packet success probability | Number of users ([/km2 or /TRxP]) |
| --- | --- | --- | --- | --- | --- |
| AI robots with 6*1080 video uploading | 15fps@1080p | ~ 20 Mbps | 15ms | 99% | ~30/TRxP （ISD 200m） |
| AI robots with 4*1080 + 2*4K video uploading | 15fps@1080p，30fps@4K | ~ 60 Mbps | 15ms | 99% | ~6/TRxP （ISD 200m） |
| AI robots with 2*1080 + 4*4K video uploading | 15fps@1080p，30fps@4K | ~100 Mbps | 15ms | 99% | ~2/TRxP （ISD 200m） |
| NOTE 1: Data Rate = Video resolution * (Bits per colour*3) * Refresh rate / Compress ratio. The data rate is estimated assuming: compression ratio of 240 and 8 bits per colour. |  |  |  |  |  |

Proposal 1:  The 6G RAN design shall support different levels of the composite requirement for different use cases as follows:

| Level of composite requirement | Service data rate (Mbps) | RAN Latency (ms) | Packet success probability | Number of users ([/km2 or /TRxP]) |
| --- | --- | --- | --- | --- |
| UL |  |  |  |  |
| AI robots with 6*1080 video uploading | 20 | 15 | 99% | ~30/TRxP （ISD 200m） |
| AI robots with 4*1080 + 2*4K video uploading | 60 | 15 | 99% | ~6/TRxP （ISD 200m） |
| AI robots with 2*1080 + 4*4K video uploading | 100 | 15 | 99% | ~2/TRxP （ISD 200m） |
| DL |  |  |  |  |
| 8K immersive gaming | 100 | 10 | 99% | ~30/TRxP （ISD 200m） |
| 12K immersive gaming | 300 | 10 | 99% | ~6/TRxP （ISD 200m） |
| 16K immersive gaming | 500 | 10 | 99% | ~2/TRxP （ISD 200m） |

# Requirement for consistent user experience

Immersive services impose stringent network performance requirements, including high data rate, low latency, and high reliability simultaneously. Such user experience should be consistent no matter whether the user is at cell center or cell edge. It is important for satisfying user experience, e.g. robust control of embodied AI robot, and smooth reaction of AI phones/wearables when the devices are moving round the cell. Considering the number of users supporting immersive communication increase gradually, it is important that the number of satisfied users can be increased to match the fast development of corresponding applications.

However, due to poor coverage (especially for uplink) and multi-cell interference, satisfying user experience becomes more challenging at cell-edge. At cell edge, users are usually in coverage hole and closer to neighbouring cells. Therefore cell edge users are more likely to suffer from low SINR, which may cause sudden degradation of transmission quality. In 5G, multiple TRP has been introduced, which is a good direction to improve cell edge performance. In 6G, multiple TRP direction shall continue to be considered to satisfy new services requirements and provide consistent user experience, by better collaboration to reduce multi-cell interference. Moreover, coverage enhancement, especially for uplink, should also be considered.

Proposal 2: 6GR shall support efficient mechanism to increase the number of immersive communication users.

For immersive interactive services as introduced above, a smooth user experience also relies on whether the end-to-end (E2E) latency is met. The latency would vary even for the same application. Take embodied AI robots as an example. When the AI robot executes actions such as obstacle avoidance, a short E2E latency is required to guarantee the safety, especially in the moderate or high mobility scenario. When the AI robot executes the actions such as route planning, the E2E latency can be longer, since the complex and precise inference is required. Therefore, the latency requirement can be changed from time to time and how to satisfy such changeable E2E latency needs to be addressed to provide consistent user experience.

As shown in Figure 2.1.1, the 3GPP E2E latency mainly includes air interface (Uu) latency and transport layer (N3) latency. These two parts of latency are usually change frequently due to the time-varying wireless channel and congestion control. For example, in current XR traffic model in TR38.838 [3], an ±4ms jitter is assumed for the N3 latency. This not only affect the traffic flow from the OTT layer, but also make the latency requirement of each part uncertain, even given the fixed 3GPP E2E latency. Due to this situation, the delay budget for air interface and transport layer may need adjustment as well. If this is not adjusted timely, the latency over air interface has to always set to satisfy the minimized latency requirement, which could be too stringent. A better way is to jointly negotiate the delay budget between different interfaces, which can give more flexibility for the network to schedule resources in an efficient way as shown in Figure 3.1.1.

Figure 3.1.1. Delay budget collaboration among different layers.

The delay budget collaboration needs accurate and timely delay measurement for each interface, and subsequently RAN and Core network can have coordination on how much delay budget can be allocated for each interface. In 5G L2 delay over air interface is measured per DRB as an average value, which cannot differentiate packets with different QoS requirements. Therefore, more timely latency measurement and QoS coordination between different interfaces shall be supported in 6G, to adapt to the varying latency requirement from applications.

Proposal 3:  6GR shall support timely measurements for latency over Uu and transport (including jitter), to enable efficient resource utilization while maintaining E2E latency requirement.

# Summary

According to the above discussion, it is proposed to adopt proposal 1 into TR 38.914 “key performance indicators” section, and adopt proposal 2 and 3 into TR 38.914 “Requirements of New and Existing Services” section.

Proposal 1:  The 6G RAN design shall support different levels of the composite requirement for different use cases as follows:

| Level of composite requirement | Service data rate (Mbps) | RAN Latency (ms) | Packet success probability | Number of users ([/km2 or /TRxP]) |
| --- | --- | --- | --- | --- |
| UL |  |  |  |  |
| AI robots with 6*1080 video uploading | 20 | 15 | 99% | ~30/TRxP （ISD 200m） |
| AI robots with 4*1080 + 2*4K video uploading | 60 | 15 | 99% | ~6/TRxP （ISD 200m） |
| AI robots with 2*1080 + 4*4K video uploading | 100 | 15 | 99% | ~2/TRxP （ISD 200m） |
| DL |  |  |  |  |
| 8K immersive gaming | 100 | 10 | 99% | ~30/TRxP （ISD 200m） |
| 12K immersive gaming | 300 | 10 | 99% | ~6/TRxP （ISD 200m） |
| 16K immersive gaming | 500 | 10 | 99% | ~2/TRxP （ISD 200m） |

Proposal 2: 6GR shall support efficient mechanism to increase the number of immersive communication users .

Proposal 3: 6GR shall support timely measurements for latency over Uu and transport (including jitter), to enable efficient resource utilization while maintaining E2E latency requirement.

# Text proposal for the TR

### 5.4.2	Immersive communication services

6GR shall support efficient mechanism to increase the number of immersive communication users.

6GR shall support timely measurements for latency over Uu and transport (including jitter), to enable efficient resource utilization while maintaining E2E latency requirement.

# References

- 3GPP TR 22.870, Study on 6G Use Cases and Service Requirements.

- https://www.displaymodule.com/blogs/knowledge/exploring-the-screen-door-effect-in-virtual-reality-phenomenon-impact-and-solutions.

- 3GPP TR 38.838, Study on XR (Extended Reality) Evaluations for NR.

- https://humanbenchmark.com/tests/reactiontime.

- G. R. Team, S. Abeyruwan, J. Ainslie, et al., “Gemini robotics: Bringing AI into the physical world,” arXiv preprint arXiv:2503.20020, 2025.

- R1-2509390 FLS#6 on evaluation assumptions for 6GR air interface

- https://std.samr.gov.cn/hb/search/stdHBDetailed?id=2001448F0D0D32F5E06397BE0A0AAD9C.