3GPP TSG RAN Meeting #110												RP-253288

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.4

Source:	Huawei, HiSilicon, CAICT, CMCC, China Unicom, China Telecom, OPPO, Lenovo, TCL, Google

Title:	Consideration on supporting new AI services for 6G

Document for:	Discussion and decision

# Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107[1]. Regarding the services in the scenario, the objective of this SID includes:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

- …..

In RAN#108, one of the objectives of the approved SID about “study on 6G Radio” is to support various services and functionalities, where AI/ML related services serve as examples[2]:

The detailed objectives of the study are:

…

(3)Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

…

e)	Data transfer design to support various types of data (e.g., AI/ML, Sensing, etc.) [RAN2, RAN3]

…

(6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g., AI/ML, sensing, etc.). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

…

(8)	AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc.…

Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc.…)

NOTE: lead WG depends on the use case.

b)	AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

i.	LCM procedures [RAN2, RAN1, RAN3, RAN4]

ii.	Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

3GPP SA1 has begun the study on AI/ML related use cases for 6G [3],  which cover aspects related to NET4AI. For the communication service to AI applications, the special characteristics related to the AI traffic for the AI applications should be considered and supported efficiently, while providing good user experience. The requirements of these use cases are identified for the whole 6G system and RAN shall also support these service requirements. This paper is to analyze RAN requirements to support AI services.

# Consideration on 6G Radio for supporting new AI traffic

The AI services powered by large AI/ML models such as large language models (LLM), vision language models (VLM), multi-modal language models (MLM), visual language action models (VLA), etc., are utilized widely for main-stream AI services, e.g., AI agent, chatbot, personal assistant, autonomous driving, AI phone/wearables, and embodied AI robots. The following analysis takes token communication as an example to illustrate the characteristics for these new AI traffic, as also introduced in SA1 use case [4][5].

## New AI traffic example: Token communication

In the AI services powered by large AI/ML models, instead of transmitting raw input data (e.g., text, images), transforming them into Tokens becomes popular for efficient transmission and processing as shown in [6]. In [7], it shows that by 2027, the daily Token consumption of Doubao (A famous mobile AI application in China) is projected to exceed 100 trillion Tokens per day. This may bring new business opportunities for operators by providing services adapting to such an application, considering such a rapid development and huge consumption of Tokens.

Figure 1 Illustration of 6G Token communication service for Agentic AI

Token is seen as a compact data carrying the context information, and can be generated and processed by AI models as shown in Figure 1. A Tokenizer converts the raw data of multiple modalities (e.g., text, image, audio, etc.) to Tokens at the transmitter side, and a corresponding de-Tokenizer converts the Tokens back to the raw data. The tokenizers for image or video data have received widespread attentions, e.g., MAGVIT-v2 [8], ElasticTok [9], Chamelon [10], TiTok [11], Liquid [12], FlexTok [13], etc. The results from the tokenizers can be processed by the AI/ML models directly, compressing the raw data and avoiding unnecessary compression and decompression operations in remote inference scenarios. Many tokenizers for audio data have also been developed, e.g., Lyra [14], EnCodec [15], FunCodec [16] etc, which supports audio data compression and downstream tasks. Some of the tokenizers have been used in the companies’ applications. For example, Lyra has been integrated in Google’s voice communication applications [17], and EnCodec has been used in Meta’s AudioCraft applications [18]. Furthermore, some popular inference service frameworks or Application Programming Interfaces (APIs) support input tokens directly [19][20]. In this scenario, the tokenizer is deployed at the client, which transforms the raw data to tokens and transmits the tokens to the server. The AI/ML model at the server processes the input tokens and generates output tokens, which are transmitted to the client for de-tokenization.

The Tokens generated by/for AI/ML models have lots of special characteristics, which are different from traditional traffics. Each Token plays a central role in enabling AI agents (as described in SA1 use cases in TR 22.870 [3]) to understand and respond to natural language instructions, interpret environmental data, make decisions for robotic systems, communicate with other AI agents, etc.

## Characteristics of New AI traffic

Figure 2 Image generation with/without Token prediction (image from ImageNet[21])

Token communication refers to the transmission of Tokens over the air interface. One important characteristic of Token transmission is error tolerance capability. The below Figure 2 shows the difference between the cases with and without Token prediction for image reconstruction, considering different Token error rate. Without Token prediction, when the error rate is larger than 5%, the image reconstruction quality is degraded significantly; and with Token prediction via generative or predictive capabilities from AI models, even when the error rate is up to 50%, the image reconstruction quality seems still acceptable.

Such resilience of error tolerance can bring new opportunities for operators to utilize spectrum resources in an efficient way. To be more specific, the existing design of transmission over radio is on a per bit basis to ensure every bit is correct, consequently reliability design is introduced from physical layer to higher layers, including PHY retransmission, L2 HARQ/ARQ etc. If the transmission is considered to be per Token basis and only part of these functions is selected according to the error tolerance capability, the resources scheduled for Token communication can be significantly saved, which will be beneficial for operators to support higher number of users, or provide more resources for edge users to improve coverage and cell edge performance.

Figure 3 Impact of different Tokens on the performance.

(Left: Tokens of different modality data for text Visual Question Answering task [24]. Right: different Tokens of image data for reconstruction.)

Tokens may also have different priorities or importance [22][23]. Firstly, Tokens serve for different modality may take different importance to the error tolerance capability. For example, as shown in Figure 3(left), the image Tokens can be predicted more easily and more tolerant to error rate, while text Tokens are more sensitive to error rate and cause accuracy degradation significantly once error rate is larger than 1%. Secondly, the Tokens of the same modality data may also have different importance weights. For example, as shown in Figure 3(right), the importance of Tokens may have a decreasing order with their Token positions for an image, where the loss of the preceding Tokens would cause higher reconstruction error than the subsequent Tokens. With the preceding Tokens being transmitted correctly, the importance of the subsequent Tokens will decrease. Hence, based on the Token importance, different transmission priority of Tokens can be utilized to improve the transmission efficiency.

The above analysis has also been discussed in SA1 6G study and [4] has been agreed to be included into the TR with the following agreements:

[PR 6.x.6-1] Subject to operator policy, the 6G network shall be able to  provide communication service for AI applications with various traffic characteristics (e.g. modality, tolerated error rate, priority, generation/arrival rate of data) efficiently for the AI applications to guarantee user experience.

The agreements reflect the general service requirements for AI applications, which includes characteristics including modality, tolerated error rate, etc. These characteristics need to be addressed in 6G RAN design to satisfy the user experience.

Observation1: Service requirments for new AI applications have been defined in TR 22.870 and shall be addressed in 6G RAN design to guarantee user experience.

## Improved RAN for new AI traffic

According to the above analysis, it is seen useful to study mechanisms considering for new AI traffic.

The network can utilize the characteristics of the above AI traffic to improve the transmission efficiency. Taking tokenized AI traffic as an example, RAN needs to be aware about the Token pattern, e.g., allowed error rate for a (group) of Tokens, importance of the Token etc. This may bring new QoS type and parameters for Token communication which needs E2E design.  One example is shown in Figure 4, which include specific QoS types (related to error tolerance and traffic patterns), enabling partial reliability to Token traffic (related to error tolerance).

Figure 4 Example of transmission mechanism to facilitate Token communication

During AI/ML related discussions in RAN1, it was agreed to study the traffic for AI/ML services in general, with coordination with other WGs. Potential RAN mechanism considering AI traffic characteristics to efficiently support AI services have also been proposed and discussed, e.g., improvement of scheduling and HARQ [25].  As shown in simulation results in [26][27][28][29][30][31][32][33], the potential improvement can increase the supported number of UEs or expanding the service area for the operators. Therefore, to provide efficient transmission mechanism for new AI services (including both token communication and non-token communication) is one important aspect to be addressed in 6G Day One.

Proposal 1: 6G RAN shall support efficient scheme(s) for AI/ML services with various traffic characteristics (e.g. modality, tolerated error rate, priority, generation/arrival rate of data) to guarantee user experience.

# Summary

Based on the above example and analysis, we propose

Observation1: Service requirments for new AI applications have been defined in TR 22.870 and shall be addressed in 6G RAN design to guarantee user experience.

Proposal 1: 6G RAN shall support efficient scheme(s) for AI/ML services with various traffic characteristics (e.g. modality, tolerated error rate, priority, generation/arrival rate of data) to guarantee user experience.

Proposal 2: Adopt the text proposal for AI/ML services requirements in TR38.914 as shown in Section 5.

# TP for 38.914

In section 5.4 Requirements of New and Existing Services in TR38.914, the text proposal is provided as follow:

-------------------------------------------------Start the text proposal ------------------------------------------------

### 5.4.5	AI

6G RAN shall support efficient scheme(s) for AI/ML services with various traffic characteristics (e.g. modality, tolerated error rate, priority, generation/arrival rate of data) to guarantee user experience.

-------------------------------------------------End the text proposal ------------------------------------------------

# References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.

- RP-251881, “New SID: Study on 6G Radio”, RAN#108, June 2025.

- 3GPP TR 22.870: “Study on 6G Use Cases and Service Requirements”.

- S1-254437, New use case on 6G system providing Token Communication service, SA1#112, Nov. 2025.

- S1-254381, Updated use case on optimizing user experience for GenAI applications, SA1#112, Nov. 2025.

- Bondcap, “Trends - Artificial Intelligence”, May 2025

- https://xueqiu.com/9332969756/321418828

- L. Yu et al, Language Model Beats Diffusion -- Tokenizer is Key to Visual Generation, ICLR, 2024.

- W. Yan, et al, ElasticTok: Adaptive Tokenization for Image and Video, ICLR, 2025.

- Chameleon Team, Chameleon: Mixed-Modal Early-Fusion Foundation Models, 2025.

- Q. Yu et al, An Image is Worth 32 Tokens for Reconstruction and Generation, NeurIPS, 2024.

- J. Wu et al, Liquid: Language Models are Scalable and Unified Multi-modal Generators, 2025.

- R. Bachmann, et al, FlexTok: Resampling Images into 1D Token Sequences of Flexible Length, ICML, 2025.

- https://research.google/blog/lyra-a-new-very-low-bitrate-codec-for-speech-compression/

- A. Défossez et al, EnCodec: High Fidelity Neural Audio Compression, TMLR, 2023.

- Z. Du et al, FunCodec: A Fundamental, Reproducible and Integrable Open-source Toolkit for Neural Speech Codec, ICASSP, 2024.

- https://research.google/blog/lyra-a-new-very-low-bitrate-codec-for-speech-compression/

- https://audiocraft.metademolab.com/

- https://developer.nvidia.com/blog/deploying-gpt-j-and-t5-with-fastertransformer-and-triton-inference-server

- https://github.com/vllm-project/vllm/blob/bda41c70ddb124134935a90a0d51304d2ac035e8/vllm/entrypoints/llm.py#L96

- J. Deng, et al, ImageNet: A Large-Scale Hierarchical Image Database, IEEE Conference on Computer Vision and Pattern Recognition (CVPR), 2009.

- Liu Z, Ma Y, Tafazolli R. ResiTok: A Resilient Tokenization-Enabled Framework for Ultra-Low-Rate and Robust Image Transmission[J]. arXiv preprint arXiv:2505.01870, 2025.

- K. Miwa, et al. One-D-Piece: Image Tokenizer Meets Quality-Controllable Compression, ICML Tokenization Workshop, 2025.

- A. Singh, et al, Towards VQA Models That Can Read, Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), 2019.

- Final Report of 3GPP TSG RAN WG1#123, November 17th-21st, 2025.

- R1-2508740,	Views on AI/ML in 6GR air interface, Huawei, RAN1#123, November 17th-21st, 2025.

- R1-2508732,	AIML use cases for 6GR air interface,	OPPO, RAN1#123, November 17th-21st, 2025.

- R1-2508947,	AI/ML in 6GR Air Interface, Google, RAN1#123, November 17th-21st, 2025.

- R1-2508460,	Discussion on AI/ML in 6GR interface, CMCC, RAN1#123, November 17th-21st, 2025.

- R1-2509004, AI/ML in 6GR, Lenovo, RAN1#123, November 17th-21st, 2025.

- R1-2509001, Views on improved scheduling/HARQ for token traffic, CAICT, RAN1#123, November 17th-21st, 2025.

- R1-2509417,	New use cases for AI/ML in 6GR interface,	Pengcheng Laboratory, ZGC Institute of Ubiquitous-X Innovation and Application, RAN1#123, November 17th-21st, 2025.

- R1-2509411, Discussion on AI/ML-enabled use cases for 6GR	, BUPT, ZGC Institute of Ubiquitous-X Innovation and Application, Pengcheng Laboratory, RAN1#123, November 17th-21st, 2025.