3GPP TSG RAN Meeting #110												RP-253291

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.1

Source:	Huawei, HiSilicon, CAICT, China Telcom, China Unicom, KT Corp., LG Uplus，SK Telecom, Telecom Italia

Title:	Text proposal of ISAC KPIs for 6G

Document for:	Discussion and decision

Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107[1]. Regarding the services in the scenario, the objective of this SID includes:

This study item aims to

- Investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

- …..

Referring to the ISAC services discussed in agenda item 8.2.1.4, and according to the detailed illustration for new ISAC use cases and services [2], we propose the following text proposals for the relevant KPIs of 6G ISAC services in this contribution.

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.

- RP-253290, Consideration on supporting ISAC services for 6G

Text proposal for 38.914

In section 5 Requirements and key technical principles in TR38.914, the text proposal for section 5.4 are provided as follow:

-------------------------------------------------Start the text proposal ------------------------------------------------

### 5.1.17	Sensing

Sensing-related capabilities are measured in terms of the following technical performance requirements:

•	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present.

•	Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error.

•	Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

•	[Sensing Resolution: TBD]

6G RAN shall support use case of target detection and tracking, e.g. road digitization, with the below performance target:

Static target or moving target up to 5km/h:

- Localization accuracy: horizontal 1m

- Velocity accuracy: 1m/s

Moving target up to 120km/h:

- Localization accuracy: horizontal 3m

- Velocity accuracy: 5m/s

The 6G system shall be able to provide sensing service by detecting and tracking the characteristics of individual environmental object as described:

- [sub meter to meter-level] reconstruction accuracy

-------------------------------------------------End the text proposal ------------------------------------------------