3GPP TSG RAN Meeting #110		RP-253295

Baltimore, USA, December 8-11, 2025

Agenda item:	9.3.1.3

Source:	         Xiaomi

Title:	Discussion on the check points of AI/ML for NR air interface Phase 2

Document for:	Discussion and Decision

- Introduction

In RAN#108 meeting, a new WID [1] on two-sided AI/ML model was achieved, the objectives are as following:

AI/ML framework for two sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_air_Ph2 and NR_AIML_air projects for CSI feedback enhancement [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model selection, activation, deactivation, switching, fallback - ID related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring. CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air, including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Data collection [RAN2]: - Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

With the WID, some progress has been made across working groups. Several checkpoints need to be confirmed or discussed in the RAN #110 meeting. In this contribution, we present our perspectives on the aforementioned checkpoints.

- Inter-vendor training collaboration

  - Check point for standardized reference model

RAN4 has provided its evaluation concerning the feasibility of the reference model directions identified in [1] According to the RAN4 liaison response in [2], RAN4 has discussed both the fully defined/specified reference model (“Direction C”) and the standardized encoder model structure with parameter exchange (“Direction A, sub-option 3a-1”, without target CSI sharing). And RAN4 confirms that the decoder and encoder structures previously assessed under Rel-19 Option 3 remain applicable and feasible, and the corresponding reference decoder/encoder mappings to Direction C are fully consistent. In addition, RAN4 notes that the encoder-side reference model for Direction A sub-option 3a-1 can be technically supported within the current study and does not have feasibility issues. The liaison also notes that the detailed procedures for proceeding these models have been discussed and agreed.

Observation 1: RAN4 confirms that both the fully defined/specified reference model (“Direction C”) and the standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) are feasible.

Given this technical confirmation from RAN4, the check points on feasibility of fully defined/specified reference model (“Direction C”) and specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) included in [1] have been addressed. The check point no longer provides additional gating value for the continuation of the WI and can therefore be removed.

Proposal 1: RAN agrees to remove the check point on Direction C and sub-option 3a-1 under Direction A from the WID, as feasibility has been confirmed by RAN4.

  - Check point for standardized dataset exchange

Recalling that, during RAN2 #129bis meeting, RAN2 sent a reply LS to RAN1 with the following options identified for dataset/model parameter transfer:

Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) Alternative 2 (OTA approach): gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server) NOTE: The data transfer (up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.

RAN2 further considered there’s no consensus on OTA solution’s feasibility, and further confirms non-OTA solution’s feasibility but further confirmations from RAN3/SA2/SA5 are needed.

Based on the progress of other WGs, we here summarize the current status of OTA and non-OTA solutions:

| Solution | Options | WG [Most Recent Reply LS] | Conclusion/Status |
| --- | --- | --- | --- |
| OTA solution |  | RAN2 [R2-2503169] | No consensus on feasibility |
| non-OTA solution | gNB -> OAM -> UE side training entity | RAN2 [R2-2503169] | Feasible |
|  |  | RAN3 [R3-253961] |  |
|  |  | SA5 [S5-254083] | Feasible via reporting and collection mechanisms defined in TS 28.622, as long as: 1) The UE-side training entity is an authorized MnS consumer. 2) The NW-side dataset/model parameter (collected from gNB) conforms to the SA5 definition for management data. (more evaluation based on RAN1 progress of dataset/model parameter definition) |
|  | gNB -> CN -> UE side training entity | RAN2 [R2-2503169] | Not supported in Rel-19 and No evaluation on feasibility |
|  |  | RAN3 [R3-253961] |  |
|  |  | SA2 [S2-2508104] | The required functionality is not supported in Rel-19. Exposure capability to UE-side training entity needs to be enhanced based on requirement. Waiting for SID updated with the corresponding objective for study the feasibility. |

Based on the above table, it is clear that dataset/model parameter transfer via non-OTA solution (gNB -> OAM -> UE-side training entity) has been confirmed feasible by all related WGs (RAN2, RAN3, SA5). Though SA5 mentioned some enhancements to make sure the dataset/model parameter defined by RAN1 can conform SA5 definition for management data, we noticed that RAN1 has made significant progress on the definition of dataset to be transferred in RAN1#123 meeting. To our understanding, the expected evaluation and normative work can be completed within R20 timeline from both RAN WGs and SA5 point of view.

Agreement in RAN1#123: When the target CSI is precoding matrix with port-subband domain representation, for the association between target CSI and CSI feedback in the exchanged dataset, - A pairing ID has K target CSI sets, - Each target CSI set has Sk target CSI samples, 1≤k≤K, Sk>1. - Each target CSI sample within the same target CSI set k has a port dimension of N1x N2x2, and the maximum number of subbands, and a rank Vk - Each target CSI sample within the same target CSI set k has Vk layers, ordered from the strongest to the weakest layer, where Vk is the rank - Target CSI set k is associated with M Mk CSI feedback sets, - Each CSI feedback set corresponds to a specific per layer CSI payload size configuration for a rank and subband configuration - FFS: rank (CSI feedback rank can be different compared to target CSI rank) - FFS: whether per layer CSI payload size is the same for all the layers of all the samples of the same CSI feedback set. - Each CSI feedback sample within the set includes CSI feedback per layer, ordered from the strongest to the weakest layer. - The target CSI set and each of the corresponding CSI feedback set has the same number of samples - Metadata: - Per Target CSI set metadata: for each 1≤k≤K set, number of samples Sk, the port configuration of N1 and N2, subband size and BWP info (NR ARFCN of the lowest PRB of the BWP and BWP size and the SCS), rank Vk, number of associated CSI feedback set Mk - FFS: the number of aggregated CSI-RS resources for total number of antenna ports larger than 32, and the portMappingMethod applicable to aggregated resources - For each CSI feedback set metadata: CSI payload size configuration, subband configuration - FFS: Additional CSI feedback metadata to indicate when specific scalability option is used in model design (e.g., truncation/masking) is used as CSI feedback payload scalability option. - FFS: rank - Other metadata are not precluded. - How the association between the kth target CSI set and Mk CSI feedback sets is indicated is up to SA WG.

However, on the other hand, SA2 has not started any evaluation on the feasibility analysis. It is further observed that whether there’s a SID objective is still subject to SA plenary approval, considering the challenging TU allocation in SA2. Therefore, considering the current progress from all WGs, it is proposed:

Proposal 2: For inter-vendor training collaboration for two-sided AI/ML models, RAN removes the check point and confirms dataset/model parameter transfer is supported via non-OTA solution gNB -> OAM -> UE-side training entity in R20 WI.

- UE-side data collection

Another check point on R20 AI/ML air interface is on whether and how UE-side data collection is supported.

- Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection

SA2 provided feedback and outcome of related SA2 study in S2-2510958 [7]

- As feedback to request in RP-243316, SA2 would like to provide information on the outcome of the related SA2 study FS_AIML_CN_Ph2 (SP-250810). - SA2 has reached interim agreements with remaining open issues on how to support the transfer of standardized data over UP for UE data collection for UE-side model training as described in clauses 7.1.1 and 7.2.1 of TR 23.700-04 v1.2.0. - Additionally, SA2 would like to ask for RAN/RAN2 to provide feedback on whether Option 2 UP is selected.

Based on SA2’s study outcome, we here summarize a high-level solution and remaining open issues:

- No consensus on whether A new 5GC entity is defined as the UP path termination entity to support the transfer of data over UP for UE data. - UP path information on the UE can be locally configured or sent from 5G CN, FFS to select which option (a or b or both). - Data transfer procedure - UMTES->CN->UE - How the mapping between the requested data id and the data to be transferred is determined is FFS. - CN->RAN, requested data id , which may assist the gNB to decide data collection configuration to UE. (depends on RAN’s feedback) - Traffic Differentiation per session or per QoS flow. - For controllability: - UP path termination entity may initiate / stop data transfer procedure - UE may accept / reject data transfer from the UP path termination entity. - How the UP path termination entity initiate / stop data transfer procedure, i.e. via CP or UP is FFS. - Whether the UE may request data transfer is FFS. - whether the UP path termination entity send to gNB via AMF an indication that data transfer is allowed for a UE is FFS. - For visibility requirement: - The UP path termination entity verifies/matches the requested data to be transferred and the data that is being reported. - whether and how to verify/match the requested data to be transferred and the data that is being reported in standardized manner is FFS and requires RAN’s feedback on visibility requirements.

Based on our understanding from SA2 conclusion, there are still some open issues on whether and how controllability and visibility can be supported via Option 2 UP solution, while feedback from RAN2 might be needed for SA2 to develop a proper solution. From RAN/RAN2 point of view, without understanding the details of how controllability and visibility works for UP solution, it is difficult for RAN/RAN2 to decide whether Option 2 UP will be selected for data transfer of UE-side data collection or not.

Observation 2: Whether Option 2 UP is selected still needs more study in RAN2.

Therefore, we believe it would be good to allow more study time for RAN2 in Rel-20 5GA to decide whether UP solution will be selected and then provide feedback to SA2 when the conclusion is made. The technical discussion for solution selection can happen between WGs, there’s no need to have check point in plenary for UE-side data collection.

Proposal 3: On check point for UE-side data collection, RAN2 to continue study and down-select between CP and UP solution in RAN2 meetings, and provide feedback to SA2 whether UP is selected based on the conclusion. Remove check point in plenary for UE-side data collection.

- Conclusion

In this contribution, the observations and proposals on the check points of AI/ML for NR air interface Phase 2 are summarized as follows:

Observation 1: RAN4 confirms that both the fully defined/specified reference model (“Direction C”) and the standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) are feasible.

Observation 2: Whether Option 2 UP is selected still needs more study in RAN2.

Proposal 1: RAN agrees to remove the check point on Direction C and sub-option 3a-1 under Direction A from the WID, as feasibility has been confirmed by RAN4.

Proposal 2: For inter-vendor training collaboration for two-sided AI/ML models, RAN removes the check point and confirms dataset/model parameter transfer is supported via non-OTA solution gNB -> OAM -> UE-side training entity in R20 WI.

Proposal 3: On check point for UE-side data collection, RAN2 to continue study and down-select between CP and UP solution in RAN2 meetings, and provide feedback to SA2 whether UP is selected based on the conclusion. Remove check point in plenary for UE-side data collection.

- References

- RP-252445 Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Qualcomm

- R4-2522434 LS on RAN check point of R20 NR_AIML_air_Ph2, RAN4

- R2-2503169 Reply LS on signalling feasibility of dataset and parameter sharing, RAN2

- R3-253961 Reply LS on signalling feasibility of dataset and parameter sharing, RAN3

- S5-254083 Reply LS on signalling feasibility of dataset and parameter sharing, SA5

- S2-2508104, Reply LS on signalling feasibility of dataset and parameter sharing, SA2

- S2-2510958, LS on AI/ML UE sided data collection, SA2