3GPP TSG RAN Meeting #110																		RP-253296

Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.1

Source:	Nokia

Title:	Text Proposal for Key Performance Indicators

Document for:	Approval

# 1	Introduction

This document provides a text proposal for TR 38.914, Section 5.1. The text proposal is aligned with the PRELIMINARY DRAFT NEW REPORT ITU-R M.[IMT-2030.TECH PERF REQ] agreed at ITU-R WP 5D, Sub-Working Group Radio Aspects, in October 2025.

# 2	Text Proposal

-------  Start of Text Proposal ------

### 5.1.10	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

The minimum requirement for the reliability is 1-10-5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms in channel quality of coverage edge for the Indoor Factory-HRLLC [test environment], assuming small application data (e.g. 20 bytes application data + protocol overhead).

-------  Unchanged part omitted  ------

### 5.1.12	Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. The proponent can report the Mobility interruption time in other scenarios, where applicable.

-------  Unchanged part omitted  ------

### 5.1.14	Positioning

Positioning is the ability to estimate the position of connected devices.

The positioning accuracy is the 90% point of the CDF of the device positioning error for [both] horizontal [and vertical] direction[s].

Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). [Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude)].

The minimum requirements for the device positioning accuracy for various [test environments] are summarized in Table 5.1.14-1.

Table 5.1.14-1: Positioning accuracy

| [Test Environment] | Horizontal Accuracy | Vertical Accuracy |
| --- | --- | --- |
| Indoor Factory - ISAC |  |  |
| [Urban Macro – ISAC / Urban – ISAC] |  |  |

-------  Unchanged part omitted  ------

### 5.1.17	Sensing

Sensing-related capabilities are measured in terms of the following technical performance requirements:

•	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing target, and false alarm probability is the associated probability that a sensing target is detected when no sensing object is actually present.

•	Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing target. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error.

•	Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing target. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

•	[Sensing Resolution: TBD]

### 5.1.18	AI

AI-related capabilities refer to the ability to provide certain functionalities to support AI enabled applications, including:

- RIT/SRIT facilitating AI applications i.e. “Network for AI”.

- AI to improve RIT/SRIT performance, i.e. “AI for network”.

The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signaling/ related to AI functionalities. This should include support for one or more of the following:

- Data collection

- Distributed processing

- Distributed learning

- AI model training

- AI computing

- AI model inference

- or other AI capabilities of the RITs/SRITs.

### 5.1.19	Resilience and Extended Connectivity

This requirement includes the following capabilities:

Resilience: It refers to capabilities of the networks and systems to continue operating correctly during and after a service disruption, such as the loss of primary source of power, etc. The RIT/SRIT should support functionalities over the radio interface that enable continuous operation or rapid temporary restoration during and after disturbance to radio infrastructure.

Extended Connectivity: It refers to the capability to provide connectivity in areas that are currently uncovered or scarcely covered due to challenges arising from geographical conditions, such as the lack of ground infrastructure (e.g., fiber cables and power supply), where users desire access to communication services. The RIT/SRIT should have functionalities that support providing geographically extended connectivity in these areas.

Functionalities related to resilience and extended connectivity may include, but are not limited to, support for connecting to one or more of the following advanced network infrastructures within the terrestrial component of IMT:

- HIBS

- Relays and repeaters

- Shared network architectures to provide redundancy and backup

- Base station onboard a vehicle, airship and/or vessel

- or other mechanisms for these capabilities using network infrastructures within the terrestrial component of IMT

This requirement is defined for the purpose of evaluation in the Ubiquitous Connectivity usage scenario and may also be applicable to other usage scenarios.

-------  End of Text Proposal ------