3GPP TSG RAN # 110										              RP-253297

Baltimore, USA, December 15-18th, 2025

Agenda Item:	8.2.1.0

Source: 	CSCN, CATT, CBN, China Broadnet, China Unicom

Title:           Views on 6G NTN Scenarios

Document for:  Discussion/Decision

- Introduction

In RAN#109, the “TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements” was approved in RP-252882 [1]. In this contribution, we present our views on NTN deployment scenarios.

- Discussion

  - Cell deployment

- Cell deployment

As studied and described in R16 TR38.821[2], “Similar to TN, one or several SSB index can be used per PCI to separate SSB transmission on different beams.” We believe that 6G NTN can reuse the designs studied in 5G TN and NTN, whether it is SSB beam or other logical beams newly defined in 6G. As a result, 6G cell deployment can include one-beam cell and multi-beam cell, as shown below.

- One-beam cell: The cell consists of only one logical beam as in Figure 1.

Figure 1: One-beam cell

- Multi-beam cell: The cell consists of multiple logical beams with beam sweeping within a cell as in Figure 2. Logical beams may be SSB beams inherited from 5G or newly defined synchronization signals in 6G. To achieve refined management, supporting more than 8 logical beams in FR1 can be considered.

Figure 2: Multi-beam cell

6G NTN shall support the one-beam cell and multi-beam cell with beam hopping and more than 8 SSBs per cell.

- Cell type

NTN can be deployed to provide coverage with earth-moving cell, quasi-earth-fixed cell and earth-fixed cell which are supported respectively by the following three types of service link [3]:

- Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites); - Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams); - Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

The Quasi-Earth-Fixed deployment can be categorized into two types in the scenario of multi-beam cell, the current serving cell is denoted by a yellow circle, with its beams denoted by blue circles, as shown in Figure 3:

- Case 1: The areas of cell coverage in two consecutive periods are completely different.

- Case 2: The areas of cell coverage in two consecutive periods partly overlap.

Figure 3: Quasi-Earth-Fixed case 1 and case 2

For Quasi-Earth-Fixed case 1, service for UE1 and UE2 starts at time T1 and stops simultaneously at time t-Service, after which the satellite starts to serve the next geographic area (i.e. another cell). The geographical area of both the serving cell and its beams remains unchanged during time T1 to time t-Service. The continuous service duration for each UE is given by t-Service – T₁.

For Quasi-Earth-Fixed case 2, with the satellite moves, the cell coverage area hops over the Earth surface with the earth-fixed beams. Different from all beams within Quasi-Earth-Fixed case 1, one or several beams covering for one geographic area switch to another geographic area after a limited period. The service for UEs in corresponding beams starts and stops simultaneously while the UEs in the other beams of this cell have the different start and stop time. From our perspective, case2 is beneficial for the link budget with ubiquitous coverage.

6G NTN should support Quasi-Earth-Fixed case 2, in which cell coverage in two consecutive periods partly overlaps in multi-beam deployment.

  - Feeder link

[PR 8.17.6-1] Subject to operator’s policy, the 6G network shall support mechanisms to connect the base stations (onboard satellite) and the ground core networks using 3GPP technology.

[PR 8.17.6-2] Subject to the regulatory requirements and operator’s policy, the 6G network shall support a mechanism to establish an efficient connection between the base station (onboard satellite) and the ground core network considering the characteristics (e.g. reliability, latency, data rate) of the carried traffic.

NOTE: Non-3GPP satellite access technologies are not precluded for the feeder link (i.e. radio link between ground station and satellite).

In [4], the 6G network shall support mechanisms to connect the base stations (onboard satellite) and the ground core networks using 3GPP technology was agreed in SA1#112 meeting.

For regenerative architecture, in order to align with SA1, even though non-3GPP satellite access technologies are not precluded for the feeder link, 3GPP defined NTN access technology should be considered for feeder link transmission in NTN deployment scenarios.

For regenerative architecture, 3GPP defined NTN access technology should be considered for feeder link transmission in 6G NTN deployment scenarios.

- Conclusion

Proposal 1:	6G NTN shall support the one-beam cell and multi-beam cell with beam hopping and more than 8 SSBs per cell.

Proposal 2:	6G NTN should support Quasi-Earth-Fixed case 2, in which cell coverage in two consecutive periods partly overlaps in multi-beam deployment.

Proposal 3:	For regenerative architecture, 3GPP defined NTN access technology should be considered for feeder link transmission in 6G NTN deployment scenarios.

- Reference

- RP-252882, TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements, 3GPP TSG#109, Beijing, China, September 15-18, 2025

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), v16.2.0

- TS 38.300, NR and NG-RAN Overall Description, v18.6.0

- S1-254403, Update 8.17 UC on 6G Satellite backhaul, 3GPP TSG SA WG 1 Meeting #112, Baltimore, USA, December 15-18th, 2025