3GPP TSG RAN Meeting #110								RP-253311

Baltimore, USA, December 8-11, 2025

Status Report to TSG

Agenda item:			9.5.1.1

| WI / SI Name | NR MIMO Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_Ph5 |  |  |  |  |
| Unique ID | 1020089 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242394 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 80% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Darcy Tsai |
|  | Company | MediaTek |
|  | Email | darcy.tsai@mediatek.com |
| Rapporteur | Name | Chai Li |
|  | Company | China Mobile |
|  | Email | chaili@chinamobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

n/a

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

In RAN4#116bis (2025-10), the following agreements were made.

RRM

WF were approved in R4-2514803 and R4-2514804.

UE-initiated/event-driven beam management

Issue 2-1: Test case list and CR split

Agreement:

| TC index | TC name | Company name |
| --- | --- | --- |
| TC1 | Event-2, FR2, Inter-cell SSB based, no window time based, mode-A | Samsung |
| TC2 | Event-2, FR2, CSI-RS based, no window time based, mode-A, with ”QCL-info” for the SSBs with the same cell ID | vivo |
| TC3 | Event-2, FR2, Intra-cell SSB based, no window time based, mode-A | Ericsson |
| FFS on TC4 | Event-2, FR2, FFS Intra-cell SSB or CSI-RS based, with window time based, and eventInstanceCount-r19 > 1 |  |
| TC5 | Event-2, FR1, Inter-cell SSB based, no window time based, mode-A | CMCC |
| TC7 | Event-2, FR1, Intra-cell SSB based, no window time based, mode-A | Nokia |
| FFS on TC8 | Event-2, FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1 |  |

CJT calibration (CJTC) reporting

Issue 1-2: Test scope for UE reporting enhancement for CJT calibration?

Agreement:

- Define 2 TCs

  - TC1: FR1 CJTC-Dd measurement period and accuracy

  - TC2: FR1 CJTC-F measurement period and accuracy

Asymmetric DL-sTRP/UL-mTRP

Issue 2-1: Test scope for asymmetric DL sTRP/UL mTRP scenarios

Agreement:

- For scenario#1,

    - TC1: Timing Advance Adjustment Accuracy Test for FR1

    - TC2: Timing Advance Adjustment Accuracy Test for FR2

- For scenario#2,

    - TC3: uplink transmission timing 2TA FR1

    - TC4: uplink transmission timing 2TA FR2

Agreement:

- Add all 4 TCs for EN-DC, and further discuss whether to add the applicability rule.

Issue 3-1: CR split for performance requirements

Agreement:

| CR title | Company name |
| --- | --- |
| Measurement accuracy requirements for CJTC-Dd and CJTC-F | Nokia |
| Mapping tables for CJTC reporting | Apple |

Issue 3-2: CR split for test case list

Agreement:

| TC index | TC title | Company name |
| --- | --- | --- |
| TC9 | Test case for FR1 CJTC-Dd measurement period and accuracy | Ericsson |
| TC10 | Test case for FR1 CJTC-F measurement period and accuracy | MTK |
| TC11 and TC12 | Test cases for Timing Advance Adjustment Accuracy for FR1, scenario#1 for SA and EN-DC | Ericsson |
| TC13 and TC14 | Test cases for Timing Advance Adjustment Accuracy for FR2, scenario#1 for SA and EN-DC | Apple |
| TC15 and TC16 | Test cases for uplink transmission timing 2TA FR1, scenario#2 for SA and EN-DC | Huawei |
| TC17 and TC18 | Test cases for uplink transmission timing 2TA FR2, scenario#2 for SA and EN-DC | Huawei |

Demod

WF were approved in R4-2514806.

General Scope for UE side

Issue 1-1-1: Clarify if PMI reporting requirements should be specified for eTypeII-r19

Agreement:

-	RAN4 shall specify PMI reporting requirements for eTypeII-r19.

Issue 1-1-2: Clarify if PMI reporting requirements should be specified for typeII-Doppler-r19

Agreement:

-	RAN4 do not specify PMI reporting requirements for typeII-Doppler-r19.

Issue 1-1-3: Clarify if UE demodulation/CSI reporting requirements should be specified for SRS port grouping

Agreement:

-	Not introduce UE demodulation/CSI reporting requirements for SRS port grouping.

Issue 1-1-5: Clarify if CSI reporting requirements should be specified for UE reporting enhancements for CJT

Agreement:

-	Not introduce CSI reporting requirements for UE reporting enhancements for CJT.

Test assumptions for typeI-SinglePanel-r19

Issue 1-2-1: How to solve the very low TP issue with random precoder

Agreement:

-	Keep current values of correlation coefficients.

Issue 1-2-3: MCS level for 64 CSI-RS ports

Agreement:

-	MCS22

Issue 1-2-4 and Issue 1-2-5: ZP-CSI-RS, CSI-IM, NZP-CSI-RS configurations for 64 CSI-RS ports

Agreement:

-	For the ZP-CSI-RS, CSI-IM and NZP-CSI-RS configurations, use below configuration

Issue 1-2-6: CQI/RI/PMI delay for FDD 64 CSI-RS ports

Agreement:

-	Configure CSI report slot offset and PMI delay as 7 slots and 11ms.

Issue 1-2-7: CQI/RI/PMI delay for TDD 64 CSI-RS ports

Agreement:

-	Configure CSI report slot offset and PMI delay as 11 slots and 9ms.

Issue 1-2-9: round-off operation of MIMO correlation matrices

Agreement:

-	Companies to provide simulation results based on either option 1 or option 2, the results based on both options will be taken into account for deriving the requirements.

-	Further discuss and decide which option to be captured in the CR in next meeting

  - Option 1: Include round-off to four digits precision operation

  - Option 2: Not include round-off to four digits precision operation

Issue 1-2-10: Scaling factors for MIMO correlation matrices

Agreement:

-	For option 1 in Issue 1-2-9, as baseline, adopt the following scaling factors for 64 CSI-RS ports XP-Medium correlation level matrices to ensure PSD matrices with a measure of numerical stability

| Array Type | Antenna Configuration | Correlation Level | Scaling Factor |
| --- | --- | --- | --- |
| Cross-polarized | 64 (8,4,2)×2 | Medium | 0.00065 |
| Cross-polarized | 64 (8,4,2)×4 | Medium | 0.00092 |

-	Further update on the scaling factor is not precluded.

Issue 1-2-11: Additional codebook mode for typeI-SinglePanel-r19?

Agreement:

-	Not specify additional test case for typeI-SinglePanel-r19, with typeI-codebookMode-r19 as Scheme B.

Issue 1-2-12: Whether to specify requirements for 128 CSI-RS ports for typeI-SinglePanel-r19?

Agreement:

-	Not specify requirements for 128 CSI-RS ports for typeI-SinglePanel-r19.

Test assumptions for eTypeII-r19

Issue 1-3-1: MCS level

Agreement:

-	MCS22 (Table 1)

Issue 1-3-2: MIMO correlation matrixes

Agreement:

-	Same as typeI-SinglePanel-r19

Issue 1-3-3: Test metric for 64 CSI-RS ports

Agreement:

-	Follow the agreement for typeI-SinglePanel-r19 as in Issue 1-2-2

Note: For eType II codebook, randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A.

BS demodulation performance requirements

Issue 2-1-1: Clarify if demodulation requirements should be specified for 3-antenna-port codebook-based UL transmissions

Agreement:

-	RAN4 should specify PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmissions.

-    FFS on whether to introduce test applicability rule for 3Tx

Issue 2-2-1: SCS and CHBW for FDD

Agreement:

-	15kHz SCS, 5MHz, 50MHz

Issue 2-2-2: SCS and CHBW for TDD

Agreement:

-	30kHz SCS, 10MHz, 100MHz

Demod CR split

| Topic | Section | Company |
| --- | --- | --- |
| PMI reporting requirements for up to 128 CSI-RS ports | 6.1.1 Applicability of requirements | CATT |
|  | 6.3.2.1.X1 Single PMI with 64TX TypeI-SinglePanel Codebook | Huawei |
|  | 6.3.2.1.X2 Multiple PMI with 64TX Enhanced Type II Codebook | Huawei |
|  | 6.3.2.2.X1 Single PMI with 64TX TypeI-SinglePanel Codebook | Nokia |
|  | 6.3.2.2.X2 Multiple PMI with 64TX Enhanced Type II Codebook | Ericsson |
|  | 6.3.3.1.Y1 Single PMI with 64TX TypeI-SinglePanel Codebook | MediaTek |
|  | 6.3.3.1.Y2 Multiple PMI with 64TX Enhanced Type II Codebook | Apple |
|  | 6.3.3.2.Z1 Single PMI with 64TX TypeI-SinglePanel Codebook | QC |
|  | 6.3.3.2.Z2 Multiple PMI with 64TX Enhanced Type II Codebook | Ericsson |
|  | A.3.2.1.1 Reference measurement channels for SCS 15kHz | Samsung |
|  | A.3.2.3.3 Reference measurement channels for SCS 30kHz | Apple |
|  | B.2.3 MIMO Channel Correlation Matrices | Nokia |
|  | Big CR for 38.101-4 | Samsung |
| PUSCH demodulation requirements for 3Tx transmission (if specified) | PUSCH requirements 8.2.1.2 in 38.104 | Samsung |
|  | PUSCH requirements in 38.141-1 for FR1 conducted test | Ericsson |
|  | Applicability rule in 38.141-1 8.2.1.0 | ZTE |
|  | Measurement of performance requirements (C.3) | Ericsson |
|  | Measurement system set-up for BS type 1-C (D.5.2B) | ZTE |
|  | Measurement system set-up for BS type 1-H (D.6.2B) | Ericsson |
|  | FRC for 3 ports in 38.104 | ZTE |
|  | FRC for 3 ports in 38.141-1 | Ericsson |
|  | FRC for 3 ports in 38.141-2 | CATT |
|  | Big CR for 38.104 | Samsung |
|  | Big CR for 38.141-1 | Ericsson |

In RAN4#117 (2025-11), the following agreements were made.

RRM

WF were approved in R4-2522636 and R4-2522638.

UE-initiated/event-driven beam management

All draft CRs for UEIBM are endorsed.

| R4-2522777 | Samsung |
| --- | --- |
| R4-2522778 | CMCC |
| R4-2522779 | Nokia |
| R4-2522780 | Vivo |
| R4-2522815 | Ericsson |

Issue 2-1: Whether to add test case(s) for window time based case?

Agreement:

- RAN4 to discuss the TC design for FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1

- RAN4 will define the TC unless it is seen as not testable.

Issue 2-2: Test configuration

Agreement:

- For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state).

- For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state).

- PUCCH configuration are different for FR1 and FR2 SCS configurations. Check test cases CRs directly.

CJT calibration (CJTC) reporting

Issue 1-1: Accuracy requirements for delay and frequency offset reporting for CJT calibration

Agreement:

- For FO, define two set of accuracy requirements with UE capability based on 3dB SNR, for 0.2ppm and 256 levels

  - For 15kHz SCS, [33] and [3]

  - For 30kHz SCS, [68] and [4]

- For DO, define two set of accuracy requirements with UE capability based on 3dB SNR, for CP and 256 levels

  - For 15kHz SCS, 14 and [3];

  - For 30kHz SCS, 18 and [4]

- Note: Achieving better accuracy requires advanced UE implementation and hence UE capability to be introduced.

  - The Note will not be captured in the CR.

Asymmetric DL-sTRP/UL-mTRP

Issue 2-1: Whether applicability rule for asymmetric DL sTRP/UL mTRP scenarios TCs is needed?

Agreement:

- For asymmetric TRPs, do not define applicability rules between EN-DC and NR-SA.

Issue 1-2: On UE capability in Issue 1-1

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_MIMO_Ph5 | 62-1 | UE accuracy for CJTC delay offset reporting | Support of high accuracy reporting for CJTC-Dd | 59-2-3-1 | Yes | N/A | UE does not support the requirements for high accuracy reporting for CJTC-Dd | Per band and per BC, or, Per UE Note: Both “Per band and per BC” and “Per UE” are feasible from RAN4 perspective, and the decision is up to RAN2 | No | FR1 only | N/A | The high accuracy reporting for CJTC-Dd is defined in TS 38.133 clause 10 | Optional |
| 62. NR_MIMO_Ph5 | 62-2 | UE accuracy for CJTC frequency offset reporting | Support of high accuracy reporting for CJTC -F | 59-2-3-2 | Yes | N/A | UE does not support the requirements for high accuracy reporting for CJTC-F | Per band and per BC, or, Per UE Note: Both “Per band and per BC” and “Per UE” are feasible from RAN4 perspective, and the decision is up to RAN2 | No | FR1 only | N/A | The high accuracy reporting for CJTC-F is defined in TS 38.133 clause 10 | Optional |

Issue 1-3: How to define report mapping tables for CJTC reporting?

Agreement:

- Discuss directly in the CR

Demod

WF were approved in R4-2522659.

UE demodulation/CSI reporting performance requirements

Issue 1-1: Test metric for 64 CSI-RS ports

Agreement:

-	For eType II codebook, RAN4 to discuss the following metrics:

  - Option 1: Defining the requirement solely on SNR measured at [70, 90] % of maximum throughput when PMI is applied

- FFS Increase the target SNR span threshold used to judge the result alignment, e.g., 2.5dB-> [3.5] dB

- Further discuss if ideal results are not well aligned:

  - Option 1a: add additional margin on the top of average impairment results to derive final requirements

  - Option 1b: use the highest impairment results among companies to derive final requirements

  - Option 2: Use the legacy test metric, with the lowest gamma among companies

Issue 1-2: round-off operation of MIMO correlation matrices

Agreement:

-	Keep round-off to four digits precision operation in the existing spec.

Issue 1-3: “regularization factor” instead of “Scaling factors” for MIMO correlation matrices

Agreement:

-	For TS38.101-4 section B.2.3.1.2 use “regularization factor” instead of currently used “scaling factor”.

Issue 1-4: Scaling factors for MIMO correlation matrices

Agreement:

-	Adapt the regularization factors which have related CSI reporting requirements into the spec.

Issue 1-5: valueOfX1-typeII-CBSR-r19 and valueOfX2-typeII-CBSR-r19 configuration

Agreement:

-	(valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) are not configured.

-	Follow the same approach for type I codebook.

Issue 1-6: typeII-CBSR-r19

Agreement:

-	typeII-CBSR-r19 is not configured.

-	Follow the same approach for type I codebook.

BS demodulation performance requirements

Issue 2-2: Clarify if test applicability rule should be specified for 3Tx

Agreement:

-	Option 1 as below is agreed. Further discuss how to capture the agreement in the CR.

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 3 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration.

Issue 2-3: MU and TT for PUSCH 3Tx tests

Agreement:

-	Applying 0.9dB for MU and TT with fading profile power uncertainty ±0.87dB.

Demod CR

Agreement:

-	All draftCRs are endorsed.

2.4.2	Remaining Open issues

RRM

- Part of accuracy requirements for CJTC reporting in [] need to be confirmed.

- FFS on Draft CRs for test cases for CJT calibration (CJTC) reporting for performance part.

- FFS on Draft CRs for test cases for Asymmetric DL-sTRP/UL-mTRP for performance part.

Demod

- Test metric for 64 CSI-RS ports

- Clarify if test applicability rule should be specified for PUSCH requirement with different Tx

- Update Draft CRs according the agreements of open issues

2.5	RAN5

n/a

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

n/a

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template