3GPP TSG RAN Meeting #110					RP-253322

Baltimore, USA, Dec 8-11, 2025

Agenda Item:	8.2.1.3.3 (”Other operational requirements”)

Source:	DISA, FirstNet, Erillisverkot, ST Engineering iDirect, DeepSig Inc, DHS Science & Technology, Tiami Networks, Lockheed Martin

Title:	RF interference detection, classification and mitigation requirements in 6G

Document for:	Approval

# 1	Introduction

Resilience is an essential 6G requirement in network, timing, and positioning as evidenced in many use cases including public safety and critical infrastructure in TR 22.870 “Study on 6G Use Cases and Service Requirements”.

A 6G system should provide resilient capabilities including high service availability, high reliability, and high mission assurance. Resilience should be a built-in capability in network design and can provide / maintain the thresholds of service requirements when encountering unforeseen risks and threats. Resilience capability is tailored to the requirements of different use cases.

The following definitions for resilience, availability, and reliability are introduced first to facilitate discussion.

3GPP does not have a good definition of resilience. The following is the definition of Network Resilience from NIST [aa].

“A computing infrastructure that provides continuous business operation (i.e., highly resistant to disruption and able to operate in a degraded mode if damaged), rapid recovery if failure does occur, and the ability to scale to meet rapid or unpredictable demands.”

The definitions of communication service availability and reliability can be found from TS 22.261 “Service requirements for the 5G system”.

Most of the resilience requirements discussed in 6G focus on network resiliency such as providing sufficient redundancy. However, resilience should also cover physical layer / lower layer resilience (e.g., against RF interference). Resilience for physical layer / lower layer provides the foundation for all the upper protocol layers. This paper discusses resilience requirements for detecting, classifying and mitigating RF interference for 6G as part of physical layer / lower layer resilience requirement in the scope of RAN.

# 2	Discussion

Resilience is an essential 6G requirement in network, timing, and positioning as evidenced in many use cases including public safety and critical infrastructure in TR 22.870 “Study on 6G Use Cases and Service Requirements”.

A 6G system should provide resilient capabilities including high service availability, high reliability, and high mission assurance. Resilience should be a built-in capability in network design and can provide / maintain the thresholds of service requirements when encountering unforeseen risks and threats. Resilience capability is tailored to the requirements of different use cases.

3GPP 5G and 6G cellular communications networks are deployed based on the design objective of having a frequency reuse factor of one for the whole network (i.e., all cells in a network use the same frequency) due to limited radio resources. As a result, there is a need to leverage interference management techniques to mitigate 3GPP inter-cell interference to improve QoS for users, especially at the cell edge. How to detect and then optimize RF interference is of great importance for cellular wireless networks. In addition to 3GPP interference, other non-3GPP types of interference experienced in a 5G or 6G cellular communication network, including internal RF interference and external RF interference, need to be detected and optimized.

To support effective interference optimization, there is a need to detect interference, identify the source of the interference in terms of interference type as well as to mitigate interference. By providing interference optimization, 6G resilience in the physical layer / lower layer is improved.

There are two categories of interference as shown below:

- Intra-3GPP interference

- Non-3GPP interference

Intra-3GPP interference includes the following:

- Inter-cell interference within one Mobile Network Operator (MNO)

- Inter-cell interference between two or more MNOs across the border of neighbouring countries

Non-3GPP interference includes the following:

- Unintentional interference

- Intentional interference

Non-3GPP unintentional interference includes the following:

- Internal RF interference

  - Passive intermodulation (PIM)

- External RF interference

  - Tropospheric ducting, especially in the coastal regions

  - Malfunctioned electronic equipment (e.g., electric power stations, RF repeaters, and home monitors)

Non-3GPP intentional interference includes the following:

- Commercial / homemade jammers

The threats of non-3GPP intentional interference are on the rise for government public safety and law enforcements personnel as well as for regular citizens as evidenced from articles and news from Internet. The reference below provides descriptions of intentional interference (i.e., jamming) and types of jammers.

- https://www.dhs.gov/science-and-technology/jamming

As a result, 6G resilience capabilities should be able to counter the threats of jammers including:

- Detecting the presence of jammers

- Mitigating intentional interference generated from jammers

- Notifying government public safety or law enforcements personnel of the presence of jammers such that they can find alternative means of communication if the intentional interference cannot be mitigated by the MNO.

# 3 	Text proposal for TR 38.914

Based on the discussion above, we propose the following text proposal.

* * * First Change * * *

# 2	References

<< text skipped>>

[aa]	https://csrc.nist.gov/glossary/term/network_resilience

* * * End of First Changes * * *

* * * Second Change * * *

# 3	Definitions of terms, symbols and abbreviations

3.1	Terms

<< text skipped>>

Network Resilience: A computing infrastructure that provides continuous business operation (i.e., highly resistant to disruption and able to operate in a degraded mode if damaged), rapid recovery if failure does occur, and the ability to scale to meet rapid or unpredictable demands [aa].

* * * End of Second Changes * * *

* * * Third Change * * *

### 5.3.5	Network Resilience

The 6G Radio and 6G RAN architecture should be designed with resilient capabilities including high service availability, high reliability and high mission assurance.

The 6G Radio and 6G RAN architecture should support detection of RF interference.

The 6G Radio and 6G RAN architecture should support classification of RF interference where RF interference includes 3GPP interference and non-3GPP interference. 3GPP interference includes inter-cell interference within one Mobile Network Operator (MNO) and inter-cell cross boarder interference between two or more MNOs. Non-3GPP interference includes unintentional interference and intentional interference. Non-3GPP unintentional interference includes internal interference and external interference (e.g., tropospheric ducting).

The 6G Radio and 6G RAN architecture should support mitigation of RF interference including non-3GPP intentional interference.

The 6G Radio and 6G RAN architecture should support notification of RF interference for inter-cell cross boarder 3GPP interference between two or more MNOs and non-3GPP intentional interference via air interface directly to the subscribers for government public safety or law enforcements personnel on site impacted by RF interference as well as via wired means to public safety or law enforcements personnel offsite in control rooms by coordinating with 6G core.

* * * End of Third Changes * * *

# 4	Conclusion

In this contribution, we propose adding the text proposal in section 5.3.5 “Network Resilience” to TR 38.914.