3GPP TSG RAN meeting #110										RP-253326

Baltimore, USA, Dec 8 – 12, 2025

## Status Report to TSG

Agenda item:			9.5.4.8

| WI / SI Name | Low NR band carrier aggregation via switching |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LBCA_Sw-Perf |  |  |  |  |
| Unique ID | 1062082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252694 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 50% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ivo Maljevic |
|  | Company | TELUS |
|  | Email | ivo.maljevic@telus.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #122bis

Maintenance

-	RAN1 has discussed specification maintenance issues summarized in [1], [2]

-	RAN1 has discussed feature list maintenance issues summarized in [14]

RAN1 #123

Maintenance

-	RAN1 has discussed specification maintenance issues and reached the following agreements [21]:

Agreement Adopt the following TP to TS38.213 Clause 24 for alignment. <Unchanged parts are omitted> 24 Downlink carrier aggregation via switching A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern switchingPattern-r19 a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots. When the UE is provided LBCA-SwitchingPattern LowBandCA-Switching-r19, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell gapDurationPCelltoSCell-r19, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell gapDurationSCelltoPCell-r19, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. <Unchanged parts are omitted>. Agreement For Rel-19 low NR band carrier aggregation via switching, the collision handling for higher-layer configured transmission and reception is per repetition. - Adopt the following TP to TS38.213 Clause 24 --- Reason for change: When the UE is provided LBCA-SwitchingPattern, it is not clear that collision handling for higher-layer configured transmission and reception is per repetition (slot) or per channel(all slots), if overlapping with at least one unavailable symbol of a slot based on the switching pattern. Summary of change: The collision handling for higher-layer configured transmission and reception is per repetition if overlapping with at least one unavailable symbol based on the switching pattern. Consequences if not approved: For low NR band carrier aggregation via switching, it is not clear the collision handling for higher-layer configured transmission and reception is per repetition or per channel. 24 Downlink carrier aggregation via switching <Unchanged parts are omitted> When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols of a slot when the set of symbols of the slot includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols of a slot on the PCell when the set of symbols of the slot includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a SS/PBCH block, PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols of a slot on the SCell when the set of symbols of the slot includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. <Unchanged parts are omitted>. --- Conclusion For Rel-19 low NR band carrier aggregation via switching, in RAN1, there is no consensus to support the following - For Type-I HARQ-ACK codebook generation, a row of the TDRA table overlapping with the invalid symbol(s) is excluded from the determination of the set of occasions for candidate PDSCH receptions. - UE does not receive SPS PDSCH if the HARQ-ACK PUCCH corresponding to the SPS PDSCH is overlapped with the invalid symbol due to semi-static switching pattern. Conclusion For Rel-19 low NR band carrier aggregation via switching, in RAN1, there is no consensus to change the following UE behavior for SPS PDSCH. - The expected UE behavior is - When there is one PDSCH on a serving cell without a corresponding PDCCH transmission in a slot, HARQ-ACK information bits for SPS PDSCH receptions overlapping with the invalid symbol(s) of switching pattern will be included. - When there are more than one PDSCH on a serving cell each without a corresponding PDCCH transmission in a slot, HARQ-ACK information bits for SPS PDSCH receptions overlapping with the invalid symbol(s) of switching pattern will be excluded.

-	RAN1 has concluded the following related to the RAN1 UE feature list for NR_LBCA_Sw:

Conclude the following: - Given RAN4 FG covers features defined in RAN1 for Rel-19 NR low-band CA, no RAN1 FG is defined for Rel-19 NR low-band CA.

#### 2.1.2	Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #131bis

RRC maintenance and ASN review

-	RAN2 has made the following agreements based on the discussion resulting from the submission of [45 – 49]:

-	Introduce an alternative FeatureSetCombinationId for a UE supporting low band carrier switching.

-	We will focus on NR standalone case for the LBCA capability signaling.

-	RAN2 assume that UE is allowed to report the super BC with type 2 and type 4 capability, with the restriction that this capability is applicable only to the fallback BC with single CC on the NR side for the EN-DC case and with two CCs for the CA case (as defined in the RAN4 spec).

RAN2 #132

RRC maintenance and ASN review

-	RAN2 endorsed the draft CR to TS38.331 on the Introduction of UE capability on low band CA via switching in [56]

-	RAN2 endorsed the draft CR to TS38.306 on the Introduction of UE capability on low band CA via switching in [57]

#### 2.2.2	Remaining Open issues

None

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #116bis – RF (Core maintenance)

RF core maintenance

-	The topic summary was prepared in [67]

-	The CR on Missing requirements for low NR band CA via switching in TS38.101-1 was endorsed in [64]

-	The draft CR to introduce RMC for LB-LB CA via switching to TS38.101-1 was endorsed in [65]

-	The TP to TR 38.768 to modify the formulas for the switching gap calculation was approved in [66]

RAN4 #116bis – RRM

General aspects

-	The topic summary was prepared in [100]

-	Adhoc minutes are captured in [101]

RRM Core maintenance

-	A CR to TS38.133 with Correction on low band carrier aggregation via switching was endorsed in [96]

-	A liaison to RAN1 with RAN4 RRM agreement update on switching pattern application was approved in [97]:

RAN4 has further discussed the applicability of switching pattern for SDL SCell for RRM requirements and would like to share the following update to RAN1. Agreement: - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied.. To RAN1: ACTION: RAN4 respectfully asks RAN1 to take the above into account in their future work.

-	A draft CR to TS38.133 on on R19 LB CA via switching was endorsed in [98]

-	RAN4 reached the following agreements related to the maintenance of the core RRM part [102]:

### Issue 1-1: SDL SCell activation/deactivation in LB CA Issue 1-1-1: interruption or scheduling restriction during SDL SCell activation/deactivation [Way forward]: FFS: - Proposal 1 (HW): - Not to define interruption requirements during SCell activation in LB-CA. Make a clarification in spec: The current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario. Issue 1-1-2: CQI/L1-RSRP reporting during SDL SCell activation in LB-CA Candidate options from Monday RRM online session: - Option 1 (Apple, Samsung, HW, MTK): - If CQI or L1-RSRP reporting is colliding with the ON duration of the being-activated SDL SCell based on the swiching pattern during the SDL SCell activation, no SCell activation requirement shall be applied; otherwise, legacy CQI and L1-RSRP reporting requirement of SCell activation can be reused for SCell activation in LB-CA. - Option 2 (vivo, Nokia, ZTE, CATT, OPPO, QC): - The issue can be closed (without any additional clarification or spec change in RAN4) - Other options are not precluded. Agreement: The issue can be closed (without any additional clarification or spec change in RAN4). Issue 1-1-3a: Application time of Switching Pattern in SCell activation Issue 1-1-3b: switching pattern based carrier switching in SCell activation Agreement: - TLBCA =1ms for MAC CE based, 0 ms for RRC based. Issue 1-1-4a: Application time of disabling switching pattern in SCell deactivation Agreement: - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation Issue 1-1-4b: switching pattern based carrier switching in SCell activation Agreement: - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation. - Following RAN1 logic, the carrier switching, if applicable, shall be placed at the end of a slot. - FFS RAN4 to capture this sub-bullet in the CR. - Further discuss for timer triggered SCell deactivation Issue 1-1-5: Delay requirement in SDL SCell activation [Way forward]: - FFS: - Proposal 1 (LGE): - RAN4 to define SCell activation delay requirements for fractional case where {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, and the principle of Kp scaling factor could be applied to SCell activation delay. - Proposal 2 (Nokia): - Switching pattern is not applied when UE is in DRX. - Remove the impact of switching pattern for Kp factor for measurements during DRX. - If above bullet is agreeable, inform RAN1 about the agreement in an LS - Proposal 3 (Ericsson): - Proposal 5: RAN4 to agree on the following text in the SCell activation delay requirement for the cases of none of the SSB overlapped with the pattern or some of the SSb/SMTC occasions are overlapped with the pattern: - For the UE supporting LowBandCA-via-Switching-r19 and configured with LBCA-SwitchingPattern in FR1, the reference time for evaluating Tactivation_time is from the switching pattern activation time. Based on the content of Tactivation_time, the following applies: - if it contains only TFirstSSB_MAX or TFirstSSB or TFirstSSB_MAX, enhanced, it corresponds to the time to the end of the first complete SSB burst that does not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern. - if it contains TFirstSSB_MAX + TSMTC_MAX, it represents the shortest combined time at which both a valid SMTC occasion and a complete SSB burst are available, without overlapping the slots indicated by '0' in the LBCA-SwitchingPattern. The order in which the first non-overlapping SMTC occasion and the first non-overlapping complete SSB burst occur is not constrained. - if it contains TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced, it refers to the time to the end of two complete SSB bursts, based on the SSB periodicity of the activated SDL SCell, where both bursts do not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern. ### Issue 1-2: BFD/CBD/RLM requirement in LB-CA Issue 1-2-1: Switching pattern application after BF(beam failure) or LF(link failure) [Way forward]: - FFS: - Discussion following options: - Option 1: No optimization for switching pattern applicability is needed related to beam or link recovery (BFD, CBD, RLM) in Rel-19. - Option 2: - No optimization for switching pattern applicability is needed related to CBD in Rel-19. - The switching pattern is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310). - The switching pattern is activated upon stopping RLF timer (i.e., upon stopping T310). - Option 3: - The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310). (Ofinno, vivo) - The switching pattern (i.e., switchingPattern-r19) is activated upon stopping RLF timer (i.e., upon stopping T310). (Ofinno, vivo) - The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering beam failure detection (BFD). - The switching pattern (i.e., switchingPattern-r19) is activated upon detecting a new beam on the PCell during the beam failure recovery procedure. Issue 1-2-2: BFR requirements for SDL SCell [Way forward]: - FFS: - Proposal 1 (LGE): - RAN4 to consider BFR requirements for SCell in 8.5.9 as follows: - The UE shall be capable of transmit PUCCH with an LRR at the first available PUCCH occasion that is not overlapped with SDL ON duration corresponding to the LB CA switching pattern after T2+D - where, T2 is CBD-RS evaluation period, and D is 2ms UE processing time ### Issue 1-3: BWP switching requirement for LB CA Issue 1-3-1: BWP switching requirement for LB CA [Way forward]: - FFS: - Proposal 1 (Apple): - RAN4 assumes that NW can avoid such scenario that active BWP switching is colliding with FDD-SDL carrier switching, and no requirement is needed to consider such colliding case. - Proposal 2 (LGE): - Regarding active BWP switching for FDD PCell and SDL SCell, support the principle as follows: - For a UE supporting LB-CA via switching and configured with switch pattern, during the active BWP switching delay and the first DL or UL slot, the UE remains on the same carrier where the active BWP switching occurs - Proposal 3 (Ofinno): - Clarify the UE behavior for both timer-based and DCI-based BWP switch by adding the following text in clause 8.6.2: - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the timer-based or the DCI-based BWP switch delay requirements refers to the first DL or UL slot, which: - is available after the completion of the BWP switching, and - is assigned to the same serving cell on which the active BWP switching occurs. - Clarify the UE behavior for the RRC-based BWP switch by adding the following text in clause 8.6.3: - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the RRC based BWP switching delay requirements refers to the first DL or UL slot, which: - is available after the completion of the BWP switching, and - is assigned to the FDD PCell. ### Issue 1-4: others for LB CA Issue 1-4-1: CSSF for LB CA [Way forward]: - FFS: - Proposal 1 (Apple): the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern. Issue 1-4-2: UE capability for RRC based SDL SCell activation in LB CA [Way forward]: - FFS: - Proposal 1 (QC): - RAN4 introduces a new capability that allows UE to signal whether it supports RRC based direct SCell activation in Rel-19 NR LB-CA. Potential details of the capability are shown in Table 1. Table 1: Details of UE capability for RRC based Direct SCell Activation in Rel-19 NR LB-CA Features; Index; Feature group; Components; Prerequisite feature groups; Consequence if the feature is not supported by the UE; Mandatory/Optional. 55. NR_LBCA_Sw; [55-x]; RRC based Direct SCell Activation in Rel-19 NR LB-CA; [UE supports RRC based direct SCell activation in Rel-19 NR LB-CA]; 55-1; [UE will not support RRC based SCell activation in Rel-19 NR LB-CA]; [Optional with capability signaling].

RRM performance

-	RAN4 reached the following agreements related to the RRM performance part [102]:

### Issue 2-2: TC configuration Issue 2-2-1: TC configuration Agreement: - General configuration: - For test configuration and parameters, existing FR1 with 15kHz SCS test configuration and parameters could be baseline - CA between FDD PCell and SDL SCell is configured - PCell and SCell are co-located deployed and keep sync between them (Max RTD is 3us) - The SCS is 15kHz and the frequencies of PCell and SCell are lower than 1GHz - As baseline, test partially overlapped scenario for LB CA RRM, i.e., SSB/SMTC is partially overlapped with LB-CA switching pattern. - Working assumption for TC: SSB/SMTC periodicity is 20ms and switching pattern periodicity is 40ms; - The details of switching pattern can be FFS. - RAN4 will pick one TC in the list to cover fully overlapping and partially overlapping cases ### Issue 2-3: TC list Issue 2-3-1: detailed TC list Agreement: TC Index; Requirement; RS used for test; Target Cell for testing; Support this TC; Leading Company. 1; Intra-frequency L3 measurement for activated SDL SCell or PCell; SSB; Both PCell and SCell in the same TC PCell and SCell can be configured with different events, details can be FFS. PCell is in partially overlapped case. SCell is in fully overlapped case.; Ericsson. 2; Deactivated SDL SCell measurement and interruption requirement; SSB; Verify measurement requirement on SDL SCell. And in the same TC verify interruption requirement to the PCell.; MTK. 3; SDL SCell activation/deactivation; SSB(for sync and L1-RSRP) and CSI-RS for CQI; SDL SCell. Only MAC CE based SCell activation is verified.; Nokia. 4; BFD/CBD for SDL SCell; Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both; Option 1: SCell only (Apple) Option 2: Both PCell and SCell (OPPO, HW, ZTE, Ericsson) [Ericsson]: If option 2 is adopted, test PCell/SCell in the different test.; FFS whether this TC shall be introduced.. 6; PL-RS switch delay for PCell; Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both; FDD PCell (Apple, OPPO, ZTE, Ericsson); FFS whether this TC shall be introduced.. 7; L1-RSRP for SDL SCell; SSB; SCell only; Huawei. 8; RLM requirement for PCell; SSB; FDD PCell; OPPO.

RAN4 #117 – RF (Core maintenance)

RF core maintenance

-	The topic summary was prepared in [130]

-	The RAN4 Chair’s minutes have captured the following agreement:

Agreement: UE-to-UE co-existence requirements for CA_n12-n29 and CA_n28-n67 for Rel-19 will not be defined.

-	The CR for TR 38.768 to modify the formulas for the switching gap calculation was agreed in [131]

-	The CR for TS 38.101-1 to introduce DL RMC for LB-LB CA via switching was agreed in [132]

RAN4 #117 – RRM

General aspects

-	The topic summary was prepared in [138]

-	Adhoc minutes are captured in [140]

RRM core maintenance aspects

-	The CR to TS38.133 on R19 LB CA via switching was agreed in [134]

-	The CR to TS38.133 on LBCA SCell activation and other requirements was agreed in [135]

-	The CR to TR38.768 for NR_LBCA_Sw RRM was agreed in [136]

-	The CR to TS38.133 on Correction on L1-RSRP/SINR measurement requirement for SDL SCell was agreed in [137]

-	The following agreements related to RRM core maintenance were captured in [139]:

### Issue 1-1: SDL SCell activation/deactivation in LB CA Issue 1-1-1: interruption or scheduling restriction during SDL SCell activation/deactivation - Proposal 1 (HW): - Proposal 1: Not to define interruption requirements during SCell activation in LB-CA. Make clarification in spec: The current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario. - Recommended WF - FFS: - Not to define interruption requirements during SCell activation in LB-CA. Make clarification in spec: The current interruption requirements in SCell activation procedure are not applicable for LB-CA scenario. Issue 1-1-2: switching pattern based carrier switching in SCell activation Agreement: For MAC CE based SCell activation procedure, If the n+Tharq+6 is the slot of SCell on-duration as per switching pattern, UE shall switch to SDL SCell by end of n+Tharq+5; otherwise UE shall switch to the SDL SCell by end of the slot immediately prior to the 1st slot in the SCell on-duration based on the switching pattern. For RRC based direct SCell activation procedure, If the n+TRRC_Process +2 is the slot of SCell on-duration as per switching pattern, UE shall switch to SDL SCell by end of n+ TRRC_Process +1; otherwise UE shall switch to the SDL SCell by end of the slot immediately prior to the 1st slot in the SCell on-duration based on the switching pattern. RAN4 can revisit this agreement if RAN1 spec has any conflicting with the above agreement. But if no any conclusion was made in RAN1 in Q1 2026, RAN4 will draft the CR based on the above agreement in Q2 2026 and the CR wording can be further discussed. Issue 1-1-3a: Application time of disabling switching pattern in timer based SCell deactivation FFS: - for timer based SDL SCell deactivation: - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(3ms+ TLBCA)/slot-length). - TLBCA =1ms - FFS: if TLBCA = 2ms can be assumed considering the TA in N2. Issue 1-1-3b: switching pattern based carrier switching in timer based SCell deactivation FFS: - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, - Option 1: UE switches to PCell by end of the n+4. - Option 2: UE’s ready to start switching to PCell from the end of n+4. Issue 1-1-4: Impact of C-DRX on LBCA measurement - Option 1 (Nokia): - Switching pattern is not applied when UE is in DRX. - a. Remove the impact of switching pattern for Kp factor for measurements during DRX. - If above bullet is agreeable, inform RAN1 about the agreement in an LS - Option 2 (HW): - Prefer not to introduce the optimization of removing Kp factor for measurement during DRX. Agreement: This issue is closed. Issue 1-1-5: Delay requirement in SDL SCell activation - Proposal 1 (Ericsson): - RAN4 to agree on the following text in the SCell activation delay requirement for the cases of none of the SSB overlapped with the pattern or some of the SSb/SMTC occasions are overlapped with the pattern: - For the UE supporting LowBandCA-via-Switching-r19 and configured with LBCA-SwitchingPattern in FR1, the reference time for evaluating Tactivation_time is from the switching pattern activation time. Based on the content of Tactivation_time, the following applies: - if it contains only TFirstSSB_MAX or TFirstSSB or TFirstSSB_MAX, enhanced, it corresponds to the time to the end of the first complete SSB burst that does not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern. - if it contains TFirstSSB_MAX + TSMTC_MAX, it represents the shortest combined time at which both a valid SMTC occasion and a complete SSB burst are available, without overlapping the slots indicated by '0' in the LBCA-SwitchingPattern. The order in which the first non-overlapping SMTC occasion and the first non-overlapping complete SSB burst occur is not constrained. - if it contains TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced, it refers to the time to the end of two complete SSB bursts, based on the SSB periodicity of the activated SDL SCell, where both bursts do not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern. - Proposal 2 (ZTE): - The partial availability of L3/L1 RS occasions during the SCell activation procedure should be considered. It could be captured into the spec with the clarification on the available RS occasions based on the switching pattern. - Recommended WF - FFS: Discuss P1/P2 or check if P1/P2 can be discussed directly in the CR. ### Issue 1-2: BFD/CBD/RLM requirement in LB-CA Issue 1-2-1: Switching pattern application after BF(beam failure) or LF(link failure) - Proposal 1 (Ofinno): - The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering radio link failure (RLF) (i.e., upon starting T310). (Ofinno, vivo) - The switching pattern (i.e., switchingPattern-r19) is activated upon stopping RLF timer (i.e., upon stopping T310). (Ofinno, vivo) - The switching pattern (i.e., switchingPattern-r19) is deactivated upon triggering beam failure detection (BFD). - The switching pattern (i.e., switchingPattern-r19) is activated upon detecting a new beam on the PCell during the beam failure recovery procedure. Exact timeline is FFS. - Proposal 2 (LGE): - RAN4 to consider not to apply switching pattern, and stay on FDD carrier until link recovery is completed after LRR (link recovery request) is provided for SDL SCell - RAN4 to consider not to apply switching pattern, and stay on FDD carrier until radio link recovery is completed after all resources configured as RLM-RS falls below the threshold Qout - RAN4 to consider not to apply switching pattern, and stay on FDD carrier until link recovery is completed after PRACH is provided for FDD PCell - Proposal 3 (vivo): - There is no need to enhance CBD measurement requirements for both FDD PCell and SDL SCell. - Proposal 4 (Ericsson): - RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritise to receive RS for for faster link recovery. - Proposal 5 (Apple, ZTE(PCell)): - No optimization for switching pattern applicability is needed related to BFD/CBD/RLM in Rel-19. Agreement: Conclude this issue in the next meeting. Issue 1-2-2: BFR requirements for SDL SCell - Proposal 1 (LGE): - RAN4 to consider BFR requirements for SCell in 8.5.9 as follows: - The UE shall be capable of transmit PUCCH with an LRR at the first available PUCCH occasion that is not overlapped with SDL ON duration corresponding to the LB CA switching pattern after T2+D - where, T2 is CBD-RS evaluation period, and D is 2ms UE processing time - Recommended WF - FFS: Discuss if P1 is agreeable. ### Issue 1-3: BWP switching requirement for LB CA Issue 1-3-1: BWP switching requirement for LB CA - Proposal 1 (Apple, HW, vivo): - No requirements are defined for overlapping of BWP switching and carrier switching. - Proposal 2 (Ofinno): - DCI or timer-based BWP switching: - Clarify the UE behavior for both timer-based and DCI-based BWP switch by adding the following text in clause 8.6.2: - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the timer-based or the DCI-based BWP switch delay requirements refers to the first DL or UL slot, which: - is available after the completion of the BWP switching, and - is assigned to the same serving cell on which the active BWP switching occurs. - RRC based BWP switching: - Clarify the UE behavior for the RRC-based BWP switch by adding the following text in clause 8.6.3: - For the UE supporting low band CA via switchig and configured with switchingPattern-r19, the first DL or UL slot in the RRC based BWP switching delay requirements refers to the first DL or UL slot, which: - is available after the completion of the BWP switching, and - is assigned to the FDD PCell. - Proposal 3 (LGE): - Regarding active BWP switching for FDD PCell and SDL SCell, support the principle as follows: - For a UE supporting LB-CA via switching and configured with switch pattern, during the active BWP switching delay and the first DL or UL slot, the UE remains on the same carrier where the active BWP switching occurs Agreement: Conclude this issue in the next meeting. ### Issue 1-4: others for LB CA Issue 1-4-1: CSSF inside gap for LB CA - Proposal 1 (Apple, HW): the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern. - Recommended WF - FFS: Discuss if P1 is agreeable. Issue 1-4-2: UE capability for RRC based SDL SCell activation in LB CA Agreement: - RAN4 introduces a new capability that allows UE to signal whether it supports RRC based direct SCell activation in Rel-19 NR LB-CA. - Potential details of the capability are shown in following Table . Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 55. NR_LBCA_Sw; 55-x; RRC based Direct SCell Activation at SCell addition in Rel-19 NR LB-CA via switching; UE supports RRC based direct SCell activation at SCell addition in Rel-19 NR LB-CA via switching; 55-1; Yes; N/A; UE does not support RRC based direct SCell activation at SCell addition in Rel-19 NR LB-CA via switching; Per band pair per band combination; FR1 only; N/A; Optional with capability signaling. - RAN4 will have one extra TC for direct SCell activation in LB-CA for UE supporting above FG. - Nokia will draft this TC. Issue 1-4-3: UL TCI state activation - Proposal 1 (Nokia): Introduce UL TCI state switching delay requirements for PCell by taking the LBCA switching pattern impact into account in PL-RS estimation delay. - Recommended WF - FFS: Discuss P1.

RRM performance aspects

-	The following agreements related to RRM performance were captured in [139]:

### Issue 2-1: General Issue 2-1-1: TC list - Whether define TC#4 BFD/CBD for SDL SCell or not? - Option 1 (HW): Yes, define TC on SSB based BFD/CBD for SDL SCell, where SSBs are partial overlapping with switching pattern. - Option 2 (vivo, MTK): No, no need to introduce test to verify BFD/CBD measurement requirements when switching pattern is configured for both FDD PCell and SDL SCell. - Whether define TC#6 PL-RS switch delay for PCell or not? - Option 1 (HW, vivo, ZTE): Define TC on SSB based PL-RS switch delay for PCell, where SSBs are partial overlapping with switching pattern. - The timeline of disabling the switching pattern by MAC CE based SCell deactivation is defined in core part but not any already approved test case to verify it. (ZTE) - (ZTE)In TC#6, the following three test motivations could be verified: - Before the switching pattern being disable, UE follows the switching pattern to pick the available RS occasions; - During the PCell measurement(PL-RS based), verify the timeline of (THARQ+3ms+ 1ms)/slot-length to disable the switching pattern when receiving the SCell deactivation; - All RS occasions are available in PCell after the processing time of disabling the switching pattern. - Option 2 (MTK): No, no need to define test case to cover PL-RS switch delay for PCell. - Recommended WF - FFS: Discuss following proposals: - Whether define TC#4 BFD/CBD for SDL SCell or not? - No need to introduce test to verify BFD/CBD measurement requirements when switching pattern is configured for both FDD PCell and SDL SCell in LB CA - Whether define TC#6 PL-RS switch delay for PCell or not? - Define TC on SSB based PL-RS switch delay for PCell, where SSBs are partial overlapping with switching pattern. - FFS: In TC#6, the following three test motivations could be verified: - Before the switching pattern being disable, UE follows the switching pattern to pick the available RS occasions; - During the PCell measurement(PL-RS based), verify the timeline of (THARQ+3ms+ 1ms)/slot-length to disable the switching pattern when receiving the SCell deactivation; - All RS occasions are available in PCell after the processing time of disabling the switching pattern. ### Issue 2-2: TC configuration Issue 2-2-1: Switching pattern related design - Proposal 1 (vivo): - The switching pattern can be configured as 10 slots for SDL SCell and 30 slots for FDD PCell. - Switching gap is 3 symbols from FDD PCell to SDL SCell and 14 symbols from SDL SCell to FDD PCell. - Proposal 2 (Ericsson): - RAN4 to agree following parameters for the switching pattern. Parameter; Value. Switching pattern; 0000001111111110000000011100000111111000. N1; 35 µs RF switching delay UE, 1 140 µs RF switching delay UE, 3. N2; 35 µs RF switching delay UE, 2 140 µs RF switching delay UE, 4. - RAN4 to agree on following SSB pattern 14 for the LB CA via switching test cases. SSB Parameters; Values. Channel bandwidth; 10 MHz. SSB SCS; 15 kHz. SSB periodicity (TSSB); 20 ms. Number of SSBs per SS-burst; 1. SS/PBCH block index; 0. Symbol numbers containing SSB Note 2; 2-5. Slot numbers containing SSB Note 2; 5. SFN containing SSB; SFN mod (max(TSSB,10 ms)/10 ms) = 0. PRB numbers containing SSB within channel BW; (RBJ, PRBJ+1,.…, PRBJ+19)Note 1. NOTE 1: PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves.. - RAN4 to agree on following SMTC pattern 13 for the LB CA via switching test cases. SMTC Parameters; Values. SMTC periodicity; 20 ms. SMTC offset; 5 ms. SMTC duration; 1 ms. - Recommended WF - FFS: Discuss the P1/P2 Issue 2-2-2: Other detailed configurations - Proposal 1 (HW): - In L1-RSRP test for UE supporting LB CA via switching, TReport is set to 40ms. - Proposal 2 (OPPO): - Focus on non-DRX case when defining test cases. - Not configure gap in the test cases. - Proposal 3 (Ericsson): - For Intra-frequency L3 measurement for activated SDL SCell or PCell test, use event A3 and A6 in the test configuration. - Switching gap duration is verified by counting the HARQ NACK percentage. The HARQ NACK percentage observed during the test shall not be lower than the HARQ NACK percentage measured for normal LB CA operation without the switching mechanism. Exact HARQ NACK percentage can be set by RAN5. - Recommended WF - FFS: Discuss P1/2/3, discuss the followings: - In L1-RSRP test for UE supporting LB CA via switching, TReport is set to 40ms. - Focus on non-DRX case when defining test cases. - Not configure gap in the test cases. - For Intra-frequency L3 measurement for activated SDL SCell or PCell test, use event A3 and A6 in the test configuration. - Switching gap duration is verified by counting the HARQ NACK percentage. The HARQ NACK percentage observed during the test shall not be lower than the HARQ NACK percentage measured for normal LB CA operation without the switching mechanism. Exact HARQ NACK percentage can be set by RAN5.

#### 2.4.2	Remaining Open issues

Core part:

None

Performance part:

-	Specify RRM test cases [RAN4]

RRM aspects to be discussed during the maintenance phase:

(1) Switching Pattern activation/deactivation time margin

(2) BFD/CBD/RLM requirement and BWP switching requirement for LB-CA

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

RAN1 #122bis

- R1-2507653	FL summary #1 of Low band carrier aggregation via switching	Moderator (Apple)

- R1-2507654	FL summary #2 of Low band carrier aggregation via switching	Moderator (Apple)

- R1-2506879	Maintenance on Low band carrier aggregation via switching	vivo

- R1-2506928	Maintenance of Rel-19 low band CA via switching	Huawei, HiSilicon

- R1-2506970	Remaining issues on low band CA via switching	Xiaomi

- R1-2507099	Maintenance of Rel-19 low band CA via switching	CATT

- R1-2507152	Maintenance on low-band CA via switching	OPPO

- R1-2507197	Maintenance on Low band carrier aggregation via switching	ZTE Corporation, Sanechips

- R1-2507235	Maintenance on other Rel-19 topics		Samsung

- R1-2507341	Correction to Low-Band CA via switching behaviour during SCell activation/deactivation				Nokia

- R1-2507356	Remaining issues on low band CA operation via switching for Rel-19	LG Electronics

- R1-2507458	Discussion on low-band CA with switching	Ofinno

- R1-2507637	On remaining issues for Low band Carrier Aggregation via switching	Google

- R1-2507977	Summary of discussion on Rel-19 NR UE features for low band CA via switching					Moderator (NTT DOCOMO, INC.)

- R1-2506930	UE features for low band CA via switching	Huawei, HiSilicon

- R1-2506976	Discussion on UE feature for low band CA via switching	Xiaomi

- R1-2507199	Discussion on UE features for for low band CA via switching	ZTE Corporation, Sanechips

- R1-2507265	UE features for low band carrier aggregation via switching	Samsung

- R1-2507878	UE features for Low Band Carrier Aggregation via switching	Google

- R1-2507979	Updated RAN1 UE features list for Rel-19 NR after RAN1 #122bis

RAN1 #123

- R1-2509089	FL summary #2 of Low band carrier aggregation via switching	Moderator (Apple)

- R1-2508357	Correction for parameter names for LBCA	Nokia

- R1-2508414	Maintenance on Low band carrier aggregation via switching	vivo

- R1-2508521	Maintenance of Rel-19 low band CA via switching	Spreadtrum, UNISOC

- R1-2508666	Remaining issues on low band CA via switching	Xiaomi

- R1-2508708	Maintenance on low-band CA via switching	OPPO

- R1-2508783	Maintenance on other Rel-19 topics	Samsung

- R1-2508898	Remaining issues on low band CA operation via switching for Rel-19	LG Electronics

- R1-2509130	Discussion on low-band CA with switching	Ofinno

- R1-2509209	Maitenance on Low-band CA via switching	Qualcomm Incorporated

- R1-2509244	Maintenance on Low band carrier aggregation via switching	ZTE Corporation, Sanechips

- R1-2508354	UE features Batch B (SBFD and MCE)	Nokia

- R1-2508416	Remaining issues of UE features for NR_duplex_evo and NR_MC_enh2	vivo

- R1-2508483	Remaining issues for Rel-19 WIs in UE features Batch B	Huawei, HiSilicon

- R1-2508509	UE features Batch B	Ericsson

- R1-2508577	UE features for SBFD	CATT

- R1-2508668	UE features for Rel-19 SBFD operation, multi-carrier enhancements and low band CA	Xiaomi

- R1-2508700	UE features for Batch B	OPPO

- R1-2508785	UE features for SBFD and MCE Phase 3	Samsung

- R1-2508816	Discussion on UE feature Batch B	ZTE Corporation, Sanechips

- R1-2508924	UE features for Msg.3 repetition in SBFD operation	Fujitsu

- R1-2509032	Discussion on UE features for evolution of NR duplex operation: SBFD	Ofinno

- R1-2509211	UE features Batch B	Qualcomm Incorporated

- R1-2509264	Discussion on UE features Batch B	NTT DOCOMO, INC.

RAN2 #131bis

- R2-2507144	Discussion on remaining issues of UE capability	OPPO

- R2-2506947	Discussion on UE capability of low band CA via switching	Huawei, HiSilicon

- R2-2507124	UE capability for LBCA via switching	Apple

- R2-2507601	Consideration on the LBCA Capability Signaling	ZTE Corporation

- R2-2507603	Backwards compatibility for low NR band carrier aggregation switching	Ericsson

RAN2 #133

- R2-2509155	Draft Reply LS on Rel-19 higher layers parameters list Post RAN1#122bis	Ericsson

- R2-2509342	Reply LS on Rel-19 higher layers parameters list Post RAN1#122bis	Ericsson

- R2-2509156	ASN.1 names in R19 RAN1 parameter list	Ericsson

- R2-2509343	ASN.1 names in R19 RAN1 parameter list	Ericsson

- R2-2508646	Introduction of UE capability on low band CA via switching	Huawei, HiSilicon

- R2-2508647	Introduction of UE capability on low band CA via switching	Huawei, HiSilicon

- R2-2509173	Introduction of UE capability on low band CA via switching	Huawei, HiSilicon

- R2-2509174	Introduction of UE capability on low band CA via switching	Huawei, HiSilicon

RAN4 #116bis

- R4-2513165	CATT	Discussion on LB-LB CA via switching

- R4-2513166	CATT	TP for TR 38.768 to modify the formulas for the switching gap calculation

- R4-2513167	CATT	draft CR for TS 38.101-1 to introduce RMC for LB-LB CA via switching

- R4-2513782	MediaTek Inc.	Discussion on remaining issues for low NR band CA via switching

- R4-2513854	Apple	On RMCs definition for Rel-19 low NR band CA via switching

- R4-2513939	MediaTek Inc.	(NR_LBCA_Sw-Core) Missing requirements for low NR band CA via switching in TS38.101-1

- R4-2514603	MediaTek Inc.	"(NR_LBCA_Sw-Core) Missing requirements for low NR band CA via switching in TS38.101-1

- R4-2514604	CATT, Mediatek, Anritsu	draft CR for TS 38.101-1 to introduce RMC for LB-LB CA via switching

- R4-2514606	CATT	TP to TR 38.768 to modify the formulas for the switching gap calculation

- R4-2514521	Moderator (Huawei) 	Topic summary for [116bis][114] R19_UERF_maintenance_Part1

- R4-2513211	CATT	Discussion on the maintenance of RRM core requirements of low band CA via switching

- R4-2513492	Huawei, HiSilicon	Discussion on low band carrier aggregation via switching

- R4-2513493	Huawei, HiSilicon	Correction on low band carrier aggregation via switching

- R4-2513605	Apple	On RRM core requirement for R19 LB CA

- R4-2513606	Apple	LS of RAN4 RRM agreement update on switching pattern application

- R4-2513727	LG Electronics	Discussion on RRM requirements for Low Band CA operation

- R4-2513860	Ofinno	Analysis of RRM Requirements for Low Band CA via Switching

- R4-2513885	ZTE Corporation, Sanechips	Discussion on maintenance of RRM aspects of R19 Low band

- R4-2514044	ZTE Corporation, Sanechips	Draft CR on R19 LB CA via switching

- R4-2514048	Nokia	On remaining LBCA requirements

- R4-2514049	Nokia	DraftCR for LBCA SCell activation and deactivation

- R4-2514075	vivo	Remaining issues on RRM requirements for LB CA via switching

- R4-2514295	Ericsson	RRM requirements for low band carrier aggregation via switching

- R4-2514296	Ericsson	Maintenance CR on LB CA via switching

- R4-2514379	vivo	Correction to SCell activation and deactivation requirements for LB-CA

- R4-2514459	Qualcomm Incorporated	RRM requirements of Rel-19 low band carrier aggregation via switching

- R4-2514794	Moderator (Apple)	WF for [116bis][202] NR_LBCA_Sw_RRM

- R4-2513100	OPPO	Discussion on RRM performance requirements for R19 LB CA

- R4-2513215	CATT	Discussion on the impacts of low band CA via switching on RRM performance requirements

- R4-2513494	Huawei, HiSilicon	Discussion on performance requirements for low band carrier aggregation via switching

- R4-2513607	Apple	On RRM performance for R19 LB CA

- R4-2513728	LG Electronics	Discussion on performance requirements for Low Band CA via switching

- R4-2513891	ZTE Corporation, Sanechips	Discussion on Perf part of R19 LB CA

- R4-2514076	vivo	On RRM performance requirements for LB CA via switching

- R4-2514243	MediaTek inc.	Discussion on performance requirements for low-band CA switching

- R4-2514258	Nokia	Discussion on RRM performance requirements for Rel-19 LBCA-sw

- R4-2514298	Ericsson	Performance requirements for low-band carrier aggregation via switching

- R4-2513861	Ofinno	Active BWP Switching Requirements under Low Band CA via Switching

- R4-2514846	Huawei, HiSilicon	Correction on low band carrier aggregation via switching

- R4-2514847	Apple	LS of RAN4 RRM agreement update on switching pattern application

- R4-2514849	ZTE Corporation, Sanechips	Draft CR on R19 LB CA via switching

- R4-2514850	Nokia	DraftCR for LBCA SCell activation and deactivation

- R4-2513508	Moderator (Apple)	Topic summary for [116bis][202] NR_LBCA_Sw_RRM

- R4-2514793	Moderator (Apple)	AdHoc minutes for [116bis][202] NR_LBCA_Sw_RRM

- R4-2514870	Moderator (Apple)	WF for [116bis][202] NR_LBCA_Sw_RRM

RAN4 #117

- R4-2520076	CATT, MediaTek Inc., Anritsu	CR for TS 38.101-1 to introduce DL RMC for LB-LB CA via switching

- R4-2520254	Huawei, HiSilicon	Co-existence on CA_n12-n29 and CA_n28-n67

- R4-2520586	Apple	DL RMC requirements for LB-LB CA via switching with 40mSec periodicity

- R4-2520470	Nokia	CR for LBCA SCell activation and other requirements

- R4-2520661	Apple	On RRM core requirement for R19 LB CA

- R4-2520662	Apple	CR on TR38.768 for NR_LBCA_Sw RRM

- R4-2520695	Ofinno	Analysis of RRM Requirements for Low Band CA via Switching

- R4-2520892	LG Electronics	Discussion on RRM requirements for Low Band CA operation

- R4-2520934	Huawei, HiSilicon	Discussion on low band carrier aggregation via switching

- R4-2520935	Huawei, HiSilicon	Correction on low band carrier aggregation via switching

- R4-2521007	vivo	Remaining issues on RRM requirements for LB CA via switching

- R4-2521013	vivo	Correction to SCell activation and deactivation requirements for LB-CA

- R4-2521343	OPPO	Discussion on RRM requirements for low band carrier aggregation via switching

- R4-2521682	Samsung	Correction on L1-RSRP/SINR measurement requirement for SDL SCell

- R4-2521910	Ericsson	RRM requirements for low band carrier aggregation via switching

- R4-2521911	Ericsson	Maintenance CR on LB CA via switching

- R4-2521932	ZTE Corporation, Sanechips	Discussion on maintenance of RRM aspects of R19 Low band

- R4-2522063	MediaTek inc.	Discussion on low-band CA switching

- R4-2522094	Nokia	On remaining LBCA requirements

- R4-2522152	Qualcomm Incorporated	RRM requirements of Rel-19 low band carrier aggregation via switching

- R4-2522304	vivo	Correction to SCell activation and deactivation requirements for LB-CA

- R4-2520936	Huawei, HiSilicon	Discussion on performance requirements for low band carrier aggregation via switching

- R4-2521008	vivo	On RRM performance requirements for LB CA via switching

- R4-2521333	OPPO	Discussion on test case configuration for low band carrier aggregation via switching

- R4-2521912	Ericsson	Test configurations for low-band carrier aggregation via switching

- R4-2521933	ZTE Corporation, Sanechips	Discussion on Perf part of R19 LB CA

- R4-2522062	MediaTek inc.	Discussion on performance requirements for low-band CA switching

- R4-2522279	Moderator (Huawei)	Topic Summary for [117][114] R19_UERF_maintenance_Part1

- R4-2520075	CATT	CR for TR 38.768 to modify the formulas for the switching gap calculation

- R4-2522441	CATT, MediaTek Inc., Anritsu	CR for TS 38.101-1 to introduce DL RMC for LB-LB CA via switching

- R4-2520696	Ofinno	Active BWP Switching Requirements under Low Band CA via Switching

- R4-2521964	ZTE Corporation, Sanechips	(NR_LBCA_Sw-Core)CR on R19 LB CA via switching

- R4-2522713	Nokia	CR for LBCA SCell activation and other requirements

- R4-2522714	Apple	CR on TR38.768 for NR_LBCA_Sw RRM

- R4-2522715	Samsung	Correction on L1-RSRP/SINR measurement requirement for SDL SCell

- R4-2520829	Moderator (Apple)	Topic summary for [117][204] NR_LBCA_Sw_RRM

- R4-2522630	Moderator (Apple)	WF on NR_LBCA_Sw_RRM

- R4-2522631	Moderator (Apple)	AdHoc minutes for [117][204] NR_LBCA_Sw_RRM

- R4-2520474	Nokia	DraftCR TC for LBCA SCell activation and deactivation

- R4-2520937	Huawei, HiSilicon	TC for SSB based L1-RSRP measurement on SDL SCell for UE supporting LB CA via switching

- R4-2521332	OPPO	DraftCR on TC8 RLM requirement for PCell

- R4-2521913	Ericsson	Draft CR on Intra-frequency L3 for a UE configured with LB CA via switching

- R4-2522064	MediaTek inc.	CR on deactivated SDL SCell measurement and interruption requirement in LBCA

04.02.2025		minor adaptations for RAN #107

17.10.2024		minor adaptations for RAN #106 and 2.6 for RAN6 removed

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template