3GPP TSG RAN meeting #110 									RP-253361

Baltimore, USA, December 8-11, 2025

## Status Report to TSG

Agenda item:			9.5.1.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_AIML_air |  |  |  |  |
| Unique ID | 1020093 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252930 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: Dec ‘25 | Performance part: March ‘26 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 100% | Performance Part: 60% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colours below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo (RAN1); Xiaofeng Liu (RAN4); Marco Belleschi (RAN2/RAN3) |
|  | Company | Qualcomm; CAICT; Ericsson |
|  | Email | juanm@qti.qualcomm.com; liuxiaofeng1@caict.ac.cn; marco.belleschi@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

None (already completed in RAN#109).

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

None (already completed in RAN#109).

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

None (already completed in RAN#109).

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	RAN4#116bis

CSI reporting requirement framework for CSI prediction

Issue 1-1: Performance monitoring

Agreement:

No performance monitoring delay requirements are specified in RAN4 for CSI prediction.

Issue 1-4: Core requirements for PMI prediction reporting

Agreement:

Introduce a new clause under Clause 6 (6.X) in 38.101-4 as “Reporting of predicted PMI”

Issue 1-7: Doppler and MCS

Agreement:

Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD

Introduce test for TDD with the details FFS

Issue 1-8: Generalization

Agreement:

Test setup for the generalization will be further discussed based on the following options

- Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points.

- Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS.

- Option 3: the combination of option 1 and 2.

It is FFS on how to quantify the generalization performance.

Issue 1-9: Activation

Agreement:

No activation requirement in RAN4 spec for CSI prediction will be introduced.

- Note: it is a general understanding that UE has to be ready to start measurements for inference after sending RRC reconfiguration complete. No details will be specified in RAN4.

RRM core requirement and testing framework for beam management

Issue 2-1: Measurement period for inference – case 2

Agreement:

Measurement period for inference – case 2

K(number of samples)*[M]*N*P

K as defined by RAN1

Issue 2-2: Reporting delay

Agreement for reporting delay:

Reuse d’ as to be agreed by RAN1

d’ to be added to the measurement period to reflect the extra time needed by the UE to perform inference

Depending on the RAN1 specifications, d’ shall not be double counted in the overall delay for periodic, aperiodic or semi-persistent reporting

Final specifications to be agreed after RAN1 includes d’ in the specs

Issue 2-3: Reporting delay requirements for monitoring

Agreement:

Do not define any explicit requirements for reporting delay of RS-PAI performance monitoring metric.

Issue 2-4: Draft CRs

CR drafting:

Introduce a new suffix for beam prediction requirements – X (applicable for all new requirements related to beam prediction)

For TCI state switching, introduce a new clause 8.10.2X replicating the known TCI state condition clause for the beam prediction use case

For L1-RSRP, introduce requirements also for SSB to SSB prediction for case 1 and case 2, reuse the same requirements as CSI-RS as applicable (for example only periodic resources)

Introduce new clause on L1-RSRP measurements for prediction 9.5X, include SSB based prediction and CSI-RS based prediction as different subclauses

For performance requirements (including test cases), consider separately SSB to SSB, SSB to CSI-RS and CSI-RS to CSI-RS prediction cases, including downselecting to even a single case.

Issue 2-6: KPIs for prediction

Agreement:

  - Regarding the metric for beam ID only prediction, where top-K predicted beam(s) are reported, RAN4 only specifies the requirements for the following scenario(s):

    - K=1

      - In this case, the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

Issue 2-8: Test system setup

Agreement:

A multi-AoA IFF based test setup with 4 active cross-polarized probes placed in the single plane (xz plane). The relative angular separation between neighboring probes is 30º, 60º, and 60º, respectively.

Issue 2-9: Simplified Channel models

Agreement:

RAN4 will further discuss the simplified Uma and UMi channel model by taking the following aspects into consideration

- Commercial test system limitation, including the number of probes and the separation

- Model simplification should not result in inference performance degradation beyond the margin when the training is done based on the original models.

  - The margin can be discussed and decided further.

- Other aspects are not precluded.

Requirements for positioning

Issue 3-1: Requirements for case 1

To be clarified in the requirements:

- Request location information message should refer to “case 1” positiniong

- Reuse the legacy RSTD measurement delay for all the cases (with gap, w/o gap, PRS aggregation)

- Beam sweeping factor for FR2 including sweeping reduction factor

  - Take the legacy values

- Rx TEG: check on signaling, take the legacy(used in RSTD measurements) approach if it is signaled

- Cover all RRC states

CR structure:

- Introduce new clause for AI/ML based positioning case 1

- Reference existing requirements wherever they are reused to avoid duplicating the same requirement

Introduce requirements for each RRC state in the corresponding clauses:

Draft for RRC idle: Nokia

Draft for RRC connected: E///

Draft for RRC inactive: Nokia

Issue 3-2: CR for case 3a/3b

Agreement:

Clause 13.2(Rx-Tx reporting) from R4-2513670 (CMCC) to be merged into R4-2514119 (Ericsson).

CMCC to further check until end of this meeting on other clauses

E/// to provide final CR in RAN4#117

Issue 3-3: Testing for case 1

Agreement:

Introduce “case 1” tests only for reporting delay, one for FR1 and one for FR2

- Only connected mode

- Only reporting delay to be tested

- Only for the case with gaps

##### 2.4.1.2	RAN4#117

CSI reporting requirement framework for CSI prediction

Issue 1-2: Averaging ofγ

Agreement:

Averaging ofγwill not be specified.

Issue 1-3: Parameters for the test cases

Agreement:

    - FDD 2Rx: 16 CSI-RS ports, TDLA30-20, MCS19

    - TDD 4Rx: 16 CSI-RS ports, TDLA30-20, MCS17

    - FFS: FDD 2Rx: 16 CSI-RS ports, TDLC300-20, MCS X (for generalization purpose)

Issue 1-4: Simulation results

Agreement:

Regarding CSI prediction, it is agreed to define the requirements based on the worst one, which is no worse than non-AI sample and hold, based one, in the current simulations without removing the outliers

RRM core requirement and testing framework for beam management

Issue 2-1: Measurement period for inference – case 2

Agreement:

For BM-case 2 measurement period for inference, RAN4 only specifies the requirements for M=1.

- K: number of RS transmission occasions, which can be measured per Rx beam.

- M:CSI reference resource occasions

Issue 2-2: TCI State Switching Clarifications

Agreement

- RAN4 to confirm that the statement ‘TCI state remains detectable’ means side condition for detectable is met throughout the time period.

- The term ”predictable” won’t be used in the description of the side condition.

Issue 2-6: Value of x

Agreement:

      - Regarding the metric for beam ID only prediction, where top-1 predicted beam(s) are reported, RAN4 requirements are specified based on the Y% of probability that the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – X dB

        - X and Y are TBD based on the simulations

Regarding the metric for RSRP+beam ID prediction, only RSRP accuracy requirement will be defined.

Issue 2-7: Channel model simplification

Agreement:

Take option 2 below as the baseline channel model simplification for Umi CDL-C at 28GHz and if no significant issue is identified on option 2,  the final agreement will be confirmed in RAN4#118.

  - The value of CASA in table 3 is FFS

  - Interested companies are encouraged to evaluate the option 2, including based on the methodology described in option 1.

Option 2: Simplification based on weak clusters

    - Consider the channel models described in Table 2 (Option A/B) and Table 3 (Option C) as candidates for CDL-based simplified channel models for multi-AoA testing of AI/ML BM.

- Table 2 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

- Table 3 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | FFS | 0.5726 | 0 | 7 |  |

Issue 2-8:	Channel quality/performance evaluation

Agreement:

Evaluation of channel model suitability to be used in the tests:

Use reference (Table 7.2.2-3 from TR 38.827 )/simplified (defined as in table 2 in R4-2521418) CDL-C in agreed as baseline in Topic 2-7(include reference to table in R&S paper)

    - Step 1: Simulate performance (inference) with the reference CDL channel.

    - Step 2: Simulate performance (inference) with the simplified CDL channel.

    - Step 3: The prediction difference between the “results” of step 1 and step 2 in the above steps should meet some criteria.

- For beam-ID only prediction, the results are defined as

  - Regarding the metric for beam ID only prediction, where top-1 predicted beam(s) are reported, RAN4 requirements are specified based on the Y% of probability that the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – X dB

    - Use same UE AI/ML model(same fully trained model with the same training data) for step 1 and step 2

Average over multiple UE orientations (one orientation means all data for a single inference is taken in a fixed orientation)

FFS how to average over different UE locations/Tx beams

FFS whether other parameters are needed

Issue 2-9:	Test cases/scenarios

Tests only for case 1

Test scenarios:

SSB to CSI-RS

CSI-RS to CSI-RS

Test case scenarios:

    - TCI state switch with predicted known conditions (CSI-RS to CSI-RS is applicable for UE supports FG59-1)

  - FFS: SSB to CSI-RS

    - Test L1-RSRP prediction reporting delay.

    - Test prediction accuracy based on measurement, and prediction covers the below options with respect to UE capability:

        - L1-RSRP prediction

        - Beam ID prediction

FFS on any other tests

FFS how many tests, testing time and coverage to also be considered

RRM core requirement and testing framework for Positioning accuracy enhancement

Issue 3-5: UE capabilities for inference time per frequency layer

Agreement: introduce the following new capability:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_AIML | 58-x-x | Inference time per positioning frequency layer for AI-ML positioning case 1 | The maximum required inference time per configured positioning frequency layer for AI-ML positioning case 1 | 58-x-x | n/a | n/a | Candidate values: {1,2,3, 4, 5, 6, 7, 8, 9, 10, 15, 20} ms | Optional with capability signaling |

- Prerequisite FG info is needed

- The granularity of the feature is to be included.

Issue 3-6: UE capabilities for positioning

Agreement: introduce the following new capability:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_AIML | 58-x-x | PRS measurement for reduced sample for AI-ML positioning case 1 | The capability to support reporting UE co-ordinate based on measuring on 1 or 2 samples of a DL PRS resource set in AI-ML positioning case 1 | 58-x-x | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_AIML | 58-x-x | Support of lower RX beam sweeping factor for AI-ML positioning case 1 | 1. Support of the lower RX beam sweeping factor than 8 for FR2 in AI-ML positioning case 1 2. Number of RX beam sweeping factor in AI-ML positioning case 1 | 58-x-x | n/a | n/a | Component 2: candidate values {1, 2, 4, 6} Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_AIML | 58-x-x | UE-RxTEGs for AI-ML positioning case 1 | 1. Support of UE-RxTEGs for UE-based AI-ML positioning case 1 2. The maximum number of UE-RxTEG, which is supported and reported by UE for UE-based AI-ML positioning case 1 | 58-x-x | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |

- Prerequisite FG info is needed

- The granularity of the feature is to be included.

- Other missing info based on the UE feature list format is to be included

Issue 3-7: Test for case 1

Agreement:

  - Reuse the existing reporting delay tests for FR1 and FR2

    - Modify configuration such that UE is configured to report AI/ML based positioning

    - FR1: test configurations in Table A.6.6.12.1.1-1, general test parameters in Table A.6.6.12.1.1-2, and cell specific test parameters in Table A.6.6.12.1.1-3

      - TDL-A channel model

    - FR2: test configurations in Table A.7.6.9.1.1-1, general test parameters in Table A.7.6.9.1.1-2, and cell specific test parameters in Table A.7.6.9.1-3

      - TDL-C channel model

General part

Issue 4-2:	Model update/activation

Agreement:

No R19 requirements will be specified in RAN4 for model update/activation

#### 2.4.2	Remaining Open issues

None for core requirements.

For performance requirements, the following items require further work:

- Performance requirements and test framework for CSI prediction

- Performance requirements and test framework for Beam management

- Performance requirements and test framework for Positioning accuracy

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

## 4.2	RAN2

## 4.3	RAN4

#### 4.3.1	RAN4#116bis

// Moderator summary and conclusions

R4-2514519	Topic Summary for [116bis][112] R19 AI for air interface

Type: other		For: Information
					Source: Moderator

Decision:		Noted.

Minutes and agreements in the online session and ad hoc

Please refer to the hyperlink below for the detailed minutes of online session:

- draft R4-2514519.docx

R4-2514629	WF on [116bis][112] R19 AI for air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Revised to R4-2514631 (from R4-2514629).

R4-2514631	WF on [116bis][112] R19 AI for air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Approved.

R4-2514632	AH minutes on [116bis][112] R19 AI for air interface

Type: other		For: Approval
					Source:

Decision:		Noted.

R4-2514634	WF on updated simulation assumption for AI based CSI prediction

Type: other		For: Approval
					Source: OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, CMCC, CTC, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple

Decision:		Approved.

R4-2514635	Summary on the CSI prediction simulation results

Type: other		For: information
					Source: OPPO

Decision:		Noted.

R4-2514639	Summary on beam management simulation results

Type: other		For: Approval
					Source: vivo

Decision:		Noted.

// General aspects

R4-2513410	Discussion on general aspects for AI/ML for NR air interface

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2513419	Discussion on general aspects on AIML for NR air interface

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2513582	On general issues on AI/ML

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Noted.

R4-2513664	Discussion on general aspects for AI/ML for NR air interface

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Noted.

R4-2513689	General Aspects – Post-Deployment Validation based on performance monitoring

Type: discussion		For: (not specified)
					Source: Korea Testing Laboratory

Decision: 		Noted.

R4-2513953	Discussion on Rel-19 RAN4 UE feature list for NR_AIML_air

Type: discussion		For: (not specified)
					Source: CAICT.

Decision: 		Noted.

R4-2514307	On General Aspects of AI/ML for NR Air Interface Phase 1

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Noted.

// CSI reporting requirement and testing framework for CSI prediction

R4-2513404	View on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		Noted.

R4-2513411	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Noted.

R4-2513420	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		Noted.

R4-2513539	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: (not specified)
					Source: CAICT

Decision: 		Noted.

R4-2513583	CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Noted.

R4-2513662	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Noted.

R4-2513765	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		Noted.

R4-2513805	CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Approval
					Source: OPPO

Decision: 		Noted.

R4-2513973	Discussion on CSI prediction

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the AI/ML-based CSI prediction.

Decision: 		Noted.

R4-2514351	CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Noted.

R4-2514456	AI-ML based CSI Prediction: Core Requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Noted.

// RRM core requirement and testing framework for beam management

R4-2513080	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		Noted.

R4-2513217	Discussion on RRM core requirements and testing framework for BM

Type: discussion		For: Discussion
					Source: CATT

Decision: 		Noted.

R4-2513412	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Noted.

R4-2513413	Draft CR on RRM core requirements for support of AIML for NR Air Interface for beam management

Type: draftCR		For: Endorsement
					38.133 v19.2.0
					Source: vivo

Decision:		Revised to R4-2514630 (from R4-2513413).

R4-2514630	Draft CR on RRM core requirements for support of AIML for NR Air Interface for beam management

Type: draftCR		For: Endorsement
					38.133 v19.2.0
					Source: vivo

Decision:		Endorsed.

R4-2513421	Discussion on RRM core requirement for AIML Beam management

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		Noted.

R4-2513453	Discussion on core part requirement and testing framework for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		Noted.

R4-2513584	RRM core requirements and testing framework Based Beam Management

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Noted.

R4-2513660	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Noted.

R4-2513691	Discussion on AI/ML beam management testability

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		Noted.

R4-2513786	Further discussion RRM core requirement for beam management

Type: other		For: Approval
					Source: Samsung

Decision: 		Noted.

R4-2513806	RRM core requirement and testing framework for beam management

Type: discussion		For: Approval
					Source: OPPO

Decision: 		Noted.

R4-2514057	RRM core requirements and test framework for AI in beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On RRM performance aspects and test cases for MIMO Ph 5

Decision: 		Noted.

R4-2514092	Discussion on the RRM requirements of AIML Beam management

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		Noted.

R4-2514248	Discussion on test setup for AI/ML based beam management

Type: discussion		For: Discussion
					Source: Rohde & Schwarz

Decision: 		Noted.

R4-2514308	RRM core requirements for AI/ML Based Beam Management

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Noted.

R4-2514311	Draft CR on RRM core requirements to support beam management use case of NR AI/ML Air Interface

Type: draftCR		For: Endorsement
					38.133 v19.2.0
					Source: Nokia

Decision:		Withdrawn.

R4-2514457	RRM Core Requirements for AI-ML based Beam Management

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Noted.

// RRM core requirement and testing framework for Positioning accuracy enhancement

R4-2513218	Discussion on RRM core requirements and testing framework for positioning

Type: discussion		For: Discussion
					Source: CATT

Decision: 		Noted.

R4-2513414	Discussion on RRM core requirement and testing framework for Positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Noted.

R4-2513422	Discussion on RRM core requirement for AIML Positioning

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		Noted.

R4-2513585	RRM core requirement and testing framework for Positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Noted.

R4-2513665	Discussion on RRM requirements for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Noted.

R4-2513670	DraftCR on requirements for positioning case 3a

Type: draftCR		For: Endorsement
					38.133 v19.2.0
					Source: CMCC

Decision:		Merged (with R4-2514608).

R4-2514093	Discussion on the RRM requirements aspects of AIML positioning

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		Noted.

R4-2514117	On core requirement for AI/ML based positioning

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses the core requirement for AI/ML based positioning use case 1.

Decision: 		Noted.

R4-2514352	RRM core requirements and testing framework for AI/ML positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Noted.

R4-2514353	draftCR to 38.133 on positioning reporting delay requirements (Rel-19)

Type: draftCR		For: Endorsement
					38.133 v19.2.0
					Source: Nokia

Decision:		Postponed.

R4-2514464	RRM Core Requirements for AI-ML based Positioning

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Noted.

// RRM performance requirements for beam management and positioning accuracy

R4-2513219	Discussion on RRM performance requirements for BM and positioning

Type: discussion		For: Discussion
					Source: CATT

Decision: 		Noted.

R4-2513415	Discussion on RRM performance requirements for beam management and positioning accuracy

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Noted.

R4-2513423	Discussion on RRM performance requirements for beam management and positioning accuracy

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		Noted.

R4-2513454	Discussion on performance requirement for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		Noted.

R4-2513586	RRM performance requirements for beam management and positioning accuracy

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Noted.

R4-2513661	Discussion on RRM performance requirements for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Noted.

R4-2514118	On testing of UE-based positioning case 1

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses test cases for AI/ML based positioning use case 1.

Decision: 		Noted.

R4-2514119	draftCR to 38.133 on AIML for positioning (Rel.19)

Type: draftCR		For: Endorsement
					38.133 v19.2.0
					Source: Ericsson

Abstract:

To implement report mapping for AI/ML positioning use cases 3a and 3b.

Decision:		Revised to R4-2514608 (from R4-2514119).

R4-2514608	draftCR to 38.133 on AIML for positioning (Rel.19)

Type: draftCR		For: Endorsement
					38.133 v19.2.0
					Source: Ericsson,CMCC

Abstract:

To implement report mapping for AI/ML positioning use cases 3a and 3b.

Decision:		Endorsed.

R4-2514404	RRM performance requirements for Beam Management and Positioning accuracy

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Noted.

R4-2514461	RRM Performance Requirements and Simulation Results of AI-ML BM

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Noted.

// Demodulation and/or CSI reporting requirements for CSI prediction

R4-2513405	Discussion and simulation results for AI based CSI prediction

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		Noted.

R4-2513416	Discussion on Demodulation and CSI reporting requirements for CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Noted.

R4-2513424	Discussion on demodulation andor CSI reporting requirements for CSI prediction

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		Noted.

R4-2513587	Demodulation and/or CSI reporting requirements for CSI prediction

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Noted.

R4-2513663	Discussion on demodulation requirements for CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Noted.

R4-2513766	Discussion on CSI reporting requirements for CSI prediction

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		Noted.

R4-2513807	Demodulation and CSI reporting requirements for CSI prediction

Type: discussion		For: Approval
					Source: OPPO

Decision: 		Noted.

R4-2513974	PMI reporting requirements with CSI prediction

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the AI/ML-based CSI prediction.

Decision: 		Noted.

R4-2514460	AI-ML based CSI Prediction: Simulation results

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Noted.

#### 4.3.2	RAN4#117

// Moderator summary and conclusions

R4-2522277	Topic Summary for [117][112] R19 AI for air interface

Type: other		For: Information
					Source: Moderator

Decision: 		Note.

Minutes and agreements in the online session and ad hoc

Please refer to the hyperlink below for the detailed minutes of online session:

- R4-2522277.docx

7.RAN4#117-AIMLAdhocMeetingMinutes_v01.docx

R4-2522430	WF on [117][112] R19 AI for air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Approved.

R4-2522431	AH minutes on [117][112] R19 AI for air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Noted.

R4-2522439	Summary on beam management simulation results

Type: other		For: information
					Source: vivo

Decision:		Noted.

R4-2522440	Summary on CSI prediction simulation results

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2522363	Big CR for R19 AI/ML Positioning

Type: CR		For: Agreement
					38.1xx-0y v18.x.0	  CR-  rev  Cat: B (Rel-1x)

					Source: Ericsson

Decision:		Approved.

// General aspects

R4-2520459	Discussion on general aspects for AI/ML for NR air interface

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Note.

R4-2520612	On general issues on AI/ML

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Note.

R4-2520705	On General Aspects of AI/ML for NR Air Interface Phase 1

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Note.

R4-2521003	Discussion on Rel-19 RAN4 UE feature list for NR_AIML_air

Type: discussion		For: (not specified)
					Source: CAICT.

Decision: 		Note.

R4-2521075	General Aspects - Post-Deployment Validation based on performance monitoring

Type: discussion		For: (not specified)
					Source: Korea Testing Laboratory

Decision: 		The document was withdrawn.

R4-2521082	General Aspects - Post-Deployment Validation based on performance monitoring

Type: discussion		For: Discussion
					Source: Korea Testing Laboratory

Decision: 		Note.

R4-2521379	Discussion on general aspects on AIML for NR air interface

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		Note.

// CSI reporting requirement and testing framework for CSI prediction

R4-2520234	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		Note.

R4-2520457	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Note.

R4-2520613	CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Note.

R4-2521016	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Note.

R4-2521237	CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Approval
					Source: OPPO

Decision: 		Note.

R4-2521380	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		Note.

R4-2521490	View on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		Note.

R4-2521898	CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Note.

R4-2522149	AI-ML based CSI Prediction: Core Requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Note.

// RRM core requirement and testing framework for beam management

R4-2520144	Discussion on RRM core requirements and testing framework for BM

Type: discussion		For: Discussion
					Source: CATT

Decision: 		Note.

R4-2520346	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		Note.

R4-2520455	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Note.

R4-2520487	Discussion on core part requirement and testing framework for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		Note.

R4-2520614	RRM core requirements and testing framework Based Beam Management

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Note.

R4-2520706	RRM core requirements for AI/ML Based Beam Management

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Note.

R4-2520709	Draft CR on RRM core requirements to support beam management use case of NR AI/ML Air Interface

Type: draftCR		For: Endorsement
					38.133 v19.2.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia

Decision: 		The document was withdrawn.

R4-2521017	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Note.

R4-2521023	CR on RRM core requirements for support of AI/ML for NR Air Interface for beam management

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6097  rev  Cat: B (Rel-19)

					Source: vivo

Decision:		Revised to R4-2522359 (from R4-2521023).

R4-2522359	CR on RRM core requirements for support of AI/ML for NR Air Interface for beam management

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6097  rev  Cat: B (Rel-19)

					Source: vivo, Nokia

Decision:		Revised to R4-2522457 (from R4-2522359).

R4-2522457	CR on RRM core requirements for support of AI/ML for NR Air Interface for beam management

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6097  rev  Cat: B (Rel-19)

					Source: vivo, Nokia, Ericsson

Decision:		Agreed.

R4-2521211	Draft CR on RRM core requirements for support of AI/ML for NR Air Interface for beam management

Type: draftCR		For: Endorsement
					38.133 v19.2.0	  CR-  rev  Cat: F (Rel-19)

					Source: Ericsson

Abstract:

Corrections on the last endorsed CR

Decision:		Merged (with R4-2522359).

R4-2521212	RRM core requirements and test framework for AI in beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

RRM core requirements and test framework for AI in beam management

Decision: 		Note.

R4-2521238	RRM core requirement and testing framework for beam management

Type: discussion		For: Approval
					Source: OPPO

Decision: 		Note.

R4-2521259	Discussion on AI/ML beam management testability

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		Note.

R4-2521381	Discussion on RRM core requirement for AIML Beam management

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		Note.

R4-2521418	Discussion on channel model for AI/ML based beam management

Type: discussion		For: Discussion
					Source: Rohde & Schwarz

Decision: 		Note.

R4-2521469	CR on RRM core requirements to support beam management use case of NR AI/ML Air Interface

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6159  rev  Cat: B (Rel-19)

					Source: Nokia

Decision: 		The document was revised to R4-2522312.

R4-2521546	Discussion on the RRM requirements of AI/ML Beam management

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		Note.

R4-2522150	RRM Core Requirements for AI-ML based Beam Management

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Note.

R4-2522312	CR on RRM core requirements to support beam management use case of NR AI/ML Air Interface

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6159  rev 1 Cat: B (Rel-19)

					Source: Nokia

(Replaces R4-2521469)

Decision:		Merged (with R4-2522359).

// RRM core requirement and testing framework for Positioning accuracy enhancement

R4-2520615	RRM core requirement and testing framework for Positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Note.

R4-2521018	Discussion on RRM core requirement and testing framework for Positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Note.

R4-2521202	On core requirement for AI/ML based positioning

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses the core requirement for AI/ML based positioning use case 1.

Decision: 		Note.

R4-2521203	CR to 38.133 on core requirement for DL AI/ML positioning in RRC_CONNECTED mode

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6141  rev  Cat: B (Rel-19)
					Source: Ericsson

Abstract:

CR to implement core requirement for DL AI/ML positioning in RRC_CONNECTED mode

Decision:		Revised to R4-2522361 (from R4-2521203).

R4-2522361	CR to 38.133 on core requirement for DL AI/ML positioning in RRC_CONNECTED mode

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6141  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

CR to implement core requirement for DL AI/ML positioning in RRC_CONNECTED mode

Decision:		Revised to R4-2522456 (from R4-2522361).

R4-2522456	CR to 38.133 on core requirement for DL AI/ML positioning in RRC_CONNECTED mode

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6141  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

CR to implement core requirement for DL AI/ML positioning in RRC_CONNECTED mode

Decision:		Endorsed.

R4-2521382	Discussion on RRM core requirement for AIML Positioning

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		Note.

R4-2521899	RRM core requirements and testing framework for AI/ML positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Note.

R4-2521901	CR to 38.133 on positioning reporting delay requirements in RRC_IDLE state

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6224  rev  Cat: B (Rel-19)

					Source: Nokia

Decision:		Revised to R4-2522360 (from R4-2521901).

R4-2522360	CR to 38.133 on positioning reporting delay requirements in RRC_IDLE state

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6224  rev  Cat: B (Rel-19)

					Source: Nokia

Decision:		Revised to R4-2522461 (from R4-2522360).

R4-2522461	CR to 38.133 on positioning reporting delay requirements in RRC_IDLE state

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6224  rev  Cat: B (Rel-19)

					Source: Nokia

Decision:		Endorsed.

R4-2521902	CR to 38.133 on positioning reporting delay requirements in RRC_INACTIVE state

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6225  rev  Cat: B (Rel-19)

					Source: Nokia

Decision:		Revised to R4-2522362 (from R4-2521902).

R4-2522362	CR to 38.133 on positioning reporting delay requirements in RRC_INACTIVE state

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6225  rev  Cat: B (Rel-19)

					Source: Nokia

Decision:		Revised to R4-2522462 (from R4-2522362).

R4-2522462	CR to 38.133 on positioning reporting delay requirements in RRC_INACTIVE state

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6225  rev  Cat: B (Rel-19)

					Source: Nokia

Decision:		Endorsed.

R4-2522157	RRM Core Requirements for AI-ML based Positioning

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Note.

// RRM performance requirements for beam management

R4-2520145	Discussion on RRM performance requirements for BM

Type: discussion		For: Discussion
					Source: CATT

Decision: 		Note.

R4-2520456	Discussion on RRM performance requirements for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Note.

R4-2520488	Discussion on performance requirement for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		Note.

R4-2520544	Simulation Results for AI-ML air interface Beam Management

Type: discussion		For: Discussion
					Source: Keysight Technologies UK Ltd

Decision: 		Note.

R4-2520616	RRM performance requirements for beam management

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Note.

R4-2521019	Discussion on RRM performance requirements for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Note.

R4-2521213	RRM performance requirements for AI in beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

RRM performance requirements for AI in beam management

Decision: 		Note.

R4-2521383	Discussion on RRM performance requirements for beam management

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		Note.

R4-2521468	RRM performance requirements for beam management

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Note.

R4-2521470	Draft CR on RRM performance requirements for beam management use case with NR AI/ML Air Interface

Type: draftCR		For: Endorsement
					38.133 v19.2.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia

Decision:		Postponed.

R4-2521547	Discussion on RRM performance requirements AI beam management

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		Note.

R4-2522154	RRM Performance Requirements and Simulation Results of AI-ML BM

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Note.

// RRM performance requirements for beam management and positioning accuracy

R4-2520617	RRM performance requirements for positioning accuracy

Type: discussion		For: Discussion
					Source: Apple

Decision: 		Note.

R4-2521015	Discussion on RRM performance requirements for positioning accuracy

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Note.

R4-2521204	On test cases for UE-based positioning case 1

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses test cases for AI/ML based positioning use case 1.

Decision: 		Note.

R4-2521205	CR to 38.133 on report mapping for AI/ML positioning use cases (Rel. 19)

Type: CR		For: Agreement
					38.133 v19.2.0	  CR-6142  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

CR to implement report mapping for AI/ML positioning

Decision:		Endorsed.

R4-2521384	Discussion on RRM performance requirements for positioning accuracy

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		Note.

R4-2521548	Discussion on RRM performance requirements AI positioning

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		Note.

// Demodulation and/or CSI reporting requirements for CSI prediction

R4-2520235	Discussion on CSI reporting requirements for CSI prediction

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		Note.

R4-2520458	Discussion on demodulation requirements for CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		Note.

R4-2520629	Simulation Results for CSI Prediction

Type: discussion		For: Information
					Source: Apple

Decision: 		Note.

R4-2520630	Draft CR to Introduce PMI reporting requirements with CSI Prediction

Type: draftCR		For: Endorsement
					38.101-4 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision:		Noted.

Chair: new CR tdoc number will be assigned by Jin.

R4-2522357	CR to Introduce PMI reporting requirements with CSI Prediction

Type: draftCR		For: Approval
					38.101-4 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision:		Agreed.

R4-2521020	Discussion on Demodulation and CSI reporting requirements for CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision: 		Note.

R4-2521233	PMI and CSI-PAI reporting requirements with CSI prediction

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the PMI reporting requirements of AI/ML-based CSI prediction and CSI-PAI reporting requirements.

Decision: 		Note.

R4-2521239	Demodulation and CSI reporting requirements for CSI prediction

Type: discussion		For: Approval
					Source: OPPO

Decision: 		Note.

R4-2521385	Discussion on demodulation andor CSI reporting requirements for CSI prediction

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		Note.

R4-2521414	On Demodulation and CSI Reporting AIML

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		Note.

R4-2521491	Discussion and simulation results for AI based CSI prediction

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		Note.

R4-2522153	AI-ML based CSI Prediction: Simulation results

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		Note.

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template