3GPP TSG RAN Meeting #110						RP-253363

Baltimore, USA, December 8-11, 2025

Agenda item:	9.7

Source: 	Qualcomm Incorporated

Title: 	On support of SIB1 repetition for TN

Document for:	Discussion

# Background

In RAN#108 the following agreement was reached in RP-251857:

Proposal: Common PDCCH repetition introduced in Rel-19 NR NTN WI is also applicable for TN for FR1 only.

This RAN endorsement resulted in the following RAN1 agreement:

Agreement Adopt the following TP for TS 38.213 related to the following Tdocs from agenda item 8.14 Rel-19 TEI R1-2506006 Discussion on common PDCCH repetition (Rel-19 NTN) for TN Spreadtrum, UNISOC R1-2506049 Draft CR on applicability of common PDCCH repetition for TN OPPO R1-2506259 Discussion on common PDCCH repetition (Rel-19 NTN) for TN ZTE Corporation, Sanechips R1-2506387 On common PDCCH repetition (Rel-19 NTN) for TN [Common_PDCCH_rep_TN] Huawei, HiSilicon WI Code: TEI19 TEI identifier: [Common_PDCCH_Rep_TN] Reason for change: In RAN#108, it was agreed to apply common PDCCH repetition for TN for FR1. Summary of change: Type0-PDCCH repetition introduced in Rel-19 NR NTN WI is also applicable for TN for FR1 only, “NTN” in “For an NTN cell in FR1” is removed. Consequence if not approved: Extension of common PDCCH repetition to TN is not captured in the specifications.. 13 UE procedure for monitoring Type0-PDCCH CSS sets *** Unchanged parts are omitted *** - For μ∈{0, 1, 2, 3} and for a SS/PBCH block index i, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+1. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+1 are provided by Table 13-11 and Table 13-12. For an NTN cell in FR1, if the PBCH payload bit aA+7 has value 1, the UE assumes that a same PDCCH candidate for a CCE aggregation level in slots n0 and n0+1 provides same information for DCI format 1_0 with CRC scrambled by the SI-RNTI. *** Unchanged parts are omitted ***.

In RAN1 specifications, SIB1 repetitions are enabled whenever common PDCCH is enabled, as per R1-2505000 (TS 38.214):

A UE capable of PDSCH repetitions for broadcast channels, which assumed the DCI format 1_0 in the Type0 PDCCH CSS of searchSpaceZero transmitted with two inter-slot repetitions may assume that PDSCHs scheduled by the DCI format 1_0 have also been transmitted with inter-slot repetitions in the same slots as the Type0 PDCCH CSS, with the same RV as indicated by the DCI format 1_0.

During the RAN1 discussions in the last few meetings, it was not decided whether the SIB1 repetitions would also apply to TN. In this contribution, we review the status of specifications and UE features, and present our views on the issue above.

# Discussion

In RAN1#122b and RAN1#123, there was some discussion (without conclusion) on whether to support SIB1 repetitions for TN. In our view, the situation is as follows:

- Current RAN1 specifications already support both Type0-PDCCH repetition and SIB1 repetition for TN (as per the CRs approved above in TS 38.213 and TS 38.214).

- UE features (as endorsed in R1-2509584) do not support SIB1 repetition for TN.

This creates a misalignment between RAN1 specifications and UE features that needs to be resolved. RAN plenary should provide guidance on how to solve this issue.

We think it would be beneficial to follow the same specifications for TN and NTN for the following reasons:

- It is in general beneficial to keep common implementations across TN and NTN. Not enabling SIB1 repetition for TN would create additional implementation burden of the UE, which would need to change its behavior depending on whether the cell is TN or NTN.

- More critically, upon decoding of the MIB the UE would not know if the cell is TN or NTN, since this information is contained in SIB1. Under current specifications, there are overlapping frequencies between TN and NTN, e.g. between n256 and n65/66, and this situation is expected to increase in the future. Therefore, the UE may not be able to uniquely determine whether SIB1 repetition is enabled or not, further increasing the UE complexity.

Observation: Not supporting SIB1 repetition in TN has the following issues:

- The PBCH payload bit aA+7 already enables both Type0-PDCCH and SIB1 repetition together in NTN. Separating these features for TN would require additional signaling mechanisms or create ambiguity in the interpretation of this bit in the case of overlapping frequencies between TN and NTN.

- Implementation complexity to handle both cases.

In view of the above, we make the following proposal:

Proposal: SIB1 repetition is supported for TN, following the exact same implementation as for NTN.

- No RAN1 specification change is needed.

- Update RAN1 UE features accordingly to align with the current specifications.

# Summary

In this contribution we provided our views on the issue of supporting SIB1 repetition for TN. We made the following observations and proposals:

Observation: Not supporting SIB1 repetition in TN has the following issues:

- The PBCH payload bit aA+7 already enables both Type0-PDCCH and SIB1 repetition together in NTN. Separating these features for TN would require additional signaling mechanisms or create ambiguity in the interpretation of this bit in the case of overlapping frequencies between TN and NTN.

- Implementation complexity to handle both cases.

Proposal: SIB1 repetition is supported for TN, following the exact same implementation as for NTN.

- No RAN1 specification change is needed.

- Update RAN1 UE features accordingly to align with the current specifications.