3GPP TSG RAN Meeting #110					RP-253392

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1

Source:	Reliance Jio, WiSig, IIT-H, CEWiT, IIT-M

Title:	6G RAN Requirements

Document for:	Discussion and Decision

# 1	Introduction

5G Fixed Wireless Access (FWA) in 5G NR technology ha sbeen used to provide high-speed, low-latency internet to a fixed location, like a home or business, via a wireless connection to a nearby cell tower. It acts as a wireless alternative to traditional wired broadband like fiber or cable, especially beneficial for areas where laying physical lines is difficult or expensive.

In recent years, mobile data usage has grown by 70%-200% per annum. The main reason for this exponential increase in mobile traffic is the new mobile applications in smartphones, which consumes almost all the available free radio resources. These applications, such as high-definition video streaming, real-time interactive games, wearable mobile devices, ubiquitous health care, do not only demand higher data rates but also require an improved Quality of Experience (QoE).

FWA provides much needed benefits such as deliver gigabit-speed internet, comparable to fiber optic connections (when bandwidth is sufficient), suitable for real-time applications like online gaming, video conferencing, and virtual/augmented reality. Installation is quicker and less expensive than running new physical cables and in countries like India it helps address the digital divide in an effective way to provide broadband to rural, remote, or underserved areas where laying cables is not cost-effective.

# 2	Discussion

In 5G, the network is designed primarily for enhanced Mobile Broadband (eMBB) services. When FWA is deployed on such a network, it has been observed that even fixedline of sight connections see substantial SINR fluctuations due to the over all mobility (an dbeam forming) optimization algorithms which were primarily developed for mobility considerations. FWA services have different device and usage patterns and even deployment scenarios can vary.

Observation 1: FWA service deployed along with Mobility service suffers from signal fluctuations when when its line of sight.

FWA service could benefit from reduced capacity base stations (with regards to Massive MIMO support, mobility algorithms etc) and the deployment consideratios vary substantially from a typical mobility deployment.

Some of the possible deployment scenarios include

- Regular FTTX mode deployment (Small cell connected directly to ISP network)

- The FWA can also be deployed as a Trusted non-3GPP access node. The Trusted non-3GPP access refers to a way for a device to connect to a mobile core network (like 5G) as a  non-3GPP technology

- FWA only mode over 5G Core network: Given that the 5G architecture supports a UE connecting to 5G Core over WLAN access (in this case it can be a 6G FWA CPE) only without requiring primary connectivity over cellular access, a FWA only UE which supports 6G NAS functionality and includes a USIM (providing 3GPP IMSI based identity and SIM credentials), can still connect directly to a 6G Core.

Observation 2: FWA service can be deployed over multiple architecture possibilities unlike mobility service.

In order to support FWA service (which could be the first service deployed in 6G given the frequency characteristics of FR3) it is important that FWA RAN nodes be seperatey defined out side the scope of the Mobility RAN definition. Further it is clear that FWA devices can support specific HP UE classes

It is proposed to study potential enhancements as follows:

- The potential air interface modifications (or possible new air interface) for FWA

- Study possible independent FWA deployment architectures (from mobility deployments) and develop appropriate protocols as necessary.

- Device capabilities specifically for FWA service / deployment architecture and the use of  mmWave (FR2) and FR3 spectrum to support FWA services

- Support for interference management/coordination schemes for independent deployment of FWA and mobility services in the same band and in adjacent bands.

Observation 3: Cost considerations will be key for FWA service deployment as these are mostly lign of signt (supporting PTP and PTM services), i.e. the base station cost will have to be substantially lower than a mobility base station.

# 3 Text proposal based on TR 38.914 V0.2.0

4	Deployment scenarios

---------

## 4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

--------------

--------------

## 4.11	[Urban grid]

## 4.12	[Highway Scenario]

## 4.13	[Fixed Wireless Access]

Table 4.13: Attributes for FWA scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | TBD |
| Aggregated system bandwidth |  |
| Layout |  |
| ISD |  |
| BS antenna elements |  |
| UE antenna elements |  |
| Service deployment and speed options (NOTE: each service is set to be evaluated individually. However, evaluation of combination of services is not precluded) |  |
| Service profile |  |

5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

- The design of the 6G RAN shall allow for the development of a seperate 6G FWA RAN node and architecture.

# 4	Conclusion

Proposal 1: RAN shall task the RAN working groups to study FWA as a separate RAN node apart from the mobility RAN node with considerations of lower FWA BS cost, specific enhancements related to, e.g., air interface, protocol and device enhancements (e.g., HPUE). Accordingly, update the requrirements and deployment sections in TR 38.914 V02.0 as indicated o include the FWA relevant aspects per the test proposal section.