3GPP TSG RAN Meeting #110															RP-253423

Baltimore, USA, 8th – 11th December, 2025

Agenda item:	8.2.2

Source:	Nokia

Title:	Minimum requirement for 6G UE channel bandwidth

Document for:	Discussion and Decision

# Introduction

In RAN1#122 in August, discussions commence on the smallest maximum supported RF and BB UE bandwidths, with further discussion being sought from RAN plenary:

Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

In the last two RAN1 meetings in October and November, RAN1 discussions on the smallest maximum supported RF and BB UE bandwidth have not progressed significantly, with only high-level agreements being made:

Agreement

If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR,

- Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

- Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations

Agreement

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - Overall device complexity

  - Overall system performance impact

  - Energy efficiency for both BS and UE

  - Aim at a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority

In RAN4 meeting#117 in November RAN4 reached following agreement,

### Sub-topic 3-2: Min Channel Bandwidth

- Agreement:

  - Define minimum CBW based on SCS (e.g., 5 MHz for 15 kHz, 10 MHz for 30 kHz, 50 MHz for 120 kHz).

  - Support 3 MHz with 15kHz SCS for particular bands subject to operator requests, commercial need and spectrum regulations,

  - Work with RAN1 for the chosen minimum CBW in initial access design to accommodate majority spectrum scenarios.

In this contribution, we propose a way forward for RAN#110 which could help RAN1 to make progress.

# Discussion

When assessing what is the smallest maximum supported RF and BB UE BWs, it is important to keep in mind that all the communication between the network and all the UEs need to take place within this BW up until the UEs’ BW capabilities are known to the network, including the cell synchronization signals, system information, random access channel, and control and data channels during the random access procedure.

It is clear that 6GR must support 3 MHz operation for specific scenarios in FDD bands where operators have 3 MHz spectrum. However, constraining the performance of the initial access signals in wider bandwidth carriers by forcing them to use exactly the same design as on 3 MHz carriers would make the performance of 6G worse than 5G on the vast majority of carriers, which are wider than 3 MHz.

In 5G NR, the initial access channels in DL (SS block and CORESET#0) define the bottleneck scenario for the minimum channel bandwidth (CBW) size. Until Release 17 the minimum CBW size was 5 MHz for 15 kHz SCS, and it turned out to be challenging to build support for the 3 MHz CBW in Release 18.

It is reasonable to assume that the initial access design will be a bottleneck for the minimum CBW​ also in 6G. Hence, we need to ensure availability of the second (narrowband) SSB/CORESET0 design for channel bandwidths below 5 MHz such as 3 MHz for predefined bands in Rel-21. This should be applied for predefined scenarios with specific sync raster points in specific bands and FDD with 15kHz SCS​.The requirements of 5G NR should be reused for 15kHz SCS FDD and 30 kHz SCS TDD for sub 6 GHz and 30 / 60 kHz around 7 GHz, i.e. defined by 25 RBs. For 120 kHz SCS for higher bands (supported at least for 24.25 – 52.6 GHz, i.e. FR 2-1), we suggest increasing the minimum CBW to 100 MHz both, to allow for a large number of SSB beams by supporting 240 kHz SCS SSB, and as we don’t see the necessity to impair the FR2-1 deployments with the minimum CBW support that may not be otherwise needed.

Table 1 below shows our proposal for smallest maximum supported RF and BB UE BW for different SCS.

Table 1: Minimum channel bandwidth size for 6GR; all UEs are required to support at least this BW

| SCS (kHz) | Min BW (MHz) |
| --- | --- |
| 15 | 5 (3*) |
| 30 | 10 |
| 60 (if supported) | 20 |
| 120 | 100 |

*	Supported only for certain specific bands and sync raster points

Some important caveats apply:

- 3 MHz channel bandwidth is only supported for certain specific bands and sync raster points.

- 3 MHz as the smallest maximum RF and BB UE BW is allowed only for UEs that can only access 3 MHz cells.

- Common signals/channels for initial access shall be designed for operation in 5 MHz (i.e. RAN1#123’s “Opt1”) except in the specific 3 MHz channel bandwidth cases for certain specific bands and sync raster points, for which “adjustments” (e.g. puncturing) are to be designed.

- UEs operating on cells of 5 MHz and above shall operate as if the smallest maximum RF and BB UE BW is as in Table 1.

# Conclusions

In order to expedite the work in RAN1, we propose that RAN#110 agrees the following:

- 3 MHz channel bandwidth is only supported for certain specific bands and sync raster points.

- 3 MHz as the smallest maximum RF and BB UE BW is allowed only for UEs that can only access 3 MHz cells.

- Common signals/channels for initial access shall be designed for operation in 5 MHz (i.e. RAN1#123’s “Opt1”) except in the specific 3 MHz channel bandwidth cases for certain specific bands and sync raster points, for which “adjustments” (e.g. puncturing) are to be designed.

- UEs operating on cells of 5 MHz and above shall operate as if the smallest maximum RF and BB UE BW is as in Table 1.

- The smallest maximum supported RF and BB UE BWs are as follows:

| SCS (kHz) | Min BW (MHz) |
| --- | --- |
| 15 | 5 (3*) |
| 30 | 10 |
| 60 (if supported) | 20 |
| 120 | 100 |

* subject to points 1-3 above.