3GPP TSG RAN Meeting #110	RP-253455

Baltimore, USA, December 8-11, 2025

# Source:	China Telecom

# Title:	New WID: Additional NR and LTE band combinations for features in Rel-20

# Document for:	Approval

Agenda Item:	9.1.5.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Additional NR and LTE band combinations for features in Rel-20

Acronym: NR_LTE_combos_Features_R20

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-20

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
|  | Stage 2 |
|  | Stage 3 |
|  | Other (e.g. testing) |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| DL_intrpt_combos_TxSW_R19 | 1040118 | Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink | 1040118 is a preceding WI which covers R19 Downlink interruption clarification for NR and EN-DC band combinations. This WI is R20 follow-up basket WI to handle additional band combinations via the basket work item approach |
| LTE_NR_R19_Simult_RxTx | 1040125 | Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19 | 1040125 is a preceding WI which covers R19 band combinations for simultaneous Rx/Tx. This WI is R20 follow-up basket WI to handle additional band combinations via the basket work item approach |
| NR_LBCA_Sw | 1060082 | Low NR band carrier aggregation via switching | 1060082 is a preceding WI which specify the generic requirements that enables Low NR band CA via switching based on example band combination of CA_n5A-n29A. This WI is to handle additional band combinations via the basket work item approach |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

Dependency on non-3GPP (draft) specification:

3	Justification

For certain features, specific requirements are defined for particular band combinations. To ensure support for each feature, these requirements must be analysed and specified on a case-by-case basis. This WI is established for this purpose by introducing a consolidated framework. Unlike previous releases which required individual WIs per feature, this WI provides a unified procedure for analysing multiple features and their band combinations for each specific feature or objective.

Objective #1: DL interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink

In Rel-16 dynamic Tx switching between two uplink carriers has been introduced under FR1 RF requirement WI in RP-192251. For some EN-DC and UL CA configurations, DL interruption from UE side is not allowed, and the configurations are requested by operators, and checked by chipset/UE vendors one by one.

In Rel-17, new EN-DC and UL CA configurations were requested by operators, and to handle these new requests, a basket WI was created to work on the new band combinations with no DL interruption. The basket WI started an approach for no DL interruption analysis as captured in TR 37.867 and also outcome into the spec of 38.101-1, which was also followed by Rel-18 basket WI to continue the work for the uplink Tx switching with two uplink bands.

In Rel-18, up to 3 or 4 bands are involved in the determination of one uplink switching, which means under the downlink 3 or 4 bands CA, the switching cases include

- Two bands involved in the uplink switching belong to different uplink serving cells.

- More than two bands involved in the uplink switching.

The first case can refer to the previous approach of Rel-17 basket WI to specify the note for no DL interruption clarification. For the second case, the newly introduced uplink switching in Rel-18 applies, if on any two of the bands the UE is configured with switchingOptionConfigForBandPair set to 'dualUL'. However, the corresponding mandating no DL interruption requirements are not specified in Rel-18 for any band combinations. Furthermore, now RAN4 existing requirement format such as “no for CA_nX-nY” of no DL interruption clarification for 2 uplink bands CA can not apply to 3 or 4 bands, which may need to be specified based on band pair rather than band combination.

In Rel-19, a basket WI was set to work on the request for the band combinations to mandate no DL interruption for dynamic Tx switching. The clarification such as “no for CA_nX-nY” was updated by “no for nX-nY” was updated in order to apply to 3 or 4 UL bands configuration cases in Rel-19 basket WI, and some of band combinations are specified with no DL interruption for 3 or 4 UL bands configured. While, there are still requests for band combinations for the 3 or 4 UL bands configuration cases. Furthermore, the advantage of uplink tx switching is not only to increase uplink throughput but also to enable flexible multi-carrier scheduling. For example, by considering the timely air interface information such as interference experienced by the UE, coverage of different frequency bands, and cell load conditions, the optimal carrier can be dynamically selected for scheduling among the entire available candidate spectrum, thereby further enhancing the utilization of operator carriers. This flexible multi-carrier scheduling scenario is not limited by FDD+TDD combinations, but the FDD+FDD combinations shall be considered. Uplink tx switching feature including FDD+FDD case has been supported by the current specification, no restriction to the duplex of the band. Existing analysis/requirements for mandating no DL interruption mainly cover the CA combinations of FDD+TDD or TDD+TDD. Some analysis provided under Rel-19 DL interruption basket WI has shown possibility of mandating no DL interruption for FDD+FDD combinations.

Objective #2: Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC

Simultaneous Rx/Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15. Specifically, for inter-band CA and EN-DC combination, the capability is used for TDD-TDD and TDD-FDD band combinations. According to the description of the capability, it is conditional mandatory and the condition is described in the field, i.e. indicated in the RAN4 spec which combinations should mandatorily support simultaneous Rx/Tx. For the combinations which have no such indication, the capability is optional, i.e. for UE supporting simultaneous Rx/Tx, the capability should be reported, otherwise, the capability is absent or not reported. Since the capability is important for network scheduling, it should be reported accurately.

In Rel-17, the principles for judging the mandatory capability for a band combination have been discussed, and the cases include:

- FR1+FR1 FDD-TDD band combination

- FR1+FR1 TDD-TDD band combination

- FR1+FR2 FDD-TDD band combination

- FR1+FR2 TDD-TDD band combination

- FR2+FR2 TDD-TDD band combination

In Rel-18, a basket WI for simultaneous Rx-Tx was created. In Rel-18 WI, as the capability is defined for CA, SUL, MR-DC and NR-DC band combinations, and applicability of the corresponding requirements cover different specifications, e.g., TS 38.101-1 and TS 38.101-3, the way to treat simultaneous Rx/Tx capability as well as the requirements is aligned among the specifications.

In Rel-19, further discussions have addressed several specification issues concerning simultaneous Rx-Tx requirements originally identified in Rel-18. Key points include simplifying or removing redundant and overlapping Notes to enhance implementation clarity, clarifying the lack of mandatory simultaneous Rx-Tx notes for certain higher-order EN-DC band combinations to aid vendor implementation and testing, and examining the CA_n40-n41 combination where 4Rx requirements on n41 may require relaxation. The need to support simultaneous Rx-Tx in the CA_n5-n8 combination with overlapping spectrum was also discussed. Additionally, case-by-case analysis of band combinations will be conducted to prevent ambiguity, with outcomes documented in TR38.793 as an extension of the Rel-18 work.

Objective #3: Low NR band carrier aggregation via switching

Operators have long sought to aggregate low-band spectrum to enhance coverage and capacity, especially as users spend significant time connected to these bands in both urban and rural areas. However, device design limitations, such as antenna and RF front-end constraints, make low-band carrier aggregation difficult to implement. To address this, operators are proposing a 3GPP specification update that introduces a switching-based CA solution. This approach allows devices to semi-static RRC-based switch to a SCell when needed, without simultaneous transmission/reception with the PCell, balancing performance and hardware feasibility.

In Rel-19, the effort was launched with band combination CA_n5A-n29A, identified by other combinations such as CA_n12A-n29A, CA_n14A-n29A, and CA_n28A-n71A, as requested by operators.

Rel-20 continues to leverage semi-static RRC-based switching to enhance low-band CA, particularly for scenarios involving switching between FDD and SDL bands, with example band combinations CA_n12-n29 and CA_n28-n67.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

4.1.1	Objective and scope

- Objective #1: DL interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink

The objectives of the core part are as follows:

- Discuss and specify inter-band uplink CA and EN-DC combinations for which DL interruption is not allowed when dynamic switching between two uplink bands is conducted, or when dynamic switching with 3 or 4 uplink bands is conducted for inter-band uplink CA.

- Based on the requested band combinations from operators, analyze if it is feasible to specify no DL interruption in RAN4 specification.

- Based on the feasibility analysis for specific combination, decide to introduce no DL interruption in RAN4 specification.

Note: The requirement of “no DL interruption” to be specified for the band combinations in the WI applies to Rel-20.

- Specify REL-independent way for CA combinations of this WI.

- For the CA combinations with three or four bands, introduce for uplink Tx switching across three or four bands in a REL-independent way starting from Rel-18.

Note: the CA and EN-DC combinations with two bands uplink switching have been introduced in a REL-independent way starting from Rel-16/17.

- Objective #2: Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC

- Identify feasibility of simultaneous Rx/Tx capability/operation for each requested FDD-TDD and TDD-TDD band combinations for CA, SUL, MR-DC based on technical analysis, especially for those with large MSD values. If needed, specify the MSD requirements for the identified band combinations.

- Note 1: Band combinations considered in this WI have to be introduced first via basket WIs or completed in previous releases if necessary.

- Note 2: Whether the simultaneous Rx-Tx capability could be supported or not depends on the evaluation of MSD for the requested band combinations case by case.

- Note 3: Applicability of simultaneous Rx/Tx capability to FDD-TDD band pairs should be reviewed

- Note 4: Band combinations of the following power classes are considered in this WI: PC3, PC2 and PC1.5.

- Align the specification treatment of simultaneous Rx/Tx capability for CA, SUL, MR-DC and NR-DC band combinations.

- Objective #3: Low NR band carrier aggregation via switching

- Specify the band combination specific UE RF requirements to allow switching between different cases to enable low band carrier aggregation via switching for scenario between FDD band and SDL band.

{case 1, case 2}

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

- Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

- Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching

- Consider the following deployment constraints:

- The carrier frequency for all cases is <1GHz.

- Co-located and synchronized network deployment for both carriers.

- Both carriers are in a single TAG.

- SCS 15kHz on both carriers.

- No in-gap interference between DL carriers is expected.

Note: For the objectives which are dependent of ongoing Rel-20 non-basket WI, these objectives will not be included in the basket WI until the completion of the related RAN4 core requirements and capability for the example bands in the corresponding Rel-20 non-basket WI.

4.1.2	Overview of identified combinations for evaluation

The band combinations requested/identified for supporting features are listed below:

- Objective #1: DL interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink

Table 4.1.2-1: Band combinations requested for mandating no DL interruption

| Band combination | Switching band pairs | Note | status |
| --- | --- | --- | --- |
| CA_n1-n5 | n1-n5 | Requested by Bo Liu, China Telecom, liubo1@chinatelecom.cn | Open |
| CA_n1-n3 | n1-n3 | Requested by Bo Liu, China Telecom, liubo1@chinatelecom.cn | Open |
| CA_n1-n3-n78 | n1-n3, n1-n78, n3-n78 | Requested by Bo Liu, China Telecom, liubo1@chinatelecom.cn | Open |
| CA_n1-n28-n78 | n1-n78, n28-n78 | Requested by Takuto Kamei, NTT DOCOMO, takuto.kamei.uk@nttdocomo.com | Open |

- Objective #2: Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC

Table 4.1.2-2: NR band combinations for evaluation to support simultaneous Rx/Tx

| Band combination | Power Class | Note | status |
| --- | --- | --- | --- |
| CA_n78-n104 | PC3, PC2 |  | Open |
| CA_n79-n104 | PC3, PC2 |  | Open |

- Objective #3: Low NR band carrier aggregation via switching

Table 4.1.2-3: Band combinations for low band CA via switching

| Band combination | Switching cases | Note | status |
| --- | --- | --- | --- |
| CA_n13A-n29A | {case 1, case 2} |  | Open |
|  |  |  | Open |

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {e.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {e.g. "TSG#87"} | {e.g. "TSG#89"} | {e.g. rapporteur: <FamilyName>, <GivenName>, <Company>, <email address>} |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | Features supported for the combinations in spec of NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone | RAN #115 | Core part |
| 38.101-3 | Features supported for the combinations in the spec of User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios | RAN #115 | Core part |
| 38.307 | NR Requirements on User Equipments (UEs) supporting a release-independent frequency band | RAN #115 | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s).}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.
Secondary co-Rapporteur(s) are possible for specific secondary task(s)}.

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

7	Work item leadership

RAN WG4

8	Aspects that involve other WGs

None

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

Supporting IM name

China Telecom

Huawei

HiSilicon

NTT DOCOMO

TELUS

Bell Mobility