3GPP TSG RAN Meeting #110	RP-253476

Baltimore, USA, December 8-11, 2025

Agenda item:	9.2.3

Source:	CMCC

Title:	Consideration on coexistence of A-IoT and legacy network

Document for:	Discussion

- Introduction

Rel-20 Ambient IoT will support various scenarios including outdoor scenarios and use cases, and type 2B/C devices do not rely on backscattering. So deployment Rel-20 Ambient IoT network and devices should consider existing operators’ deployment. NB-IoT deployment will continue to exist for a long time, so the coexistence with NB-IoT should be considered.

- Deployment of Ambient -IoT Network

The vast majority of IoT networks are highly cost-sensitive, often operating on significantly lower ARPU compared to traditional smartphone networks. Given these financial constraints, it is particularly advantageous to explore deployment strategies that leverage existing network infrastructure and spectrum allocation. This approach can effectively reduce both CAPEX and OPEX. From a maintenance standpoint, utilizing established networks minimizes the need for extensive new hardware installations and ongoing maintenance efforts, thereby further optimizing operational costs and ensuring long-term sustainability.

Spectrum perspective

Most of the interested carrier frequency for Ambient-IoT is 900MHz, as discussed in WGs, which is targeting for global usage and better coverage in practice.

For 900MHz, LTE FDD(4G), GSM and NB-IoT are deployed in CMCC network. Considering future potential refarming, 5G can also be possible.

Upgrading perspective

The expected upgrade of legacy networks to support Ambient-IoT represents a strategic move towards enhancing connectivity and efficiency in the Internet of Things ecosystem. Here are detailed considerations for different network upgrades:

- Upgrading from 2G Networks

Upgrading from 2G networks to support Ambient-IoT is not considered a viable option. This decision is primarily driven by the fact that 2G networks are expected to be phased out in the near future. As a result, investing in upgrades from 2G infrastructure would not only be economically inefficient but also counterproductive in the long run, given the rapid evolution of communication technologies.

- Upgrading from NB-IoT Networks

Upgrading from NB-IoT networks to support Ambient-IoT is of particular interest. NB-IoT and Ambient-IoT share similarities in transmission bandwidth, which makes the transition more seamless and technically feasible. In practical deployments, many NB-IoT networks experience relatively low traffic loads. This presents an opportunity to share the existing NB-IoT infrastructure with Ambient-IoT, thereby optimizing resource utilization and reducing the need for additional hardware investments. This shared approach can be highly beneficial in scenarios where both technologies coexist, allowing for more efficient spectrum and hardware utilization.

- Upgrading from 4G/5G Networks

Upgrading from 4G/5G networks to support Ambient-IoT is also a promising option. The hardware of 4G/5G networks is sufficiently powerful to handle the processing requirements of Ambient-IoT, ensuring robust and reliable connectivity. However, it is important to note that 4G/5G networks operate on a wider bandwidth compared to Ambient-IoT, which is a narrowband transmission system (e.g., a 1 PRB system with 180 kHz). This wider bandwidth may result in a lower Power Spectral Density (PSD) for Ambient-IoT downlink transmission compared to 4G/5G. Despite this, the existing 4G/5G infrastructure can still serve as a strong foundation for Ambient-IoT deployment, especially when considering the cost and time savings associated with leveraging existing hardware.

Observations 1:

The upgrade path for Ambient-IoT should be carefully evaluated based on the existing network infrastructure and its compatibility with the specific requirements of Ambient-IoT. While upgrading from NB-IoT and 4G networks presents immediate opportunities, the potential of 5G networks should not be overlooked, especially considering its advanced features and future-proof capabilities.

Proposal 1: Upgrading of the legacy network (e.g., NB-IoT or 4G/5G) to support Ambient-IoT network from RAN perspective is considered.

Design consideration for NB-IoT and Ambient-IoT coexistence

- NB-IoT and Ambient-IoT spectrum sharing

With consideration of the spectrum reuse and site upgrading of the NB-IoT network, Ambient-IoT can be deployed in a fast and cost-effective way.

Particular design should be taken into consideration in order to allow Ambient-IoT device to be operated and sharing spectrum with NB-IoT. Always on NB-IoT signals/channels should be avoided, such as in Table 1.

Table 1. Always on signals in NB-IOT[2]

| Signals/channels | Characteristics | Values |
| --- | --- | --- |
| NB-IoT NPSS | Subframe | 5 |
|  | Subframe periodicity | 10 ms |
|  | Sequence pattern periodicity | 10 ms |
|  | Duration | last 11 OFDM symbols are used to carry NSSS in the subframe |
| NB-IoT NSSS | Subframe | 9 |
|  | Subframe periodicity | 20 ms |
|  | Sequence pattern periodicity | 80 ms |
|  | Duration | last 11 OFDM symbols are used to carry NSSS in the subframe |
| NB-IoT NRS | Subframe | Any |
|  | Basic TTI | 1 ms |
|  | Sequence pattern periodicity | 10 ms |
| NB-IoT PBCH | Subframe periodicity | 160ms |
| NB-IoT SIB | Subframe periodicity | 80ms or 160ms |

Figure.1 Always on signals in NB-IoT

In summary, in the case of no data transmission in NB-IoT, if the whole NRS symbols are excluded, Ambient IoT can use no more than (14x20 – 11x5 – 4x15) / (14x20) = ~59% of the resources of the 180 kHz carrier. And if NRS symbols are used by Ambient IoT system, i.e. 4 NRS REs are treated as interference, the available REs for Ambient IoT is about (14x20 – 11x5) / (14x20) = ~80% of the resources of the 180 kHz carrier. SIB1 overhead is not considered here.

For uplink, since both NB-IoT and Ambient-IoT are able to be operated in a subcarrier (15kHz) way. Hence, it is more easier to be FDMed in a PRB for these two systems.

- NB-IoT and Ambient-IoT adjacent carrier

For this case, Ambient-IoT device does not requires to share spectrum. Additional spectrum for Ambient-IoT is required for operator.

Proposal 2: Both NB-IoT and Ambient-IoT spectrum sharing and adjacent carrier deployment are considered for Rel-20 Ambient-IoT.

- Conclusion

In this contribution we discuss the coexistence deployment R20 A-IoT and legacy network, and the following proposal and observation are made,

Observations 1:

The upgrade path for Ambient-IoT should be carefully evaluated based on the existing network infrastructure and its compatibility with the specific requirements of Ambient-IoT. While upgrading from NB-IoT and 4G networks presents immediate opportunities, the potential of 5G networks should not be overlooked, especially considering its advanced features and future-proof capabilities.

Proposal 1: Upgrading of the legacy network (e.g., NB-IoT or 4G/5G) to support Ambient-IoT network from RAN perspective is considered.

Proposal 2: Both NB-IoT and Ambient-IoT spectrum sharing and adjacent carrier deployment are considered for Rel-20 Ambient-IoT.

References

- 3GPP TR 38.914 V0.2.0, Study on 6G Scenarios and Requirements; (Release 20).

- 3GPP TS36.211 v15.0.0, Physical channels and modulation.