3GPP TSG RAN Meeting #110										 RP-253493

# Baltimore, USA, December 8-11, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.194 | CR | 0006 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.194 on including n5 and n28 for A-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | China Telecom, CMCC |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a demand in China to deploy A-IoT in band n5 and n28. Considering n5, n28 and the already-specified band n8 are both sub1GHz frequency bands. Given the spectral proximity and similar RF properties between n5, n28 and n8, co-existence conclusion of n8 can be applied to n5 and n28. For BS and CW RF requirements, it is band-ignored. The same core RF requirements can apply regardless of the specific band within this frequency range. Due to the technical similarity to n8 and the band-ignored nature of the BS and CW RF requirements, band n5 and n28 can be directly incorporated into the A-IoT band list without additional analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add band n5 and n28 into the A-IoT operation band list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band n5 and n28 is not supported for A-IoT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.3.1.5, 5.4.1.3, 6.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This company CR is same as endorsed CR: R4-2523105 in Nov. RAN4#117 meeting. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2523105 |  |  |  |  |  |  |  |  |

5.2	Operating bands

A-IoT is designed to operate in the operating bands defined in table 5.2-1

Table 5.2-1: A-IoT operating bands in FR1

| A-IoT operating band | Uplink (D2R&CW) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (R2D) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- | --- |
| n5 | 824 – 849 | 869 – 894 | FDD |
| n8 | 880 – 915 | 925 – 960 | FDD |
| n28 | 703 – 748 | 758 – 803 | FDD |

< Non-changed part is omitted >

<<Next Change>>

5.3.1.5 	R2D channel bandwidth per operating band

The requirements in this specification apply to the combination of BS channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1. The transmission bandwidth configuration in table 5.3.2-1 shall be supported for each of the BS channel bandwidths within the BS capability. The BS channel bandwidths are specified for the Tx path.

Table 5.3.5-1: BS channel bandwidths and SCS per operating band

| A-IoT Bands | SCS (kHz) | Reader channel bandwidth (kHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 200 | 400 | 600 | 800 |
| n5, n8, n28 | 15 | 200 | 400 | 600 | 800 |

< Non-changed part is omitted >

<<Next Change>>

5.4.1.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each A-IoT operating band are given through the applicable NR-ARFCN in table 5.4.1.3-1, using the channel raster to resource element mapping in clause 5.4.1.2.

Channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.1.3-1 is given as <2>.

Table 5.4.1.3-1: Applicable NR-ARFCN per operating band for enhanced channel raster

| A-IoT operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n5 | 10 | 164800 – <2> – 169800 | 173800 – <2> – 178800 |
| n8 | 10 | 176000 – <2> – 183000 | 185000 – <2> – 192000 |
| n28 | 10 | 140600 – <2> – 149600 | 151600 – <2> – 160600 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.1.3 and 5.3.2.2 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |

< Non-changed part is omitted >

<<Next Change>>

6.5.3.2	Limits and Basic limits

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

For operation in paired spectrum, the ACLR shall be higher than the value specified in table 6.5.3.2-1 in band n5, n8 and n28.

Table 6.5.3.2-1: Base station ACLR limit

| BS R2D channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted (kHz) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 200 | 300 | A-IoT of same BW | Square (180 kHz) | 40dB |
|  | 500 | A-IoT of same BW | Square (180 kHz) | 45dB |
| 400 | 500 | A-IoT of same BW | Square (360 kHz) | 40dB |
|  | 900 | A-IoT of same BW | Square (360 kHz) | 45dB |
| 600 | 700 | A-IoT of same BW | Square (540 kHz) | 40dB |
|  | 1300 | A-IoT of same BW | Square (540 kHz) | 45dB |
| 800 | 900 | A-IoT of same BW | Square (720 kHz) | 40dB |
|  | 1700 | A-IoT of same BW | Square (720 kHz) | 45dB |

<<End of Change>>