3GPP TSG RAN #110			RP-253499

Baltimore, Baltimore, December 8-11, 2025

Agenda Item:	8.2.1.0

Source: 	LG Electronics

Title: 	Consideration of deployment scenarios for vehicular use cases

Document for:	Discussion and decision

Introduction

We discuss recommendations on deployment scenarios for vehicular use cases in this contribution.

Discussion

- 2.1	Urban grid and Highway scenario

Considering the expanded commercial needs and deployments of autonomous driving, the automotive-related applications are one of the most important and promising motivation towards 6G era. From communication perspective, 6GR can not only support the techniques for low latency and high reliability communication, but also support rich services providing infotainment experience to the driver based on e.g. XR-based immersive communication. From sensing perspective, 6GR should be able to provide safe driving through situational awareness to avoid collisions, as well as to make driving more efficient in both energy consumption and traffic management point of view, based on sensing.

With these reasons, Urban grid and Highway scenario that supports automotive-related deployment scenarios should be considered for 6G deployment scenario. We suggest removing the brackets in Section 4.11 and 4.12 in TR 38.914, and study the relevant deployment scenarios.

Proposal 1: For 6GR deployment scenarios, urban grid and highway scenario should be studied. Remove the bracket in [Urban grid] and [Highway Scenario] in Section 4.11 and 4.12 in TR 38.914.

As a starting point of discussion for Urban grid and Highway scenario, we can consider the relevant deployment scenarios and parameters from TR 37.885 [1]. We propose to include the following tables for Urban grid and Highway scenario in Section 4.11 and 4.12 in TR 38.914.

Table 1: Attributes for Urban grid

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro BS: 4GHz |
| Aggregated system bandwidth | Up to 200MHz (DL+UL) |
| Layout | Baseline: Macro only (with the road configuration in Figure 6.1.9-1 in [2] and BS placement as depicted in Figure A.1.3-1 in [3]) |
| ISD | Inter Macro: 500m |
| BS antenna elements | Macro BS antenna height: - Macro BS: 25m Macro BS antenna element pattern/array configuration in Section 6.1.4 of [1] |
| UE antenna elements | UE antenna height: - Type 1 vehicle (passenger vehicle with lower antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 0.75 meters - Type 2 vehicle (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters - Type 3 vehicle (truck/bus): length 13 meters, width 2.6 meters, height 3 meters, antenna height 3 meters UE antenna element pattern/array configuration in Section 6.1.4 of [1] |
| User distribution and UE speed | Vehicles are dropped according to the following process: - The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane. Distribution of vehicle type/speed for the urban grid scenario in Section 6.1.2 of [1] |

Table 2: Attributes for Highway

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro BS: 2GHz or 4GHz |
| Aggregated system bandwidth | Up to 200MHz (DL+UL) |
| Layout | Baseline: Macro only (straight line BS placement with Road configuration in [3]) |
| ISD | Inter Macro: 1732m, 500m (optional) |
| BS antenna elements | Macro BS antenna height: - 35m for ISD 1732m - 25m for ISD 500m Macro BS antenna element pattern/array configuration in Section 6.1.4 of [1] |
| UE antenna elements | UE antenna height: - Type 1 vehicle (passenger vehicle with lower antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 0.75 meters - Type 2 vehicle (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters - Type 3 vehicle (truck/bus): length 13 meters, width 2.6 meters, height 3 meters, antenna height 3 meters UE antenna element pattern/array configuration in Section 6.1.4 of [1] |
| User distribution and UE speed | Vehicles are dropped according to the following process: - The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane. Distribution of vehicle type/speed for the highway scenario in Section 6.1.2 of [1] |

Proposal 2: The attributes for Urban grid and Highway scenario in Table 1 and Table 2 are included in Section 4.11 and 4.12 in TR 38.914.

Summary

In this contribution, we discussed deployment scenarios for vehicular use cases, and the following proposals were made:

Proposal 1: For 6GR deployment scenarios, urban grid and highway scenario should be studied. Remove the bracket in [Urban grid] and [Highway Scenario] in Section 4.11 and 4.12 in TR 38.914.

Proposal 2: The attributes for Urban grid and Highway scenario in Table 1 and Table 2 are included in Section 4.11 and 4.12 in TR 38.914.

Reference

[1]	3GPP TR 37.885: "Study on evaluation methodology of new V2X use cases for LTE and NR".

[2]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies".

[3]	3GPP TR 36.885: "Study on LTE-based V2X services".