3GPP TSG RAN Meeting #110								RP-253516

Baltimore, USA, December 8-11, 2025

Status Report to TSG

Agenda item:			9.5.4.2

| WI / SI Name | # NR base station (BS) RF requirement evolution for FR1/FR2 and testing |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_BS_RF_req_evo |  |  |  |  |
| Unique ID | 1030078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252578 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 12/2025 (updated from 09/25) | Performance part: 03/2026 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 100% | Performance Part: 90% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Xue, fei |
|  | Company | ZTE Corporation |
|  | Email | Xue.fei25@zte.com.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4 #116bis:

Core part for Tx spurious emissions co-existence limits re-evaluation:

- Polarization gain

For BS-UE case, as well as for BS-BS:

- Consider 0 dB gain for calibration.

- For requirement derivation keep both 0 dB and 3 dB for polarization gain. Final decision to be taken during RAN4#117 meeting.

- Antenna / array gain considerations

FFS

- Correlation factor for OoB

FFS

- Grid shift

100% for both calibration and requirement derivation.

Clarifications on the underlying cells arrangements to be captured in the final simulation assumptions documentation.

- Performance metrics

CDF %-tile value to be used for the requirement derivation to be agreed at RAN4#117.

- Summary of system-level simulation assumptions; other assumptions

See annex A and B.

- Simulation results collection for calibration

Excel spreadsheet with updated calibration results:

- Requirement’s impact

Final requirement to be derived at RAN4#117 to conclude technical work.

- Documentation of findings (TR)

Aim to handle documentation of findings under Maintenance from RAN4#118. Companies to investigate workable alternatives (as TR 38.817-02 could not be re-used).

Perf part for Test scope simplification and TRP simplification:

TRP simplification:

- Anechoic chamber definition

Introduce generic anechoic chamber definition into the TR 37.941, listing only few examples of different anechoic chambers (i.e., not to have explicit list of specific chambers to avoid the need for future updates).

- “Reference” vs. “preferred” wording

Use the “preferred” wording for anechoic chamber, as well as for the reference TRP sampling grid.

- Content of modifications

Limit modifications in CRs to TS 38.141-2, TS 37.145-2 to reflect WID objective. Other corrections to be handled under Rel-15 maintenance.

Modifications to the TRP annex to be first concluded based on TS 38.141-2. Afterwards, those are to be mirrored to the related Annex in TS 37.145-2.

Test scope simplification-Out of band blocking

- Agreement:

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

RAN4 #117:

Core part for Tx spurious emissions co-existence limits re-evaluation:

- Simulation assumptions: BS total output power is 46 dBm for two polarizations.

- Summary of the RAN4 work: Capture under post-meeting process for Information.

Perf part for Test scope simplification and TRP simplification:

TRP simplification:

Reuse of notes from ERC/REC 74-01:

- Capture modifications required to reflect 3GPP drafting rules.

- Capture modifications to follow existing terminology, i.e., “systematic error” vs. “ΔTRP correction factor”.

2.4.2	Remaining Open issues

Perf part:

Formal CRs to be approved for Test scope simplification and TRP simplification.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

RAN4#116bis

- R4-2513156	Discussion on angle and coordinate system for Expected EIRP mask for upper 6GHz	CATT

- R4-2513157	(NR_BS_RF_req_evo-Perf)CR for TS 38.141-2, On conformance testing requirement for EEIRP for upper 6GHz	CATT

- R4-2513775	OTA OOB blocking test case reduction	Huawei, HiSilicon

- R4-2513776	Draft CR to TS 38.141-2 OTA test case reduction for TX IM requirement	Huawei, HiSilicon

- R4-2513777	Draft CR to TS 38.141-2 Correction on EEIRP test procedure	Huawei, HiSilicon

- R4-2513816	Simulation results for transmitter co-existence spurious emission requirements	Nokia

- R4-2513817	Transmitter co-existence spurious emission requirements	Nokia

- R4-2513818	Simplification of BS TRP test methods	Nokia

- R4-2513819	Draft CR to TR 37.941 on definition of anechoic chamber	Nokia

- R4-2513820	Draft CR on TRP sampling grid for the preferred test method	Nokia

- R4-2513862	On Transmitter Spurious Emission Co-existence discussion	Ericsson

- R4-2513863	On Transmitter Spurious Emission Co-existence BS conformance discussion	Ericsson

- R4-2513964	OTA test scope reduction	Ericsson

- R4-2513965	Draft CR to 38.141 on OTA test reduction for receiver OOBB	Ericsson

- R4-2514107	draft CR to TS 38.141-2: Improvements on Annex I - TRP measurement procedures	Ericsson

- R4-2514112	draft CR to TS 38.141-2: improvements related to Expected EIRP for band n104	Ericsson

- R4-2514270	Further discussion on Coexistence spurious emission requirements	ZTE Corporation, Sanechips

- R4-2514271	Discussion on the OTA testing scope reduction	ZTE Corporation, Sanechips

- R4-2514272	Discussion on the applicability improvement of TRP measurement	ZTE Corporation, Sanechips

- R4-2514428	Topic summary for [116bis][305] NR_BS_RF_Part1_SE_CLTA	Moderator (Huawei)

- R4-2514429	Topic summary for [116bis][306] NR_BS_RF_Part2_EIRP_OTA	Moderator (ZTE)

- R4-2514451	DraftCR to TS 38.115-2 with improvements for EIRP mask for NCR	Nokia

- R4-2514452	On BS OTA test scope reduction	Nokia

- R4-2514453	Draft CR to TS 38.141-2 OTA test reduction for out-of-band blocking requirement	Nokia

- R4-2514455	Draft CR to TS 38.141-2 with correction to EEIRP mask	Nokia

- R4-2514469	Spurious emission for co-existence simulation results alignment	Huawei, HiSilicon

- R4-2514473	Discussion on the proposed conclusions for the preferred OTA test methods	Huawei, HiSilicon

- R4-2514474	Draft CR to TS 38.141-2: implementation of the preferred test method solution	Huawei, HiSilicon

- R4-2514475	Draft CR to TS 37.145-2: implementation of the preferred test method solution	Huawei, HiSilicon

- R4-2514476	Draft CR to TR 37.941: Definition of anechoic chamber	Huawei, HiSilicon

- R4-2515062	Ad-hoc meeting minutes for [116bis][305] NR_BS_RF_Part1_SE_CLTA	Huawei

- R4-2515072	Draft CR to TR 37.941 on definition of anechoic chamber	Nokia

- R4-2515073	draft CR to TS 38.141-2: Improvements on Annex I - TRP measurement procedures	Ericsson

- R4-2515074	Draft CR to TS 37.145-2: implementation of the preferred test method solution	Huawei, HiSilicon

- R4-2515075	Way Forward for [116bis][305] NR_BS_RF_Part1_SE_CLTA	Huawei

- R4-2515076	draft CR to TS 38.141-2: improvements related to Expected EIRP for band n104	Ericsson

- R4-2515077	Draft CR to TS 38.141-2 OTA test reduction for out-of-band blocking requirement	Nokia

- R4-2515078	DraftCR to TS 38.115-2 with improvements for EIRP mask for NCR	Nokia

- R4-2515079	Way Forward for [116bis][306] NR_BS_RF_Part2_EIRP_OTA	ZTE

- R4-2515168	Draft CR to TS 38.141-2 OTA test reduction for out-of-band blocking requirement	Nokia

- R4-2515171	Way Forward for [116bis][305] NR_BS_RF_Part1_SE_CLTA	Huawei

RAN4#117

- R4-2520063	Discussion on angle and coordinate system for Expected EIRP mask for upper 6GHz	CATT

- R4-2520064	Draft CR for TS 38.141-2, On conformance testing requirement for EEIRP for upper 6GHz	CATT

- R4-2520275	Simulation results for transmitter co-existence spurious emission requirements	Nokia

- R4-2520276	Transmitter co-existence spurious emission requirements	Nokia

- R4-2520277	Simplification of BS TRP test methods	Nokia

- R4-2520278	CR to TR 37.941 on definition of anechoic chamber	Nokia

- R4-2521466	CR to TS 38.115-2 with improvements for EIRP mask for NCR	Nokia

- R4-2521703	Draft CR to TS 38.141-2 correcction on Expected EIRP for n104	Huawei, HiSilicon

- R4-2521761	Further discussion on Coexistence spurious emission requirements	ZTE Corporation, Sanechips

- R4-2521762	CR to TS 38.106: update on coexistence spurious limit for protection of band n104	ZTE Corporation

- R4-2521763	CR to TS 38.174: update on coexistence spurious limit for protection of band n104	ZTE Corporation

- R4-2521784	On Transmitter spurious emission co-existence discussion	Ericsson

- R4-2521785	(FS_NR_IMT_param) (NR_BS_RF_req_evo-Core) draftCR to TR 38.921 Addition of BS Transmitter Spurious emission co-existence work	Ericsson

- R4-2522074	CR to TS 38.141-2: improvements related to Expected EIRP for band n104	Ericsson, Nokia, ZTE, NEC, CATT, Huawei

- R4-2522079	CR to TS 38.141-2: Improvements on Annex I - TRP measurement procedures	Ericsson

- R4-2522134	CR to TS 38.141-2: Improvements on Annex I - TRP measurement procedures	Ericsson

- R4-2522175	Concluding on remaining issues on the preferred OTA test methods and TRP simplifications	Huawei, HiSilicon

- R4-2522176	Draft CR to TS 38.141-2: Improvements on Annex I on TRP measurement procedures	Huawei, HiSilicon

- R4-2522177	Draft CR to TS 37.145-2: implementation of the preferred test method and TRP annex improvements	Huawei, HiSilicon

- R4-2522178	Draft LS on implementation of the preferred test method for TRP measurements of the NR BS and AAS BS	Huawei, HiSilicon

- R4-2522179	Tx spurious emission co-existence simulation results	Huawei, HiSilicon

- R4-2522180	Draft LS on the status of the work on TX spurious emission co-existence requirements limits for high frequency bands	Huawei, HiSilicon

- R4-2522213	Topic summary for [117][305] NR_BS_RF_Part1_SE_CLTA	Moderator (Huawei)

- R4-2522214	Topic summary for [117][306] NR_BS_RF_Part2_EIRP_OTA	Moderator (ZTE)

- R4-2522322	CR to TS 38.141-2: improvements related to Expected EIRP for band n104	Ericsson, Nokia, ZTE, NEC, CATT, Huawei

- R4-2522958	Draft LS on the status of the work on TX spurious emission co-existence requirements limits for high frequency bands	Huawei, HiSilicon

- R4-2522959	Ad-hoc meeting minutes for [117][305] NR_BS_RF_Part1_SE_CLTA	Huawei

- R4-2522960	Way Forward for [117][305] NR_BS_RF_Part1_SE_CLTA	Huawei

- R4-2522961	CR to TS 38.141-2: Improvements on Annex I - TRP measurement procedures	Ericsson, Nokia, Huawei

- R4-2522962	Draft CR to TS 37.145-2: implementation of the preferred test method and TRP annex improvements	Huawei, HiSilicon

- R4-2522963	Draft LS on implementation of the preferred test method for TRP measurements of the NR BS and AAS BS	Huawei, HiSilicon

- R4-2522964	CR to TS 38.141-2: improvements related to Expected EIRP for band n104	Ericsson, Nokia, ZTE, NEC, CATT, Huawei

- R4-2522965	CR to TS 38.115-2 with improvements for EIRP mask for NCR	Nokia

- R4-2522994	(FS_NR_IMT_param) (NR_BS_RF_req_evo-Core) CR to TR 38.921 Addition of BS Transmitter Spurious emission co-existence work	Ericsson, Nokia

- R4-2523108	Collection of simulation assumptions and results for transmitter spurious coexistence	Ericsson, Huawei, ZTE

- R4-2523109	Way Forward for [117][305] NR_BS_RF_Part1_SE_CLTA	Huawei