3GPP TSG-RAN5 Meeting #109	R5-255615

# Dallas, Texas, USA, 17 – 21 Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 1091 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of A-MPR test point analysis for NS_05/NS_01 and NS_05U/NS_01 for CA_n1-n78 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing A-MPR test point analysis for NS_05/NS_01 and NS_05U/NS_01 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added A-MPR test point analysis for NS_05/NS_01 and NS_05U/NS_01 into Table 4.1.2.2-1. Configuration CA_n1-n78 is introduced in Rel-17. However, RAN4 has adopted the new A-MPR requirement in Rel-19 with the UE capability of modifiedMPR-Behaviour, which need to be considered for A-MPR TCs from R16-R19 UE. Therefore, the TPA is made according to Rel-19 specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR test point analysis for NS_05/NS_01 and NS_05U/NS_01 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 CR 3535 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Requirement CR in R5-255616 (CR 3535) Added file: 38.521-1_TPanalysis 6.2A.3 NS_05_and_NS_05U+NS_01.zip |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

#### 4.1.2.2	A-MPR test cases for FR1 UL CA

This section contains information on test point selection for test case 6.2A.3.1 in [2], UE additional maximum output power reduction for CA.

TS 38.101 [3] specifies band dependent NS-values, which in the inter-band UL CA test cases become a combination of two NS-values. For the inter-band UL CA configurations included in table 6.2A.3.1.3-1 in TS 38.101-1[3], the combined AMPR requirements per CA configuration will be tested based on following general principle:

-	Test the minimum Total power backoff value

-	Test the maximum Total power backoff value

-	Test the maximum unbalanced Total power backoff among CCs (max PCMAX,c difference).

Where the Total power backoff value means: MAX[MPR, A-MPR]

The analyses are performed per NS-values combination and are stored as zip-files as defined in annex A and documented in table 4.1.2.2-1.

For the inter-band UL CA configurations that are NOT included in table 6.2A.3.1.3-1 in TS 38.101-1[3], the AMPR testing for the CA configuration can be skipped. The testing could be covered by the testing of 6.2.3 for each single band in this configuration.

Table 4.1.2.2-1: A-MPR test coverage per CA configuration for inter-band CA with 2 CC

| CA config with UL CA support (Note 1) | NS values in same order as Uplink CA Configuration column |  | Number of test points | Applicable test case | Justification | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n78 | NS_05 and NS_05U | NS_01 | 12 | N/A | “38.521-1_TPanalysis 6.2A.3 NS_05_and_NS_05U+NS_01.zip” | RAN5#109 |
| CA_n3-n78 | NS_100 | NS_01 | 24 | N/A | “38.521-1_TPanalysis 6.2A.3 NS_100+NS_01.zip” | RAN5#87-e |
| CA_n8-n78 | NS_43 | NS_01 | 12 | N/A | “38.521-1_TPanalysis 6.2A.3 NS_43+NS_01.zip” | RAN5#88-e |
| CA_n8-n78 | NS_43U | NS_01 | 12 | N/A | “38.521-1_TPanalysis 6.2A.3 NS_43U+NS_01.zip” | RAN5#88-e |
| CA_n28-n78 CA_n28-n79 | NS_17 | NS_01 | 12 for PC3 and PC2 | N/A | “38.521-1_TPanalysis_6.2A.3_NS_17+NS_01_v1.zip” | RAN5#106 |
| CA_n40A-n41A | NS_01 | NS_47 | 3 | N/A | “38.521-1_TPanalysis_6.2A.3_NS_47+NS_01_v1.zip” | RAN5#108 |
| CA_n41A-n79A | NS_47 | NS_01 | 3 | N/A | “38.521-1_TPanalysis_6.2A.3_NS_47+NS_01_v1.zip” | RAN5#103 |
| CA_n28A-n41A | NS_17 | NS_47 | 12 | N/A | “38.521-1_TPanalysis_6.2A.3_NS_17+NS_47.zip” | RAN5#101 |
| Note 1: As per TS 38.101. |  |  |  |  |  |  |

-------- End of changes --------

Add attached .zip file to TR 38.905: “38.521-1_TPanalysis 6.2A.3 NS_05_and_NS_05U+NS_01.zip”