3GPP TSG-RAN5 Meeting #109	R5-256440

# Dallas, USA, November 17 - 21 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4245 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SA event triggered reporting tests without gap without SSB index reading under non-DRX test case 19.5.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will remain incomplete and WP cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

#### 19.5.1.2	SA FR1 event triggered reporting tests with per-UE gaps under non-DRX

Editor's Note: This test case is incomplete in following aspects:

TT analysis is missing.

AT Command to set the UE speed and position is not specified

The specific gNB reference location is missing

Antenna array is FFS

19.5.1.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.2D.6.2 and 9.2D.6.3.

19.5.1.2.2	Test applicability

This test applies to all types of NR ATG UE from release 18.

19.5.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.1.2.

19.5.1.2.4	Test description

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.5.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.5.1.2.4.1-1

Table 19.5.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.5.1.2-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.2-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.2-3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations |  |

Test environment parameters are given in Table 19.5.1.2.4.1-2.

Table 19.5.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.5.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.5.1.2.4.1-3 and 19.5.1.2.4.1-4.

2.	Message contents are defined in clause 19.5.1.2.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in the Table TBD.

Table 19.5.1.2.4.1-3: General test parameters (same as Table 6.6.1.3.4.1-2 with following exceptions)

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Propagation Condition |  | 1, 2 | AWGN+220 Hz |  |  |  |
|  |  | 3 | AWGN+500 Hz |  |  |  |

19.5.1.2.4.2	Test procedure

Same as the test procedure given in clause 6.6.1.3.4.2 with following exceptions:

-	Table 6.6.1.3.4.1-3 is replaced by Table 19.5.1.1.4.1-3;

19.5.1.2.4.3	Message contents

Same as the message contents given in clause 6.6.1.1.4.3.

19.5.1.2.5	Test requirements

Same as the test requirements given in clause 6.6.1.3.5.

/// End of changes