3GPP TSG-RAN WG5 Meeting #109	R5-256042

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-2 | CR | 1079 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to R18 IoT NTN enh RRM TCs applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Applicability for newly added IoT NTN RRM TCs needs to added in TS 36.521-2. Also conditions and PICS need to be updated as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following newly added IoT NTN RRM TCs have been added: - 13.5.1 HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access - 13.5.2 HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access - 13.5.3 HD-FDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access - 13.1.1.5 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3, A.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-3 CR 3105, 3111 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related TC CRs are R5-255564(CR 3105), and R5-256041 (CR 3111). The interim number “xxx”, “yyy” and “zzz” in Table 4.3.3-1a and Table A.4.5-1 needs to be assigned with hard number by MCC. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.3.3	RRM conformance test cases for NB-IoT/eMTC NTN

Table 4.3.3-1: Applicability of RRM conformance test cases for NB-IoT/eMTC NTN, ref. TS 36.521-3 [2]

| Clause | TC Title | Release | Applicability |  | Additional Information |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comments | Number of TC Executions | Release on other RAT | Branch |
| 13 E-UTRAN Standalone Tests for UE Category NB for Satellite Access |  |  |  |  |  |  |  |
| 13.1 RRC_IDLE state for satellite access |  |  |  |  |  |  |  |
| 13.1.1 Cell re-selection for satellite access |  |  |  |  |  |  |  |
| 13.1.1.1 | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage | Rel-17 only | C05 | UE supporting NB-IoT HD-FDD and GSO |  |  | 1Rx |
| 13.1.1.1_1 | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage for GSO and NGSO | Rel-18 | C05a | UE supporting NB-IoT HD-FDD and satellite access |  |  | 1Rx |
| 13.1.1.2 | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation | Rel-17 only | C06 | UE supporting wakeUpSi Relaxed RRM measurements,gnal, NB-IoT HD-FDD and GSO |  |  | 1Rx |
| 13.1.1.2_1 | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation for GSO and NGSO | Rel-18 | C06a | UE supporting wakeUpSignal, NB-IoT HD-FDD and satellite access |  |  | 1Rx |
| 13.1.1.3 | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX | Rel-17 only | C07 | UE supporting NB-IoT HD-FDD, GSO and UE specific DRX |  |  | 1Rx |
| 13.1.1.3_1 | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX for GSO and NGSO | Rel-18 | C07a | UE supporting NB-IoT HD-FDD, satellite access and UE specific DRX |  |  | 1Rx |
| 13.1.1.4_1 | HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage | Rel-18 | C05a | UE supporting NB-IoT HD-FDD and satellite access |  |  | 1Rx |
| 13.1.1.5 | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO | Rel-18 | C05c | UE supporting NB-IoT HD-FDD, NGSO satellite access and Location-based measurement |  |  | 1Rx |
| 13.1.1.6 | HD – FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO | Rel-18 | C05b | UE supporting NB-IoT HD-FDD, NGSO satellite access and time-based measurement |  |  | 1Rx |
| 13.3 RRC connection mobility control for satellite access |  |  |  |  |  |  |  |
| 13.3.1 RRC re-establishment for satellite access |  |  |  |  |  |  |  |
| 13.3.1.1 | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage | Rel-17 only | C08 | UE supporting NB-IoT HD-FDD, GSO and User Plane CIoT |  |  | 1Rx |
| 13.3.1.1_1 | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage for GSO and NGSO | Rel-18 | C08a | UE supporting NB-IoT HD-FDD, satellite access and User Plane CIoT |  |  | 1Rx |
| 13.3.1.2 | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | Rel-17 only | C08 | UE supporting NB-IoT HD-FDD, GSO and User Plane CIoT |  |  | 1Rx |
| 13.3.1.2_1 | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage for GSO and NGSO | Rel-18 | C08a | UE supporting NB-IoT HD-FDD, satellite access and User Plane CIoT |  |  | 1Rx |
| 13.3.1.3 | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | Rel-18 | C08a | UE supporting NB-IoT HD-FDD, satellite access and User Plane CIoT |  |  | 1Rx |
| 13.3.2 Random Access for Satellite Access |  |  |  |  |  |  |  |
| 13.3.2.1 | Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage | Rel-17 | C01 | UE supporting NB-IoT HD-FDD and NTN | NOTE 1 |  | 1Rx |
| 13.3.2.2 | Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage | Rel-17 | C01 | UE supporting NB-IoT HD-FDD and NTN | NOTE 1 |  | 1Rx |
| 13.3.2.3 | Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage | Rel-17 | C10 | UE supporting NB-IoT HD-FDD, NTN and NPRACH on non-anchor carrier | NOTE 1 |  | 1Rx |
| 13.4 Timing and signalling characteristics for satellite access |  |  |  |  |  |  |  |
| 13.4.1 UE transmit timing for satellite access |  |  |  |  |  |  |  |
| 13.4.1.1 | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access | Rel-17 | C01 | UE supporting NB-IoT HD-FDD and NTN | NOTE 1 |  | 1Rx |
| 13.4.1.2 | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access | Rel-17 | C02 | UE supporting NB-IoT HD-FDD, NTN and DRX | NOTE 1 |  | 1Rx |
| 13.4.1.3 | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access | Rel-17 | C03 | UE supporting NB-IoT HD-FDD and NGSO, but not supporting to report the supported gap length between segments for TA pre-compensation |  |  | 1Rx |
| 13.4.2 UE timing advance for satellite access |  |  |  |  |  |  |  |
| 13.4.2.1 | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access | Rel-17 | C01 | UE supporting NB-IoT HD-FDD and NTN | NOTE 1 |  | 1Rx |
| 13.4.2.2 | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access | Rel-17 | C01 | UE supporting NB-IoT HD-FDD and NTN | NOTE 1 |  | 1Rx |
| 13.4.3 Radio Link Monitoring for satellite access |  |  |  |  |  |  |  |
| 13.4.3.1 | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage | Rel-17 | C02 | UE supporting NB-IoT HD-FDD, NTN and DRX | NOTE 1 |  | 1Rx |
| 13.4.3.2 | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage | Rel-17 | C02 | UE supporting NB-IoT HD-FDD, NTN and DRX | NOTE 1 |  | 1Rx |
| 13.4.3.3 | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage | Rel-17 | C02 | UE supporting NB-IoT HD-FDD, NTN and DRX | NOTE 1 |  | 1Rx |
| 13.4.3.4 | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage | Rel-17 | C02 | UE supporting NB-IoT HD-FDD, NTN and DRX | NOTE 1 |  | 1Rx |
| 13.4.3.5 | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Rel-17 | C01 | UE supporting NB-IoT HD-FDD and NTN | NOTE 1 |  | 1Rx |
| 13.4.3.6 | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage | Rel-17 | C01 | UE supporting NB-IoT HD-FDD and NTN | NOTE 1 |  | 1Rx |
| 13.4.3.7 | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Rel-17 | C01 | UE supporting NB-IoT HD-FDD and NTN | NOTE 1 |  | 1Rx |
| 13.4.3.8 | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage | Rel-17 | C01 | UE supporting NB-IoT HD-FDD and NTN | NOTE 1 |  | 1Rx |
| 13.5 UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access |  |  |  |  |  |  |  |
| 13.5.1 | HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access | Rel-18 | C12 | UE supporting NB-IoT HD-FDD, NTN, User Plane CIoT and neighbour cell measurement | NOTE 1 |  | 1Rx |
| 13.5.2 | HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access | Rel-18 | C12 | UE supporting NB-IoT HD-FDD, NTN, User Plane CIoT and neighbour cell measurement | NOTE 1 |  | 1Rx |
| 13.5.3 | HD-FDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access | Rel-18 | C12a | UE supporting NB-IoT HD-FDD, NTN, User Plane CIoT and location based neighbour cell measurement | NOTE 1 |  | 1Rx |
| 13.6 Measurement performance requirements for UE for satellite access |  |  |  |  |  |  |  |
| 13.6.2 Channel quality reporting accuracy for satellite access |  |  |  |  |  |  |  |
| 13.6.2.1 | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage | Rel-17 | C11 | UE supporting NB-IoT HD-FDD NTN and DL channel quality reporting for anchor carrier for FDD in Msg3 | NOTE 1 |  | 1Rx |
| 13.6.2.2 | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage | Rel-17 | C11 | UE supporting NB-IoT HD-FDD NTN and DL channel quality reporting for anchor carrier for FDD in Msg3 | NOTE 1 |  | 1Rx |
| 13.6.2.3 | E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage | Rel-17 | C09 | UE supporting NB-IoT HD-FDD, NTN and DL channel quality reporting for a non-anchor carrier for FDD in Msg3 | NOTE 1 |  | 1Rx |
| 13.6.2.4 | E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage | Rel-17 | C09 | UE supporting NB-IoT HD-FDD, NTN and DL channel quality reporting for a non-anchor carrier for FDD in Msg3 | NOTE 1 |  | 1Rx |
| 13.6.2.5 | E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage | Rel-17 | C04 | UE supporting NB-IoT HD-FDD, NTN and DL channel quality reporting in RRC_CONNECTED | NOTE 1 |  | 1Rx |
| 13.6.2.6 | E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage | Rel-17 | C04 | UE supporting NB-IoT HD-FDD, NTN and DL channel quality reporting in RRC_CONNECTED | NOTE 1 |  | 1Rx |
| NOTE 1: Refer to Table 4.3.3-1b. |  |  |  |  |  |  |  |

Table 4.3.3-1a: Applicability of RRM conformance test cases Conditions for NB-IoT/eMTC NTN

C01 IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) THEN R ELSE N/A

C02 IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND A.4.4-1a/5 AND (A.4.5-1/103 OR A.4.5-1/104) THEN R ELSE N/A

C03 IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND A.4.5-1/103 AND (NOT A.4.5-1/105) THEN R ELSE N/A

C04 IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) AND A.4.5-1/106 THEN R ELSE N/A

C05 IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND A.4.5-1/104 THEN R ELSE N/A

C05a IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) THEN R ELSE N/A

C05b IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND A.4.5-1/103 AND A.4.5-1/114 THEN R ELSE N/A

C05c IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND A.4.5-1/103 AND (A.4.5-1/yyy OR A.4.5-1/zzz) THEN R ELSE N/A

C06 IF (A.4.1-1/8b AND A.4.3-7/2 AND A.4.5-1/108 AND A.4.5-1/112) AND A.4.5-1/99 AND A.4.5-1/104 THEN R ELSE N/A

C06a IF (A.4.1-1/8b AND A.4.3-7/2 AND A.4.5-1/108) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) THEN R ELSE N/A

C07 IF (A.4.1-1/8b AND A.4.3-7/2 AND A.4.5-1/107) AND A.4.5-1/99 AND A.4.5-1/104 THEN R ELSE N/A

C07a IF (A.4.1-1/8b AND A.4.3-7/2 AND A.4.5-1/107) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) THEN R ELSE N/A

C08 IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND A.4.5-1/104 AND A.4.5-1/34 THEN R ELSE N/A

C08a IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) AND A.4.5-1/34 THEN R ELSE N/A

C09 IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) AND A.4.5-1/109 THEN R ELSE N/A

C10 IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) AND A.4.5-1/110 THEN R ELSE N/A

C11 IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) AND A.4.5-1/111 THEN R ELSE N/A

C12 IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) AND A.4.5-1/xxx AND A.4.5-1/34 THEN R ELSE N/A

C12a IF (A.4.1-1/8b AND A.4.3-7/2) AND A.4.5-1/99 AND (A.4.5-1/103 OR A.4.5-1/104) AND A.4.5-1/xxx AND (A.4.5-1/yyy OR A.4.5-1/zzz) AND A.4.5-1/34 THEN R ELSE N/A

Table 4.3.3-1b: Number of TC Executions - Notes

Note 1: If UE supports both NGSO and GSO, the GSO test case can be skipped if the UE passes NGSO test case, this according to the guidance in each test cases of TS 36.521-3 [2].

<< Unchanged Parts Skipped >>

A.4.5	Additional information

Table A.4.5-1: Additional UE radio access capabilities

| Item | Additional capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Support of CSG | 36.331, Annex B.2 | Rel-8 |  |
| 2 | Support of intra-frequency SI acquisition for HO in FDD | 36.306, 4.3.11.1 | Rel-9 |  |
| 3 | Support of inter-frequency SI acquisition for HO in FDD | 36.306, 4.3.11.2 | Rel-9 |  |
| 4 | Need for inter-frequency gaps (Note 1) | 36.306, 4.3.6.1 | Rel-8 |  |
| 5 | Need for inter-RAT gaps (Note 1) | 36.306, 4.3.6.1 | Rel-8 |  |
| 6 | Support of E-UTRA bands within band group FDD_Nonly | 36.133, Annex A.3.7.2 | Rel-12 |  |
| 7 | Support of rsrqMeasWideband | 36.306, 4.3.6.2 | Rel-11 |  |
| 8 | Support of Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 | 36.306, 4.3.5.5 | Rel-11 |  |
| 9 | Void |  |  |  |
| 10 | Disable E-UTRA capability if IMSVoIP not supported by the network | 23.221, 7.2a, 24.301, 4.5 | Rel-8 | pc_Disable_E-UTRA_NOIMSVoIP |
| 11 | Support of Maximum CSI processes of Three on a component carrier within a band with PDSCH transmission mode 10 | 36.306, 4.3.5.5 | Rel-11 |  |
| 12 | Support of Maximum CSI processes of Four on a component carrier within a band with PDSCH transmission mode 10 | 36.306, 4.3.5.5 | Rel-11 |  |
| 13 | Support of multiClusterPUSCH-WithinCC-r10 | 36.306, 4.3.4.13 | Rel-10 |  |
| 14 | Support of FDD-TDD CA with PCell in TDD band | 36.306, 4.3.4.28 |  | The UE may not send the IE tdd-FDD-CA-PCellDuplex-r12 |
| 15 | Support of FDD-TDD CA with PCell in FDD band | 36.306, 4.3.4.28 |  | The UE may not send the IE tdd-FDD-CA-PCellDuplex-r12 |
| 16 | Support of interRAT-PS-HO-ToGERAN | 36.306, 4.3.7.11 | Rel-8 |  |
| 17 | Support of 64QAM in UL | 36.306, 4.3.4.39 | Rel-12 |  |
| 18 | Support of 256QAM in DL | 36.306, 4.3.5.7 | Rel-12 |  |
| 19 | Support CRS based discovery signals measurement | 36.306, 4.3.6.9 | Rel-12 |  |
| 20 | Support CSI-RS based discovery signals measurement | 36.306, 4.3.6.10 | Rel-12 |  |
| 21 | Support the behaviour on DL signals and physical channels when SCell is deactivated and discovery signals measurement is configured | 36.306, 4.3.4.38 | Rel-12 |  |
| 22 | Support of 4Rx antenna ports | 36.101, 7.2 | Rel-13 |  |
| 23 | Support of ProSe direct communication | 36.306, 4.3.21.1 | Rel-12 |  |
| 24 | Support of ProSe direct discovery | 36.306, 4.3.21.3 | Rel-12 |  |
| 25 | Support of CE mode A | 36.306, 4.3.8.3 | Rel-13 | Mandatory for CAT M1 UE |
| 26 | Support of CE mode B | 36.306, 4.3.29.1 | Rel-13 |  |
| 27 | Support of DC ASYNCH | 36.306, 4.3.29.2 | Rel-12 | The UE supports asynchronous dual connectivity and power control mode 2 |
| 28 | Support of DC SCG DRB | 36.306, 4.3.20.2 | Rel-12 | The UE supports dual connectivity and DRB type of SCG bearer |
| 29 | Support of DC Split DRB | 36.306, 4.3.20.1 | Rel-12 | The UE supports dual connectivity and DRB type of Split bearer |
| 30 | Support of MPR for intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation | 36.306, 4.3.5.10 36.101, H.1 | Rel-10 | ModifiedMPR_Behavior bit 0 (leftmost bit) |
| 31 | Support of A-MPR associated with NS_05 for Band 1 | 36.306, 4.3.5.10 36.101, H.1 | Rel-10 | ModifiedMPR_Behavior bit 1 |
| 32 | supports downlink LAA operation | 36.306, 4.3.23.1 | Rel-13 |  |
| 33 | supports measurement and reporting for RSSI and channel occupancy | 36.306, 4.3.6.19 | Rel-13 |  |
| 34 | Support of User plane CIoT | 24.301, 5.3.15 | Rel-13 |  |
| 35 | Support of EMM-REGISTERED without PDN | 24.301, 5.3.15 | Rel-13 |  |
| 36 | Support of EMM-REGISTERED with PDN | 24.301, 5.3.15 | Rel-13 |  |
| 37 | Support of 4Rx antenna ports in at least one FDD frequency band | 36.101, 7.2 | Rel-13 |  |
| 38 | Support of 4Rx antenna ports in at least one TDD frequency band | 36.101, 7.2 | Rel-13 |  |
| 39 | Support of FDD-TDD CA with PCell in FDD band and SCell with 4Rx supported TDD RF band | 36.306, 4.3.4.28, 36.101, 7.2 | Rel-13 |  |
| 40 | Support of 4Rx antenna ports on all supported FDD operating bands | 36.101, 8.1.2.6.1, 36.133, A.3.8.1 | Rel-13 | UE with same FDD band support declared in tables A.4.3-3 and A.4.5-5 |
| 41 | Support of 4Rx antenna ports on all supported TDD operating bands | 36.101, 8.1.2.6.1, 36.133, A.3.8.1 | Rel-13 | UE with same TDD band support declared in tables A.4.3-3 and A.4.5-5 |
| 42 | Support of A-MPR associated with NS_04 for Band 41 | 36.306, 4.3.5.10 36.101, H.1 | Rel-12 | ModifiedMPR_Behavior bit 2 |
| 43 | Support of RSSI and Channel occupancy reporting | 36.306, 4.3.6.19 | Rel-13 | Support of RSSI and Channel Occupancy. |
| 44 | Support of intra-frequency SI acquisition in TDD for HO | 36.306, 4.3.11.1 | Rel-9 |  |
| 45 | Support of inter-frequency SI acquisition in TDD for HO | 36.306, 4.3.11.2 | Rel-9 |  |
| 46 | Support of 4-layer spatial multiplexing with transmission mode 3 and transmission mode 4 | 36.306, 4.3.5.14. | Rel-10 |  |
| 47 | Void |  |  |  |
| 48 | Support of autonomous resource selection mode with full sensing for V2X sidelink communication | 36.306, 4.3.21.15 | Rel-14 |  |
| 49 | Support of SLSS transmission and reception for V2X sidelink communication | 36.306, 4.3.21.17 | Rel-14 |  |
| 50 | Support of maximum transmit power associated with Power class 2 V2X UE | 36.306, 4.3.21.22 | Rel-14 |  |
| 51 | Support of TM-9 in CE Mode A | 36.306 4.3.29.10 | Rel-13 |  |
| 52 | Support of TM-9 in CE Mode B | 36.306 4.3.29.11 | Rel-13 |  |
| 53 | Support of 4-layer spatial multiplexing with transmission mode 9 and transmission mode 10 | 36.306, 4.3.4.7 | Rel-10 |  |
| 54 | Support of TDD UL/DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI on a TDD PCell, and HARQ feedback according to UL and DL HARQ reference configurations | 36.306 4.3.4.31 | Rel-12 |  |
| 55 | Support of Rel-12 DL CSI subframe set configuration | 36.306 4.3.4.29 | Rel-12 |  |
| 56 | Support of tm9 operation on LAA cell(s). | 36.306, 4.3.23.6 | Rel-13 |  |
| 57 | Supports of RRM measurements on LAA cell(s) based on CSI-RS-based DRS. | 36.306, 4.3.23.3 | Rel-13 |  |
| 58 | Support of 256QAM in UL | 36.306 4.3.4.73 | Rel-14 |  |
| 59 | Support of SRS switching between a band pair | 36.306, 4.3.5.24, 4.3.5.25 | Rel-14 |  |
| 60 | Support of Make-Before-Break handover | 36.306, 4.3.30.1 | Rel-14 |  |
| 61 | Support of csi-RS-DiscoverySignalsMeas-r12 | 36.331, 6.3.6 | Rel-12 |  |
| 62 | Support of UL LAA | 36.306, 4.3.23.8 | Rel-14 |  |
| 63 | Support of TM-6 in CE Mode A | 36.306 4.3.29.12 | Rel 13 |  |
| 64 | Support of high speed measurement enhancements | 36.306 4.3.33.1 | Rel-14 |  |
| 65 | Support of PUCCH transmission on SCell in CA | 36.306 4.3.4.47 | Rel-13 |  |
| 66 | Support of simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 36.306 4.3.5.27 | Rel-14 |  |
| 67 | Support of transmitting PSCCH/PSSCH using dynamic scheduling | 36.306 4.3.21.14 | Rel-14 |  |
| 68 | Support of Channel Busy Ratio measurement and reporting of Channel Busy Ratio measurement to eNB for V2X sidelink communication | 36.306 4.3.21.18 | Rel-14 |  |
| 69 | Support of transmission and reception in the configuration of non-adjacent PSCCH and PSSCH for V2X sidelink communication | 36.306 4.3.21.21 | Rel-14 |  |
| 70 | Support of reception of 20 PSCCH in a subframe and decoding of 136 RBs per subframe counting both PSCCH and PSSCH in a band for V2X sidelink communication | 36.306 4.3.21.13 | Rel-14 |  |
| 71 | Support of Increased UE carrier monitoring E-UTRA | 36.306 4.3.6.6 | Rel-12 |  |
| 72 | Support of codebookConfig 1 for non-precoded EBF/ FD-MIMO operation in TM9 | 36.306 4.3.28.6 | Rel-13 |  |
| 73 | Support of codebookConfig 2 for non-precoded EBF/ FD-MIMO operation in TM9 | 36.306 4.3.28.6 | Rel-13 |  |
| 74 | Support of codebookConfig 3 for non-precoded EBF/ FD-MIMO operation in TM9 | 36.306 4.3.28.6 | Rel-13 |  |
| 75 | Support of codebookConfig 4 for non-precoded EBF/ FD-MIMO operation in TM9 | 36.306 4.3.28.6 | Rel-13 |  |
| 76 | Support of Increased UE carrier monitoring UTRA | 36.306 4.3.6.7 | Rel-12 |  |
| 77 | Support of multi-carrier operation | 36.306 4.3.4.115 | Rel-13 |  |
| 78 | Support of reception ending with a subframe occupied for a DwPTS-duration on LAA cell(s) | 36.306 4.3.23.4 | Rel-13 |  |
| 79 | Support of reception of subframes with second slot starting position on LAA cell(s) | 36.306 4.3.23.5 | Rel-13 |  |
| 80 | Support of enhanced 4Tx codebook | 36.306 4.3.4.33 | Rel-12 |  |
| 81 | Support of PUSCH feedback mode 3-2 | 36.306 4.3.4.34 | Rel-12 |  |
| 82 | Support for channel measurement restrictions in TM9 and TM10 | 36.306, 4.3.28.3 | Rel-13 |  |
| 83 | Support for interference measurement restrictions | 36.306 4.3.28.5 | Rel-13 |  |
| 84 | Support of 64QAM for non-repeated unicast PDSCH in RRC_CONNECTED when operating in coverage enhancement mode A | 36.306 4.3.4.126 | Rel-15 |  |
| 85 | Support of alternative CQI table in RRC_CONNECTED when operating in coverage enhancement mode A | 36.306 4.3.4.127 | Rel-15 |  |
| 86 | Support of eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE mode | 36.306, 4.3.6.31 | Rel-15 |  |
| 87 | Support of having SCell configured in dormant SCell state | 36.306, 4.3.19.18 | Rel-15 |  |
| 88 | Support of asynchronous DAPS handover in source PCell and intra-frequency target PCell | 36.306, 4.3.5.39 | Rel-16 |  |
| 89 | Support of DAPS handover in source PCell and intra-frequency target PCell | 36.306, 4.3.5.40 | Rel-16 |  |
| 90 | Support of asynchronous DAPS handover in source PCell and inter-frequency target PCell | 36.306, 4.3.5.42 | Rel-16 |  |
| 91 | Support of DAPS handover in source PCell and inter-frequency target PCell | 36.306, 4.3.5.43 | Rel-16 |  |
| 92 | Support of conditional handover | 36.306, 4.3.30.3 | Rel-16 |  |
| 93 | Support of conditional handover between FDD and TDD cells | 36.306, 4.3.30.5 | Rel-16 |  |
| 94 | Support of short TTI and/or short processing time | 36.306 4.3.4.150 | Rel-15 | pc_sTTI_SPT |
| 95 | Support of sTTI combination {slot, slot} | 36.306 4.3.4.103 | Rel-15 |  |
| 96 | Support of sTTI combination {subslot, subslot} | 36.306 4.3.4.103 | Rel-15 |  |
| 97 | Support of sTTI combination {subslot, slot} | 36.306 4.3.4.103 | Rel-15 |  |
| 98 | Support of short processing time for the corresponding frame structure types | 36.306 4.3.4.100 | Rel-15 | pc_spt_Parameters |
| 99 | Support NTN access | 36.306 4.3.38.1 | Rel-17 |  |
| 100 | Support Timing advance reporting in NTN cell | 36.306 4.3.38.2 | Rel-17 |  |
| 101 | Support delaying the start of the pur-ResponseWindowTimer for NTN operation | 36.306 4.3.38.3 | Rel-17 |  |
| 102 | Support timing relationship enhancements using Differential Koffset | 36.306 4.3.38.4 | Rel-17 |  |
| 103 | Support NTN features in NGSO scenario | 36.306 4.3.38.5 | Rel-17 |  |
| 104 | Support NTN features in GSO scenario | 36.306 4.3.38.5 | Rel-17 |  |
| 105 | Support to report the supported gap length between segments for PUSCH and PUCCH required by a UE supporting ce-ModeA-r13 or for NPUSCH required by a UE supporting ue-category-NB, for TA pre-compensation. | 36.306 4.3.38.6 | Rel-17 |  |
| 106 | Support DL channel quality reporting of the configured carrier for FDD in RRC_CONNECTED | 36.306 4.3.6.37 | Rel-16 |  |
| 107 | Support UE specific DRX | 24.301 9.9.3.63 | Rel-16 |  |
| 108 | Support of WUS (wake up signal) for FDD | 36.306 4.3.4.113 | Rel-15 |  |
| 109 | Support DL channel quality reporting for a non-anchor carrier for FDD in Msg3 | 36.306 6.17.8 | Rel-16 |  |
| 110 | Support NPRACH on non-anchor carrier | 36.306 4.3.4.75 | Rel-14 |  |
| 111 | Support DL channel quality reporting for an anchor carrier for FDD in Msg3 | 36.306 6.17.2 | Rel-16 |  |
| 112 | Support Relaxed RRM measurements | 36.306 6.17.11 | Rel-16 |  |
| 113 | Support of protection against improper reselection to GERAN/UTRAN | 36.306, 6.8.16 | Rel-18 |  |
| 114 | Support of time-based measurement trigger in RRC_CONNECTED | 36.306, 4.3.38.13 | Rel-18 |  |
| xxx | Support of NTN neighbour cell measurements | 36.306, 6.19.4 | Rel-18 |  |
| yyy | Support of location-based measurement trigger in RRC_CONNECTED in (quasi-)earth fixed cell | 36.306, 4.3.38.11 | Rel-18 |  |
| zzz | Support of location-based measurement trigger in RRC_CONNECTED in earth moving cell | 36.306, 4.3.38.12 | Rel-18 |  |
| Note 1: Need for inter-frequency gaps or inter-RAT gaps indicates that the UE does not support corresponding measurement without gaps. |  |  |  |  |

<< END OF CHANGES >>