3GPP TSG-RAN5 Meeting #109	R5-255774

# Dallas, Texas, USA, 17th - 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1637 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of eCall test case 21.32 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Specific message contents are not clearly specified to serve the test purposes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification of specific message contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 21.32	eCall capable / Manual initiation / Emergency registration / 200 OK without ACK/NACK / PSAP Callback / Success

### 21.32.1	Definition

Test to verify that the UE can correctly perform eCall over IMS when initiated manually, continues with call establishment after 200 OK without ACK or NACK is received from SS and handles PSAP callback after completion of eCall over IMS. This process is described in 3GPP TS 24.229 [10].

### 21.32.2	Conformance requirement

[Rel-19, TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

- the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

NOTE 0:	In some countries, the type of emergency service is required, due to national regulations, so a Request-URI with ecall sub-type is needed to avoid failure in routing, e.g. when the IP-CAN indicates that eCall over IMS is not supported.

2)	if the IP-CAN indicates the eCall over IMS support defined in 3GPP TS 36.331 [19F] or in 3GPP TS 38.331 [19G], the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

1)	if the UE receives a 200 (OK) response to the INVITE request not containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to either "true" or "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then, if the PSAP requests it, the UE shall send the MSD using audio media stream encoded as described in 3GPP TS 26.267 [9C] and in accordance with the audio media procedures in EN 17184:2024 [300] and EN 16062:2023 [304];

[Rel-19, TS 24.229, clause 5.1.6B]:

This clause specifies a feature which applies to UEs subject to national regulations in EN 17184:2024 [300]. The procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD shall be supported by a UE during an incoming IMS call when the following conditions are fulfilled:

1)	the UE previously established or attempted to establish an emergency call either over IMS or in the CS domain of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service;

2)	the incoming IMS call occurred within an implementation dependent time period of at least one hour;

3)	the INVITE for the incoming IMS call contains a Priority header field with header field value "psap-callback";

4)	the INVITE for the incoming IMS call contains an Accept header field that includes the value “application/EmergencyCallData.Control+xml”; and

5)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

NOTE 1:	When there are multiple attempts to establish the emergency call, the emergency call in condition 2) refers to the last attempt.

When conditions 1) to 5) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

When conditions 1) and 2) and, as an implementation option, condition 3) are fulfilled as well as the PSAP requests it, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C] and in accordance with the audio media procedures in EN 17184:2024 [300] and EN 16062:2023 [304].

NOTE 2:	A PSAP can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the PSAP, the serving PLMN, the HPLMN or by some other intermediate network.

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.2 and 5.1.6B.

### 21.32.3	Test purpose

1) 	To verify that the UE sends a correctly composed initial INVITE request for eCall over IMS and will continue with call establishment after receiving 200 OK without ACK or NACK.

2) 	To verify that an ecall capable UE checks the header fields in incoming call for PSAP callback and responds with 200 OK.

### 21.32.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription according to TS 36.508 [94] table 4.9.3.5-1. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE subjected to national regulations in CEN EN 17184:2024 for IMS eCall over EPS (TS 34.229-2 [5] A.4.5/67)

1)	Manual eCall is initiated at UE.

2-5) Emergency registration according to C.20 is executed.

6) SS waits for UE to send an INVITE request.

7)	SS sends 200 OK not containing received attribute set to either "true" or "false" in "ack" element.

8)	UE responds with ACK for 200 OK.

9)	SS waits for 5 seconds.

10-13) MT Call release according to steps 1-4 of procedure C.33.

14) SS sends SIP INVITE with correctly composed Priority header.

15) SS expects call setup with correctly composed 200 OK and sends ACK.

16)	SS waits for 5 seconds.

17) SS sends BYE to the UE.

18) SS expects and receives 200 OK for BYE from the UE.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate a manual eCall |  |
| 2-5 | - |  | Steps 1-4 defined in Annex C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 6-8 | - |  | Steps 1-3 defined in Annex C.47 | eCall setup. Referred from 36.508 [94] table 4.5A.26.3-1. |
| 9 |  |  |  | SS waits for 5 seconds. |
| 10-13 | - |  | Steps 1-4 defined in Annex C.33 | SS releases the eCall. |
| 14-26 | - |  | Steps 1-13 defined in Annex C.11 | MT call establishment. Referred from 36.508 [94] table 4.5A.7.3-1. |
| 27 |  |  |  | SS waits for 5 seconds. |
| 28-31 | - |  | Steps 1-4 defined in Annex C.33 | SS releases the call. |

Specific Message Contents:

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A20.

200 OK for INVITE (Step 7, step 2 of C.47)

Use 200 OK as specified for step 2 in annex C.47 with condition A6 and without conditions A12, A13 and with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message body |
| Message-body | SDP message as specified in the specific message contents for the 200 OK for INVITE at step 2 of C.47 |

INVITE (Step 14, step 1 of C.11)

Use INVITE as specified for step 1 in annex C.11 with condition A11 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Accept |  |
| media-range | application/sdp, application/3gpp-ims+xml |
| Recv-Info | not present |

### 21.32.5	Test requirements

The UE shall send requests and responses as described in clause 21.32.4.