3GPP TSG-RAN5 Meeting #109	R5-256454

# Dallas, Texas, USA, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5556 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 6.2.5_1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The applicability of test point 5 is not clear in the configured UE transmitted output power for HPUE test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability and intention of test point 5 have been clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will remain ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5_1.4.2, 6.2.5_1.4.3, 6.2.5_1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 6.2.5_1	Configured UE transmitted Output Power for HPUE

#### 6.2.5_1.1	Test purpose

To verify the UE does not exceed the minimum between the PEMAX maximum allowed UL TX Power signalled by the E-UTRAN and the PPowerClass maximum UE power the UE power class.

Additionally, for power class 2 UE in band 41, to verify that PCMAX,c is reduced when the uplink/downlink configuration is 0 or 6 in the cell.

#### 6.2.5_1.2	Test applicability

This test applies to all types of E-UTRA power Class 1 or power Class 2 UE release 10 and forward.

#### 6.2.5_1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2.5.3 with the following exceptions:

-	Instead of Table 6.2.2.3-1  use Table 6.2.2_1.3-1

-	Instead of Section 6.2.3  use Section 6.2.3_1

-	Instead of Section 6.2.4  use Section 6.2.4_1

The normative reference for this requirement is TS 36.101 [2] clause 6.2.5.

#### 6.2.5_1.4	Test description

##### 6.2.5_1.4.1	Initial conditions

Same initial conditions as in clause 6.2.5.4.1 with the following exceptions

-	Instead of clause 6.2.5.4.3  use clause 6.2.5_1.4.3

-	Instead of table 6.2.5.4.1-1  use table 6.2.5_1.4.1-1

Table 6.2.5_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Configured UE transmitted Output Power test case |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 5 | 5 |
| 3MHz |  |  | QPSK | 4 | 4 |
| 5MHz |  |  | QPSK | 8 | 8 |
| 10MHz |  |  | QPSK | 12 | 12 |
| 15MHz |  |  | QPSK | 16 | 16 |
| 20MHz |  |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For the uplink RB allocation the RBstart shall be RB #0. Note 3: For power class 2 operation in band 41, RMC with UL-DL config 1 is used in test points 1 to 4 and RMC with UL-DL config 0 is used in test point 5. |  |  |  |  |  |

##### 6.2.5_1.4.2	Test procedure

Same test procedure as in clause 6.2.5.4.2 with the following exceptions:

-	Instead of Table 6.2.5.5-1  use Tables 6.2.5_1.5-1 and 6.2.5_1.5-2

-	Test point 5 is executed only in band 41 for UEs supporting power class 2

##### 6.2.5_1.4.3	Message contents

Same message contents as in clause 6.2.5.4.3 with the following additional test point:

Table 6.2.5_1.4.3-1: SystemInformationBlockType1: Test point 4 for Power class 1

| Derivation Path: TS 36.508 [7] clause 4.4.3.2, Table 4.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 25 |  |  |

Table 6.2.5_1.4.3-2: SystemInformationBlockType1: Test point 4 for Power class 2

| Derivation Path: TS 36.508 [7] clause 4.4.3.2, Table 4.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 21 |  |  |

Table 6.2.5_1.4.3-3: SystemInformationBlockType1: Test point 5 for Band 41 and Power class 2

| Derivation Path: TS 36.508 [7] clause 4.4.3.2, Table 4.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 26 |  |  |
| NOTE: The intention of this test point is to test the fall back to power class 3 requirements for power class 2 capable UE operating on Band 41 with UL-DL config 0 or 6. |  |  |  |

Table 6.2.5_1.4.3-4: TDD-Config-DEFAULT: Test point 5

| Derivation Path: TS 36.508 [7] clause 5.3.1, Table 5.3.1-1: TDD-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| subframeAssignment | sa0 | To enable PC3 fallback operation |  |
| specialSubframePatterns | ssp4 |  |  |
| } |  |  |  |

#### 6.2.5_1.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2.5_1.5-1 and Table 6.2.5_1.5-2.

Table 6.2.5_1.5-1: PCMAX configured HPUE output power for Power class 1

|  | Channel bandwidth / maximum output power |  |
| --- | --- | --- |
|  | 5MHz | 10MHz |
| Measured UE output power test point 1 | -10 dBm ± 7.7 |  |
| Measured UE output power test point 2 | 10 dBm ± 6.7 |  |
| Measured UE output power test point 3 | 15 dBm ± 5.7 |  |
| Measured UE output power test point 4 | 25dBm ± 2.7 |  |

Table 6.2.5_1.5-1a: PCMAX configured HPUE output power for Power class 1 from Release 11 and forward for the band with lower tolerance, E-UTRA bands ≤ 3GHz

|  | Channel bandwidth / maximum output power |  |
| --- | --- | --- |
|  | 5MHz | 10MHz |
| Measured UE output power test point 1 | -10 dBm ± 7.7 |  |
| Measured UE output power test point 2 | 10 dBm ± 6.7 |  |
| Measured UE output power test point 3 | 15 dBm ± 5.7 |  |
| Measured UE output power test point 4 | 25dBm + 2.7/-3.7 |  |

Table 6.2.5_1.5-2: PCMAX configured HPUE output power for Power class 2

|  | Channel bandwidth / maximum output power |  |
| --- | --- | --- |
|  | 5MHz | 20MHz |
| Measured UE output power test point 1 | -10 dBm ± 7.7 |  |
| Measured UE output power test point 2 | 10 dBm ± 6.7 |  |
| Measured UE output power test point 3 | 15 dBm ± 5.7 |  |
| Measured UE output power test point 4 | 21 dBm ±2.7 |  |
| Measured UE output power test point 5 (Note 1) | 23 dBm ± 2.7 |  |
| Note 1: Only applicable to power class 2 capable UE operating on Band 41 with UL-DL config 0 or 6 where UE is required to fall back to power class 3. |  |  |

Table 6.2.5_1.5-2a: PCMAX configured HPUE output power for Power class 2 from Release 11 and forward for the band with lower tolerance, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Channel bandwidth / maximum output power |  |
| --- | --- | --- |
|  | 5MHz | 20MHz |
| Measured UE output power test point 1 | -10 dBm ± 8 |  |
| Measured UE output power test point 2 | 10 dBm ± 7 |  |
| Measured UE output power test point 3 | 15 dBm ± 6 |  |
| Measured UE output power test point 4 | 21 dBm +3/-4 |  |

< End of changes >