3GPP TSG-RAN5 Meeting #109	R5-255589

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3107 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction to NB IoT RRM TC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test mechanism is disrupted by Wrong “skipped step number” in test procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the incorrect step number. Fix Typo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedure remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

4.2.24	HD - FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage

4.2.24.1	Test purpose

To verify the requirement for the HD-FDD inter frequency cell reselection requirements for Cat-NB1 UE specified in clause 4.6.2.6.

4.2.24.2	Test applicability

This test applies to all types of NB-IoT HD-FDD UE release 13 and forward of UE Category NB1 and higher.

4.2.24.3	Minimum conformance requirements

The cell re-selection delay shall be less than Tevaluate, NB_Inter-EC + TSI in RRC_IDLE state.

The UE shall be able to identify new inter-frequency cells and perform NRSRP measurements of identified inter-frequency cells if carrier frequency information is provided by the serving NB-IoT cell, even if no explicit neighbour list with physical layer cell identities is provided.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in 3GPP TS 36.304 within Pcarrier * Tdetect,NB_Inter_EC. An inter-frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.5 for a corresponding Band.

The UE shall filter NRSRP measurements of each measured inter-frequency cell using at least [2] measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NB_Inter_NB-IoT_EC/2.

If an inter-frequency cell has been already detected but that has not been reselected to the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 36.304 within Pcarrier * Tevaluate,NB_Inter_EC. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and inter-frequency NB-IoT cells.

If Treselection timer has a non zero value and the inter-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

The UE shall evaluate the inter-frequency cell re-selection criteria as defined in 3GPP TS 36.304 [6] at least every DRX cycle. The DRX cycle length is 1.28 seconds.

At inter-frequency cell re-selection, the UE shall monitor the downlink of serving NB-IoT cell for paging reception until the UE is capable to start monitoring downlink channels of the target inter-frequency cell for paging reception. The interruption time shall not exceed TSI + 100 ms. TSI is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks as defined in 3GPP TS 36.331 [5] for a NB-IoT cell.

The normative reference for this requirement is TS 36.133 [4] clause 4.6.2 and A.4.2.24.

4.2.24.4	Test description

4.2.24.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: EUTRA Bandwidth is 10 MHz, as defined in 3GPP TS 36.508 [7] clause 4.3.1.1, except for band 31 where 5 MHz bandwidth is used. Ncell bandwidth is as specified in Table 4.2.19.5-1.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.93.

2.	The parameter settings for the cells are set up according to Table 4.2.24.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	There are one NB-IoT carrier and one E-UTRA FDD carrier and three cells specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

Table 4.2.24.4.1-1: General test parameters for HD-FDD
inter frequency cell reselection test case for Cat-NB1 UE in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  | In-band |  |
| Initial condition | Active cell |  | Ncell1 |  |
|  | Neighbour cells |  | Cell1, Ncell2 |  |
| T2 end condition | Active cell |  | Ncell2 |  |
|  | Neighbour cells |  | Cell1, Ncell1 |  |
| Final condition | Visited cell |  | Ncell1 |  |
| E-UTRA RF Channel Number |  |  | 1 | One carrier frequency is used for Cell. |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH Configuration |  |  | NPRACH.R-1 | Refer to Table A.10.3-1 |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| T0 |  | S | 5 | During T0, UE decodes SIB3-NB and SIB5-NB to acquire the inter-frequency carrier information. |
| T1 |  | s | >7 | During T1, Ncell2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Ncell2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 67 | T2 is defined so that cell re-selection time is taken into account. Once the UE has reselected to Ncell2 (within T2) T3 starts. |
| T3 |  | s | 22 | T3 is defined so that cell re-selection time is taken into account. |

4.2.24.4.2	Test procedure

The test scenario comprises of 1 E-UTRA carrier with one Cell and one NB-IoT carrier with 2 Ncells of different physical cell ID. The test consists of four successive time periods, with time duration of T0, T1, T2 and T3 respectively. Only Ncell1 is already identified by the UE prior to the start of the test, i.e. Ncell 2 is not identified. Ncell 1 and Ncell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Ncell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure according to 3GPP TS 36.508 [7] clause 8.1.5A.5"

1.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 in Ncell 1.

2.	Set the parameters according to T0 in Table 4.2.24.5-1 and 4.2.24.5-2. Propagation conditions are set according to Annex B clause B.1.1. T0 starts.

3.	Set the parameters according to T1 in Table 4.2.24.5-1 and 4.2.24.5-2. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

4.	Alternate between Ncell 2 physical cell identity =  6 and Ncell 2 physical cell identity = 12 for one iteration of the test procedure loop.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 4.2.24.5-1 and 4.2.24.5-2.

6.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Ncell 2.

7.	If the UE responds on the newly detectable cell, Ncell 2, during time duration T2 within 67 seconds from the beginning of time period T2, then count a success for the event "Re-select newly detected Ncell 2". Otherwise count a fail for the event "Re-select newly detected Ncell 2".

8.	If the UE has re-selected Ncell 2 within T2, after the re-selection or when T2 expires, continue with step 9.
Otherwise, if T2 expires and the UE has not yet re-selected Ncell 2, skip to step 13.

9.	The SS shall switch the power setting from T2 to T3 as specified in Table 4.2.24.5-1 and 4.2.24.5-2.

10.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Ncell 1.

11.	If the UE responds on the already detected cell, Ncell 1, during time duration T3 within 22 seconds from the beginning of time period T3, then count a success for the event "Re-select already detected Ncell 1". Otherwise count a fail for the event "Re-select already detected Ncell 1".

12.	If the UE has re-selected Ncell 1 within T3, after the re-selection or when T3 expires, skip to step 14.
Otherwise, if T3 expires and the UE has not yet re-selected Ncell 1, continue with step 13.

13.	Switch off and on the UE and ensure the UE is in State 3A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5 in Ncell 1.

14.	Repeat step 2-13 until a test verdict has been achieved.
Each of the events "Re-select newly detected Ncell 2" and "Re-select already detected Ncell 1" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

4.2.24.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 8.1.6 and clause 8.1.4.3 using conditions "Inband_Different" and "RRM-InterFreq".

4.2.24.5	Test requirement

Tables 4.2.24.4.1-1, 4.2.24.5-1 and 4.2.24.5-2 defines the primary level settings including test tolerances for HD-FDD inter frequency cell re-selection test case.

Table 4.2.24.5-1: Ncell 1, Ncell 2 specific test parameters for HD-FDD
inter frequency cell reselection test case for Cat-NB1 UE in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  | nCell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T3 | T0 | T1 | T2 | T3 |
| BWchannel | kHz | 180 |  |  |  | 180 |  |  |  |
| PRB location within eCell | - | 30 |  |  |  | 35 |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  | -3 |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |  |
| NOCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| NOCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  | -140 |  |  |  |
| Pcompensation | dB | 0 |  |  |  | 0 |  |  |  |
| Qhysts | dB | 0 |  |  |  | 0 |  |  |  |
| Qoffsets, n | dB | 0 |  |  |  | 0 |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | NRSRP |  |  |  | NRSRP |  |  |  |
|  | dBm/15 kHz | Specified in Table A.4.2.24.1-3 |  |  |  |  |  |  |  |
|  | dB | 10 | -12 | -12 | -2.25 | -infinity | -infinity | -2.25 | -12 |
| Note2 | dB | 10 | -12 | -12 | -2.25 | -infinity | -infinity | -2.25 | -12 |
| NRSRP Note2 | dBm/15 kHz | -88 | -110 | -110 | -100.25 | -infinity | -infinity | -100.25 | -110 |
| Treselection | s | 0 |  |  |  | 0 |  |  |  |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  | 2x1 |  |  |  |
| Timing offset to nCell 1 | ms | 0 |  |  |  |  |  |  |  |
| NOTE 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

Table 4.2.24.5-2: Cell 1 specific test parameters for HD-FDD inter frequency cell reselection test case for Cat-NB1 UE in enhanced coverage

| Parameter | Unit | eCell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  |  |
| OCNG Pattern | - | NOP.1 FDD |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  |
| Pcompensation | dB | 0 |  |  |  |
| Qhysts | dB | 0 |  |  |  |
| Qoffsets, n | dB | 0 |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |
|  | dB | 3 | 3 | 3 | 3 |
| Treselection | s | 0 |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |
| NOTE 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Ncell 2, and starts to send preambles on the NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Ncell 2.

The cell re-selection delay to a newly detectable cell shall be less than 66.32 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Ncell 1, and starts to send preambles on the NPRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Ncell 1.

The cell re-selection delay to an already detected cell shall be less than 21.12 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect,NB_Inter_EC + TSI, and to an already detected cell can be expressed as: Tevaluate, NB_Inter_EC + TSI,

Where:

Tdetect,NB_Inter_EC		as specified in 3GPP TS 36.133 [4] Table 4.6.2.6-1 in clause 4.6.2.6.

Tevaluate, NB_Inter_EC	as specified in 3GPP TS 36.133 [4] Table 4.6.2.6-1 in clause 4.6.2.6.

TSI	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 8.32 s is assumed in this test case.

This gives a total of 66.32 s, allow 67 s for the cell re-selection delay to a newly detectable cell and 21.12 s, and allow 22 s for the cell re-selection delay to an already detected cell in the test case.

==============End of change==============