3GPP TSG-RAN5 Meeting #109	R5-255968

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3645 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 38.508-1 cl 4.8.2.3 (Reference QoS flow descriptions) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If a UE supports interworking with EPS, then it would include the Mapped EPS Bearer Contex IE in the PDU Session Modification Command, and the current message content of PDU Session Modification Command to add RTT (38.508-1 cl 4.9.46) does not handle this. A separate Prose is raised for this. However, it is also observed that the reference IE authorized QoS flow Descriptions in 38.508-1 Table 4.9.4.8.2.3-11) which also does not handle the case where interworking with EPS is supported. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 4.8.2.3-11 to handle the case where Interworking with EPS is supported. Also added a Table describing this condition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN based on current Prose will fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.8.2.3	Reference QoS flow descriptions

Table 4.8.2.3-1: Reference QoS flow #1

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0001’B | QFI 1 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 1001’B | 5QI 9 |  |
| EPS bearer identity | Any not yet assigned value different to '5' |  | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-2: Reference QoS flow #2

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0010’B | QFI 2 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0101’B | 5QI 5 |  |
| EPS bearer identity | ‘0101 0000’B | EBI 5 | Interworking_with_EPS |
| EPS bearer identity | Any not yet assigned value different from ‘5’ |  | EmergencySession AND Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |
| EmergencySession | If this flow is used in the Authorized QoS flow descriptions IE of a PDU Session Establishment Accept message for an emergency PDU session |

Table 4.8.2.3-2a: Reference QoS flow #2a

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0100’B | QFI 4 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-3: Reference QoS flow #3

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0101’B | QFI 5 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-4: Reference QoS flow #4

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0110’B | QFI 6 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-5: Reference QoS flow #5

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0111’B | QFI 7 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0101’B | 5 parameter |  |
| Number of parameters | ’00 0110’B | 6 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0001’B | 5QI 1 |  |
| GFBR uplink | ‘040002’H | 128 Kbps |  |
| GFBR downlink | ‘040002’H | 128 Kbps |  |
| MFBR uplink | ‘040005’H | 320 Kbps |  |
| MFBR downlink | ‘040005’H | 320 Kbps |  |
| EPS bearer identity | Any not yet assigned value different to '5' | EBI 6 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-6: Reference QoS flow #6

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1000’B | QFI 8 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0101’B | 5 parameter |  |
| Number of parameters | ’00 0110’B | 6 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0010’B | 5QI 2 |  |
| GFBR uplink | ‘020012’H | 72 Kbps |  |
| GFBR downlink | ‘020012’H | 72 Kbps |  |
| MFBR uplink | ‘030033’H | 816 Kbps |  |
| MFBR downlink | ‘030033’H | 816 Kbps |  |
| EPS bearer identity | Any not yet assigned value different to '5' | EBI 7 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-7: Reference QoS flow #7

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1001’B | QFI 9 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0101 0010’B | 5QI 82 |  |

Table 4.8.2.3-8: Reference QoS flow #8

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1001’B | QFI 10 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0101 0010’B | 5QI 9 |  |

Table 4.8.2.3-9: Reference QoS flow #9 (MCX signalling)

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1011’B | QFI 11 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0100 0101’B | 5QI 69 MCX signalling according to TS 23.501 [68] Table 5.7.4-1 |  |

Table 4.8.2.3-10: Reference QoS flow #10 (MCX user plane data)

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1100’B | QFI 12 | MCPTT |
|  | ‘00 1101’B | QFI 13 | MCVIDEO |
|  | ‘00 1110’B | QFI 14 | MCDATA |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0100 0001’B | 5QI 65 MCPTT user plane according to TS 23.501 [68] Table 5.7.4-1 | MCPTT |
|  | ‘0100 0011’B | 5QI 67 MCVideo user plane according to TS 23.501 [68] Table 5.7.4-1 | MCVIDEO |
|  | ‘0100 0110’B | 5QI 70 MCData user plane according to TS 23.501 [68] Table 5.7.4-1 | MCDATA |

Table 4.8.2.3-11: Reference QoS flow #11 (RTT)

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1111’B | QFI 15 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0101’B | 5 parameters |  |
| Number of parameters | ’00 0110’B | 6 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0001’B | 5QI 1 |  |
| GFBR uplink | ‘010004’H | 4 Kbps |  |
| GFBR downlink | ‘010004’H | 4 Kbps |  |
| MFBR uplink | ‘010004’H | 4 Kbps |  |
| MFBR downlink | ‘010004’H | 4 Kbps |  |
| EPS bearer identity | Any not yet assigned value different to '5' | EBI 7 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-12: Reference QoS flow #12

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘01 0000’B | QFI 16 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 1001’B | 5QI 9 |  |
| EPS bearer identity | Any not yet assigned value different to '5' | EBI 8 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message or PDU SESSION MODIFICATION COMMAND message also including the Mapped EPS bearer context IE. |