3GPP TSG-RAN5 Meeting #109	R5-255784

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3770 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 38.523-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The R5-254942 has introduced a new pixit px_NR_NTN_TxRx_FrequencySeparation that supports two values, “default” and “flexible”. However, “default” is a reserved word in TTCN-3 language and cannot be applied. - The execution guidelines need to be updated to consider test specification updates and the new delivered test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified “default” to “default_” to allow TTCN-3 implementation. - Updated clause 11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Implementation of px_NR_NTN_TxRx_FrequencySeparation will not be possible. - Execution guidelines will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 9.2-1, 11.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 9.2	E-UTRA and NR PIXIT

Table 9.2-1: E-UTRA and NR PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_ENDC_BandCombination | ENDC_BandCombination_Type | DC_1A_n28A |  | EN-DC Band Combination |
| px_ENDC_CA_BandCombination | ENDC_CA_BandCombination_Type | DC_1A_n28A_n78A |  | EN-DC CA Band Combination |
| px_ENDC_SecondaryBandCombination | ENDC_BandCombination_Type | DC_1A_n77A |  | Secondary EN-DC Band Combination |
| px_IPv4_Address_Other | charstring |  |  | Other IPv4 Address (see NOTE 4) |
| px_IPv6_Address_Other | charstring |  |  | Other IPv6 Address (see NOTE 4) |
| px_IPv4_Address4_NW | charstring |  |  | IPv4 Gateway Address in PDN4 |
| px_IPv6_Address4_NW | charstring |  |  | IPv6 Gateway Address in PDN4 |
| px_IPv4_Address4_UE | charstring |  |  | IPv4 Address connected to PDN4 |
| px_IPv6_Address4_UE | charstring |  |  | IPv6 Address connected to PDN4 |
| px_IPv4_Address5_NW | charstring |  |  | IPv4 Gateway Address in PDN5 |
| px_IPv6_Address5_NW | charstring |  |  | IPv6 Gateway Address in PDN5 |
| px_IPv4_Address5_UE | charstring |  |  | IPv4 Address connected to PDN5 |
| px_IPv6_Address5_UE | charstring |  |  | IPv6 Address connected to PDN5 |
| px_IPv4_Address6_NW | charstring |  |  | IPv4 Gateway Address in PDN6 |
| px_IPv6_Address6_NW | charstring |  |  | IPv6 Gateway Address in PDN6 |
| px_IPv4_Address6_UE | charstring |  |  | IPv4 Address connected to PDN6 |
| px_IPv6_Address6_UE | charstring |  |  | IPv6 Address connected to PDN6 |
| px_MUSIM_PeriodicGapPattern | integer | 0 |  | The ID of a MUSIM periodic gap pattern supported by the UE |
| px_NEDC_BandCombination | NEDC_BandCombination_Type | DC_n28A_3A |  | NE-DC Band Combination |
| px_NEDC_SecondaryBandCombination | NEDC_BandCombination_Type | DC_n28A_39A |  | Secondary NE-DC Band Combination |
| px_NR_CA_BandCombination | NR_CA_BandCombination_Type | CA_n1A_n3A |  | NR CA Band Combination with 2CC |
| px_NR_CA_3CC_BandCombination | NR_CA_3CC_BandCombination_Type | CA_n257H |  | NR CA Band Combination with 3CC |
| px_NR_DC_BandCombination | NR_DC_BandCombination_Type | DC_n78A_n257A |  | NR-DC Band Combination |
| px_NR_DC_CA_BandCombination | NR_DC_CA_BandCombination_Type | DC_n78A_n257G |  | NR-DC CA Band Combination |
| px_NR_CipheringAlgorithm | CipheringAlgorithm | nea2 |  | Ciphering Algorithm (see Note 1) |
| px_NR_IntegrityProtAlgorithm | IntegrityProtAlgorithm | nia2 |  | Integrity Algorithm (see Note 1) |
| px_NR_NTN_TxRx_FrequencySeparation | TxRx_FrequencySeparation_Type | default_ | default_, flexible | DL and UL frequency separation applicable to NTN test case scenario. |
| px_NR_OverlappingNotSupportedBand_MFBI | integer | 1 |  | A not supported NR band that is overlapping with a supported band (px_NR_PrimaryBand). Applied to MFBI test case scenario. |
| px_NR_OverlappingSupportedBand_MFBI | integer | 1 |  | Another supported NR band that is overlapping with a supported band (px_NR_PrimaryBand). Applied to MFBI test case scenario. |
| px_NR_PrimaryBand | integer | 1 |  | NR primary band |
| px_NR_SecondaryBand | integer | 2 |  | NR secondary band. Applied to inter-band and SUL test cases. |
| px_NR_SidelinkBand | integer | 47 |  | NR sidelink band |
| px_NR_SidelinkPDCP_ReorderTimer | integer | 1500 |  | The value of t-Reordering timer configured by UE-Value in ms |
| px_NR_RATComb_Tested | NR_RATComb_Tested_Type | NR_UTRA | NR_UTRA, NR_GERAN | This parameter represents the network RAT capability / preference and indicates which, if any is supported, RAT combination is to be tested. |
| px_NR_V2X_ConcurrentOperationBandCombination | NR_V2X_ConcurrentOperationBandCombination_Type | V2X_n71_n47 |  | V2X operating bands for con-current operation |
| NOTE 1: Unless specified otherwise in the test case prose, the null algorithm shall not be used for verification. NOTE 2: Void. NOTE 3: Void. NOTE 4: IP address used for specific IP-address-related test requirements. The only requirements applying to this IP address is that its value shall be different from the values of all other IP address PIXITs (see TS 36.523-3 [12] clause 9) and it shall not be used within more than one PDN (PDU session) within a test case. In the present version of this specification, it is only used within PDN1 NW. |  |  |  |  |

==============Next change==============

## 11.3	Test execution restrictions

### 11.3.1	Bands / band combinations

#### 11.3.1.1	Introduction

The restriction on test case execution is due to the number of frequencies available for the specific band under test specified in TS 38.508-1[5] and the number of frequencies used by the test cases specified in TS 38.523-1[8].

#### 11.3.1.2	EN-DC, NR/5GC, Inter-RAT

The list of test cases, that cannot be executed due to restrictions on bands and/or band combinations, is specified in Table 11.3.1.2-1.

Table 11.3.1.2-1: Restriction on bands / band combinations for test case execution

| Category of test case | Category of restriction | Bands / band combinations with restrictions | TS 38.523-1 [8] test cases that cannot be executed |
| --- | --- | --- | --- |
| EN-DC |  |  |  |
| Single NR carrier | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | DC_2A_n2A, DC_21A_n28A, DC_41A_n41A, DC_66A_n66A | 8.2.3.6.1a, 8.2.3.7.1a, 8.2.3.8.1a, 8.2.3.10.1, 8.2.3.11.1, 8.2.3.18.4 |
| Single NR carrier | More than two NR radio frequencies (i.e., using NRf3 or NRf4) | DC_(n)41AA | 8.2.3.18.4 |
| NR/5GC |  |  |  |
| Single RAT / Single NR carrier | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | n31, n51, n54, n72, n91, n93, n100, n106, n110 | 6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.3a, 6.1.2.7, 6.1.2.11, 6.1.2.13, 6.1.2.14, 6.1.2.16, 6.1.2.18, 6.1.2.20, 6.1.2.21, 6.1.2.22, 6.1.2.24, 6.1.2.25, 6.1.2.27, 6.1.2.32, 6.1.2.33, 6.1.2.34, 6.1.2.36, 6.3.1.1, 6.3.1.2, 6.3.1.3, 6.3.1.4, 6.3.1.5, 6.3.1.8, 6.3.1.9, 6.3.1.10, 6.3.2.1, 6.3.2.2, 6.3.2.3, 6.3.2.4, 6.3.2.5, 6.3.2.6, 6.3.3.1, 6.3.3.2, 6.3.3.3, 6.3.3.4, 6.4.1.1, 6.4.1.2, 6.4.2.2, 6.4.2.3, 6.5.1.2, 6.5.2.4, 6.5.2.6, 6.5.3.1, 6.5.3.4, 6.5.3.5, 6.5.3.7, 6.5.3.8, 6.5.3.9, 6.5.3.10, 6.5.3.11, 6.5.3.12, 6.5.3.13, 7.1.3.4.4.1, 7.1.3.6.5, 8.1.1.2.1, 8.1.1.2.5, 8.1.1.3.1, 8.1.1.3.3, 8.1.1.3.7, 8.1.1.3.7b, 8.1.1.3.8, 8.1.1.3.9, 8.1.1.4.10, 8.1.2.1.6, 8.1.3.1.3, 8.1.3.1.6, 8.1.3.1.9, 8.1.3.1.11, 8.1.3.1.12, 8.1.3.1.14A, 8.1.3.1.15A, 8.1.3.1.20, 8.1.3.21, 8.1.4.1.2, 8.1.4.1.10, 8.1.4.3.4.1, 8.1.4.3.5.1, 8.1.5.10.2, 8.1.5.10.3, 8.1.5.10.5, 8.1.5.11.1, 8.1.5.11.3, 8.1.5.11.4, 8.1.6.1.2.2, 8.1.6.1.2.10, 8.1.6.1.3.2, 8.1.6.1.3.7, 8.1.6.1.4.3, 8.1.6.1.4.7, 8.1.6.1.4.9, 9.1.4.1, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.8, 9.1.5.1.10, 9.1.5.1.12, 9.1.5.1.14, 9.1.5.1.16, 9.1.5.2.10, 9.1.5.2.11, 9.1.7.4, 9.1.9.3, 9.1.12.3, 11.3.5, 11.3.6, 11.3.6a, 11.3.8, 11.3.9, 11.3.9a, 11.3.11, 11.4.7, 11.4.8, 14.1.2.1, 14.1.2.2 |
| Single RAT / Single NR carrier | More than two NR radio frequencies (i.e., using NRf3 or NRf4) | n13, n14, n24, n30, n53 | 6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.11, 6.1.2.20, 6.1.2.24, 6.1.2.25, 6.3.1.1, 6.3.1.2, 6.3.1.3, 6.3.1.5, 6.3.1.8, 6.4.1.1, 6.4.1.2, 6.4.2.2, 6.4.2.3, 6.5.2.4, 6.5.2.6, 8.1.1.3.3, 8.1.1.3.9, 9.1.5.1.2, 9.1.5.1.8 |
| Single RAT / NR CA inter-band | Switched allocation of PCell and SCell if SCell frequency has no UL | CA_n29A_n66A, CA_n29A_n70A, CA_n29A_n71A | 8.1.4.1.7.2, 8.1.4.1.9.2 |
| Single RAT / NR CA intra-band Contiguous | Switched allocation of PCell and SCell if SCell frequency has no UL or DAPS | CA_n5B, CA_n66B | 7.1.3.4.4.2, 8.1.4.1.7.1, 8.1.4.1.9.1, 8.1.4.3.4.2, 8.1.4.3.5.2 |
| Single RAT / NR additional BWP without CORESET#0 | No frequency specified | n29, n51, n54, n75, n76, n91, n93, n100, n101 | 7.1.1.8.4, 7.1.1.8.6 |
| Single RAT / Inter-band | More than one NR radio frequency on the primary band (i.e., using NRf2 or NRf3 or NRf4) | n31, n51, n54, n72, n91, n93, n100, n106, n110 | 8.1.5.10.6 |
| NR/5GC Inter-RAT |  |  |  |
| NR / E-UTRA Inter-RAT | More than one E-UTRA radio frequency (i.e., using f2 or f3 or f4) | 13, 18, 31, 54, 72, 73, 87, 88, 106 | 6.4.3.1, 8.1.1.3.4, 11.1.4, 11.1.5. |
| NR / E-UTRA Inter-RAT | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | n31, n51, n54, n72, n91, n93, n94, n100, n106, n110 | 6.2.1.1, 6.2.1.2, 6.2.1.3, 6.2.1.4, 8.1.1.3.7a. |
| NR / E-UTRA Inter-RAT | More than two NR radio frequencies (i.e., using NRf3 or NRf4) | n13, n14, n24, n30, n53 | 6.2.1.1 |
| NR / EN-DC Inter-RAT | More than one NR radio frequency (i.e., using NRf2 or NRf3 or NRf4) | DC_2A_n2A, DC_21A_n28A, DC_41A_n41A, DC_66A_n66A | 8.1.4.2.1.2 |

==============End of change==============